LPT2DSK - Capture printer output to disk file.

Version  1.0

October 3, 1986

(c) Copyright by George G. Bouche

P.O.Box 65111    Lubbock, TX  79464

This program works by intercepting the printer bios interupt (17h)

and capturing the would be printed data in a buffer which is later

written to a disk file.  By capturing at the bios level, all dos

printing (file redirection, Ctl-PrtSc, LPT?, PRN, ...) and programs

which call the bios directly (BASIC, Shft-PrtSc) are handled.

Capturing is for a specific printer (LPT1, LPT2 or LPT3) and output

to the other two printers is passed thru to the original INT17

interupt address.

Usage:

LPT2DSK [p] [nnK] [F=[d:][\path]filename[.ext]]

p=1, 2 or 3   printer number for LPT1, LPT2, or LPT3

Default=1  LPT1

nn=1 to 64    the amount of memory (in multiples of 1024 bytes) to

allocate to the printer buffer. The K is required.

Default=64K

filename      any valid dos filename (can be a device) with

optional drive, path and/or extension. The F=

is required.

Default=LPT2DSK.p  where p=printer number as above

on the current drive, current directory.

Requirements:

- Dos 2.0 or later.

- The parameters can be in any sequence but must be delimited by

space(s).

- Any or all parameters can be ommitted.

- The environment must have an entry for locating COMMAND.COM

(COMSPEC= ) and COMMAND.COM must be available as it will be

loaded.

- LPT2DSK.EXE needs 9120 bytes + XXXX bytes for the resident

portion of COMMAND.COM(for dos 3.1 this is 3200) + buffer size.

- Multiple printers can be captured - just run the program again

for the other printer.  Remember, last one started is the first

one to exit from.

- Dos redirections of the printer (i.e. to COM?) are not detected

and no data will reach the buffer.

- The printer port hardware need not be present to capture data.

The printer status of the captured printer is returned as

ready/not busy unless the buffer has filled.

Examples:

LPT2DSK 2 32K F=CAPTURE.DAT

capture LPT2 and reserve a 32k buffer. A secondary level of

command.com is loaded. Interupt 17h is replaced; no hardware or dos

interupts are disturbed.

...                    run the programs that print

and/or hit shift-print-screen

and/or hit control-print-screen

printing to LPT1 or LPT3 goes on un-affected

if the buffer gets full, dos returns an

"Not ready error writing device LPT2

Abort, Retry, or Ignore"

the only option is to abort.

EXIT                    exit secondary command.com and end LPT2DSK

the file CAPTURE.DAT is opened and data is

written. if the file existed before, it is

overwritten. if no data was printed, the

file will not be opened. interupt 17h is

restored to value before capture. all

allocated memory is returned - nothing

resident.

LPT2DSK F=LPT1 3        capture LPT3 and reserve a 64k buffer. the

buffer will be written to printer 1 when

done.

0 L