VPCI PREFIX DOCUMENTATION

Version 1.01a 18 Mar 1996

Copyright (C) 1995-1996 Michael Hypytsch.  All rights reserved.

VPCI in the current version outputs 'straightforward' code for

printers and plotters, i.e., it uses commands which are common

on all devices of a printer/plotter family.

However, some printers and plotters have a mixed-language envi-

ronment, which may rely on the individual print jobs to switch

between languages accordingly.

To make advanced use of the data generated by VPCI in such envi-

ronments, and still get the benefit of multiple languages, the

VPCI package contains prefix and suffix code which can be combined

with VPCI output data.

All prefix code must be sent to the output device before the

actual graphics data, and all suffix code must be sent after

that data.

This can be achieved by (manually) embedding VPCI output between

the desired prefix and suffix code via the DOS 'COPY' command.

In the following example, the resulting file FILE.PCL, to be sent

to a LaserJet IIISi, contains prefix code to switch to the PCL

language, the actual graphics code, and suffix code to end the PJL

job:

COPY /B LJPS.__1 + OUTPUT.PCL + LJPS.__3 FILE.PCL

Always use the '/B' switch with binary data.

In most cases, the prefixes can also be sent individually to

perform a non-temporary language switch.

HEWLETT-PACKARD HP-GL/2 LASER PRINTERS

Laser printers with PCL level 5 (HP LaserJet III and newer) can be

used with HP-GL/2 data.

Data in user units (SC command) is scaled to fit automatically,

without any setup, however, the printer must already be in HP-GL/2

mode (see section 'HP7600 SERIES PLOTTER' for switching code).

Data in plotter units (used by VPCI) must use the PCL5 picture

frame scaling.

The following code does the required setup and switches to HP-GL/2

mode and back to PCL mode.

LJHPGL2.__1  -  PCL prefix code, brings un-scaled A4-sized HP-GL/2

onto the paper

LJHPGL2.__2  -  PCL prefix code, brings un-scaled A3-sized HP-GL/2

onto the paper

LJHPGL2.__3  -  PCL prefix code, brings un-scaled A2-sized HP-GL/2

onto the paper

LJHPGL2.__4  -  PCL prefix code, brings un-scaled A1-sized HP-GL/2

onto the paper

LJHPGL2.__5  -  PCL prefix code, brings un-scaled A0-sized HP-GL/2

onto the paper

LJHPGL2.__6  -  PCL suffix code, must always be appended to

HP-GL/2 code when using the above prefixes

The above prefixes might not give the best results since it is

not always easy to output pen plotter data without alterations to

the LaserJet series printers.

'Brings onto the paper' means that the plot may not be properly

scaled, may have unsuitable margins, or may even be clipped.

Initializing commands in the HP-GL/2 data override the minimum,

one dot wide line thickness defined in the prefix code (line

thickness defaults to 0.35mm).  Then, strip all IN and DF

commands, usually the first commands in a plot.

HEWLETT-PACKARD POSTSCRIPT LASER PRINTERS

In a multi-language environment (PCL/PostScript) it is necessary

to switch between PCL and PostScript, otherwise, PCL code will be

interpreted as PostScript code and vice-versa.

The HP PostScript laser printers (e.g., LaserJet IIISi) support

the Printer Job Language (PJL) switching.

LJPS.__1  -  Prefix code, switches to PCL via PJL

LJPS.__2  -  Prefix code, switches to PostScript via PJL

LJPS.__3  -  Suffix code, ends a PJL job (expects immediate

further language switching, omit if not wanted)

Make sure that PJL switching is not disabled from the printer

control panel.

Newer LaserJet printers and printers of other manufacturers may

automatically sense the type of data.  Consult your documentation.

HEWLETT-PACKARD 7600 SERIES PLOTTER

Since the 7600 Series plotters accept both vector (HP-GL/2) and

raster (HP-RTL, CALS CCITT) data, language switching might be

desired.

The 7600 series plotters also support the Printer Job Language

(PJL) switching.

* HP7600.__1  -  Reset plotter

HP7600.__2  -  Prefix code, enters HP-GL/2 mode, 'stand-alone

plotter'

* HP7600.__3  -  Prefix code, enters HP-GL/2 mode, restoring the

previously used pen position and palette

* HP7600.__4  -  Prefix code, enters HP-GL/2 mode, maintaining the

current HP-RTL print position

* HP7600.__5  -  Prefix code, enters HP-RTL mode, restoring the

previously used position and palette

* HP7600.__6  -  Prefix code, enters HP-RTL mode, maintaining the

current HP-GL/2 pen position

HP7600.__7  -  Enters HP-RTL mode via PJL

HP7600.__8  -  Enters CALS CCITT Group 4 Type I mode via PJL

Note that you must switch to HP-RTL via PJL (using HP7600.__7)

before the plotter accepts any non-PJL switching code.

The code marked with an asterisk '*' can also be used on PCL5-type

laser printers to switch to HP-GL/2 and back to PCL (which is

roughly the same as RTL), and to reset the printer.

TRADEMARK ACKNOWLEDGEMENT

Company and product names are trademarks or registered trade-

marks of their respective owners.

End