eSoft NetCalc

Copyright 1996 by eSoft Inc., All rights reserved. No modification allowed.

This program may be freely distributed as long as eSoft, Inc. is credited.

This program is designed to aid in the calculation of the IP numbers and

netmasks a router requires to correctly connect both standard class A, B

and C and non-standard "subnet" Internet networks. The original effort was

to try and make the subnet selection process as graphical and easy to

understand as possible.  This is a complex subject and we strongly suggest

that you find out more about Internet routing from the eSoft IPAD manual

(chapter 9) or one of the many TCP/IP books.

When the program first starts it will ask you for an Internet Protocol (IP)

address. You may optionally type all or part of the IP number on the DOS

command line. The complete address must be a "4D" address in the form of FOUR

numbers each in the range of 0 to 255 separated by periods.

For example: 199.45.143.2

The last digit may not be either 0 or 255 as these are reserved for broadcast

address which is also called multiplex addresses. You may optionally also

follow the IP address with a slash and a number of active bits in the

netmask. This optional number must be in the range of 1 to 32.

For example: 199.45.143.2/28

If part of the number is missing then Zeros are added to the left side until

the address is complete. This will cause the information about your address

to be inaccurate, but it provides a quick way to check for the available

addresses in a subnet.

For example:

"NETCALC 2/31" would become "0.0.0.2/31" and show that the netmask of 31

active bits does not allow any IP addresses to be used.

The IP number will be displayed in Decimal, Hexadecimal and Binary at the top

of the screen. An arrow will point to the dividing line between the network

part of the address, the part which a router looks at, and the host part of

the address, which is the part you can use to number hosts within a network

or subnet. You can move the arrow to the right and left with the cursor keys

and create a custom subnet from the address you have at the top of the

screen. Pressing the spacebar will toggle the bit of the address at the arrow

location between 1 and 0.

Below the arrow is the netmask required by a router to subnet the network

part of the address and filter out the host part of the address. The mask is

shown in Binary, Hexadecimal, and Decimal form.

Below the netmask is an indication of the class of network address listed on

the top line. This is determined by the first One to Three bits in the

network address. There are four possible settings for these bits;

0   = Class A (used for backbone carriers)

10  = Class B (used for backbone carriers)

110 = Class C (used for companies and access providers)

111 = Class D (only used in MBONE and multi-cast networks)

Below the class is the starting and ending address for the selected subnet

and the possible number of hosts in the netmask you have selected. Again,

the pointer at the top of the display selects how much of the address is

network and how much is host.

Notice that the IP address on the top line does not have to be the first

address in the subnet that the netmask selects. It only must be inside

the subnet selected by the netmask.

The next line shows the network address for the subnet you have selected.

This is one of the two parts required by a router to send packets to a

network.

The next line is the multicast address which is used to broadcast to all

hosts inside of a subnet. There are always two addresses within a netmask

which are reserved for broadcasts and may not be used for actual IP addresses

except in the special case of a 32 bit network where all addresses are the

same IP number.

You can enter a different address at any time by pressing the Enter key. A

prompt will display at the bottom of the screen. This address must be a "4D"

address in the form of FOUR numbers each in the range of 0 to 255 separated

by periods as explained above.

Remember that the last digit may NOT be either 0 or 255 as these are reserved

for broadcast addresses.

You may optionally also follow the IP address with a slash and a number of

active bits in the netmask. This optional number must be in the range of 1 to

32.

Command line example:

NETCALC [IPnumber[/bits]] [/Wide] [/NoCheck]

Command line options:

IPnumber/bits  - Starting IP address and optional active netmask bits.

/W or /Wide    - Will show wide (large) text display. This makes it

easier to see the numbers from across the room. :)

/N or /NoCheck - Will prevent class limit checking. This allows non-

standard networks to build netmasks outside of their

normal network class.