Spheric

=========

A Spherical Geometry Calculator and Navigation Aid

Version 1.42

by J. Andrzej Wrotniak

March 5, 1997

Table of Contents

1.  Introduction

1.1.  What does Spheric do?

1.2.  PC-DOS and HP Palmtop compatibility

1.3.  Extended (registered) version

1.4.  Disclaimer

2.  The Main Screen

2.1.  The History Log

2.2.  The Status Line

2.3.  The Fix Database Display

2.4.  The F-Key Menu

2.4.1.  Nested menus

2.4.2.  Nested F-key notation

2.4.3.  Help and Exit keys

2.4.4.  The Top Menu

2.5.  The Prompt Line

3.  The Fix Database

3.1.  Fix display format

3.2.  Moving around

3.3.  Finding a fix

3.4.  Inserting a fix

3.5.  Appending a fix

3.6.  Modifying a fix record

3.7.  High accuracy fix display

3.8.  Deleting a fix

3.9.  Saving a fix file

3.10. Loading a fix file

3.11. Sorting fixes by name or category

3.12. Sorting fixes by proximity

3.13. Clearing the database

4.  Computational operations

4.1.  Distance between fixes

4.1.1. True ellipsoidal great arc distance

4.1.2. Geodesic distance

4.1.3. Cumulative distance for a series of fixes

4.2.  Azimuth from a fix to another

4.2.1. Geodesic azimuth

4.3.  Midpoint between two fixes

4.4.  Fix at a given azimuth and distance

4.5.  Intersection of two great circles

4.6.  Projection of a fix on a great circle

4.7.  Distance from a fix to a great arc and its circle

4.8.  Distance between two great arcs

5.  Data entry

5.1.  Editing the input

5.2.  Special characters

5.3.  Entering a single fix name

5.3.1. Name completion

5.3.2. Recalling previous inputs

5.3.3. New and old fix names

5.4.  Entering two fix names

5.5.  Entering fix coordinates

5.6.  Entering an azimuth

5.7.  Entering a distance

6.  The History Log

6.1.  Display format

6.2.  Scrolling through the log

6.3.  Deleting a log entry

6.4.  Clearing the log

7.  Program settings

7.1.  Distance units

7.2.  Distance display precision

7.3.  Angular modes

7.4.  Sign convention

7.5.  Azimuth display modes

7.6.  Earth model

7.6.1.  Spherical Earth

7.6.2.  Elliptical Earth

7.6.3.  Accounting for the ellipticity

7.7.  Sound alerts

7.8.  Fix category display

8.  Miscellaneous operations

8.1.  On-line help

8.2.  Clearing history and data

8.3.  Exiting the program

9.  Program installation and use

9.1.  The packing list

9.2.  Installing Spheric on your computer

9.3.  Character set compatibility

9.3.1.  Screen

9.3.2.  Printer

9.4.  Running Spheric from DOS

10. Registration and support

10.1. How to register

10.2. The extra goodies

10.3. Distribution

10.4. Support

10.5. Improvements and new features

10.6. Credits

1. Introduction

1.1. What does Spheric do?

Spheric allows you to define a number of  points (fixes)  on the

Earth sphere and then to perform various calculations  involving

these fixes: distance, azimuth, arc intersections etc.

1.2. PC-DOS and HP Palmtop compatibility

Spheric has been originally written for the Hewlett-Packard 95LX

palmtop computer.  Until version 1.26, there were two  mutations

of the program: one running on  the HP  95/100/200 LX  Palmptops

(denoted, e.g., as 1.26/HP) and another to be used on  "regular"

displays under PC-DOS (1.26/PC).

Starting from Version 1.30 the same executable runs  properly on

both systems. On non-palmtop  machines, it  centers the  Spheric

display  in  the screen,  uses color  and provides  function key

labels.  A math coprocessor,  if present,  will be  detected and

used.

The additional information, specific to the HP  Palmtops can  be

found in the supplied file SPH_HP.DOC.

1.3. Extended (registered) version

Spheric  is  shareware;  the  details  on  its distribution  and

registration are given in Chapter 10.

*** Some program features  may be  available only  in registered

copies of Spheric, distributed exclusively to those who paid

the registration fee.  These are designated with letter  "R"

in the version number (e.g., 1.20R).

*** Parts  of  this  document  referring  specifically   to  the

registered version are marked with a "***" string.

1.4. Disclaimer

Although  the  author has  extensively tested  the software  and

reviewed   the   documentation,   he   makes   no   warrant   or

representation,  explicit  or  implied,  with  respect  to  this

package, its  quality, performance,  merchantability or  fitness

for  a  particular  purpose,   in  any   hardware  or   software

environment.

In no event  shall the  author be  liable for  direct, indirect,

special, incidental or consequential damages arising out of  the

use, misuse or inability to use the software or documentation.

2. The Main Screen

When you run Spheric, you will first find  yourself in  the Main

Screen, which consists of four parts:

��������������������������������������Ŀ

�Fix at distance 1200mi and            �

�azimuth -75�19' from HONOLULU,HI      �

�                      24�42'N 176�23'W�

��������������������������������������Ĵ

� 4� NE � mi � DMS �Ellip� N = 32  �","�

��������������������������������������Ĵ

�    12  HONOLULU  HI 21�19'N 157�52'W �

�    13  HOUSTON   TX 29�46'N  95�22'W �

�    14  INDIANAPO IN 39�46'N  86�09'W �

�    15  LASVEGAS  NV 36�10'N 115�09'W �

�    16  LOSANGELE CA 34�04'N 118�15'W �

�    17  MEMPHIS   TN 35�08'N  90�03'W �

�    18  MIAMI     FL 25�47'N  80�11'W �

����������������������������������������

CAL FFd Azi Mid Aim FAd Prj AAd Int TOP

These are, from top to bottom, the History Log, the Status Line,

the Fix Database and the F-Key Menu.  The last one  is sometimes

taken over by prompts asking for some input.

2.1. The History Log

This  is  the  long,  three-line  area  near  the   top.   Every

calculation performed by Spheric is displayed  here.  The  first

two lines show the input data, the third one -  the result.  For

more details - see Chapter 6.

2.2. The Status Line

This line is located below the Expression Line and contains  the

following information (left to right).

- The number of the History Log entry currently displayed  (see

Chapter 6).

- The  default direction  indicator, showing  whether North  or

South  (East  or  West)   will  be   assumed  when   latitude

(longitude) is entered without the direction letter N or S (E

or W).  See also Section 7.4.

- Unit  indicator,  showing  the  units  in  which distance  is

displayed or expected on input (see Section 7.1).

- Angular  mode:  DMS  (degrees  and  minutes),  DMF  (degrees,

minutes and decimal fraction), Dcm (decimal  degrees) or  Inf

(infix notation, as in  56N09).  This  is the  mode used  for

display of latitude and longitude.  See Section 7.3 for more.

- The  Earth  model  used  in  calculations  -   elliptical  or

spherical (see Section 7.6).

- Number of fixes currently in the database.

- The  comma  mode  indicator,  ",",  or  an  empty  space (see

Sections 6.1 and 7.4).

2.3. The Fix Database Display

Most of the Main Screen is  taken by  the Fix  Database Display.

The fix number is shown at the left (if there  is enough  space;

in  some  cases just  its two  last digits  will be  displayed),

followed by the fix name and  category, latitude  and longitude.

The display shows up to seven fixes at the time.

The Fix Database is described in detail in Chapter 3.

2.4. The F-Key Menu

The bottom line is taken by the Function Key Menu with mnemonics

for the function keys F1..F10.  We will be referring to function

keys with these abbreviations  in square  brackets, e.g.,  [Set]

instead of F2.

2.4.1. Nested menus

Some of the function  keys do  not cause  any immediate  action.

Pressing such  a key  will, instead,  bring up  a new  menu with

different operations assigned to the F-keys.

For example, F3, marked as [Cal], will bring up the  Calculation

(CAL) Menu;  while that  one is  active, F2,  denoted as  [Azi],

performs the azimuth calculation.

The name of the  currently active  menu is  always displayed  in

uppercase letters above the F1 key.

2.4.2. Nested F-Key notation

Nested function key sequences  are denoted  in this  manual with

key mnemonics separated by  a vertical  bar; thus  the operation

described above would be referred to as [Cal|Azi].

As the current menu is indicated  with the  key mnemonic  in the

Status Line,  [Cal|Azi] may  be also  interpreted as  "Press the

[Azi] key while the Calculation (CAL) Menu is active, i.e. 'CAL'

is being displayed at the bottom left of the screen".

2.4.3. Help and Exit keys

In  all  function  key  menus,  F1  activates the  on-line help.

Because this is true for every menu, the space above the F1  key

is not  used  for the  [Hlp] mnemonic,  but for  the menu  name,

displayed in capital letters, e.g. "SET".

In all menus F10 is used to return to the next higher menu level

(the name of that menu will be shown here in  caps) or,  in case

of the top menu, to leave the program (in this case it  shows as

[Exit]).

2.4.4. The Top Menu

This  is  the  F-key  menu  visible  first when  the program  is

executed.  In addition to [Hlp] and [Exit] functions,  it allows

for  switching  to  other,  more  specialized  menus, containing

groups of related operations:

[Set] brings up the Settings Menu, from where  different program

settings can be adjusted, as described in Chapter 7

[Cal] switches to the  Calculations Menu,  with all  the numeric

operations described in Chapter 4

[Dat] calls  the  Database  Menu,  with the  database operations

(mostly those from Chapter 3)

2.5. The Prompt Line

Some times Spheric will ask you for a  numeric or  textual input

by displaying a prompt at the bottom of the screen,  temporarily

replacing the F-key menu.

Data input at prompts is discussed in detail in Chapter 5.

3. The Fix Database

Spheric is capable of storing up to 32 geographic locations (for

short: fixes).

*** The registered version can store up to about 1800  fixes (if

there is enough memory in the system).

At any time, one of the fixes (or,  sometimes, the  empty record

at the end) will be selected, i.e.  highlighted.   This will  be

always a fix within the currently displayed range.  Up to  seven

fixes will be shown in the Fix Database Display at one time.

3.1. Fix display format

Each fix  is shown  as one  line of  text.  It  consists of  the

following:

* Fix name (up to twelve characters starting from a letter; all

letters will be in uppercase)

* Fix  category,  if present;  up to  ten characters  (denoting

state, country or any other information you wish)

* The latitude in  a format  depending on  the current  angular

display mode

* The longitude, as above

Some examples of this format could be seen  in the  illustration

in Chapter 2.

Individual  fixes  can  be  displayed  with  higher accuracy  as

described in Section 3.7.

3.2. Moving around

At most seven fixes are visible at a  time in  the Fix  Database

Display.  Others may be viewed by  scrolling the  display up  or

down.

* The up- and down-arrow keys  select the  previous (next)  fix

with respect to the currently selected one. If the latter  is

already at  the top  (bottom) of  the display,  the displayed

range will be scrolled by one record.

Obviously, it is impossible to scroll above the first record  in

the database or more than one record beyond the last one.

* The Page Up and Page Down keys will take you to the top or to

the bottom of the current page or, if you are  already there,

will scroll one page up or down, respectively.

* The Home and End keys will cause a jump to the top or  to the

bottom of the whole database.

3.3.  Finding a fix

In order to find a fix by its name, just start typing this name.

A one-second pause (or pressing the Return key) marks the end of

the input.  Spheric will then jump to the first record  matching

what you have typed in.  After that, pressing Tab will  take you

to the next such  record (if  it exists).   The display  will be

scrolled so that the newly selected fix will be in the displayed

range.

You  can  also  search   for  fixes   with  specified   category

abbreviation (for example, to find fixes in California,  denoted

with  "CA"  in  the  small  database  US.FIX, enclosed  with the

program).  To do this, type a comma and then the  category, e.g.

",CA".

3.4. Inserting a fix

To insert a new fix into the database, press the Insert key. You

will be prompted first for the new fix  name, and  then for  its

latitude and longitude.  (See Chapter 5 for the details on  data

entry). If your answers do not contain errors, the new  fix will

be inserted into the database before the currently selected one.

If the name you have entered already exists,  Spheric will  warn

you about it.  You will be then able to cancel the operation  or

to overwrite the old fix of that name.

3.5. Appending a fix

Another way to add a fix, instead of pressing the Insert key, is

by  going  to  the  bottom  of  the database  (the empty  record

following the last one) and pressing Enter.

The  remaining  part of  the procedure  is as  described in  the

previous section, except that the new fix will then be  appended

at the end of the database.

3.6.  Modifying a fix record

To change the name and/or the coordinates of a fix, select it as

described in Section 3.2, and then press the Enter key. You will

be prompted for the new name, and then for the new latitude  and

longitude.

If you are renaming a fix, the new name has to be different than

that of any other fix already in the  database. (Entering  a fix

name is described in Section 5.3.)

3.7. High accuracy fix display

In the  low [Set|Acc]  setting, fix  coordinates are  displayed,

depending on the [Set|Ang]  status, with  an accuracy  of 1'  or

0.01�, which is high enough for  all practical  purposes. Still,

sometimes you may want  to see  a fix  with a  higher precision,

without changing the format in which the database is shown.

Pressing the "."  key will show  the currently  selected fix  at

the bottom of the screen, with an extra two digits  of accuracy.

Any key  will then  bring the  menu display  (and functionality)

back.

*** This  feature  is  available  only in  registered copies  of

Spheric.

3.8. Deleting a fix

Pressing the Delete key will remove the currently  selected fix,

after asking for a confirmation.

3.9. Saving a fix file

Spheric will save the fix database as an ASCII (plain text) file

with fix names and coordinates. This  file can  be then  used by

another program or by Spheric itself.

To save a fix file, press [Dat|Sav].  You  will be  prompted for

the file name which may contain up to eight characters.   Do NOT

enter the path (which will default to the current Spheric path);

the program will; the program will  append the  extension ".FIX"

to the name you enter.

If the name thus created is not a legal DOS file name,  an error

message will be displayed.

Pressing the Tab key will recall  the last  file name  used; the

down-arrow  will  scroll  through  all  .FIX  file names  in the

Spheric directory, and the up-arrow will jump back to the  first

name in the list.

If a .FIX file with the selected name already exists, it will be

overwritten without a warning.

The  output  file format  is simple:  each fix  takes one  line,

consisting  of  the  fix  name  and  category (separated  with a

comma), a space, latitude, a space again, and then longitude.

The angle notation will depend on the current angular mode (DMS,

Infix or Decimal, see Section 7.3). For example, the last of the

fixes from the screen shot in Chapter 2 will be  written in  one

of the following formats:

NEWARK,NJ 40�44'N 74�10'W

NEWARK,NJ 40N44 74W10

NEWARK,NJ 40.7333N 74.16677W

(When writing to a file, the degree symbol in the Decimal format

is dropped, as in the third line.)

3.10. Loading a fix file

A fix file can be loaded into the program with [Dat|Loa]. Again,

you will  be prompted  for the  .FIX file  name without  path as

described above (including the use of Tab and arrow keys).

The program will also  ask, whether  you want  to clear  the fix

database first; if not, the new  fixes will  be appended  to the

database.

The input file standard is the same as the output  one described

in  Section  3.8,  except  that  the  coordinate  values  can be

preceded by a sign instead of being followed by a letter, as in

NEWARK,NJ 40�44' -74�10'

NEWARK,NJ 40.73 -74.17

The sign convention (what directions correspond  to "+"  or "-")

can be changed by  [Set|+/-] (see  Section 7.3),  so it  is also

possible to have the last record interpreted as,  e.g., (40.73S,

74.17E). To avoid confusion, do not switch between  various sign

conventions unless you really have to.

3.11. Sorting fixes by name or category

Pressing [Dat|Nam] will sort the fixes in ascending alphabetical

order (as per the ASCII codes of the characters). This  is quite

useful, especially if you intend to search for fixes by name, as

described in Section 3.2.

[Dat|Cat]  will  sort  the  database alphabetically  by category

abbreviations; within the same category fixes  will be  arranged

by names.

3.12. Sorting fixes by proximity

Fixes can be also  be sorted  in the  increasing order  of their

distances  from  a  given  fix.   To  do this,  select a  fix by

highlighting it in the database window and press  [Dat|Prx]. The

fix, distances from which have been  used as  the basis  for the

sort will be shown at the top of the database.

3.13. Clearing the database

Pressing [Dat|Clr]  will bring  up the  Clear (CLR)  Menu.  From

here, [Fix] will clear  the fix  database, [His]  - the  history

log, while [All] will clear both.

4. Computational operations

Here  we  come  to  what  Spheric  actually  does.    All  these

operations are  performed from  the Calculations  Menu.  To  get

here, press [Cal] in the Top Menu.  (There  is also  a shortcut:

just press the space bar at any time,  and you  will jump  right

here.)

4.1. Distance between fixes

[Cal|FFd]  ("Fix-to-Fix  distance")  will  compute the  distance

between two fixes. You will be prompted for both fix names.

The  distance  is  defined  as  the  length  of  the great  arc,

connecting both fixes on the Earth sphere.

4.1.1. True ellipsoidal great arc distance

An additional distance value  is also  computed, and  denoted as

"Ell" on the display (only if the distance units are set  to km,

mi or nm).  This distance is measured along the line  running on

the  ellipsoidal  Earth  surface  "under" or  "above" the  great

circle arc.

In  other  words,  this  is  the  distance  measured  along  the

intersection  line  of  the  Earth's  ellipsoid  with the  plane

defined by both fixes and the Earth center.

4.1.2. Geodesic distance

Pressing Shift-[FFd] instead of just [FFd] will replace the true

ellipsoidal distance described above with the geodesic distance,

denoted as "Geo". This is the length of the geodesic line, i.e.,

the shortest  path   connecting  both   fixes  on   the  Earth's

ellipsoid.

Interestingly enough, the geodesic distance barely  differs from

the  true  elliptical  one:  with the  exception of  some exotic

situations  involving  almost-opposite  (antipodal)  fixes,  the

differences are within a 10-meter  range! This  means that,  for

any  practical  purposes,  following  a great  circle arc  is as

efficient as following a geodesic line.

The method employed in Spheric for geodesic distance computation

(and also for azimuth, see 4.2.1) is the  same as  the algorithm

used by  the U.S.   Federal Aviation  Administration in  the NAS

system.  The iterative process does not converge if  both points

are very near to the Equator and, at the same  time, almost  180

degrees of longitude apart.  If this is the  case, Spheric  will

not compute the geodesic distance, displaying only the great arc

length value.

4.1.3. Cumulative distance for a series of fixes

Instead of the first fix name you may enter just a "+"  (without

quote marks). Spheric will then replace it with the name  of the

second fix from the previous distance  calculation, compute  the

arc length, add it to the previous result and display the total.

This way, a total length of a route defined by a series of fixes

can be easily computed.

4.2. Azimuth from one fix to another

Pressing  [Cal|Azi]  will compute  the azimuth  from one  fix to

another. You will have to identify both fixes by their names.

The  azimuth  from  fix  A  to  fix B  is defined  as the  angle

(measured at A) between two great arcs - one running  from A  to

the  North  Pole,  and  the other  from A  to B.   The clockwise

direction is defined as positive ("+").

The result will be between -180 and 180 degrees or between 0 and

360 degrees, depending on the [Set|360] setting.

If both points have the same coordinates, or  if one  is exactly

opposite on the sphere to the other, an error will be reported.

The azimuth from the North Pole to anywhere else is 180 degrees;

from  the  South Pole,  0 degrees.  Similarly, the  azimuth from

anywhere to the North Pole is 0, to the South Pole, 180 degrees.

4.2.1. Geodesic azimuth

Pressing Shift-[Azi] instead of [Azi] will compute, in  addition

to the great circle azimuth as described above, the value of the

geodesic azimuth, i.e. the angle under  which the  geodesic line

intersects, at  its  starting  point,  the local  meridian.  The

displayed value will be denoted as "Geo".

See Section 4.1.2 for more about the geodesic line.

4.3. Midpoint between two fixes

This point is computed with [Cal|Mid]. A midpoint  between fixes

A and B is defined as a fix C, located on the  great arc  from A

to B, and such that the distances A-C and B-C are equal.

You will be prompted for the names of the original two fixes  (A

and B), and then you will be able to assign the midpoint a name.

If you do that, a new fix will be added to the database.

An  error  will  be  reported if  one of  the points  is exactly

opposite on the sphere to the other.

4.4. Fix at a given azimuth and distance

This  operation  is  invoked  with  [Cal|Aim].   Given a  fix A,

azimuth angle a (any  values between  -360 and  360 degrees  are

allowed), and distance d, it will compute a new fix B, such that

the azimuth from A to B will equal a, and the distance, d.

4.5. Intersection of two great circles

Two great arcs may or may  not intersect  within their  lengths,

but  their  great circles  always do  (as long  as they  are not

identical).  This operation, activated  by [Cal|Int],  will find

the intersection between the  great circles  to which  two given

great arcs belong.

First you will be prompted for the names  of two  fixes defining

the first great arc (A,B), then for those defining the other arc

(C,D). You will be also given an option to assign a name  to the

intersection point E; in this case the fix will me added to  the

database.

Two great circles (different from each  other) intersect  in two

points. Out of these two possible solutions, Spheric  will chose

the one which is closer to the arcs  involved (more  accurately:

to the midpoint between their respective midpoints).

The result displayed in the History Log will be followed  by two

characters;  each  of  them  denotes  the  relationship  of  the

intersection E to the first or the second arc:

"*" means  that the  intersection belongs  to (falls  within the

length of) the given arc

"-"  denotes the intersection preceding the arc's first fix

"+"  means that the intersection follows the arc's second fix.

For  example,  let us  consider two  great arcs:  the first  one

running from (5N,0) to (10N,0) and  the second  from (0,10W)  to

(0,5W).  It is not difficult to  predict, that  the intersection

point will be at  (0,0).  It  will be  marked with  "+-", as  it

follows the end of the first arc, but precedes the start of  the

second one.

On the other hand, with the first arc from (5S,0) to (5N,0)  and

the second from (0,5W) to (0,5E), the intersection point will be

(0,0) but it will be denoted as "**",  falling within  the spans

of both arcs.

4.6. Projection of a fix on a great circle

The projection  of a  fix can  be defined  as the  point on  the

circle at the closest distance from that fix.  For example,  the

projection of (10N,5E) on the Equator will be (0,5E).

We may be, for instance, interested in finding  the point  on an

aircraft's route (a great arc) from Warsaw to Chicago, which  is

closest  to  London.   This  will be  exactly the  projection of

London on the great arc (or its circle) Warsaw-Chicago.

This computation is activated by pressing  [Cal|Prj].  You  will

be prompted: first for  the names  of the  fix being  projected,

then for the names of two fixes defining the great arc.

The displayed result will be followed with a character, "*", "-"

or  "+"  denoting whether  the projection  falls within  the arc

span, precedes its start, or follows its end,  respectively. For

example,  if  our  fix is (10N,0) and the great arc  runs  from

(0,5E)  to (0,10E),  then the projection will be (0,0)  and  it

will  precede  the arc's start fix,  so the  solution  will  be

marked with a "-".

4.7. Distance from a fix to a great arc and its circle

Pressing [Cal|FAd]  ("Fix-to-Arc  distance")  will  compute  and

display two results:

- The "passing distance": this  is the  distance between  a fix

and  its  projection  on  a great  circle (see  the preceding

section).

- The "closest distance", i.e.   the distance  from the  fix to

the closest point of the arc.

The prompts and data input will be the  same as  in the  case of

[Cal|Prj], and the character flag following the result value has

the same meaning as well.

If that character is a  "*" (i.e.   the projection  falls within

the arc length), both results will be the same. For  a "-"  or a

"+", the second result (greater than the first one) will  be the

distance between the fix and the start or  the end  of our  arc,

respectively.

4.8. Distance between two great arcs

[Cal|AAd] ("Arc-to-Arc distance") will prompt you for fix  names

defining two arcs, and then the distance between those arcs will

be computed.  This is the shortest distance between any point on

one arc and any point on the other.

Obviously,  if  the  arcs  intersect (within  their lengths)  or

overlap, the computed result will be zero.

5. Data entry

At certain times, Spheric  will prompt  you for  input from  the

keyboard.   Sometimes  it  will  be  a  simple Yes/No  question,

sometimes you  will have  to answer  with a  textual or  numeric

input.

The prompt is always displayed in  the Prompt  Line, temporarily

replacing the  F-Key Menu  at the  bottom of  the screen.   Your

answer will be also echoed here.

The Yes/No questions require you to press just the Y  or N  key.

Instead of "Y" you may press again the same key which caused the

prompt to  be displayed,  and pressing  Escape will  abandon the

whole operation.

For example, [Dat|Clr|All] will  prompt you  with "Clear  all?".

Instead of moving your  finger to  the Y  key, just  press [All]

again, and the clear-all operation will be performed.

The following general rules are applicable to all other kinds of

keyboard input.

5.1. Editing the input

Sometimes  a  default  (or  just  the  previous) answer  will be

displayed after the prompt.  The answer can be  edited with  use

of  the  customary  line-editing  keys: left-   and right-arrow,

Delete, Backspace, Home and End.

Pressing Escape when any  input is  present on  the Prompt  Line

will clear it. The same key with no input present will exit  the

prompt, and  the operation  of which  the prompted  input was  a

part, will be abandoned.

After  having  the appropriate  input typed  in, press  Enter to

continue. The same effect can be achieved by pressing again  the

same F-key in response to which the prompt has been displayed.

5.2. Special characters

In all inputs involving angles, the degrees character "�" can be

entered by  pressing Alt+D  (hit the  D key,  while holding  Alt

down).  The minutes symbol can be entered as an apostrophe,  but

also as Alt+M, and seconds -  as double quote or Alt+S.

Alnernatively, a "d" or "D" can  be used  instead of  the degree

symbol, as in 12d30'21".

5.3. Entering a single fix name

The fix name can  be typed  in either  upper-  or  lowercase; it

will be  converted to  uppercase anyway.   Leading and  trailing

spaces are discarded; spaces  inside the  name are  not allowed.

The maximum name length is twelve characters.

The name can be immediately followed by a  comma and  a category

abbreviation (up to ten  characters), as  e.g.  in  "PARIS,FRA".

The total length of  name+category is  limited to  12 characters

(not including the comma).

If these limits are exceeded on input, name and/or category will

be truncated appropriately and redisplayed. Pressing Enter again

will accept the truncated version.

If possible, Spheric will keep the  category intact,  truncating

just the name itself. Only if the name is down to one character,

the xategory will be shortened as well.

For example,

"STOCKHOLM,SWEDEN" will become "STOCKH,SWEDEN"

"NEWBIGGERPOINT,X" will become "NEWBIGGERPO,X".

5.3.1. Name completion

Instead of entering the whole name  (or name+category),  you may

type in just one or more of its first characters  and press  the

Tab key.  The program will then display the first matching  name

(as ordered in the database) on the Prompt  Line, together  with

the category abbreviation.

If more names match, subsequent uses of the Tab key  will scroll

through all matching names, finally returning  to the  first one

again. If there are no matching names, the entered text  will be

just reformatted to uppercase.

5.3.2. Recalling previous inputs

If  the  Tab key  is pressed  when the  prompt line  contains no

input, the last used (and accepted) fix name  will be  displayed

there. Press Enter to accept this input.

Alternatively,  the  up-   and  down-arrow keys  can be  used to

browse through the three most recent inputs.

*** In the registered version, last ten inputs  (not three)  can

be recalled.

5.3.3. New and old fix names

In some situations Spheric expects you to enter  the name  of an

existing fix (i.e. one already in the database). If this  is not

the case, an error message will be shown.

An opposite case is also possible, when Spheric waits for a name

of a new fix (different than any already  in the  database), but

you enter an "old" one.  If this happens, the program will alert

you that the operation will overwrite the existing  fix and  you

can proceed or not.

5.4. Entering two fix names

This case is very  similar to  the one  described above,  except

that Spheric expects two existing fix names, separated  with one

or more  spaces.  Each  name can  be immediately  followed by  a

comma and category abbreviation.

Pressing  the  Tab key  will, obviously,  complete just  one fix

name: the last one in the Prompt Line.

For example, if the input is just "lo", it may be redisplayed as

"LONDON,GBR ",  but if  it is  "LO PA",  then pressing  Tab will

bring up "LO PARIS,FRA".   To get  "LONDON,GBR PARIS,FRA"  enter

"lo", press Tab, then add "pa" and press Tab again. It can't  be

more simple than this.

If  the  Tab key  is pressed  when the  prompt line  contains no

input, the  last used  and accepted  pair of  fix names  will be

displayed there.  The up-  and down-arrow  keys allow  to scroll

through three  (in the  registered version:  ten) most  recently

entered fix name pairs.

5.5. Entering fix coordinates

At many points Spheric will expect you to enter the  coordinates

of a fix. It will then display a prompt

Lat, Lon:

followed by  the current  coordinates of  this fix,  if the  fix

already exists in the database.

Now the program will expect you to enter values of  latitude and

longitude (in this particular order), separated with one or more

spaces (but no spaces within values are allowed!).

The program  will understand  quite a  variety of  input formats

here, and they can  be mixed  in almost  any way  you wish  (the

current angular mode setting does not affect this).

For example, the same coordinates can be entered  in any  of the

formats shown below:

13�30'36"N 5�15'W

13�30.6' 5�15'W

13.51N -5�15'

13.51 -5.25

13N3036 5W15

(Note  that  the  meaning  of "-"  in coordinate  values can  be

changed as described in Section 7.4.)

Additionally, instead of the degree symbol, a letter "D" or  "d"

can be used.

Any  illegal  input (and  there are  quite a  many possibilities

here) will cause an error sound  (no message,  though!) and  you

will be able to correct the mistake.

5.6. Entering an azimuth

When Spheric prompts you for an angular value of an  azimuth, it

will be able to recognize input in all  angular formats  used by

Spheric, (see Section 7.3),  regardless of  the current  angular

mode.

Azimuth is always measured clockwise from North, usually from -

180  to  +180  degrees,  although values  between -360  and +360

degrees will be interpreted properly.  This  does not  depend on

the [Set|+/-] setting (described in Section 7.4).

Instead of "-" or  (optional) "+",  letters "W"  and "E"  can be

used.

For example, an azimuth of -90 degrees denotes the direction due

West.  Instead of entering "-90", you can enter  "270" or  "90W"

here.

5.7. Entering a distance

Every prompt for value of a distance explicitly states the units

in which the input  is expected.   This depends  on the  current

setting of distance units (see Section 7.1).

The entered value may (but does not have  to) contain  a decimal

point.

6. The History Log

Every computational operation performed  by Spheric  is entered,

if successful, into the History Log.  The most  recent entry  is

displayed in the three-line area at the top of the Main Screen.

Up  to  12 recent  entries (in  the non-registered  version) are

stored and can be reviewed as described in Section 6.2.

*** The registered version stores up to 60 log entries.

When the full capacity of the history log has been reached, each

next entry will cause the oldest one to be removed from the log,

and all others will be pushed up by one number.

6.1. Display format

A log entry consists of three  lines: the  first two  define the

computational problem, the third shows the result.

Coordinates and distances are displayed according to the current

settings (see Sections 7.1-7.3).  Changes to these settings will

not affect the existing log entries,  only those  which will  be

added afterwards.

Display of fix categories in the history log  can be  enabled or

disabled.  Switching between these two formats is done  with the

[Set|Com]  function  key  sequence,  and if  the second  mode is

active, a "," symbol will be shown on the Status Line.

6.2. Scrolling through the log

The left- and right-arrow keys can be used to move from one  log

entry to another.

Pressing one of these these keys  while Ctrl  is held  down will

cause a jump to the first or to the last entry, respectively.

6.3. Deleting a log entry

The  currently  displayed  entry  can be  (after a  confirmation

query) deleted by pressing the Backspace key.

6.4. Clearing the log

[Dat|Clr|His] will  clear the  history log,  while [Dat|Clr|All]

clears also the fix database.

7. Program settings

To get to the Settings (SET) Menu, press [Set] while in the  Top

Menu. There is also a shortcut: while in the Calculations  Menu,

you can get directly to the Settings Menu  by just  pressing the

Space key (press Space  again to  get back  to the  Calculations

Menu).

7.1. Distance units

This setting  affects in  what units  Spheric displays  distance

values and in what units it expects them on input.

Pressing  the  [Set|Uni]  function  key  repeatedly  will change

distance  units  between  kilometers  (km), statute  miles (mi),

nautical miles (nm), minutes of arc (') and degrees of arc (�).

The last two settings  are useful  primarily, but  not only,  in

astronomical applications  (and you  thought Spheric  dealt just

with Earth?).

One statute mile is  defined as  1.609344 km,  and one  nautical

mile - as 1.852 km.

The relationship between degrees/minutes  of arc  and the  other

units depends,  obviously, on  the assumed  Earth radius.   This

will be described in Section 7.6.

7.2. Distance display precision

Depending  on  the  [Set|Uni]  setting,  the   distance  results

displayed in the History Log are rounded to the nearest  km, mi,

nm, arc' or 0.01 arc�.

This  accuracy  is sufficient  for practical  purposes but  if a

higher display  precision is  needed, [Set|Acc]  can be  used to

switch to a mode where three more decimal digits are displayed.

*** The  high-accuracy  display  mode   is  available   only  in

registered copies of Spheric.

7.3. Angular modes

This  setting   decides  how   Spheric  displays   latitude  and

longitude. It does not affect the data  input (all  formats will

be always accepted on input).

Pressing the [Set|Ang] function key toggles you between the DMS,

Infix, Decimal and DMF modes.

In the DMS mode  angles are  displayed as  degrees and  minutes,

e.g.  23�15',  in the  Decimal mode  - as  degrees with  decimal

fraction, like in 23.25�. The Infix mode is similar to  DMS, but

the direction character follows the degree  value, as  in 23N15.

Finally, the DMF (Deg/Min/Frac) mode shows  degrees followed  by

minutes and fraction, e.g. 23�15.28' (in  low precision  display

this is the same as the DMS mode).

When  angular  mode is  changed, all  entries shown  in the  Fix

Database Display will be reformatted.

The angular mode does not affect the way in which azimuth values

are  displayed:  decimal degrees  will be  always used  for that

purpose.

7.4. Sign convention

Some of us prefer to use "-" and (optionally) "+"  signs instead

of  the  N,  S,  W  and  E  letters  when entering  latitude and

longitude values.

The usual convention seems to be that positive values correspond

to North and East.  Spheric allows this to  be tailored  to your

personal  preferences.   Pressing the  [Set|+/-] key  repeatedly

will change the sign convention from  NE (where  North and  East

are  "+")  to  NW  (North  and  West, often  used in  the United

States), SE (useful for our Australian friends) and SW.

One has to remember, that if a fix file using  signs instead  of

letters  is  read  by  Spheric (see  Section 3.10),  it will  be

interpreted according to the current setting. This is  important

only for files generated  by other  programs, as  Spheric always

uses the letter (not sign) convention when saving fix files.

7.5. Azimuth display modes

The  [Set|360]  function  key  toggles  between  two  modes   of

displaying azimuth values: from -180 to 180 degrees,  or from  0

to 360 degrees. This does not affect how the azimuth  is defined

(always clockwise from the North).

7.6. Earth model

Spheric  allows,  in  a  somewhat  limited  way,  to  use either

spherical or  ellipsoidal model  of Earth  in its  calculations.

Pressing [Set|Ert] toggles between these two models.

This may cause some delay, especially  for large  fix databases,

as the program has to recompute  the internal  representation of

all stored fixes.

7.6.1. Spherical Earth

This model assumes that  Earth is  a sphere  with the  radius of

6367.4357 kilometers.

The  spherical  model is  useful when  comparing the  results of

calculations by Spheric with  those of  other programs;  it also

should be used when Spheric is used for astronomical (as opposed

to geographic/navigational) calculations. In the latter case the

distance units should be set to degrees or minutes of arc (miles

or kilometers do not make much sense on the sky!).

7.6.2. Elliptical Earth

This model, often referred to as GRS 1980, assumes that Earth is

an ellipsoid with the polar axis of 6356.7523 and the equatorial

axis of 6378.1370 km.

The  geometric  average  of  these two  values is  equal to  the

"spherical" Earth radius as shown in Section 7.6.1.

7.6.3.  Accounting for the ellipticity

For most purposes you may just  skip this  section and  remember

only  that  the  elliptical  model  is  more  accurate  than the

spherical  one.   Now more  explanations for  those who  have to

know.

In  the  first  approximation  Earth is  a sphere.   It can  be,

however, represented more accurately  as an  ellipsoid with  the

shorter  axis  about 1/300  less than  the longer  one.  (Better

approximations are also available).

When  dealing  with the  elliptical Earth,  one may  distinguish

between two different definitions of latitude.

The geocentric latitude of a fix P can be defined  as the  angle

between the Equator plane  and the  line connecting  P with  the

Earth's center.

The  geographic  (geodetic) latitude  is the  angle between  the

Equator plane and the line perpendicular to the ellipsoid at  P.

It is slightly greater than the geocentric latitude (except at 0

and 90  degrees S  or N,  where it  is the  same).  This  is the

latitude usually given for a point.

Most of navigational and  cartographic applications  account for

Earth's ellipticity just by performing their operations in three

steps:

(1) Translating the geographic latitude into geocentric for  any

points on input;

(2) Doing  all  calculations  on  a sphere  with some  "average"

radius;

(3) Translating the geocentric latitude into geographic  for any

points on output.

Spheric also mostly follows this method, with the average radius

assumed as 6367.4357 km (the same as the  one for  the spherical

model).

Such a procedure accounts for most, if not  all, of  the Earth's

ellipticity. In principle, the shortest path length between  two

points can be computed on an ellipsoid with the full accuracy (a

slow, iterative procedure), but it would be no longer a distance

along a great arc: there is no such thing like a great arc on an

elliptical Earth!  All other  concepts used  in our  program are

based on the concept of great arc -  and this,  obviously, makes

sense only on a sphere.

There are two enhancements to the above, where computations on a

truly ellipsoidal Earth are performed:

- For fix-fix distances, an additional  result is  computed and

displayed:  it  may  be  either  distance  along  the  "truly

ellipsoidal great arc" (see  Section 4.1.1)  or the  geodesic

distance on ellipsoid (Section 4.1.2).

- Similarly, the geodesic azimuth can be  computed in  addition

to the "spherical" one (see Section 4.2.1)

7.7. Sound alerts

Spheric  uses  two  kinds  of  sound  alerts:  one  to signal  a

completion of a computation and one to alert you to an error.

These sounds can be disabled (or enabled again) by pressing  the

[Set|Snd] function key.  Enabled sound is denoted with  an arrow

preceding the [Snd] menu entry.

7.8. Fix category display

The [Set|Cat] function key  toggles between  the two  formats of

fix name/category display in  the History  Log, as  described in

Section 6.1.

When the category display is enabled, a "," symbol will be shown

on the Status Line, and an arrow sign  will be  pointing to  the

[Cat] menu abbreviation.

8. Miscellaneous operations

Here we describe all  remaining features  of Spheric,  which did

not fit into any of the preceding chapters.

8.1. On-line help

Spheric has a simple on-line help, activated by pressing the  F1

key in  any menu.   The Help  Screen contains  a list  of topics

available.   Entering  the  topic's  first  one  or two  letters

(enough  to  distinguish  it  from  others)  at  the  prompt and

pressing Return,  will bring  the proper  help page.   Some help

pages will offer you a further choice of  topics -  the help  is

structured in a tree-like fashion.

Pressing Return, Escape or F10 at a topic prompt (or any key  in

a prompt-less  screen) will  take you  back to  the next  higher

level or to the Main Screen.

8.2. Clearing history and data

[Dat|Clr] will bring up  the Clear  Menu, denoted  as CLR,  from

where three operations can be performed:

[Fix] - clear the fix database

[His] - clear the history log

[All] - clear all (i.e. both of the above).

8.3. Exiting the program

To leave  the program,  press [Exit]  in the  Top Menu.   Before

exiting  Spheric  will  allow  you  to  save its  current status

(including  fixes,  history  and  settings) to  disk file  named

SPH.ENV. There will be also the final request  for confirmation,

with  the  exit  prompt  showing  the  program version  and (for

registered copies) the serial number.

To  exit  quickly without  saving (and  with no  extra queries),

press Shift+F10; going to the Top  Menu is  not necessary  to do

this.

The size of SPH.ENV depends on the number of fixes stored in the

database and on the number of entries in the history log.

SPH.ENV is automatically read in as Spheric is executed, so that

you can  exit the  program, do  something else,  and reenter  it

without losing continuity of your work.

9. Program installation and use

Installing  the  program  requires  that  some  or  all  of  the

distributed files are copied to your disk drive.

9.1. The packing list

Whatever was  the source  from which  you obtained  Spheric, you

should have the following files:

SPH.EXE -      the program itself

SPH.HLP -      the text file used by on-line help

SPH.ERR -      the text file with error messages

SPH.DOC -      the manual (you are reading it now)

SPH_HP.DOC -   addendum to the manual (HP Palmtops only)

WD.FIX -       a fix file with 30 major world cities

US.FIX -       another one, with 30 major U.S. cities

WHATS.NEW -    a brief list of changes since the last version

READ_ME.1ST -  the introductory file

*** Two larger data files will be useful only when  you register

your copy of Spheric:

USA.FIX -      a file with coordinates of 240 U.S. cities

WRD.FIX -      same, for 300 cities of the world

*** Upon registration you will receive one more tiny file:

SPH.KEY -      your personal registration key file

The key  file  should be  backed up  and placed  in the  Spheric

directory.  It  activates the  "registered" enhancements  of the

program and will do it also for all future versions of Spheric.

9.2. Installing Spheric on your computer

This is a simple task. Just copy the files SPH.EXE,  SPH.HLP and

SPH.ERR to one directory, e.g., C:\SPH.   If you  decide to  use

some of the supplied data files, copy them there, too.

If that directory is  included in  the PATH  definition in  your

AUTOEXEC.BAT  file,  you  will  be  able  to  run  Spheric while

anywhere in the system, as explained in Section 9.4.

9.3. Character set compatibility

9.3.1. Screen

The only non-standard ASCII characters used  by Spheric  are the

degree symbol  (�), corresponding  to the  value of  248 in  the

ASCII  table,  and the  graphic symbols  used to  draw the  Main

Screen.

These symbols  are available  on the  HP Palmtop  as well  as on

desktop machines, at least these using English or  International

character sets (code page 437 or 850, respectively).

9.3.2. Printer

Not all printers may print the above symbols correctly; this may

cause minor problems in a few  places of  this documentation;  a

minor annoyance.

On the HP LaserJet III (and perhaps some other models) all these

problems can  be avoided  by setting  the character  set to  850

Multilingual.

9.4. Running Spheric from DOS or Windows

To run Spheric we must assure that

* DOS will find the program file, SPH.EXE, and

* Spheric  will find  its related  files: SPH.HLP,  SPH.ERR and

SPH.ENV (in registered copies: also SPH.KEY).

The first is done either by moving (with use of the CD  command)

to the directory where  Spheric resides  before running  it, or,

preferably, by specifying this directory in  the PATH  statement

in your AUTOEXEC.BAT file.

The second is done by keeping all files listed in (ii) above  in

the same directory where SPH.EXE is.

Spheric will also run  (as a  DOS session)  from within  Windows

3.1, NT or 95.

10. Registration and distribution

Spheric is a copyrighted  program, released  as shareware.   You

are free to use any evaluation (non-registered) copy of it  on a

trial  basis  for  a  month  or  so.  If  you decide  to use  it

afterwards,  you  are  expected  to  register  it (as  described

below); otherwise please delete it from your system.

Note that your registration covers this DOS/HPLX version as well

as the versions for Windows 3.1 and 95 - not too shabby!

10.1. How to register

=== DIRECT ORDERS FROM ME, PAID BY CHECK OR MONEY ORDER

To register,  send your  name and  address (including  email, if

applicable) together with the registration fee  of US  $30 -   a

check or money order (U.S. funds, drawn on a U.S. bank) to

Dr. J.A. Wrotniak

2057 Happy Lane

Crofton, MD 21114-1917

Please state your current version number.  If you have an email,

address, I'll send you the registration key by  email, otherwise

(or if you ask for it  explicitly), I'll  send you  a floppy  by

mail. If you would like to get the Windows version  at to  extra

cost,  state  so  and  say which  Windows you  are using.   And,

finally,  for  an  extra  $5 you  may also  register the  Midget

sci/eng calculator for Windows 3.1 or 95.

=== ORDERING VIA SWREG ON THE COMPUSERVE

CompuServe subscribers can also register online:  GO SWREG,  the

file ID  is 10078;  $30 will  be added  to your  next CompuServe

bill.

=== CREDIT CARD ORDERS

Credit  card  orders  are  handled  by  a respectable  shareware

distributor from Texas, Public Software Library.  In this  case,

however, I have to split with you the  extra cost  involved; the

all-inclusive price of Spheric ordered via  PsL is  $32. For  an

extra $7 (50% off!) you may register the  Midget calculator  for

Windows.

To order via PsL using your Visa, MasterCard or AmEx, choose one

of the following:

WORLD WIDE WEB: go to the Spheric Web page:

http://www.freeflight.com/wrotniak/sph.html

and click on the PsL order link. This will take  you to  the PsL

order form for Spheric which you will have to fill. PsL offers a

secure server for transactions,  but even  if your  browser does

not support the secure mode, PsL  uses some  tricks to  minimize

the risk of any fraud.

BY PHONE:  call PsL  orders-only line

800-2424-775  ext.1563  or  713-524-6394   ext.1563

Monday to Thursday 7:00-6:00 or Friday  7:00-12:30 Central  Time

(London minus 6 hrs) and specify Spheric, product #1563.

If you have an email address, please give it as well -  I prefer

to use it to email you the registration key. If not, I will mail

you a 3.5-inch floppy with the  latest version  and the  key. If

you would also like to get the Windows version at no extra cost,

say so and also mention which  Windows you  are running  (3.1 or

95).

PLEASE NOTE:  THESE NUMBERS  ARE FOR  ORDERS ONLY!   PsL handles

only  the  monetary  side  of  the  transaction.  All  technical

questions, queries on the status of  your order  etc. should  be

addressed to me directly.

BY MAIL OR FAX: fax your order to 713-524-6398 or send it to

PSL Registration Service

P.O.B. 35705

Houston, TX 77235-5705

USA

and mention Spheric, product #15163 (at $32; you may also bundle

it with Midget for Windows for the total of $39).   Include your

name,  shipping  address, billing  address (if  different), card

number, expiration date, phone number and your signature.

10.2. The extra goodies

To  increase  chances  that  you  keep  your  end  of the  deal,

registration activates some enhancements to the program:

*** An option to  display additional  digits in  the display  of

fixes and of calculation  results; instead  of rounding  the

result to the nearest unit (km, mi, nm), 0.001 of  that unit

is used.

*** An increase in the fix database size: from 30 to about  1800

points; this allows you to use two large data files, USA.FIX

and WRD.FIX, supplied with the program.

*** A  number  of  minor  enhancements:  larger  history buffer,

larger input buffers, etc.

The  owner  may make  backup copies  only for  his/her own  use,

safety and convenience.

Every registered copy of Spheric comes withs a unique key  file,

SPH.KEY,  with  the  registered  user name  and a  serial number

embedded inside and displayed at the program exit.  Please refer

to this number in all related correspondence.

The updates and upgrades of Spheric  are free:  any new  version

will  recognize  the key  file. (Registered  owners of  versions

below 1.30 will have the update mailed or emailed for  free upon

request.)

Most  importantly,  your  registration  key  file  will also  be

recognized by the Windows 3.1 and Windows 95 versions of Spheric

which should be available in the Spring of 1997. This means that

switching to the Windows version of Spheric is free (as  long as

you download the program yourself from my Web  site or  from the

CompuServe). All you need is to copy SPH.KEY to the Spheric  for

Windows directory as SPHERIC.KEY.

You are free (and encouraged) to pass to your friends, relatives

and  strangers  all  files,  except  SPH.KEY,  which  is  to  be

installed on your computer only.

10.3. Distribution

You are free to  distribute Spheric  to anyone,  as long  as all

original files, as in the first part of the Packing  List above,

are enclosed, with the exception of  your personal  registration

file,  SPH.KEY.   This  permission  extends  to  the  electronic

distribution.

Public Domain  and shareware  vendors are  granted the  right to

include  complete  evaluation  copies  of  Spheric   into  their

libraries as long as their policies meet the  guidelines set  by

the Association of Shareware  Professionals (ASP);  otherwise my

explicit permission is required.

I would, however, recommend contacting me in either case; I  may

already have a newer version.

10.4. Support

If you have any questions, suggestions  or problem  reports, the

best way is to reach me by email:

* Internet:         70611.2552@compuserve.com

* CompuServe:       70611,2552

If you  do not  have an  email access,  write under  the address

shown above.  I read all letters and answer most of them.

The most recent version of the program can be downloaded  at

no cost from one of the following sources:

* World Wide Web: http://www.freeflight.com/wrotniak/ -   this

is the primary Spheric distribution site

* CompuServe: HP Palmtop  Forum (GO  HPHAND), Library  9 (95LX

General)

Upon request, the newest version can be also mailed to  you, for

a modest shipping fee of $5.

10.5. Improvements and new features

Unavoidably, the program reflects, to a  large extent  at least,

my  own  needs,  preferences  and working  habits.  In  your own

interest, let me know what you like and  dislike about  Spheric,

and how would you like it to evolve.

10.6. Credits

Spheric  has  been developed  in Turbo  Pascal 7.0  from Borland

International, and contains library code copyrighted by Borland.

Some of the math used in Spheric has been borrowed  from one  of

my earlier programs, Star Base, running on the Atari ST series.

I have also used some algorithms I developed  for various  Civil

Aviation  applications  at  Aerospace  Engineering and  Research

Associates, Inc., in Landover, MD, and vice versa:  some of  the

algorithms   developed   for  Spheric   found  their   way  into

Aerospace's software.