Ein Auszug aus MPLAY204.ARJ von Mark J Cox betreffend Selbstbau D/A-Wandler.

Die ZN426E und Widerstandsloesung habe ich beide nachgebaut. Beide liefern

ausgezeichnete Qualit�t. Der Klang der D/A's gef�llt mir pers�nlich besser als

der der Soundblaster.

Jan Ole Suhr

MODPLAY  *********                            (C) Mark J Cox

MODPLAY  **                                        1990/1991

MODPLAY  *******

MODPLAY  **************

Amiga four�channel sound module player for PC's

Hardware devices for great sound output!

I. CONTENTS

���������������������������������������������������������

How to make:

Mono output from a parallel port for about 5 pounds

Mono output from a parallel port for around 1 pound

Stereo from one parallel port for about 12 pounds

This document has been supplied with MODPLAY or MODRES and the

license agreement attached to the MODPLAY documentation applies

to this file.

I. INTRODUCTION

���������������������������������������������������������

MODPLAY will play modules through the PC Speaker as mentioned above, via

an D to A converter on any of the three possible parallel ports, in stereo

if you have an D to A converter on both LPT1 and LPT2 (this sounds really

good) and also through a special board that plays stereo from one

printer port ("stereo�on�1")

1. HOW TO MAKE A D/A CONVERTER FOR FIVE POUNDS

���������������������������������������������������������

A simple circuit for HiFi quality sound needs only a few components (and

this circuit will work with other software).   It even sounds better than

a Sound Blaster for sampled output!

You need:

IC1 a ZN426E (about 3 pounds) (Ferranti or Plessey are UK manufacturers)

C1 a 10micro-farad electrolytic capacitor (10V or greater)

C2 a 4.7micro-farad electrolytic capacitor (10V or greater)

R1 a 380ohm resistor (5%, 1/8W)

CON1 a phono socket for connecting a lead to your amplifier

CON2 a 25 way male D-type connector for your parrallel port

A bit of 10 way cable - or 10 short wires.

A +5v supply

Zn426e    +5V

��������Ŀ  �

(2)  D0  ��Ĵ9     14��Ĵ

(3)  D1  ��Ĵ10      � ��

(4)  D2  ��Ĵ11      � � � 390�

(5)  D3  ��Ĵ12      � ���

(6)  D4  ��Ĵ13     5��Ĵ

(7)  D5  ��Ĵ1      6��Ĵ       +

(8)  D6  ��Ĵ2      4��ĳ�������Ĵ�����> TO AMPLIFIER

(9)  D7  ��Ĵ3   7   �  �        10�F

���������� +�        (may not be needed)

� 4.7�F�

�      �

(25) GND ������������������������������ GND

2. GETTING A +5V POWER SUPPLY

���������������������������������������������������������

Catalogues such as MAPLIN (uk) provide simple circuits for obtaining

a +5v power supply (it only has to be 100mA maximum).  Alternatively

you could tap +5v off the keyboard connector or use 3 1.5volt

batteries (or even 4 1.5volt batteries with two diodes in series to

drop the voltage).  In fact, anything that produces a voltage in the

range 4.5 � 5.5 volts. (in fact you can even tap +5v from the game

port).  Don't forget to connect the 0v of your power supply to the GND

rail shown.

3. HOW TO MAKE AN EVEN CHEAPER D/A

���������������������������������������������������������

For the price of 20 1% resistors and a plug, with no power supply

needed you can make a D/A converter that sounds close to the IC

one above (it seems only the UK can get a Plessey ZN426E)

You need one plug for your amp, one for your parallel port and

9 20k and 9 10K resistors

Use 1% resistors if possible � and 20k sound much better than 22k

Printer Port:

signal   pin        20k   20k

D0       2      >��İ����İ����0v

20k � 10k

D1       3      >��İ��Ĵ

20k � 10k

D2       4      >��İ��Ĵ

20k � 10k

D3       5      >��İ��Ĵ

20k � 10k

D4       6      >��İ��Ĵ

20k � 10k

D5       7      >��İ��Ĵ

20k � 10k

D6       8      >��İ��Ĵ

20k � 10k

D7       9      >��İ��Ĵ

20k � 10k  100nF

������Ĵ�����> TO AMPLIFIER

� 10k

GND      20     >��������������������>

0v

Thanks to Harry Stox for the above circuit.  Again, make sure all

the 0v lines are connected together.  The 100nF capacitor may not

be needed, try removing it if this circuit sounds poor.

4. STEREO-ON-1 BOARD

���������������������������������������������������������

It is possible to play in stereo with only one parallel port (MODPLAY

version 2.00 onwards) using the circuit below.  MODPLAY will

automatically detect this circuit if it is present and work out what

address it is at � so you can plug it into any of your parallel ports.

MODPLAY needs no further options and will default to playing through

this device.  The +5v supply can be  anywhere between 4.5 and 5.5 volts

so it could be run from three 1.5v  batteries. (Or from four

rechargable batteries as these are only 1.2v)

* Stereo-on-1 works from MODPLAY 2.03 onwards where it did not in 2.00 *

ZN428

��������������Ŀ         390�

�����������������Ĵ11           6��������ı������+5v

� ���������������Ĵ12           7���    �

� � �������������Ĵ13           3�NC    �

� � � �����������Ĵ14            �      �  + 10�F

� � � � ���������Ĵ15           5������ĳ��Ķ������> Left out

� � � � � �������Ĵ16            �      �     0v���>

20 >�0v � � � � � � �����Ĵ1           10���+5v �

� � � � � � � ���Ĵ2            9����0v �

14 >���ĳĳĳĳĳĳĳĳ��Ĵ4            8���    �

� � � � � � � �   ����������������      �

11 >���Ĵ � � � � � � �   ZN428                 �  + 1�F

� � � � � � � �   ��������������Ŀ      ���Ķ��0v

9 >�����ĳĳĳĳĳĳĳ��Ĵ11 D7   VRIN 6��������

8 >�������ĳĳĳĳĳĳ��Ĵ12 D6  VROUT 7� NC

7 >���������ĳĳĳĳĳ��Ĵ13 D5     NC 3� NC

6 >�����������ĳĳĳĳ��Ĵ14 D4         �        + 10�F

5 >�������������ĳĳĳ��Ĵ15 D3    OUT 5���������Ķ������> Right out

4 >���������������ĳĳ��Ĵ16 D2         �           0v���>

3 >�����������������ĳ��Ĵ1 D1     +V 10���+5v

2 >���������������������Ĵ2 D0     0v  9���0v

1 >���������������������Ĵ4 !EN    0v  8���

25 pin male D connector   ����������������

Notes:

Make sure all the 0v lines are connected together and connected to the

0v line of your power source.  Also, make sure the +5v lines are

connected together.  The lines marked 'NC' do not need to be connected.

If a ZN428 is not available, any D/A with input latches will work -

the D/A should latch the data bus when pin 4 (!EN) is low.

ĳ� = No connection, ��� = joined

The above circuit (Stereo-on-1) is (C) Mark J Cox 1991, 29 Lundie Close

Stenson Fields, Derby DE2 3AN; and is supplied for personal use only.

For any other use, please contact the author at the address above. If

this circuit is abstracted or incorporated into any other document or

circuit then that document or documentation supplied with that circuit

must contain this paragraph of text.  Any commercial use where not

agreed by written contract with the author (Mark J Cox) is prohibited.

13. UK SUPPLIERS

���������������������������������������������������������

A lot of people have been having difficulty obtaining the ZN426/ZN428

IC's.  If you cannot find an alternative, there is a company in the

UK that will do overseas orders.   Carriage charges vary from 10%

of the total order value (Ireland) to 25% (Fiji) and payment can be

by Eurocheques, bank drafts, credit cards etc. Maplin: +44 702 554161

14. CONTACT/SUPPORT

���������������������������������������������������������

Before June 1992:

-----------------

Mark J Cox                 Electronic Mail:

17 Kirkburn Place           (JANET) m.j.h.cox@uk.ac.bradford (UK)

Bradford                            m.j.h.cox@bradford.ac.uk (other)

W. Yorks, UK              (NETMAIL) Post a message in PLAY SUPPORT on

BD7 2BZ                             Kingdom of Greyhawk BBS

(+44 332 756414)

After June 1992:

----------------

29 Lundie Close           (NETMAIL) Post a message in PLAY SUPPORT on

Stenson Fields                      Kingdom of Greyhawk BBS

Derby, UK                           (+44 332 756414)

DE2 3AN

All comments on PLAY, RESPLAY, MODPLAY or future software are welcomed!

Please enclose a SSAE to ensure reply.