������������������������������������������������������������������������Ŀ

� QuadCOM - A four port serial system for QuickBasic 4.x                 �

�                                         Basic Compiler 6.x             �

�                                         Basic Compiler 7.x PDS         �

�                                         Visual Basic for DOS 1.x       �

��������������������������������������������������������������������������

Four serial ports open at once, user definable card addresses, user

definable IRQ's, automatic XON/XOFF RTS/CTS handshaking, baud rates up

to 115,200 bps, use 16550A's FIFO's, no more buffer loss on SHELLs, no

more dropping DTR when closing the port, no more ON ERROR to catch

transmission errors, and more... WITH QUICKBASIC... This is it.

QuadCOM Version 4.1 (c) Nick Davison 1995

This software is supplied "as is" without a warranty of any kind.

This shareware version of QuadCOM is issued free of charge. Please feel

free to distribute it to others who may find it useful. If you continue

to use QuadCOM on a regular basis, please register your copy.

Registration for the modest sum of 10 UK pounds will also include a

free upgrade to the next version when available and telephone support.

Nick Davison               Email  nick.davison@almac.co.uk

6 Tithe Barn Lane,

Bardsey, Leeds

West Yorkshire

England

LS17 9DX

The QuadCOM system is distributed as several object files ready to

link into a Quick Library and be incorporated into a .LIB module.

I have not attempted to include a sample program illustrating the use of

QuadCOM, as there are a numerous combinations possible when using

the system, and trying to interpret hundreds of lines of another person's

code, written in the style of THEIR choice is a little tedious to say the

least. However the SUB and FUNCTION descriptions are fairly comprehensive,

so you shouldn't have any trouble utilising QuadCOM (easy for me to say,

I wrote it <g>).

������������������������������������������������������������������������Ŀ

� QB serial I/O                                                          �

��������������������������������������������������������������������������

QuickBasic (QB) will only support COM ports 1 and 2, and expects

these ports to reside at the standard addresses of 3f8h and 2f8h, using

IRQ4 and IRQ3. Even if QB did allow the opening of COM ports 3 and 4,

they would share IRQ lines with COM1 and COM3 respectively, hence it

is not feasable to have COM1 and COM3, or COM2 and COM4 open at the same

time, unless they all use different IRQ's.

Four port serial cards which support this mode of operation tend

to be quite expensive, whereas a normal 8 bit twin serial card can be

picked up for about 10 UKP, and cards using the high speed 16550A UARTs

on multi I/O cards can be found for around 20 UKP.

These cards generally allow you to select any of the four ports,

but again restricting you to the shared IRQ regime, but can be modified

to use other IRQ's, so allowing a cheap upgrade to 4 useable ports.

QuadCOM is a purpose written set of routines to allow the

simultaneous operation of four serial ports with user definable port

addresses and IRQ lines. The input buffers are set at 2000 bytes per

port, and avoid the buffer loss associated with QB's release of memory

when performing SHELL's. Unlike QB, DTR is not de-asserted when closing

a port, and avoids the subsequent problems when writing "doors".

Both RTS/CTS and XON/XOFF handshaking are supported.

QuadCOM is written in 8086 assembler, is extremely fast, does NOT act

as an interface to a fossil, and can totally replace QB's COM port routines.

If you wish, you may continue to use QB to access COM 1 and 2, using

QuadCOM for COM 3 and 4, but the mix of syntax for the two systems may

become confusing.

������������������������������������������������������������������������Ŀ

�  Using QuadCOM                                                         �

��������������������������������������������������������������������������

Routines provided by mixed language programs are placed into Quick

libraries (QLB's) which are loaded with the BASIC environment if indicated

on the command line when BASIC is started.

Two object files must be included in the QLB, quadcom.obj and the

appropriate string interface object file. QuickBASIC stores all variable

length string data in one memory segment and refers to those strings by

offset only (near string pointers) yet PDS and VBDOS store variable

length strings in far memory, and therefore are forced to use both memory

segment and offset (far string pointers) when referring to those strings.

QuadCOM uses a small external procedure to retrieve string data, the

procedure used is dependant upon the pointer method employed by BASIC.

Using this scheme ensures that only one main procedure need be written

for all the MS BASIC's which have a mixed language interface.

STRNEAR.OBJ is the near string handler.

STRFAR.OBJ is the far string handler.

The examples below show how to incorporate quadcom.obj and the

appropriate string handler into a Quick Library for use within the BASIC

environment, and into library module for linking into an EXE file.

LINK.EXE and LIB.EXE expect their command lines to be correct, so ensure

correct syntax is used. The examples assume that you are in the

directory which contains the BASIC .EXE and support files, and also

contains the object files.

Linking for QB4.5

-----------------

LINK /Q quadcom strnear, comms.qlb, ,bqlb45;

Analysing the statement above from left to right.

1.  LINK            The name of the linker LINK.EXE

2.  /Q              Command line swith to indicate the formation of a

Quick Library

3.  quadcom         An object file name to include in the Quick Library

4.  strnear.obj,    An object file name to include in the Quick Library

The comma indicates the end of the object file list.

strfar.obj,     (Use strfar for PDS and VBDOS)

5.  comms.qlb,      The name of the Quick Library to be formed, comma

6.   ,              The name of a map file, leave empty

7.  bqlb45;         The name of the Quick Library support routine file.

bqlb4-          - indicating version number

qbxqlb          PDS

vbdosqlb        VBDOS

8.  ;               Prevents further prompting from the program.

If you wish to incorporate the object files into an existing Quick

Library, add the name of the .lib file in section 4 above.

Eg : LINK /Q quadcom.obj strnear.obj existing.lib,comms.qlb,,bqlb45;

The resultant Quick Library, COMMS.QLB is then loaded with the BASIC

environment by using the command line option /L COMMS, allowing access

to the QuadCOM functions.

When a QLB is formed, a corresponding library module containing all the

object files must also be created to allow the subsequent compilation of

your programs.

LIB comms.lib quadcom.obj strnear.obj;

1.  LIB          The name of the library manager LIB.EXE

2.  comms.lib    The name of the library to be formed

3.  quadcom.obj  The name of an object file to add to the library.

4.  strnear.obj  The name of an object file to add to the library.

5.  ;            Prevents further prompting from the program.

If you wish to incorporate the object files into an existing library,

add the name of the .lib file in section 4 above.

LIB comms.lib quadcom.obj strnear.obj existing.lib;

To compile programs from the command line :-

BC program.bas /O/T;

LINK program+nocom.obj /NOE,program.exe,,bcom45.lib+comms.lib

This assumes that BC and LINK are in the current directory or path, that

the basic souce file is called program.bas, the EXE file will be called

program.exe, the code is in comms.lib and you do not require any QB

com port facilities (linking in nocom.obj, saves a couple of Kb).

QuadCOM required declarations :-

------------------------------ Cut Here -----------------------------------

DECLARE FUNCTION ComSetUp% (Port%, Card%, Irq%, Subst%, MaxWait%, Inhibit%)

DECLARE FUNCTION ComOpen% (Port%, BaudRate&, SetupWord$, HandShake%)

DECLARE FUNCTION ComSetPort% (Port%)

DECLARE FUNCTION ComSend% (StrToTx$)

DECLARE FUNCTION ComGet% ()

DECLARE FUNCTION ComPeek% ()

DECLARE FUNCTION ComStat% ()

DECLARE FUNCTION ComFree% ()

DECLARE FUNCTION ComDCD% ()

DECLARE FUNCTION ComRING% ()

DECLARE FUNCTION ComDSR% ()

DECLARE FUNCTION ComCTS% ()

DECLARE FUNCTION ComBaudSet% (BaudRate&)

DECLARE FUNCTION ComError% ()

DECLARE FUNCTION ComUart% (Port%)

DECLARE SUB ComPurge ()

DECLARE SUB ComSetDTR (HighOrLow%)

DECLARE SUB ComSetRTS (HighOrLow%)

DECLARE SUB ComBuffer (Seg&, Os&, Length&, RdPtr&, WrPtr&, WrapCount&)

DECLARE SUB ComClose ()

------------------------------ Cut Here -----------------------------------

���������������������������������������������������������������������������Ŀ

�DECLARE FUNCTION ComSetUp% (Port%, Card%, Irq%, Subst%, MaxWait%, Inhibit%)�

���������������������������������������������������������������������������Ĵ

� IF ANY OF THE FACILITIES PROVIDED BY THIS FUNCTION ARE REQUIRED, IT MUST  �

�                   BE CALLED BEFORE OPENING THE PORT.                      �

�����������������������������������������������������������������������������

Port%          1,2,3,4

Card%          Base address of serial card, or -1 to use normal address

as identified by the BIOS.

Irq%           0 - 15, -1 to leave as set (see list of available IRQ's)

Subst%         0 - 255, -1 to leave as set (~)

ASCII value with which to replace any character received

which exhibits an error. -1 to leave as set.

Setting this variable to zero will cause the receiver

routine to ignore errored characters and not to insert

them into the buffer.

MaxWait%       Time out value in timer ticks (18.2 per second) that the

transmission routine (ComSend) will wait for CTS/DSR lines

to be asserted or XON to be received after the reception

of XOFF (if flow control is selected).

These lines do not need to be active for the port to be

opened, only when transmitting.

Inhibit%       A bit encoded switch which turns off specific funtions.

-1, leave as set

0, clear inhibit functions

Bit 0, if set, disables UART programming on opening/closing

a port. Enables production of BBS doors. Only the

interrupt vector will hooked and released.

Bit 1, if set, disables the use of 16550A UART FIFO

buffers.

THE VALUES SET BY THIS FUNCTION REMAIN ACTIVE UNTIL THEY ARE CHANGED,

EVEN IF THE PORT IS SUBSEQUENTLY CLOSED.

Returns -1 if successful or 0 if not.

�����������������������������������������������������������������������Ŀ

�                      QuadCOM Default Settings                         �

�����������������������������������������������������������������������Ĵ

�   COM  Address  BIOS Storage Address  Irq  Subst  TimeOut  Inhibit    �

�����������������������������������������������������������������������Ĵ

�    1    03f8h   0000:0400 - 0000:0401  4     ~      180       -       �

�    2    02f8h   0000:0402 - 0000:0403  3     ~      180       -       �

�    3    03e8h   0000:0404 - 0000:0405  10    ~      180       -       �

�    4    02e8h   0000:0406 - 0000:0407  11    ~      180       -       �

�����������������������������������������������������������������������Ĵ

�               Normal IRQ useage in an AT class machine                �

�����������������������������������������������������������������������Ĵ

�               0    Timer               8   Real time clock            �

�               1    Keyboard            9   Redirected to IRQ2         �

�               2    Cascaded 8 - 15    10   Free                       �

�               3    COM2,COM4          11   Free                       �

�               4    COM1,COM3          12   PS2 mouse                  �

�               5    LPT2               13   Coprocessor                �

�               6    Floppy controller  14   Hard drive controller      �

�               7    LPT1               15   Free                       �

�                                                                       �

�������������������������������������������������������������������������

Notes

-----

Ports%

Normally, the values 1 - 4 relate to COM1 - COM4, but with the

use of ComSetUp, card addresses could be passed to QuadCOM (or poked into

the BIOS storage area) which could "rearange" the logical pattern to the

port numbering system, hence care is needed when using such a flexible

system.

Card%

On powering up the PC, one of the tasks performed by the BIOS

is to determine how many serial cards the system contains, and the

port addresses. Each card's base address is stored in the BIOS data area,

and is used as a default by the OpenCom routine for opening the port.

Some BIOS's don't check for more than 2 serial ports, and may be unable

to determine the base address of a non-standard card. If this is the

case, use ComSetUp to identify the card's base address.

Irq%

COM1 and COM2 default to using the standard IRQ's 4 and 3. COM3

and COM4 default to using IRQ's 10 and 11. If you wish to use

alternative IRQ's then use ComSetUp to pass the IRQ numbers to QuadCOM.

If you wish to use COM1 and COM2 on a shared IRQ basis with COM3 and

COM4, then change the default IRQ's for COM3/4 from 10 and 11, to 4 and

3 repectively. Use with caution. The system does not check whether the

IRQ is free to use, it just takes it.

Subst%

Whereas QB causes a fatal error if there is a problem with the

reception of an individual character (parity and frame errors or overrun)

requiring an ON ERROR trap, QuadCOM aknowledges the reception error by

setting the character received to a tilde (~) character, and sets the

error code appropriately (See ComError). You may change this substitution

character for each port to a more suitable character if required, or

instruct QuadCOM to discard such characters.

��������������������������������������������������������������������������Ŀ

�  Use of ON ERROR without specifying labelled returns will bloat          �

�  your programs. A table of return values has to be kept for every single �

�  statement in your program. Event trapping uses a similar method,        �

�  placing a procedure call after every  statement in your program.        �

����������������������������������������������������������������������������

In the 12+ years I have used RS232 ports to transmit data, (moving >2mb of

data per day between PC's and devices), once the two communicator's UART's

are using the same configuration (speed, data length, parity, stop bits)

and cable runs are conform to RS232 specification in terms of screening,

length etc, I have NEVER seen a transmission error, (am I just lucky ?),

and so don't consider this to be of great concern.

�������������������������������������������������������������������������Ŀ

�  Examples of ComSetUp                                                   �

�������������������������������������������������������������������������Ĵ

�  Eg: Ok3% = ComSetUp% (3, -1, 4, -1, -1, 0)                             �

�      Ok4% = ComSetUp% (4, -1 ,3, -1, -1, 0)                             �

�                                                                         �

�  COM3/4 to use standard BIOS card addresses with IRQ lines 4 and 3.     �

�  Use the tilde character as error substitute, 10 seconds as the time    �

�  out and clear all inhibit flags, allowing UART programming and the     �

�  use of 16550 buffers (if detected).                                    �

�������������������������������������������������������������������������Ĵ

�  Ok3% = ComSetUp% (3, &H300, 15, 0, 91, 2)                              �

�                                                                         �

�  COM3 to use prototype board at &H300, IRQ 15, dump errored             �

�  characters, allow a maximum flow restraint time of 91 ticks            �

�  (approx 5 seconds), and inhibit the use of 16550 buffers               �

�������������������������������������������������������������������������Ĵ

�  Ok1% = ComSetUp% (1, -1, -1, 255, -1, 1)                               �

�                                                                         �

�  COM1 to use standard BIOS card address, default IRQ, CHR$(255)         �

�  as error substitute character, default time out of 10 seconds,         �

�  and no programming of UART, hook vector only.                          �

�                                                                         �

���������������������������������������������������������������������������

��������������������������������������������������������������������������Ŀ

� DECLARE FUNCTION ComOpen% (Port%, Baud&, Word$, Hs%)                     �

����������������������������������������������������������������������������

Returns port selected if successful or 0 if not.

Port%       1,2,3,4

Baud&       110 - 115,200 bps (0 to leave as set)

Word$       "LPS"  ("" to leave as set)

L = 5,6,7,8  (Number of data bits)

P = N,E,O    (Parity, none, even, odd)

S = 1,2      (Number of stop bits, 1.5 stop bits will be used

if 2 are selected with a 5 bit data length.)

Hs%         0,1,2,3      (Handshaking is enabled on both receive and

transmit)

0 = no handshaking

1 = XON/XOFF

2 = RTS/CTS

3 = Both 1 and 2

OpenCom will accept any baud rate as long as 115,200 is EXACTLY

divisable by the rate selected. The only rate which doesn't conform to

this principle is 110bps, which will also be accepted.

Both transmission and reception of data will be controlled by the

handshaking system selected. Flow control will be asserted if the input

buffer has only 100 free bytes remaining, and will be released when 50%

free.

Only use XON/XOFF handshaking for non binary operations. The characters

used for software flow control are CHR$(17) and CHR$(19) (XON/XOFF).

Reception of these during a transmission indicates that no further data

is to be sent, or further data can now be sent, hence files containing

these characters would not be transmitted correctly, as flow control

characters are discarded.

Eg  OpenedOk% = ComOpen% (3, 9600, "8N1", 2)

Open port 3 at 9600 bps, 8 data, no parity, 1 stop, using RTS/CTS

handshaking. OpenedOk% will = 0 if open failed, and ComError% will

indicate the nature of the problem.

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComSetPort% (Port%)                                    �

����������������������������������������������������������������������������

Port%       1,2,3,4

This function sets the port number for all the subsequent Com

operations listed below, as the following functions and subroutines

do not specify a port to use, thereby minimising coding overhead when

calling the routines, and increasing speed of response.

It acts as a switch between all the ports which are open. If there's

only one port open, then it's use is not necessary.

The current port is initially set to the port used for the last ComOpen

function above.

Returns port selected if successful or 0 if not.

Eg  Port% = ComSetPort% (2)                     ' Make current port COM2

CharsSent% = ComSend% ("Output via Com2")   ' Tx via COM2

Port% = ComSetPort% (4)                     ' Make current port COM4

WHILE ComStat%                              ' Retrieve and

PRINT CHR$(ComGet%);                      '   print the buffered

WEND                                        '     contents from COM4

�������������������������������������������������������������������������Ŀ

� ���������������������������������������������������������������������Ŀ �

� �       ALL OF THE FOLLOWING FUNCTIONS APPLY ONLY TO THE PORT         � �

� �           CURRENTLY SELECTED BY THE FUNCTIONS ABOVE.                � �                              �

� ����������������������������������������������������������������������� �

���������������������������������������������������������������������������

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComSend% (Tx$)                                         �

����������������������������������������������������������������������������

This function transmits a string and returns the number of characters

actually sent. A transmit buffer is not implemented as interrupts from

the UART are used for data reception only due to the possibility of data

loss during periods of intense activity which can be generated when using

4 serial devices communicating at speed.

When QuadCOM opens a port, incoming data causes an interrupt which enables

the CPU to shelve what it's doing and get the character from the

8250/16450/16550 UART before the next one arrives, and place it in the

buffer. If the transmitter holding register becomes free at the same time

as a character arrives at the port, and THR empty interrupts are enabled,

as the former interrupt has priority over the latter, it may be lost causing

data loss, hence QuadCOM uses a polling technique for transmission,

waiting for the THR to empty rather than allowing the CPU to return to

the program before all the characters of a string are transmitted. I

very much doubt whether you would notice the difference in program speed

if transmission buffers WERE used.

QuadCOM checks whether the UART for the port is a 16550A, and if so, uses

the 16550A's ability to buffer Rx/Tx data.

The string is transmitted as is, CR & LF are NOT appended to the string.

Eg:

To send a string followed by a carriage return and line feed :-

CrLf$ = CHR$(13) + CHR$(10)

A% = ComSend% ("This is a test" + CrLf$)

or

A% = ComSend% ("This is a test" + CHR$(13) + CHR$(10))

This routine will fail if flow control exerted by the receiver is not released

within the time set by ComSetUp or the default of 10 seconds.

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComPeek% ()                                            �

����������������������������������������������������������������������������

Retrieves the next character to be read from the input buffer and returns

it's ASCII value or -1 if the buffer is empty. This function does not

remove the character from the buffer, thereby allowing a single

character look ahead.

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComGet% ()                                             �

����������������������������������������������������������������������������

Removes one character from the input buffer and returns it's ASCII value

or -1 if the buffer is empty. This is the only means of removing

characters from the buffer, there is no equivalent to BASIC's INPUT$

etc, strings have to be built up.

Eg:     IF ComStat% THEN Char$ = CHR$(ComGet%)

FUNCTION ComInput$ (Port%)                       ' Input a string

'   terminated by CR

Build$ = ""                                      ' Empty target string

'

IF ComSetPort% (Port%) THEN                      ' Ensure port is OK

DO                                             '

IF ComStat% THEN                             ' Buffer not empty ?

IF ComPeek% <> 13 THEN                     ' Is next character CR

Build$ = Build$ + CHR$(ComGet%)          ' No, accumulate string

ELSE                                       '

Discard% = ComGet%                       ' Yes, discard CR

IF ComPeek% = 10 THEN Discard% = ComGet% '   and LF if present

EXIT DO                                  '    and exit

END IF                                     '

END IF                                       '

LOOP                                           '

END IF                                           '

'

ComInput$ = Build$                               ' Return string

END FUNCTION

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComStat% ()                                            �

����������������������������������������������������������������������������

Returns number of characters in the input buffer

Eg:     IF ComStat% = 0 THEN

PRINT "Buffer is empty"

ELSE

PRINT "Buffer contains "ComStat%" Characters"

END IF

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComFree% ()                                            �

����������������������������������������������������������������������������

Returns number of free spaces in the input buffer

Eg:     FreeSpace% = ComFree%

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComDCD% ()                                             �

����������������������������������������������������������������������������

-1 if DCD of COM port is high, 0 if low

Eg:     IF NOT ComDCD% THEN PRINT "Carrier Lost"

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComRING% ()                                            �

����������������������������������������������������������������������������

-1 if RING indicator of COM port is high, 0 if low

Eg:     IF ComRING% THEN PRINT "The phone is ringing"

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComDSR% ()                                             �

����������������������������������������������������������������������������

-1 if DSR of COM port is high, 0 if low

Eg:     IF NOT ComDSR% THEN PRINT "Turn that modem back on"

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComCTS% ()                                             �

����������������������������������������������������������������������������

-1 if CTS of COM port is high, 0 if low

Eg:     IF NOT ComCTS% THEN PRINT "The device is not ready to receive"

��������������������������������������������������������������������������Ŀ

�  DECLARE SUB ComClose                                                    �

����������������������������������������������������������������������������

Unhooks the COM interrupt vector and disables the COM irq (if COM chip

programming has not been disabled by opening ports 5 - 8).

You MUST call this SUB if you have used ComOpen, as interrupts have been

enabled and vectors changed which must be replaced. If you don't then

machine will eventually crash.

Ok% = ComSetPort% (PortToBeClosed%)     ' if not current port

Eg:     CALL ComClose

��������������������������������������������������������������������������Ŀ

�  DECLARE SUB ComPurge ()                                                 �

����������������������������������������������������������������������������

Clears the input buffer and releases any flow restraint

Eg:     CALL ComPurge

��������������������������������������������������������������������������Ŀ

�  DECLARE SUB ComSetDTR (HighOrLow%)                                      �

����������������������������������������������������������������������������

Asserts or drops DTR according to whether HighOrLow% is non zero

Eg:     CALL ComSetDTR (0)      'Drop DTR

CALL ComSetDTR (1)      'Assert DTR

��������������������������������������������������������������������������Ŀ

�  DECLARE SUB ComSetRTS (HighOrLow%)                                      �

����������������������������������������������������������������������������

Asserts or drops RTS according to whether HighOrLow% is non zero

Eg:     CALL ComSetRTS (0)      'Drop RTS

CALL ComSetRTS (1)      'Assert RTS

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComBaudSet% (Baud&)                                    �

����������������������������������������������������������������������������

Baud&       110 - 115,200 bps

Returns -1 if successful or 0 if not.

Used to adjust the baud rate after opening the com port.

Eg:     IF INSTR( ModemReply$, "CONNECT 9600") THEN Ok% = ComBaudSet% (9600)

��������������������������������������������������������������������������Ŀ

�  DECLARE SUB ComBuffer (Seg&, Os&, Length&, RdPtr&, WrPtr&, Wrap&)       �

����������������������������������������������������������������������������

Returns data about the communications buffer for the currently selected

port.

Seg&      Buffer memory segment

Os&       Buffer memory offset

Length&   Buffer length

RdPtr&    Current read pointer position within buffer

WrPtr&    Current write pointer position within buffer

Wrap&     Number of times the buffer write pointer has wrapped

back to the beginning of the circular buffer.

These statistics allow calculations of data throughput to be performed,

and can be used to search for previously received data, (restricted by

buffer size).

Eg:

' Calculate the total number of characters received since the port was

' opened. (Closing the port or purging the buffer will reset the

' read/write pointers)

CALL ComBuffer (Seg&, Os&, Length&, RdPtr&, WrPtr&, Wrap&)

CharsReceived& = (WrPtr& - Os&) + (Wrap& * Length&)

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComError%                                              �

����������������������������������������������������������������������������

Returns last error/warning number generated by the serial system, and

resets the error number to zero.

1    Bad port number

2    Port number in use

3    No Com card found

4    Bad baud rate (110 or 115,200/rate = whole number)

5    Bad word string length (0 or 3 characters only)

6    Bad data bits (5 - 8 only)

7    Bad parity (N,E,O only)

8    Bad stop bits (1 or 2 only)

9    Bad handshake selection (0 - 3 only)

10   Bad IRQ selection (0 - 15 only)

11   Receive error (overrun, parity, frame errors or break condition)

12   Buffer full - no handshake used

13   Buffer filling, flow restraint asserted (if selected)

14   Buffer empty

15   Flow restraint released

16   THR did not become available within time out period during transmit

17   Flow restraint not released by receiver during transmit

18   XOFF received

19   XON received

��������������������������������������������������������������������������Ŀ

�  DECLARE FUNCTION ComUart%(Port%)                                        �

����������������������������������������������������������������������������

This function tries to identify the UART for the port requested, using

the BIOS card addresses.

Eg:     UartId% = ComUart% (1)  ' Get UART ID number for port 1

UartId% = 0     No card found for this port

1     8250    Bugs in operation, XT/PC BIOS adjusted to cope

2     16450   Faster version of 8250, bugs corrected

3     16550   Fast 16450, design problem, dodgy FIFO's

4     16550A  Fast 16450 with operational FIFO's

8250    Original chip for the PC serial port. Had a few bugs for which

the PC and XT BIOS had to be adjusted to compensate.

8250A   Upgrade chip without the bugs. Won't work in a PC or XT as it

expects and corrects for the bugs. Will work in an AT but not

fast enough.

8250B   Last 8250, with a bug PUT BACK IN to allow it to work in the PC

and XT. Doesn't work properly in AT.

16450   High speed 8250 without the bugs.

16550   Improved 16450, but design fault renders FIFO buffers non

viable.

16550A  Fixed 16550. FIFO buffers work, fast, essential for multitasking

operating systems.

��������������������������������������������������������������������������Ŀ

� How to modify a twin serial card to use other IRQ lines.                 �

��������������������������������������������������������������������������Ĵ

�  WARNING - you do this at your own risk. The author assumes NO           �

�  responsibility whatsoever for mistakes within the text, or for problems �

�  arising from following these instructions. The following processes      �

�  assume a passing knowledge of PC architecture and the ability to do a   �

�  little fine soldering.                                                  �

�                                                                          �

�          UNLESS YOU'RE CONFIDENT, BOLD OR MAD  DON'T DO IT !!!!          �

����������������������������������������������������������������������������

Twin serial cards are usually 8 bit cards, which sit in an expansion

slot on the PC motherboard, allowing you to set up each port as COM

1,2,3 or 4 (but not both the same), and allowing the selection of IRQ 3

or IRQ 4 for each port. An example of such a board is that supplied by

Watford Electronics - twin serial card - 8ukp. The cards have edge

connectors, the fingers of which make contact with springy connectors in

the expansion slots. The connecting fingers down one side of the board

have a totally different function from the fingers on the other side and

are electrically isolated, so DON'T think that A1 and B1 in the diagram

below, are the same, COS THEY AINT !

For example - to change IRQ 3 to IRQ 10 (AT class machine)

�Ŀ            Serial card

������������������������������������Ŀ

�                                    �

Rear of PC   �        Track side of board         �

�      (Chips on the other side)     �

�                                    � 0   Stripboard   1

�    0        1         2         3  � 1                8

� �Ŀ1        0         0         0  �������������������Ŀ

��� �|||||||||||||||||||||||||||||||���||||||||||||||||||�

�����           �����

IRQ 7 ��ٳ���           ������ IRQ 14

6 ���ٳ��           ������     15

5 ����ٳ�           ������     12

4 �����ٳ           ������     11

3 �������           ������     10

Rear of

computer

AT expansion slot on PC motherboard

�  � Blanking

�  � Plate

�  �   �    A1                           A31  C1              C18

� �ſ  �   ����������������������������������������������������Ŀ

� ���<��   �|||||||||||||||||||||||||||||||   ||||||||||||||||||�

� ���      �|||||||||||||||||||||||||||||||   ||||||||||||||||||�

� ���      ������������������������������������������������������

�  �        B1       (8 bit slot)        B31  D1   (16 bit)   D18

�  �

�  �

REFERRING TO THE DIAGRAM :-

To connect to IRQ line 10 (D3), cut a piece of stripboard to length so

that it can be inserted into the 16 bit AT extension slot (standard

stripboard is 0.1 inch spaced, same as expansion slots). Solder a short

piece of insulated wire (approx 4 inches) to the IRQ 10 strip on the

board. Do the same for IRQ 11 if required.

On the serial card, there will be 2 tracks which travel from positions

B24 and B25 to a jumper block which allows IRQ selection, or directly to

chips if no selection is permitted.

In the first case where selection is permitted :-

������������������������������������������������������������������������Ŀ

�

�

�

�

COMA COM B                                           �

^   ^ To chips                                     �

�   �                                              �

�   �                                              �

1  0-0 0-0  4  Jumper block                           �

���Ŀ       (on other side of card)                �

5  0 0 0 0  8                                         �

� ���Ĵ                                            �

�     �������������Ŀ                              �

���������������Ŀ   �                              �

�   �                              �

�   �                              �

�� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �� �����

� � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � � �

Ĵ �Ĵ �Ĵ �Ĵ �Ĵ �Ĵ �Ĵ �Ĵ �Ĵ �Ĵ �Ĵ �Ĵ �Ĵ �Ĵ �Ĵ �Ĵ �Ĵ ���

��� ��� ��� ��� ��� ��� ��� ��� ��� ��� ��� ��� ��� ��� ��� ��� ���

B24 B25                     B31

The 0's represent the jumper pins on the board. Counting from the left,

if pin 1 is jumpered to pin 5, then COM A will use IRQ 4. However if we

solder the other end of the lead that is connected to IRQ10 to a jumper

and attatch that to pin 1, or 2, or both, and leave pins 5 and 6 out of

it, then COM A will use IRQ 10, get it ? If not then I don't think you

should really be reading this :-) Using this method, you don't even need

to modify the board.

If selection isn't permitted, then you have to solder the wire from

IRQ10 to the top of the finger for IRQ3 and cover the IRQ3 finger on the

board with tape, covering only the track side of the finger, so that

when the card is inserted, the masked off finger(s) do not make

electrical contact with the connector(s).

Insert the strip board so that the leads soldered to the IRQ 10 & 11

strips are in full contact with the IRQ 10 & 11 connectors in the

expansion slot (D3 and D4). Make sure that you don't bridge any

connectors. Insert the serial card into the 8 bit slot and away you go.

A similar method would be used on a 16 bit board, except there is no

need for the strip board.