MODEM BATCH RESET

Version 1.00

(C)opyright 1997 by Carlton Griffin

All Rights Reserved

--- INTRODUCTION

Modem Batch Reset is an assembly of EXEs, batch commands and ideas in

effort to make sure your modem always gets reset after every caller.  I've

owned several kinds of modems and they all seem to get "hung up" sometimes

where nothing but a hard reset of the computer will get them to accept

commands again.

The included batch and exe files should work well with any BBS or other

modem communications program that can run a DOS batch file.  By studying

and implementing the ideas based in this document and example batch file,

you should be able to get your modem to automatically reset 99.9% of the

time.  Under normal conditions (no modem lockup), this process will only

take about 1 second to complete.

--- THEORY OF OPERATION

NON-DOS AND OTHER APPLICATIONS

--- EXAMPLE BATCH FILE STUDY

BATCH FILE NOTES

DTR

FOSSIL DRIVERS

USING FOSSIL DRIVERS BESIDES X00

USING NO FOSSIL DRIVER

--- INSTALLING BATRESET.BAT

--- TESTING BATRESET.BAT

TEST.BAT

TEST.BAT USAGE

--- MULTINODE OPERATIONS

--- THE "OUT OF TOWN" NO FAIL MODEM RESET SOLUTION

--- THE AUTHOR

--- THEORY OF OPERATION

Let me start with a quick tip, end your modem's init string with H, the

universal command to hang up the modem.  This way, each time your BBS

program re-inits the modem, it tries to hang it up.

The batch file takes advantage of several things:

COMSET.EXE  Version 12.0S by Hank Volpe

ASKMODEM.EXE Version 2.2 by Mark Rogers

AT.COM   (c)1985 by Donavon Kuhn and Jon Niedfeldt

DTR.COM  (c)1985 by Donavon Kuhn and Jon Niedfeldt

Dos ECHO commands

Your Fossil driver's possible utilities and commands

To use BATRESET.BAT, you must have your bbs program run the batreset.bat

program after every caller, before it goes back to the Waiting For Call

(WFC) screen.  Many BBSs that do not run a front end mailer do not exit

back to dos after each caller, but most can be configured so that they

will.  For many DOS bbs programs, the start batch file for your bbs would

look something like this:

:BEGIN

cd\bbs

bbs.exe

if errorlevel 255 goto end

call batreset.bat

goto begin

:END

In this example, BBS.EXE exits with errorlevel 255 when you press QUIT,

otherwise batreset.bat runs and then the bbs starts back up again.

Consult your bbs documentation for it's start up parameters & configuration

settings.

For sysops that are running a front end, simply have your existing batch

file call batreset.bat just before it starts your front end in the waiting

for call mode.

Also you should consider running BATRESET.BAT from your autoexec.bat file,

just be sure you don't get it caught in a resetting loop, more on that

later.

NON-DOS AND OTHER APPLICATIONS

Even if you do not normally run in a dos environment, try running

BATRESET.BAT as a dos door before you go back to the Waiting For Call

Screen.  At least some of the commands used in BATRESET.BAT may be

successful with your application.  If you're having modem reset problems,

its worth a try.

--- EXAMPLE BATCH FILE STUDY

BATRESET.BAT:

:start

;;; MC=Modem Com port, MP=Modem Port # (for fossils),

;;; MI=Modem Irq, MS=Modem com port Speed

set mc=3

set mp=2

set mi=5

set ms=57600

:Stage1

;;; first attempt to reset modem

xu dtr:%mp:on

;;; Fossil Driver X00's attempt to get modem's attention

dtr.com com%mc: on

;;; Dos program DTR.COM's attempt to get modem's attention

askmodem /p%mc /i%mi /b%ms /t3 atz

;;; Command line for ASKMODEM to send an ATZ, the internal command that

;;; makes all modems reset themselves.

if errorlevel 1 goto Stage2

goto end

;;; If Askmodem failed, goto stage2, otherwise exit.

:Stage2

xu dtr:%mp:off

xu dtr:%mp:on

;;; Attempt to toggle the DTR signal line off and back on again using

;;; X00's XU utility

dtr.com com%mc: off

dtr.com com%mc: on

;;; Attempt to toggle the DTR signal line off and back on again using the

;;; dos program DTR.COM

at com%mc: +++

at com%mc: s0=0h1

;;; Attempt to send the escape sequence +++ and the command to stop

;;; answering all calls and hang up immediately, using AT.COM

xu dtr:%mp:off

xu dtr:%mp:on

;;; Have the fossil driver continue to nag the modem with DTR toggles

dtr.com com%mc: off

dtr.com com%mc: on

;;; Keep nagging the modem with DTR toggles

echo +++>com%mc

echo ats0=0h0>com%mc

;;; Attempt to send the escape sequence +++ and the command to stop

;;; answering all calls and hang up immediately, using the dos echo command

askmodem /p%mc /i%mi /b%ms /t1 +++

askmodem /p%mc /i%mi /b%ms /t10 atz

;;; Have Askmodem attempt to send the escape sequence and have the modem

;;; reset itself.

if errorlevel 1 goto Stage3

goto end

;;; Exit if Askmodem was successful, otherwise continue to Stage3

:Stage3

dtr.com com%mc: off

dtr.com com%mc: on

;;; Keep nagging the modem with DTR toggles

xu dtr:%mp:off

xu dtr:%mp:on

;;; Have the fossil driver continue to nag the modem with DTR toggles

comset %ms %mc,%mi

;;; Haul out the big guns, and have COMSET.EXE attempt to reset the modem.

;;; This program can take up to 60 seconds to execute, but if all the above

;;; didn't reset the modem already, its time to let COMSET try.

if errorlevel 1 goto Stage4

goto end

;;; If even COMSET was unsuccessful, continue to Stage4, otherwise exit.

:Stage4

COLDBOOT.COM /B

;;; Reset the entire computer in hopes of resetting the modem.

:end

set mc=

set mp=

set mi=

set ms=

;;; Clear out all variables previously set in your environment.

---

BATCH FILE NOTES

As you can see, BATRESET.BAT tries several different approaches to

resetting your modem, and tries many of them more than one time.  This is

important.  Most of the commands take only a fraction of a second each, so

running them all isn't a problem and the more times you try, the better

your success will be.

RESETTING YOUR COMPUTER

You may want to reconsider resetting your computer at all.  (Stage4)

I run a multinode BBS, therefore I don't use this method myself.  For

single node BBSs, it would be a good idea, however most resets you can

achieve from a batch file won't reset your modem(s).  If you do choose to

reset your computer after all else fails, don't run this from Autoexec.bat

or your computer could get stuck in an endless resetting loop.

DTR is Data Terminal Ready.  DTR is both a Serial line and a Serial device

command.  This hardware command tells any serial device, such as an

attached or installed modem, to reset itself and become ready for incoming

data and commands.  By turning this on you can hope to communicate with the

modem.  By toggling it off and back on, you can attempt to reset the modem.

DTR can be turned on/off by programs such as the included DTR.COM, by some

Fossil drivers, and also by communications programs such as your BBS.

FOSSIL DRIVERS are small program that remain resident in memory after being

loaded.  These programs deal directly with the modem and act as a middle

man between your modem and your BBS program.  If you are using any kind of

Fossil Driver, check its documentation for any commands or utilities that

may be included with it that will attempt to toggle the DTR, such as X00's

external utility XU does with the commands:

xu dtr:%mp:off

xu dtr:%mp:on

USING FOSSIL DRIVERS BESIDES X00

Replace your fossil driver's commands to turn the DTR on and off with the

above commands as they occur in BATRESET.BAT.  If you can find no such

command in the fossil's documentation, remove all the command lines in

BATRESET.BAT that begin with "xu".

Note: The Fossil driver BNU has no DTR commands.

USING NO FOSSIL DRIVER

If you use no Fossil Driver, remove all the command lines in BATRESET.BAT

that begin with "xu".

Note, if you still have trouble with your modem even after trying

everything in this documentation, consider running a fossil driver.  Even

if your BBS isn't made to work with a fossil, having one installed may not

interfere with normal operation and could, conceivably, help your modem

get reset properly.  Install a fossil when its not needed only as a last

resort.

--- INSTALLING BATRESET.BAT

BATRESET.BAT must be custom installed for each application.  By reading the

above documentation you should be familiar with what you now need to do.

You should unzip the BATRESET.ZIP file into a convenient directory, the

example used in the batch file is C:\BBS\BATRESET.  Use whatever directory

you like, but be sure to modify the CD command at the beginning of

BATRESET.BAT.  Unzip COMSET.EXE from CONST12.ZIP and ASKMODEM.EXE from

ASKMOD22.ZIP.  Fill in your correct COM PORT, PORT #, IRQ, and Com Port

Speed in the beginning section of BATRESET.BAT.

COM PORT   FOSSIL PORT   DEFAULT IRQ

1          0             4

2          1             3

3          2             4

4          3             3

Your IRQ may vary, but if it does you probably know about it.  In other

words, if you're not sure what IRQ you're using, use the default value

shown above.

--- TESTING BATRESET.BAT

Not all modems respond to all the commands used in BATRESET.BAT.

In fact some of the commands are not compatible with some modems.

Modify and install BATRESET.BAT as directed in the above documentation and

observe what happens when your bbs program attempts to init the modem.

If, after modifying and installing BATRESET.BAT your modem no longer

responds, you need to find out which commands in BATRESET.BAT are

incompatible with your modem using the TEST.BAT file.

TEST.BAT

:start

set mc=3

set mp=2

set mi=5

set ms=57600

if "%1"=="1" goto 1

if "%1"=="2" goto 2

if "%1"=="3" goto 3

if "%1"=="4" goto 4

if "%1"=="5" goto 5

:1

ECHO TESTING ASKMODEM.EXE

askmodem /p%mc /i%mi /b%ms /t3 atz

PAUSE

goto end

:2

ECHO TESTING DTR.COM/AT.COM

dtr.com com%mc: off

dtr.com com%mc: on

at com%mc: +++

at com%mc: s0=0h1

dtr.com com%mc: off

dtr.com com%mc: on

PAUSE

goto end

:3

ECHO TESTING DOS ECHO

echo +++>com%mc

echo ats0=0h0>com%mc

PAUSE

goto end

:4

ECHO TESTING COMSET.EXE

comset %ms %mc,%mi

PAUSE

goto end

:5

ECHO TESTING X00 FOSSIL COMMANDS

REM replace with your own fossil commands for testing

xu dtr:%mp:off

xu dtr:%mp:on

PAUSE

goto end

:end

set mc=

set mp=

set mi=

set ms=

TEST.BAT USAGE

If, after installing BATRESET.BAT, your modem no longer responds, you need

to reset your computer by pushing RESET or powering down, then on again

after 30 seconds.  CTRL-ATL-DEL will almost never reset your modem, you

need a HARD (or COLD) reset to ensure your modem is reset.  Then replace

the command

CALL BATRESET.BAT

in your bbs batch file with the command

CALL TEST.BAT 1

Now run your batch program that starts your bbs and verify whether or not

your modem inits properly.  Then exit your bbs program and replace 1 with

2, and do it all over again.  Do this with 3, 4, and 5.  As you can see

from the TEST.BAT file, the numbers 1-5 correspond to the resetting

commands used in BATRESET.  Remove any of the commands that are found to

be incompatible with your system from the BATRESET.BAT file.  Leave the

rest and hopefully this will still do the job just fine.  Not all the

commands are required, but the more you can use, the better your chance of

getting the modem reset.  Remember your ultimate goal is to keep your bbs

up and running.

ASKMODEM and COMSET should work with all systems, but that's no guarantee.

The COM programs AT and DTR have been known to be incompatible with some

modems, but I've included them because I've seen other modems that would

respond to nothing else.  Even the DOS ECHO commands might not work well

with all modems.  By using the TEST.BAT to individually test each resetting

command, you should be able to determine which ones can be used with your

system.  Your first configuration of BATRESET.BAT may work beautifully, but

even if you have to take the time to run all five TEST.BAT tests and

experiment to find out which commands will help you, its well worth the

effort if you've been having trouble with your modem not responding.

--- MULTINODE OPERATIONS

For multinode BBSs, special considerations need to be taken.

The example batch file for multinode operations (BATCH-2.BAT) takes one

parameter, the node of the bbs you wish to reset.  Inside the batch file

you must set the com port, port #, and irq for each node.  Also, in the

example batch file, stage4, where the entire computer is reset, is removed.

For my BBS, I'd rather not reset the entire computer just because one line

is down.  Use your judgement as to whether or not you wish to reset if all

else fails.

--- THE "OUT OF TOWN" NO FAIL MODEM RESET SOLUTION

If you must leave your BBS unattended for a few days, there is one way to

insure your modem and your computer gets completely reset at least once a

day.  I have one of those lamp timers that you can use to make people think

you are at home by turning the lights on and off automatically.  Get a

heavy duty one that has a three prong plug so your computer will still have

ground protection.  Set the timer to be on at all times, but to turn off for

the minimum amount of time during the middle of the night.  The minimum off

time on my timer is one hour, so when I leave town I have it turn the

computer off from 4-5am and then the system will get at least one good,

hard reset per day.

--- THE AUTHOR

I wrote this file because it took me a long time to resolve my modem

resetting problems.  I searched high and low for modem resetting programs

that aren't crippled with pauses.  PRESS ANY KEY TO CONTINUE really puts a

cramp on an automated modem resetting utility.  The truth is there are

other, better, modem resetting dos utilities if you want to PAY for them.

Most sysops have little extra money for such things, so I share with you

what I have been able to find.  This document file and example batch

programs are distributed as copyrighted THANKware.  Please distribute this

package freely.  Do not modify this package or it's included files.  If you

receive benefit from the time I have taken to write this file, please write

to me and thank me so I'll know my time has not been wasted.  It brightens

my day to know that I have helped people, please write me if I have.

Please contact me if you have any good modem utilities to share!

Good luck with your bbs.

Carlton Griffin

Gangster's Hideaway 770-307-0331

71053.1043@Compuserve.com