- THE OUTLAW TRIAD DEMO-SERIES -

�������������������������������� PART IV ������������������������������������

Written by : Vulture/OT

Code in    : Assembler

Topic      : High resolution

������������������������������ Introduction ���������������������������������

Welcome to the Outlaw Triad demo-series! In these series we will be talking

about programming demo-effects in either pascal or assembler. Theory behind

the effects shall be discussed while a full sourcecode is also provided.

In this fourth release we will discuss the basics of high resolution modes

like 640*480*16. We will make simple stringwriter for use in these modes.

A full sourcecode in assember is included. It's a part of the sourcecode of

a BBS intro coded for Magicware. The Outlaw Triad Italian HQ. Enjoy!

��������������������������������� Theory ������������������������������������

Ok, I will not waste much time explaining how a stringwriter works since

these routines are fairly easy to make. However, I will spend a few bytes

on it. I mean, this still is a trainer, right? ;-)

To show some text on the screen, you will have to state the text you

want. What we want is a label representing the text to show. Like this:

Label Text Byte

DB   '   the text you want to show   ',0

The major advantage of doing it this way is the fact that you can change

the text very easily. Another way I've seen in some intro sourcecodes is

stating the numeric values of the text, or something like that. A rather

odd way of doing things, IMHO.

Anyway, the "0" at the end of the string means that the text has come to

an end there. We will take care of that in our stringwriter routine.

Ok, now how to we show this text? That's the main question here. First, we

need a font to work with. In most small intros, the fontdata is organised

in a lineair way. For example, you could scan a pcxfile from left to right,

top to bottom and save the bytes to a constant. Ok, you also need to know

the x,y size of the font. We need them for calculating the exact starting

position in the fontdata of the specific character we want to draw. Next

thing you need is the character order. An example:

Label Order Byte

DB   'abcdefghijklmnopqrstuvwxyz!?.,()":+-'

Let's take a look at an example of how to locate a character in the data.

I assume the data is organised in a lineair way like a stated above. Also,

let's say the x and y size are both 16. If we wanted to locate the letter

'g', we would add (6*16) to the offset (start) of the fontdata. Why? Take

a look: The letter 'g' is the seventh letter in the fontorder. We want to

locate the upperleft corner (!) of that letter. So, by adding (6*16) to the

offset we are exactly where we want to be! We have simply skipped the first

6 characters (that is, the first scanline of those characters). Ok, when we

have located that particular position, we can draw the char. You could do

something like this:

- Go into a loop where you:

- Draw 16 pixels

- Update pointer to fontdata (remember: fontdata is lineair)

- Update vga position

Do this 16 times (y was 16 here) and you're done. Ok, I don't want to waste

much more space explaining this so here are the steps you could take to show

some text:                                                ~~~~~

1 - Load a character from the text

2 - Do appropiate action (in case of "0")

3 - Reset counter

4 - Go into a loop:

a - Load a character from order

b - Compare with the character from the text

c - If not the same, increase counter, back to A

5 - They were the same, now use the counter to find

the position of the character in the fontdata

(The way this is done depends on how the data is

organised. Check out the source to see what I mean)

6 - Draw the character

7 - Do next one

Well, and that's it! You can use these steps in any resolution you want.

In our case, high resolution 640*480*16. Let's take a closer look at this

particular mode, shall we?

Each byte in high resolution vgamemory represents 8 neighboring horizontal

pixels! In a byte are 8 bits, so each bit is 1 pixel on the screen! Quite

a difference compared to standard mode 13h where 1 byte represented one

pixel on the screen. Ok, since you are working with groups of 8 horizontal

pixels at a time, the logical screenwidth is now 80. Think about it! The

screenwidth in pixels is 640. Because 1 byte represents 8 horizontal pixels,

the logical screenwidth is 640/8=80. You use the logical screenwidth when

drawing a character on the screen.

When you write a byte to the screen, you can select the bits (pixels) you

want by writing the byte into the Bit Mask Register. This is index 08h of

the Graphic Controller. All enabled bits in this register will be written

to the screen and all not-enabled bits will not be written to the screen!

So, with this register you could write a PutPixel routine. Anyway, typical

code to write a byte (a pattern of 8 horizontal pixels) to the screen can

look something like:

mov   dx,03ceh       ; Graphic Controller

mov   ah,ds:[si]     ; The byte to write (pointed to by ds:si)

mov   al,08h         ; Bit Mask Register

out   dx,ax

movsb                ; From data to vga (es:di points to vga)

This is exactly what I did in the sourcecode. Note when using the Bit Mask

Register, all pixels you write to the vga will have the same color. If you

want them to have different colors you could use a PutPixel procedure in

which you set the color for the dot to write. Ok, you would like to have

a PutPixel routine for use in this mode, right? Well, I could give you one

but I won't. Instead, I will give you some hints on how to code your own.

- Set color of dot

- Multiply Y with 80

- Divide X with 8

- Y=Y+X

- Set correct bit (using Bit Mask register => X and 7)

- Read dummy byte from vga (loading latches)

- Write byte to screen

There you go! It shouldn't be hard to write a PutPixel routine now. If you

don't succeed, please mail me. I will give you my own routine instead.

Ok, one more thing to discuss and that is setting the color for a dot. Like

I stated before, you should load the byte you want to write to the screen

into the Bit Mask Register before you write it to the screen. All enabled

bits will be written to the screen with the same data as presented in the

Set/Reset Register! With this, we can set the color of the dot we want to

write to the screen. The Set/Reset Register is 4 bits width so this leaves

us with a maximum number of 16 colors. Colorsetting code in assembler:

mov     dx,3ceh           ; Graphic Controller address register port

mov     ax,0f01h          ; Index 01h  (enable set/reset register)

out     dx,ax             ; Enable all 4 bits to be altered

xor     al,al             ; Index 00h  (set/reset register)

mov     ah,cl             ; Color (0 to 15)

out     dx,ax             ; Set color

There are 2 registers to be altered here. The Enable Set/Reset Register and

the Set/Reset Register. They are both 4 bits in width. If you enable a bit

in the Enable Set/Reset Register, the corresponding bit in the Set/Reset

Register can be altered. And, when you disable a bit in the Enable Set/Reset

Register, the corresponding bit in the Set/Reset Register can't be altered.

So, we set all (4) bits in the Enable Set/Reset Register to 1. This way, all

bits (4) of the Set/Reset Register may be altered. Then, we load the color

into the Set/Reset Register. Any byte (bits) that is written to the screen

will have this color. Why don't you make a sample program to test this code?

It should work just fine. (note: I used this code in my own PutPixel proc).

One advantage of these high resolution modes is that you need just a few

bytes to represent an entire character. Like in this example source, you

only need 2*18=36 bytes. If you want to show a character this size in plain

mode 13h, you would need 16*18=288 bytes. Quite a difference, eh?

(note: this doesn't have to be true. When using a single color font, you

can also store the pixels in bits in mode 13h).

A disadvantage of high resolution mode is that you only have 16 colors to

you disposal.

Now go and take a look at the sourcecode provided in this package. It is

fully documented so there shouldn't be any problems. If you have troubles

after all, e-mail me. I like to hear from you!

Ok, this is all for now. Happy coding!

- Vulture / Outlaw Triad -

������������������������������� Distro Sites ��������������������������������

Call our distros to get all our releases.

BlueNose    World HQ          +31 (0)345-619401

FireHouse   Distrosite        +31 (0)528-274176

The Force   Distrosite        +31 (0)36-5346967    More distros wanted!

MagicWare   Italian HQ        +39  6-52355532

ShockWave   South African HQ  +27 (011)888-6345

���������������������������������� Contact ����������������������������������

Want to contact Outlaw Triad for some reason? You can reach us at our

distrosites in Holland. Or if you have e-mail access, mail us:

Vulture  (coder/pr)     comma400@tem.nhl.nl

Our internet homepage:

http://www.tem.nhl.nl/~comma400/vulture.html

These internet adresses should be valid at least till june 1996.

������������������������������������������������������������������������������

Quote:  Does killing time hurt eternity?