--------------------------

| Doc for VGAColors unit |

--------------------------

The VGAColors unit contains procedures for accessing the 256-color VGA palette.

Here's a brief introduction on how the VGA generates color :

In the 256-color modes on a VGA card, the pixels are represented by one byte,

i.e. a value between 0 and 255.  However,  as opposed to, e.g., textmode, the

colors that correspond to a value aren't fixed.  Instead, the byte value of a

pixel represents an index to a table called the DAC table, which contains the

actual colors.  Each entry of this table contains an RGB triple, which is one

byte for Red, one byte for Green and one byte for Blue (hence RGB).  However,

these bytes are not fully used. Instead, only 6 bits are used, which gives us

a value ranging from 0 to 2^6-1 = 63 per Red, Green, or Blue byte. Hence, the

number of colors you can choose from is 64^3 = 262144. To change a color, you

must change an entry in the DAC table.

In VGAColors,  you can use SetPalette or SetPaletteNoNoise to change the DAC

table. The first parameter is a VGAPalette, the Pascal representation of the

DAC table, i.e. an array of 256 RGB-triples.

The difference between SetPalette and SetPaletteNoNoise is that,  on some cards

SetPalette produces snow, while SetPaletteNoNoise doesn't. Therefor, you should

stick to using SetPaletteNoNoise, although it is slower than SetPalette.

A remark on the Fade effects: these effects (i.e. FadeToPalette, FadeToRGB and

FadeOut) use the Timer unit for timing.  Although the procedures check for the

installation of the timer,  it is recommended that you preset the timer before

calling them, especially if you plan on using mod-players or other units which

modify the timer. Also, if you use the timer unit, you should save TimerTicks

before doing a fade, as the timerticks value is modified. Here's an example:

InitTimer;

{Some actions}

OldTimer := TimerTicks;

FadeOut (1000);

TimerTicks := OldTimer + 1000;

{Some more actions}

DoneTimer;

Refer to the docs for the Timer unit for more information on how to use that

unit.

Finally, something about the border color (overscan color, color 0):

The first entry in the DAC table (entry# 0) contains the color that is used for

(1) all pixels with value 0 (as explained above) and (2) the overscan or border

color.  By changing this color, the border color is changed.