The SuperVGA Test Library

-------------------------

The SuperVGA Test Library is a set of routines for working with SuperVGA

video cards that have a VESA VBE compliant Video BIOS. This library was

an offshoot from the development of the Universal VESA VBE, which is an

installable TSR to provide VESA VBE video BIOS extensions for most

SuperVGA cards. It is intended to show how to program the SuperVGA

cards through the VESA VBE and uses some of the more powerful features

that the latest standard include in your own software. It was not

designed with speed in mind, but more as an explanatory and exploratory

tool.

This library supports many of the extended features of today's SuperVGA

adapters, such as being able to separately set the read and write banks

and support for extended page flipping. This is all done via the

standard VESA VBE programming interface.

NOTE: All source code is formatted with 4 space tabs, including the

assembly language source.

NOTE: This library has '386 specific code in it, so it will _not_ work

with machines without a '286 or lower processor. Some of the code

such as the screen clearing code etc uses fast 32 bit string

instructions to run at full speed on new VESA VLB Local Bus

video cards.

Using it in your own programs

-----------------------------

The SuperVGA test programs all call upon a single C library to access the

SuperVGA using the VESA VBE programming interface. You can use this

same library to provide SuperVGA support in your own applications, or

you can take the source code for the library as a starting point and

expanding on it from there.

As the library was not designed with speed in mind, some of the functions

such as line drawing can be sped up significantly by recoding them in

assembly language.

Assembling the source files

---------------------------

All of the assembler source files have to be assembled with Borland's TASM,

since they are written using TASM's IDEAL mode. If you do not have TASM,

you can simply link with the pre-assembled object files provided.

Test programs

-------------

The library has five test programs, for 16 color, 256 color, 32k color,

64k color and 16 million color SuperVGA video modes. Simply run the programs

and follow the instructions. The code in these test program should give

you an idea of how to use the SuperVGA routines in your own programs.

Note that the test programs require a VESA VBE compliant SuperVGA in

order to run, so if your video card does not have the VESA VBE in the BIOS,

then you will need to install the Universal VESA VBE before running the

test programs.

Paranoid

--------

The SuperVGA kit also included the executable and source code for the

PARANOID.EXE program. This program is a stress test program for the VESA

VBE interface to check the BIOS of your video card (or the installed version

of UniVBE if you are using this) for conformance. It fully tests all the

standard and advanced functions of the VESA VBE 1.2 interface, and will

report any errors that it detects to the PARANOID.LOG file.

Unfortunately many video BIOS'es pass the conformance test, however the results

that are displayed visually on the screen (such as when doing wide virtual

screens and scrolling) are not correct. If this is the case then you can

consider that the BIOS has failed the conformance test. If you have a BIOS

that does not pass the test, you might like to contact your video card's

manufacturer and report the problems to them.

PMODE library

-------------

Before you can build the SuperVGA kit you need to unzip the PMODE library

into the same directory as the SuperVGA kit source code. The PMODE library

is provided with the SuperVGA kit.