COREtest - Disk Performance Test Program v2.92

This program will measure the performance of one or two hard disk

drives.  The evaluation is made using three tests:  data transfer,

random seek, and track-to-track seek.  The program will not destroy any

data on the disk and may be run as many times as desired.

The data transfer test measures the performance of the computer, disk

controller, and drive functioning as a combined unit.  This is

accomplished by reading repeatedly a specific block of data and timing

each transfer.  The resulting effective transfer rate is produced from

the combined relationship of the computer bus structure (8, 16, or 32

bits), the transfer rate of the controller with the drive, the

interleave factor for which the drive was formatted, and the size of the

data block transferred.

The block size is automatically calculated or may be explicitly set from

1 to 64K in 1K increments.  If automatically calculated, the block size

will be set to 64K or the amount of data stored on one cylinder,

whichever is less.  The transfer rate is displayed in units of kilobytes

per second (KB/sec).  This indicates how much data will be moved from

the disk to memory in one second.

The next two tests measure the time required to position the read/write

heads to a specific location where data is stored.  The first of these

tests will measure what is referred to as the "average access time" by

moving the heads across one-third of the cylinders, simulating true

random access.  The final test will measure the time to move the heads

from one track to an adjacent track, measuring the "track-to-track

access time." Both these results are reported in milliseconds (ms =

thousandths of a second).  The smaller the access time, the faster the

data may be located and transferred to the computer.

After the tests are complete, a "performance index" is calculated for

the tested system.  This number measures the overall performance of a

given computer, the disk controller, and its hard disk.  It may be used

to evaluate the performance of different computer systems on a

comparable scale.  For example, a standard IBM 10MB XT system will

typically report an index of one while an IBM 20MB AT will yield an

index of approximately 2.4.  The higher the index, the more powerful the

computer system.  It can generally be said that a computer with a

performance index of two is twice as fast as a computer with an index of

one.

Caching  Information

If COREtest detects that a cache program is installed, it will display

a message indicating same.  The presence of a cache program can mis-

represent the actual drive performance.  When comparing the performance

of disk drives, you should do so without any cache programs installed.

This will give a true representation of the actual drive performance.

Ranking screen

The final screen displays where your drive(s) rank compared to others

in the industry.

Various options are available to change the test parameters to better

compare performance across a wide variety of applications.  These

options must be placed on the command line when program execution is

started.  The format of the command line and the option descriptions are

given below:

CORETEST [?] [HELP] [/B:xx] [/C:xxx] [/D:n[="xxx"]] [/P:xx] [/T:xxx]

Format  Description

------  -----------

/B:xx   Use a block size of xxK.  The block size may be up to

64K.  Default is the largest transfer possible without

moving the disk heads.

/C:xxx  Set maximum seek stroke to xxx.  This is used to set drives to

equal cylinder comparisons. If not specified, then the full

cylinder range of the drive will be used (default).

/D:n[="xxx"]  Test disk number n (n = 0-7).  This parameter may appear

up to two times. If a string is enclosed in quotes, then this

string will be used to identify the drive on the display.  If

omitted, up to the first two drives will be tested.

/P:xx   Pause for xx seconds when test complete (for batch files).

/T:xxx  Perform transfer test for xxx seconds (default = 4 seconds).