NAME

graph - plot a tabulated function

SYNOPSIS

graph [file file file] [options]

DESCRIPTION

Graph takes pairs of points (two numbers per line) from the given

file as x- and y- values and plots them on the screen, connected by

straight lines.  A string surrounded by quotes "..." may follow a

pair of points.  This string is a "point label", and will be

displayed starting at that point.  The program draws axes with

convenient labels (integers times powers of 1000), plots the data,

waits for a keypress, then resets the screen parameters and clears

the screen before exiting.

If no file is specified on the command line, data is read from the

standard input, which may be redirected.  If data is entered from

the keyboard, end of data is signaled by typing <cntl>Z.

In the data, blank lines and lines beginning with ';' are ignored.

More than one input file can be specified.  All the data will

be read before scaling parameters are calculated.  The plot

will be "broken" at the end of each file.  That is, there will

be no line connecting the last point of the first file with the

first point of the second file.  Similarly, if a line starts

with a null byte (that is, if a line feed is followed by a

null) the plot will be broken.  Separate curves may also be

delimited with a point label on the same line as the last point

on the curve, or by a null byte immediately following the last

point (see the -b switch).

The -m switch can be used to select the line style or marker

style for each curve.  If both markers and connecting lines are

wanted, list the data file twice and specify different styles.

For example,

graph gamma gamma -m 1 -30

would give red squares connected by white solid lines on an EGA.

Graph should no longer leave the display in a nonstandard mode.  If it

does, please let me know.  In the mean time, the display can be

restored using the DOS command MODE:

mode co80      or     mode mono

as appropriate.

OPTIONS

Options can appear before, among, or after input file names

provided a file name cannot be mistaken for the parameter of an

option.

&<name>		(hard copy versions only) Device driver switches are

in the configuration file <name>.  See "DEVICE DRIVER

CONFIGURATION FILES" below.

@<name> or -f <name>

More parameters (filenames and/or switches) are in the script

file <name>.  This can be used for lists of parameters

that are too long for a command line, or merely to save

retyping lists of parameters.  The parameters in the

script file have the same format as those on the

command line, but may be separated by linefeeds as well

as spaces or tabs.  More than one script file switch

may appear on the command line, and script files may

refer to other script files to any depth.  All the

script files will be read.  For example, if the file

"curves" contains the line

parabola elipse circle

and the file "options" contains the lines

-g -e -m 20 30 40

then the command

A>graph  @curves  @options

is equivalent to

A>graph parabola elipse circle -g -e -m 20 30 40

In a script file, a semicolon introduces a comment that

extends to the end of the line.

-a  [<step> [<start>]]

Automatically generate abscissas.  The first argument,

if present, is the step between values (default 1).

The second argument, if present, is the starting value

(default 0, or first argument from -x switch).

-b 		break (disconnect) the plot after each label.  (See

switch -m to plot the line segments with different

linestyles.  To break the plot without printing labels,

use empty labels: "".) This option is set automatically

if more than one input file or more than one linestyle

is specified.

-c <text>	argument is default label for data points

-e		equal vertical and horizontal scales (same number of

inches on the display per unit in problem space), so that

circles don't look like elipses

-f <name>	See @<name>.

-g		-g with no arguments eliminates the axes.

-g <style> <tics> <width>

First argument is grid style:

0 = no grid and no numeric labels,

1 = frame with tic marks inside (default),

-1 = frame with tic marks outside the graph area,

2 = full grid

3 = bottom and left axes only

-3 = bottom and left axes, tic marks on outside.

4 = bottom and left axes only, separated from plot

-4 = bottom and left axes, separated from plot,

tic marks on outside.

Second argument is approximate number of small

tic marks per numeric label (1 to 10, default 5).  Set

<tics> to 1 to eliminate small tic marks.  Third

argument is width of grid lines (default 1).

-l <text>	next argument is label for graph (printed at top).  If the

label starts with a double quote it may contain spaces:

-l "three cornered hat"

-m <n1> <n2> <n3>...

numeric arguments are linestyles for successive line

segments (see switch -b).  Each positive argument is a

number of up to three hex digits WCS, where

W is the linewidth (0-9, with 0 giving width 1),

C is the color (0-f, with 0 being brightest), and

S is linestyle (1-5, 1 for solid, 2 for dashed,

3 for dotted, etc.)

If an argument is -1, the data points are plotted as

dots not connected by lines.  In general, a negative

argument -CS generates a marker:

C is the color (0-f, with 0 being brightest), and

S is the marker style (0-9 for box, dot, plus,

asterisk, hexagon, cross, box, diamond,

triangle with point down, triangle with point

up)

The special style FF causes a curve not to be displayed

at all.  An argument can also include a repeat count,

as in 3*45 or 4*-205.

The -m switch will use as many of the following

parameters as appear to be hex numbers, either negative

or positive, noteably including the switches -a through

-f.  This misinterpretation may be avoided by putting

the -m switch and its arguments last in the command

line.

-n		Omit numeric labels on axes.  This uses more of the screen

for the graph, therefore effectively increases

resolution slightly.  It may also be used when you want

to make a hard copy and label it with a typewriter.  To

eliminate both grid and labels, but keep the plot

scaling the same as if labels were printed, use "-g"

alone.  For plotter versions, this can be used to add

more curves to an existing graph.

-t		Transpose x and y values before plotting (-x now refers to

vertical axis).

-x[l] [min [max [junk]]]

If l is present, x axis is logrithmic. First argument,

if present, is lower x limit.  Second argument, if

present, is upper x limit.  A third numeric argument

may be present but will be ignored.  By default, these

values are determined automatically.

-y[l]   [min [max [junk]]]

Similarly for y axis

-y[l]m  [min [max [junk]]]

-ya[p]  [min [max [junk]]]

-yp[p]  [min [max [junk]]]

Any of these signal that each line of the input

contains the real and imaginary parts of a complex

number (after the abscissa, unless -a is also used).

An 'm' requests that the magnitude of the number

(square root of sum of squares of real and imaginary

parts) be plotted.  'p' or 'a' requests the phase

(atan(imaginary part/real part), also known as the

argument) be plotted.  The signs of both parts are

taken into account, so the phase ranges from -pi to

+pi.  If both real and imaginary parts are zero, the

phase is arbitrarily assumed to be zero.  An additional

'p' requests that the phase be divided by pi before

plotting.

-h  <n>		argument is the fraction of the vertical space to be

used for the graph.

-u  <n>		argument is the fraction of the vertical space to move

up before plotting.

-w  <n>		argument is the fraction of the horizontal space to be

used for the graph (useful if screen dump routine

doesn't dump the entire width of the screen).

-r  <n>		argument is the fraction of the horizontal space to move

right before plotting.

INTERACTIVE USE

Several attempts are sometimes required to get the display

options set correctly.  There are two ways to simplify this

process.  The first is to use the command line edit facilities

provided by DOS.  To add an option: return to DOS, type F3 to

redisplay the previous command, add the option, and type

<return>.  You can use also use this procedure to invoke a

hardcopy version of GRAPH with the same options.  A more

powerful command line editor like CED makes this procedure

easier.  The disadvantage is that both GRAPH and the data must

be read in each time, which can be slow.

In display versions of GRAPH, the user may also adjust display

options interactively via a series of pop-up menus.  To display the

top level menu, type a slash '/'.  The highlighted option may be

selected by typing <return> (sometimes labeled <enter>).  Any

options may be selected by typing the initial letter.

>>	The following mouse functions may be expected in a future

>>	version.  In the mean time, all menu functions can be

>>	selected via the keyboard.

If a mouse or other pointing device and a Microsoft-equivalent

driver (MOUSE.COM or MOUSE.SYS) are installed, the mouse may also

be used to adjust parameters.  Mouse commands are of three kinds.

"Pressing" a mouse button has an effect which doesn't depend on the

location of the cursor.  "Clicking" means to point with the cursor

and press the left mouse button quickly.  "Dragging" means to point

one place with the cursor, press the left button, and hold it down

while moving the cursor to another place.

Pressing the left mouse button will pop up the top level menu and

make the mouse cursor visible.  On a two- or three-button mouse,

pressing the right button is equivalent to pressing <esc> on the

keyboard.  This will clear the most recent menu or, if there is no

menu, terminate the program.  When the top level menu is cleared,

the data are redisplayed using the new settings.  (You may

envision the left button sending you further "in", and the right

button backing you "out".)

When the menu is visible, an item may be selected by clicking on

it.

The cursor may be used to adjust the portion of the user's data

space being displayed (the "window") in four ways.

The X and Y menu items can be used to set limits explicitly.

To zoom in, drag from one corner of the desired display area to

the opposite corner.

The "zoom" menu item zooms out - enlarges the window in all

directions.

To enlarge the area being displayed in only one direction, drag

from somewhere near the center of the display area toward that

edge of the display.

For the first method, you must explicitly request a redisplay.  For

the other methods, the redisplay is automatic.

To save the current options for a future plot (in particular,

for a hard copy), use the Output command under File.  For

example, if you display some data by starting with the command

C>graph stuff -m 50 60 70

and use the menubars to set additional options, you can get a

hardcopy by entering

/ F O <return> Q

followed by the DOS command

C>graph stuff -f f

Note that these commands use default format output file name,

'f'.

VERSIONS

Graph is available in several versions for different hardware.

When necessary for configuration control, these versions are

given different file names (see FILES section below).  It is

suggested that the "normal" version for a given computer be

renamed GRAPH for simplicity.  Two measures are taken to ensure

no damage is done by inadvertently executing the wrong version.

First, any version of graph can be executed with a question

mark as the only parameter:

GRAPH ?

It will then print a help screen which indicates the hardware it

supports, and exit immediately without accessing any

machine-specific hardware.  Second, graph tests that the hardware

it needs is present before using it (except for the plotter and

laser printer versions).

For the display version of GRAPH, you will need a device driver in the

form of a .BGI (for Borland Graphics Interface) appropriate for

your hardware.  If you have one of the following standard graphics

cards, you can use one of those supplied by Borland:

CGA

MCGA

EGA

ATT400

VGA

PC3270

IBM8514

Hercules mono

If so, you need only put the .BGI file where GRAPH can find it.  GRAPH

will look in the directories listed in the environment variable

BGIPATH if it is set, otherwise it will look in the directories

indicated by PATH.  For example:

C>set bgipath=C:\MY\BGIFILES;C:\TC\BGI

C>graph mydata

If you put both GRAPH.EXE and the required .BGI file into one of the

directories in your PATH, you will not need to add anything to

your environment.

If you want to select a particular graphics mode, you may set the

environment variable GRAPHMODE to one of the numeric constants of

"graphics_modes" enumeration type as defined by Borland.  The

constants are as follows:

mode           value

------------------    -----

CGA	320*200	C0	0

CGA	320*200	C1	1

CGA	320*200	C2	2

CGA	320*200	C3	3

CGA	640*200 	4

MCGA	320*200 C0	0

MCGA	320*200 C1	1

MCGA	320*200 C2	2

MCGA	320*200 C3	3

MCGA	640*200		4

MCGA	640*480		5

EGA	640*200		0

EGA	640*350		1

EGAMONO	640*350		3

HERC	720*348		0

ATT400	320*200	C0	0

ATT400	320*200	C1	1

ATT400	320*200	C2	2

ATT400	320*200	C3	3

ATT400	640*200		4

ATT400	640*400		5

VGA	640*200		0

VGA	640*350		1

VGA	640*480		2

PC3270	720*350		0

IBM8514	640*480		0

1024*768	0

...where C0, C1, C2, and C3 refer to the four palettes on the CGA

and compatible systems.

If you have nonstandard video hardware, you will have to obtain or

write a .BGI file.  In that case, set the environment variable

BGIDRIVER to the name of the file.  For example, if your driver

is named SGA.BGI, you would set the environment variable as

follows:

C>set bgidriver=SGA

No hardware autodetection of nonstandard video cards is supported.

For the Houstin Instruments plotter, HP plotter, or LIPS-10

laser printer, or Postscript versions, the output port may be

specified by setting an environment variable:

C>set plot_port=COM1        (the default)

C>set plot_port=COM2

C>set plot_port=PRN

Alternately, the output can be saved in a file for later use:

C>set plot_port=c:\path\filename

The device driver asks about several other options: pen speed,

margins (or pen displacement before plotting begins), and

dimensions of desired plotting area.  Note that the area

includes space for numeric labels (unless defeated by the "-n"

switch) and tic marks (if moved outside the graph by the "-g"

switch).

The Houstin Instruments plotter version asks which plotter is in use:

a DMP-29, which has eight pens, or a DMP-7, which has only two.  The

DMP-29 is the default.  The question can be suppressed by setting the

environment variable PLOTTER to DMP-7 or DMP-29.

By default an HI plotter is initialized with the character string

\n\n\n;: EC5 V8 A 0,40 O @

which the plotter interprets as follows:

Carriage return (allows auto baud rate),

plotter select, mode 1, .005" resolution,

8 in/sec,

absolute mode,

move .2 inches up and set origin,

deselect

If necessary, a different setup string may be specified using the

environment variable "plot_setup".  As with the PROMPT command in

MS-DOS, the device driver interprets the sequence "$e" to represent

an ASCII escape character.  The escape sequences available are:

$e  ->  escape character

\n  ->  line feed

\r  ->  carriage return

\t  ->  tab character

\\  ->  \ (escape an otherwise special character)

\$  ->  $ (escape an otherwise special character)

For example, the default setup string could be specified using the

following command:

C>set plot_setup=\n\n\n;: EC5 V8 A 0,40 O @

The HP plotter version assumes an RS-232 connection with

hardware flow control: The plotter pulls pin 20 (DTR) low to

stop data transmission.  Other conditions are set to their

defaults.  To implement this, HP recommends wiring a "null

modem" RS-232 cable which exchanges pins 2 and 3, and connects pin

20 on each end to pins 5 and 6 on the opposite end.  A second

adaptor cable may be used to connect the null modem cable to the

9-pin connector on an IBM PC serial adaptor, or the two functions

may be combined in a single cable.  The cable schematics are as

follows:

IBM                   plotter

9-pin     25-pin     25-pin DTE

chassis ground    .. -------- 1 ---------- 1         chassis ground

signal ground     5 -------- 7 ---------- 7         signal ground

DTR     4 ------> 20 ------+--> 5         CTS

+--> 6         DSR

CTS     8 <------- 5 <---+---- 20         DTR

DSR     6 <------- 6 <---+

TxD     3 -------> 2 ---------> 3         RxD

RxD     2 <------- 3 <--------- 2         TxD

gender     >--------->>------------>

The last line indicates, for example, that the plotter has a female

connector, so the mating cable must have a male connector.  The IBM

BIOS serial port handler will stop transmitting data when

requested.  However, it interprets the signal as an error, and

prompts the operator with the message

Not ready error writing to COM1:

Abort, Retry, Ignore?

Type "r" to continue plotting.  These messages may be avoided by using

a communications program to send the file.

By default, the HP plotter is initialized with the character string

DF; <esc>.R

which the plotter interprets as follows:

default conditions

reset handshake to hardware flow control

(The plotter uses RS-232 connector pin 20, or DTR.

It pulls the pin low to stop data transmission.)

If necessary, a replacement string may be specified using the

environment variable "plot_setup", as above.  The default

setup string could be specified using the following command:

C>set plot_setup=DF; $e.R

The LIPS-10 version clears all macros and all fonts while

initializing to maximize the memory space available, with the

setup string

DAF; DAM; UNIT D; SCF; FONT 2;

As with the HP plotter version, the environment variable

plot_setup can be set to a replacement string.  For example,

"DAM;" or "DAF;" can be omitted to preserve the macros or

fonts, respectively.

DEVICE DRIVER CONFIGURATION FILES

The hard copy versions (GRAPHHI, GRAPHHP, GRAPHLI, GRAPHPS) can also

accept parameters from an ASCII configuration file.  Entries in

the file are switches, some with arguments, as on a command line.

The following switches are available:

-p		portrait orientation (LI and PS only)

-l		landscape orientation (the default) (LI and PS only)

-o <xoffset> <yoffset>

-r <xoffset>

-t <yoffset>

offset from top left corner of page

-s <width> <height>

-w <width>

-h <height>

plot size

-f <name>	name of output file (or port)

-v <velocity>	pen velocity (HP and HI versions only).

Units are cm/sec for HP and in/sec for HI.

-n <name>	Plotter name (HI version only).  Choices

are "DMP-29" (the default) and "DMP-7".  Case is not

distinct, and "29" and "7" are also accepted.

None of the arguments can be omitted.  A semicolon introduces a

comment that extends to the end of the line.

A switch may start with '+' instead of '-', in which case it will

establish the default, but the program will still ask for the

final value.

The device driver configuration file name will be, in decreasing order

of priority:

1. An "&filename" entry on the command line or script file,

2. An "&filename" entry in the environment variable GRAPH,

or a script file referenced by that environment

variable.

3. GRAPHPS.CFG in the current directory.

If more than one &filename entry appears, only the last

configuration file referenced is read.

A PostScript output file name will be, in decreasing order of

priority:

1. The argument of the -f switch in the device

driver configuration file,

2. The value of the environment variable PLOT_PORT, or

3. FOO.PS, where FOO is the root name of the first

data file.

Output file names for the other versions are generated similarly, but

with extensions .HI, .HP, or .LI.

FILES

These are the currently available executable files:

GRAPH.EXE     IBM PC with any standard graphics adaptor

GRAPHHI.EXE   Houstin Instruments DMP-29 plotter

GRAPHHP.EXE   HP 7470A plotter

GRAPHLI.EXE   CIE Laser Image Printing System, or LIPS 10

GRAPHPS.EXE   Postscript

GRAPHZ.EXE    Zenith Z-100 (previous version with menubar)

All these files have been compressed with LZEXE.

NOTES

Graph is largely equivalent to the program of the same name on

Unix, with the following exceptions: Data may come from one or

more files rather than the standard input.  Line segments that

go out of bounds are windowed rather than being dropped.  Graph

may adjust the specified lower or upper limits (outwards) to

give convenient axis labels.  Steps along axes may not be

specified.  The screen is always cleared before plotting.  The

default grid type is frame with tics rather than full grid.

Axes may not be reversed by specifying a lower limit greater

than the upper limit.

UPDATE HISTORY

15 Jul 90	All hard copy versions accept parameters from

a configuration file.

5 Jul 90	Ver 3.21: Script files may have comments.

GRAPHPS default output filename is FOO.PS.

4 Jul 90	Ver 3.20: Added support for device driver

configuration files (see &<name> switch).

16 Jun 90	Ver 3.19: Accepting @file as well as -f file as a

script file reference.  If a log scale switch causes a

labeled point to be discarded, the label is

moved to the last retained point.

14 Jun 90	Ver 3.18: Fetching options from environment variable

GRAPH.

19 Apr 90	Ver 3.17: Displaying up to 16380 data points, if

user memory permits.

6 Mar 90	Ver 3.16: allowing axes separated from graph area.

27 Feb 90	Ver 3.15: no longer attempts to restore video mode

if graphics mode was never set.  Fixed occasional

divide-by-zero bug for screens with subsquare

pixels (pixel width > pixel height, like the

VGA).

19 Feb 90	Ver 3.14: No more text mode error messages if a memory

allocation fails while in graphics mode.  Coherent

error message if script file cannot be opened.

Default script file name is constructed from

data file name.  Menu starts at top right.

Fixed occasional divide-by-zero bug for screens with

subsquare pixels (pixel width > pixel height,

like the VGA).

16 Sep 89	Ver 3.13: Switching to log scales no longer temporarily

drops a linestyle.  "abscissa" menu option can

now discard y values.

15 Sep 89	Ver 3.12: Adding styles interactively automatically

enables breaking, keystrokes entered during a redraw

are not echoed through DOS when they are

discarded.

13 Aug 89	Ver 3.11: Automatically adjusting width to be

consistent with right move, etc.

1 Jul 89	Ver 3.10: graphics mode menus, mouse handler.

16 May 89	Cleaned up nonportable constructions.

4 May 89	Ver 3.09: user can adjust grid width & number of tic

marks interactively.  Markers don't have corner glitches.

25 Apr 89	Ver 3.08: not adjusting # tic marks for label size

if no numeric labels are requested.

24 Apr 89	using intensity if color isn't available.

8 Apr 89	Ver 3.07: name of input script file becomes default for

output script file

6 Apr 89	Ver 3.06: optionally writes data file names to script file

5 Apr 89	Ver 3.05: writes -c and -n switches to output script file

The part of argv[] referring to data files is preserved.

22 Mar 89	Ver 3.04: requests correct number of tic marks in

script file.

27 Feb 89	Ver 3.03: quits gracefully if there are no data points

23 Nov 88	Ver 3.02: Menubars allow interactive update of switches.

1 Aug 88	Ver 2.49: Allowing 100 styles rather than 30.

Style of FF suppresses display entirely.

4 May 88	Ver 2.48: Allows axis labels as high as 10000 before

switching to scientific notation.

23 Mar 88	Ver 2.47: Warns about data points dropped after memory

is full.

4 Mar 88	Ver 2.46: Can display bottom and left axes only.

10 Feb 88	Ver 2.45: Automatic abscissas restart with each

new curve.

23 Jan 88	Ver 2.44: Reads parameters from script file.

25 Nov 87	Ver 2.42: The linestyle is interpreted as a hex

number, so the color digit can range from 0 (white)

through f.  Plot area is shrunk if necessary so

wide grid stays within specified width and height.

29 Oct 87	Ver 2.41: Marker color controlled only by the

second digit of the style number.

25 Aug 87	Ver 2.40: Large number of labels permitted, via

dynamic reallocation (shrinking) of data arrays.

Saving repeat counts.

20 Aug 87	Ver 2.39: Tic marks can be outside graph area.  # small

tic marks is adjustable.

17 Aug 87	Ver 2.38: width of grid lines can be specified.

AUTHOR

Copyright (c) 1985 - 1988 by James R.  Van Zandt

(jrv@mitre-bedford.arpa) 27 Spencer Dr., Nashua NH 03062,

603-888-2272.  Resale forbidden, copying for personal use

encouraged.  Constructive comments welcome.