CalcQF  by Ken Hipple (CIS:74076,2331) and Conrad Smith (76701,107)

V1.0      Mississippi Data Equipment Co.

625C Lakeland East Drive

Jackson, MS 39208               (601) 932-6332

This program calculates and sets the optimum memory refresh rate for your

machine.  It will then create a short .COM file that you can run to quickly

set the refresh rate.  The memory refresh concept is from an article by

Brian Roemmele in the 7/88 issue of PC Magazine.  The .COM file produced

is the same as the QFRESH.COM program from that article.  Please see that

article for a thorough discussion of memory refresh rates.

NOTE: It is possible that your machine will 'hang' before the point

CalcQF considers optimum is reached.  This will not harm your

machine but you will have to re-boot.

Switches:

/C - Calculate the optimum memory refresh rate.  /I, /O and /T imply /C

/I - change the default Increment rate of 16.  In general, larger values

will produce numerically higher optimum rates and smaller values

will produce lower ones

/O - don't estimate the Original refresh rate.

/T - set the Tolerance.  The default value is 0.00001. This determines what

rate CalcQF considers optimum.  Larger values (i.e. 0.01) should produce

numerically lower optimum rates and smaller values (i.e. 0.000001)

should produce higher ones.

/S - Skip the calculation phase and go straight to file creation.

Do not press any keys while this program is running as this can affect

the calculations.  Some TSRs and device drivers may also affect them.

Because of this you may want (or need) to run CalcQF with nothing loaded

into memory.  The rate referred to as the original rate is the refresh

rate in effect when CalcQF was started.  The rate CalcQF says is the best

rate is really only an approximation but it should be close to the best rate

for any given application.  At the very least it will provide you a good

starting point for further experimenting.