*************************** EXPANDED MEMORY ********************************

ASMLIB's EMS subroutines detect and manipulate Expanded Memory hardware

and software.  Expanded memory, when installed in any PC, can help solve

many memory limitation problems.  Call IsEMS before using any other

ASMLIB EMS subroutines.

NOTE: ASMLIB's EMS subroutines are under development.  Additional EMS

subroutines may be available by the time this message reaches you.

EMS memory is allocated in blocks of 16k pages

Error codes returned by EMS functions are:

AH = 0      no error

AH = 80h    error in memory manager software

AH = 81h    error in expanded memory hardware

AH = 84h    bad function code passed to memory manager

AH = 85h    no EMM handles available

AH = 89h    you tried to allocate zero pages

EMMVER:     determines EMS driver version

Source:     emmver.asm

Call with:  no parameters

Returns:    if CF = 1, AH = EMS error code

if CF = 0, AH = major version number

AL = minor version number

Uses:       AX, flags

Example:

include  asm.inc

extrn    isems:proc, emmver:proc

.code

.

.

.

call   isems

jc     no_ems

or     ah,ah      ; test for EMM error

jnz    ems_error

call   emmver     ; get emm version

jc     ems_error

cmp    ah,4       ; version 4.x?

jb     version3x  ; nope, an older EMM version

ISEMS:      detects EMS driver

Source:     isems.asm

Call with:  no parameters

Returns:    if CF = 1, no Expanded Memory manager installed

if CF = 0, AH = EMM error code

if AH = 0, BX = page frame segment address

Uses:       AX, BX, CF

Example:

include  asm.inc

extrn    isems:proc

.code

.

.

.

call   isems

jc     no_ems     ; CF = 1 if no EMS memory

or     ah,ah      ; test for EMM error

jnz    ems_error