1  Introduction

---------------

This help section is a tutorial style introduction

to the basic features of Quickroute 1.5 for DOS. There is

enough information in this section to get you started, and

it would be a good idea to try out the features as they are

introduced.

2  Running Quickroute

---------------------

Normally Quickroute will chose the most appropriate

graphics mode for your computer automatically. However,

you can force Quickroute to use a specific mode

using a command line option. Run quickoute using

QR /? to find out the current list of options.

The Quickroute window is split into 4 main areas; the main

design area which is always coloured black, the standard

drop down menu bar at the top of the window from which

you can select all Quickroutes major functions, the status

line at the bottom of the screen which shows the current

cursor position and memory usage, and just below the

menu bar the 'button bar'.

3  The Button Bar

-----------------

The button bar allows you to select the type of object which

you are going to place onto the design, and provides quick

access to 'block' and 'pick and place' editing.

Moving from left to right, the first four buttons select the

object type. This can be a pad, a track, an integrated circuit

pin out, or a symbol. To place an object onto the design,

move your mouse to one of the buttons (for example the

pad button), and click the left mouse button. Notice that the

object type shown in the status line at the bottom of the screen

changes to show the current object selected. Next, move the pointer

onto the design area and click the left mouse button. A pad, or

other object type, will then appear at the cursor position.

The cursor position is always shown as an X,Y co-ordinate

in the status line. The units are in inches (default) or cm

depending on the Cursor Imp or Cursor Met setting (see

later).

4  Select Window

----------------

You can change the type or size of the object you are

currently using by clicking on the select button, which has

a question mark on it. A window will then open showing the

range of object types available.

To select a new size of pad, point the cursor at the desired

pad, and click the left mouse button. The select window will

then close and you will be able to use the new pad

size/shape.

The default size of an object, can be changed whilst the

select window is open by moving the cursor to point at the

object you want to change and clicking the right mouse

button. You will then be prompted to enter the new size in

the status line. This will be a diameter in the case of pads

or a width in the case of tracks.

5  Objects

----------

There are five types of object which you can place onto your

design; pads, tracks, integrated circuit pin outs (IC's),

symbols, and graphic primitives. Pads correspond to

regions on your PCB design to which component leads will

be connected and tracks are the copper 'wires' which

interconnect the pads.A range of Track types exist

including filled polygons for large copper areas.

Graphic primitives are a set of shapes including text,

circles, ellipses, arcs, etc which you can use

to build up more complex graphic designs. Symbols are

combinations of all the object types, which are stored as a

single unit for convenience. Symbols include, surface

mount designs, connector pin outs, etc. The final object

type is IC's, a set of 32 different dual in line (DIL) designs

are available.

Changing between the different object types is just a matter

of selecting the appropriate button, or in the case of the

primitives, pulling down the Tools menu, and selecting the

appropriate primitive. The following sections look at each

object type in turn, and shows how you can use them

effectively.

Pads

----

Pads are small regions of copper through which holes are

usually drilled, and component leads are soldered. In the

case of integrated circuits, an arrangement of pads will be

designed, to suit the pin-out of the particular chip. There

are 3 groups of pad types; circular, square and oval. Each

type is available in 16 different diameters, and with or

without a central hole to guide drilling.  In generally use the

circular pads for discrete components, square pads for

surface mount components and edge connectors, and the oval pads

for integrated circuits.

To place a pad on your design just move the mouse cursor

to the Pad button on the Button Bar, click the left mouse

button, and move the cursor to the position on the design

where you want your pad. Click the left mouse button. You

can then continue placing the current style of pad on your

design, changing it using the Select Window as required.

Tracks

------

Tracks are the copper wires on a PCB which connect pads

or other objects. A range of widths are available and in

general you should use thicker tracks where larger currents

are being carried, eg. power rails. Four Track types are also

available, namely Normal (Norm), 45 degree (45), Polyline

(Plin) and Polygon (Pgon).

To place a track onto your design first click the mouse

cursor on the Track button on the Button Bar. Next move

the cursor onto the design area and click the left mouse

button. Moving the cursor will then 'rubber-band' a track so

that you can see exactly where it will be placed. Click the

left mouse button when you are happy with the position, or

the right mouse button to quit. If the Track type is set to 45,

then tracks will be made up of segments with node angles which

are multiples of 45 degrees automatically.

Polylines and polygons are entered as strings of tracks in a

sequential fashion. In each case click the right mouse

button to quit entering lines. Note that the first and last

nodes of polygons will be joined together, and the resulting

shape will be filled. This feature is particularly useful for

entering large copper areas.

Integrated Circuits

-------------------

There are 2 types of DIL integrated circuit pin out which

can be placed onto the design with a single mouse click.

The first type is a simple square pad design, and the second

type uses rounder pads and automatically places a silk

screen identifier onto the design. The two types are

available with 8 different pin combinations and can be

either horizontally or vertically aligned.

To place an IC onto your design just move the mouse

cursor to the Integrated Circuit button on the Butto Bar,

click the left mouse button, and move the cursor to the position

on the design where you want your IC. Click the left mouse button.

You can change the design or style using the Select Window button

as required.

Symbols

-------

Symbols are simply groups of pads, tracks, graphic

primitives, IC's and other symbols, which are stored as a

single unit. They can then be stored in a symbol library,

and used at any stage in your design. Symbols can be

rotated/translated before they are placed onto your design,

using the symbol Translate option. Typical symbols include

edge connector pin outs, surface mount designs, and

transistor pin outs. You can create your own symbols, and

store areas of an existing design as a symbol.

To place a symbol onto your design just move the mouse

cursor to the Symbol button on the Button Bar, click the

left mouse button, and move the cursor to the position on

the design where you want your symbol. Click the left

mouse button. You can continue placing symbols onto your

design, changing the design or style using the Select

Window button as required.

Graphic Primitives

------------------

Graphic Primitive are a range of basic shapes which you

may wish to use on your designs. They include ellipses,

rectangles, arcs, and text. See the reference section on the

Tools menu in chapter 3 for more details.

6  Using Zoom and Pan

---------------------

The Quickroute design area that you see within the window

is in fact a small part of the maximum possible design area.

PCB and schematic designs can be over 30"x30" in size and

to allow you to move around your total design area there

are a range of pan, zoom and scroll options.

First of all try loading the demonstration PCB design called

'DEMO.PCB'. This should be installed in the same directory

as Quickroute. You can load it by selecting the File menu,

and clicking the mouse on the Open... option. You will then be

prompted to enter a file name. Enter 'DEMO.PCB' and press the

return key.

The PCB will then be displayed in the main window. Move

the cursor to the centre of the design area and press the

numeric key marked '5'. The design will then zoom in

around the cursor. It is now drawn at a scale of X2.0. Move

the cursor to the bottom right of the window, and press the

right mouse button. The PCB will then be redrawn, with the

region selected at the centre of the window. This is the pan

option.

A range of preset zooms exist from X0.1 to X10.0. You can

select these by pulling down the View menu, selecting

Zoom, and clicking the mouse on the required zoom. The

numeric keys '1' to '8' are hot keys which correspond to the

various zooms as shown on the menu. There is also a

Custom Zoom feature which allows you to enter a positive

zoom.

Note that the cursor step is selected using Cursor Imp

(imperial measurements) or Cursor Met (metric

measurements)and that the currently selected step is

maintained in real terms no matter what the zoom level is.

7  Cursor Step

--------------

As you move around your design, you will notice that the

cursor always locks onto a fixed cursor step, by default this

is 0.1 inches. No matter what the zoom is, the step will

always correspond to 0.1 inches in the 'real world'. To

change this cursor step just pull down the Options menu

and select Cursor Imp. You can chose any of the steps from

0.001 inch, up to 1 inch. Note that you must increase the

zoom level (for example to X5.0) to use the smaller cursor

steps though, otherwise your cursor will not appear to

move. Similarly, only use large cursor steps in small zoom

levels (for example X0.1).

For many applications, an imperial measurement system is

preferable as many integrated circuits and discrete

components have pin spacings which are in imperial

measurements. However, in certain circumstances you may

wish to use metric measurements and this can be done by

pulling down the Options menu and clicking on Cursor Met.

8  Turbo Draw and Grid Operations

---------------------------------

When you are using a lot of zoom and pan operations it is

useful to select the accelerated or 'turbo' graphic feature,

which can be toggled on and off by pulling down the

Options menu and selecting Turbo. When on, all tracks and

pads are drawn as outlines, showing the correct size but

being unfilled. Where detail is required, simply toggle the

feature off.

The Grid feature is also toggled on and off from the Options

menu. Once on a grid is drawn on the screen, providing a

useful reference. Note that turbo draw and grid can also be

toggled on/off by pressing 't' and 'g' respectively.

9  Rounded Tracks and Quick Zoom

--------------------------------

One of the new features of Quickroute 1.5 for DOS, is that

all tracks are drawn with rounded ends. This gives a better

appearance when thicker tracks are joined together. The

negative side is that tracks take longer to draw. Therefore

the option of turning on/off the feature is available by

toggling Rounded Tracks on the Options menu.

Quick Zoom can also be toggled on/off on the Options

menu. When turned on, an attempt is made to calculate as

much of the scaling as possible using integer arithmetic.

This has the effect of reducing redraw times, especially on

computers which don't have a numeric co-processor.

For minimum redraw speed use Turbo ON, Quick Zoom ON,

Grid OFF, and Rounded Tracks OFF. For maximum quality

redraw use Turbo OFF, Quick Zoom OFF, and Rounded

Tracks ON.

10  Editing Designs

-------------------

Once you have entered your design it is highly likely that at

some stage you are going to want to edit it! This is where

the real power of computer based design systems comes to

the fore. Quickroute gives you two types of editing feature;

block based editing for groups of objects, and 'pick and

place' editing for single objects.

Block Based Editing

-------------------

The Button Bar gives you fast access to the block based

editing features. The sixth, seventh, and eighth buttons

from the left allow you to copy, move, and delete

rectangular regions of a design. Load the demonstration file

'DEMO.PCB' into Quickroute, and select the copy block button. Move

the cursor onto the design area and click the left mouse

button. Move the mouse cursor until the rubber-banded

box outlines a region of interest and click the left mouse

button again (you can click the right mouse button to quit

incidently). Any objects that are fully contained within the

rectangle are selected (except for graphics primitives). As

you move the cursor now, a rectangle which is the same

size as the selected region will move with the cursor

allowing you to judge positions easily. Click the left mouse

button to copy the highlighted objects to the new position.

Move and delete work in a similar manner.

Pick and Place Editing

----------------------

When using the block editing features you will notice that

only objects (tracks, pads, etc) that are entirely within the

rubber-banded rectangle are 'selected'. For more precise

selection of objects, the pick and place editing feature can

be used. In order to use this move the cursor to the tenth

button along the Button Bar and click the left mouse

button. Move the cursor onto the design area, and move the

cursor as close as possible to the object you want to select.

Click the left mouse button to 'pick' the object, it should

then be highlighted with a cross. Note that in the case of a

track, the cross will appear towards the centre point of the

track, so make sure the all the track is on screen.Note also

that polygons and graphic primitives (circles, ellipses, etc)

can not be picked.

Once an object is 'picked' you will be asked to select to

either move, copy or delete the object. Enter the required

option or '0' to quit. In the case of move and copy you will

be prompted to move the cursor to the new required

position. Click the left mouse button to copy or move the

object to the new position.

11  Saving, Loading and Printing

--------------------------------

To load a design into Quickroute, simply pull down the File

menu and select Open... You will be prompted to enter a

filename. Note that you can use Directory to check the

contents of the current directory.

Parameters such as the Zoom setting and the position of

the design in the window will also be automatically set to

those which were used when the design was originally

saved. Similarly, you can also Save a file. You will be

prompted to confirm the current filename in the status line.

The File menu also contains the Print and Plot functions.

Print and Plot are used to output hardcopy of your design

in the current Zoom or scale, and with the same top left

hand corner co-ordinate. Thus you can chose which bits of

a design are to be printed.

12  Advanced Features

---------------------

This tutorial has covered the basic features of Quickroute,

but there is much more to explore. In particular the

autorouter Tool which is included has not been mentioned,

as well as the Library features for designing and editing

symbols and including them in custom or standard

library's. All these functions are detailed in the manual

which you will be sent when you register.

13 Registration

---------------

To register your copy of Quickroute, just send a cheque payable

to POWERware, for 60 US dollars to

POWERware, 14 Ley Lane, Marple Bridge, Stockport, SK6 5DD, UK.

We will then send you,

1 - The latest enhanced version of Quickroute for DOS

2 - All the latest printer and plotter drivers

3 - The latest symbol libraries

4 - The full A5 size, quality manual with tutorial and reference

sections.

5 - The POWERware technical help phone number.

Thankyou for registering.      Copyright (c) 1993 Dr I R Frost