DLGLX is a DLG to DXF command line converter.

Converts USGS (United States Geological Survey) DLG (Digital Line Graph) data

to DXF format to import into various CADD systems.  Layers the attribute

data & allows for including & excluding particular layers.  This is a

shareware program.  Filling out & sending me the registration form along with

$10(US) will make you a registered user of this program.

This program is freely distributable.

DISCLAIMER:

DLGLX are being distributed on an as-is-basis.  There is no

warranty of any kind.  The author is not responsible for any damages,

including lost profits or other incidental or consequential damage

that may result from the use of this program.

DLGLX###.EXE	DLG to DXF converter that adds layer mapping (### is version)

ADMIN.DLG	Sample Data from Hawaiian islands in Standard format

CULTURAL.DLG

POLITIC.DLG

RAILRDS.DLG

ROADS.DLG

STREAMS.DLG

WATER.DLG

INCLUDE.LAY	Sample include layer map

EXCLUDE.LAY	Sample exclude layer map

LAYER.MAP	Description of most of the layers you will run into with

the 100K.  The 2M files you will get some different ones.

If you know of some others, please email them to me.

DLGLX.DOC	This file, which can be recreated with DLGLX /? > DLGLX.DOC

REGISTER.FRM	Registration form

FILE_ID.DIZ	File briefly describing what this program does

AD.NFO		Advertisement

Usage:

DLGLX [switches] FILE.EXT DXFNAME.DXF LINE_WIDTH

/? (displays complete documentation without pause or can be redirected)

/Header (mainly for acad release 13, adds release 12 header to file)

/Include=FILE (where FILE is a file that contains layers to include)

/No=Lines  (don't output line numbers as they are processed)

/No=Comments  (don't output Comments to .DXF file)

/R (displays Registration form without pause or can be redirected)

/Xclude=FILE (where FILE is a file that contains layers to exclude)

/Type=S, O, or G (Standard, Optional, or Graphics format)

Will handle wildcard names

You must enter at least 3 arguments

/? (displays complete documentation without pause or can be redirected)

You can recreate this documentation by entering the following:

DLGLX /? > DLGLX.DOC

Display a screen at a time:

DLGLX /? | MORE

Print it:

DLGLX /? > PRN

/Header (mainly for acad release 13, adds release 12 header to file)

If you don't add this switch, only layer names are added to the

header.  Release 13 does not seem to like that as one of my beta

testers, Pat Harman informed me.

/Include=FILE (where FILE is a file that contains layers to include)

This switch is most useful when you want to bring in only certain

layers & discard all the rest.  See explanation following /X for

more information on this switch an the /X switch.

/No=Lines (don't output line numbers as they are processed)

The program normally outputs the current Node, Area, & Line that it

is processing.  If you don't care to see this information, you can use

this switch.  Very little speed difference with or without this.

/R (displays Registration form without pause or can be redirected)

You can use similar methods as shown above with /? to create or

print this form.

/Xclude=FILE (where FILE is a file that contains layers to exclude)

This switch is most useful when you want to exclude particular layers.

A good example would be to exclude all roads but the major interstate

roads.

Information that pertains to both /Include & /Xclude

You can only use 1 of these switches at a time.  Maximum of 1024

include or exclude layers and 8k file size.  This should be more

than plenty.  You can add comments to the file as long as you leave a

non-numeric value after the layer name.  A tab or multiple spaces is

probably best.  Keep in mind that adding comments increases your file

size & you will hit the 8k barrier a lot quicker.

See below for an example

/Type=S, O, or G (Standard, Optional, or Graphics format)

I doubt you will need this, but added it just in case.

Here is a sample of what a /Include or /Exclude file should look like

-------- Cut Here --------

default

170_209 Road or street, class 3

170_210 Road or street, class 4

170_211 Trail, class 5, other than

50_412  Stream

50_414  Ditch or canal

190_205 Arbitrary line extension

-------- Cut Here --------

General Information

This program has been well tested by a few people for a couple of

weeks & at the time of this release, there are no known bugs.

Memory usage is less than 46k, so you should be able to shell out of

most any program & run it.

You can also use a ":" for the switches that have an "=" after the

switch.  Switches only need the 1st letter of the switch name entered.

You can use upper or lower case for the switches or file names.  The

program converts them all to upper case before processing.  Files that

have been massaged with the unix "dd" command or similar program will

take a bit longer to process because of the variable record lengths it

creates.  If the program sees a carriage return and/or line feed

character in the 1st 1024 bytes of the file, it will consider the

file has been massaged.

Colors are generated in the following fashion.  The default layer is

set to red.  The attribute layer color's range in color numbers from

1-7 depending on the minor attribute code.  What I'm currently doing

is taking the minor code dividing it by 7 & using the remainder for the

color number.  In the case of no remainder, the color is 7.

For example:

layer 170_209	209/7=29 w/ a remainder of 6, so it is set to color 6.

layer 170_210	210/7=30 w/ no remainder, so it is set to color 7.

Hitting the ESCAPE key should get you out of the program fairly

quickly.  You may end up with a partial .DXF file if you do this when

the program is on the 2nd pass.

If you don't want any output going to your screen, try the following:

DLGLX /N=L *.DLG AREA.DXF 0 > NUL

This will turn off line counting & send the screen output to the NUL

device.

Formats (brief explanation, see USGS specification for complete explanation)

Standard

This format uses integer numbers from -32767 to +32767 for all the

coordinates.  There is some translation coordinates at the beginning

of the file that make it possible to convert it to the optional format.

144 byte records, unless someone used the unix "dd" command on the file.

Optional

This format contains F12.2 fortran format numbers (real numbers to the

1/100 of a meter in UTM projection.

80 byte records, unless someone used the unix "dd" command on the file.

Graphics

This format contains Longitude & latitude coordinates represented in

Degrees, minutes & seconds.  I convert this to a decimal number by

cheating & 8 byte integer math on the minutes & seconds, then writing

the degrees,decimal point & the 8 byte integer converted to ASCII.

I use 19 digit numbers for the calculations so I can get 16 digits

past the decimal point of accuracy.

20 byte records, unless someone used the unix "dd" command on the file.

Ftp sites as of this writing for further USGS information & .DLG files

edcftp.cr.usgs.gov:/pub/data/DLG       (contains 2M & 100K data files)

nmdpow9.er.usgs.gov:/public            (information on USGS's various

standards)

Trouble shooting

If you have any problems, try using 1 or more of the switches.

Acadr12 works should work fine with the defaults.  Acadr13 will need

the /H switch added & you can also use this switch with Acadr12.  This

adds a release 12 header to the .DXF file.

I've added comments to the beginning of the .DXF file using the DXF

code 999.  Some other CADD systems may not recognize this.  If so,

try the "/N=C" switch to turn off output of comments.

Future versions

Am considering the following, but don't hold me to any of them.  May

or may not release another public version if I find it is not worth

my time (ie. no one registers), but will send email updates to those

that register.

1. Buffered reads, should get a 3 to 4 times increase in conversion, at least

on larger files.  Initial tests showed a 7meg file on my machine took 3:44

& with buffered reads took 52 seconds. I don't feel comfortable with my

method on this yet, so have not added it to the release version.

2. Better color mapping & possibly line width mapping.  One of my beta testers,

Pat Harman is getting the colors that USGS uses.

3. Conversion of Standard format to Optional format while converting.  This

looks fairly easy to do, but will require some floating point math.  All

the math in the program is integer, even the Degrees,minutes & seconds

conversion to Decimal degrees.

4. Possibly conversion between a couple of map projections.  Don't hold your

breath on this.

5. Conversion of DLG data to a postscript file.  Someone just asked me about

this, & after looking at my postscript book, it looks quite possible.

6. Possibly a user interface.

I wish to thank the people that were kind enough to be guinea pigs & test

my program out.  They found some bugs that I would not have found myself.

A special thanks goes to:

Pat Harman helped me get it to work with r13 & tested it quite extensively.

Jim Day found the bug where using the Unix "dd" command.  See history.txt

Jim Davidson found the bug in the input file names without an extension.

Electronic feedback:

Internet Address:  ddbunch@infi.net

My mailing address for non electronic feedback:

David Bunch

4 Gillis Road

Portsmouth, Virginia (USA)  23702-2214

(804) 393-4347