page 1

��������������������������������������������������������Ŀ

� ASTATER.EXE Version 1.0 - AHDL State Machine Generator �

����������������������������������������������������������

(C)Copyright & Support - Amos Zaslavsky @ 972-4-230219.

Description ........................................................  1

Command line syntax ................................................  2

Example 1 : A Mealy machine with fully encoded state assignment ....  3

Example 2 : A Mealy machine with One-Hot state assignment .......... 11

Example 3 : A Moore machine with direct outputs .................... 15

Example 4 : A Moore mixed example .................................. 21

Errors, Warning & Note Messages .................................... 28

Limitations & Legal details ........................................ 32

Description

-------------

ASTATER.EXE is a utility for generating an ABEL file for state machines.

It will generate a file template that will fit your requirements of:

- number of inputs

- number of outputs

- maximum number of relevant conditions per state

- number of flip flops

- state names

After generation of the state machine file you will have to make a few

changes to the file with your text editor. This utility can save time and let

you concentrate on the logic of the machine an not on the syntax. The program

is simpler to use then some fancy graphical state machine generators that

are on the market (which are also expensive).

The original idea of a stater program came from Ofer Hoffman (A VHDL shark

@ 972-4-230048) how wrote a program like this for VHDL users !

page 2

Command line syntax

---------------------

ASTATER must be followed by eight command line parameters:

ASTATER <module> <in> <out> <con> <ff> /1|2 <state_name> <state_name>...

<module>  Is the module name. This will also be the file name and device

name.

<in>      Is the number of inputs. At least one input must be defined.

<out>     Number of indirect outputs. Indirect output is an output that is

not a direct flip flop output.

<cond>    Is the maximum number of relevant condition per state. A machine

can have many inputs (for example 15: I1,I2...I15), but in each

state only a small number of conditions is relevant. for example:

condition1 -   I1 & !I2

condition2 -   I1 &  I2

condition3 -  !I1

(in this example only 3 conditions). The maximum number of

conditions is 2^(inputs). A condition is not only a single input

signal (literal) but a can be a function of such signals (Like in

the previous example).

<ff>      Number of flip-flops. A state machine must have at least one

flip-flop. If it has no flip-flops it is not a synchronous state

machine. If you plan to use one of the Flip-Flop outputs as an

output of the machine (a good way to prevent output spikes), don't

count it as an output of the machine. Such an output is called in

this document - "A direct output". Only indirect outputs should be

counted as outputs of the machine.

/1|2      Type of machine:

/1 - Moore machine (outputs depend only of the present state of

the machine)

/2 - Mealy machine (outputs depend both on the present state of

the machine and the inputs)

/3 - Dummy machine (A moore machine that can be compiled with on

modifications. The machine is a cyclic counter

enabled by the "input1" signal. The number of

conditions that you enter as command line is

not relevant.)

<state>   A state name. A list of state names must have at least tow

members, but can have more.

page 3

Example 1 : A Mealy machine with fully encoded state assignment

---------------------------------------------------------------

In this example we will describe a Mealy state machine with 3 inputs and 3

outputs.

���������Ŀ

U ���>�         ���>� R

Inputs     V ���>�    M1   ���>� S     Outputs

W ���>�         ���>� T

�����������

�

Cp ��>���

This is a state-table of the machine:

����������������������������������������������������Ŀ

� Present State �  Input  ��  Next State   � Output  �

����������������������������������������������������Ĵ

�       AA      �   !U    ��      AA       �    R    �

�               �    U    ��      BB       �    R    �

����������������������������������������������������Ĵ

�       BB      �   !V    ��      CC       �   S,T   �

�               �    V    ��      DD       �   S     �

����������������������������������������������������Ĵ

�       CC      � !V & W  ��      CC       �    -    �

�               �  V & W  ��      DD       �    -    �

�               �     !W  ��      AA       �    -    �

����������������������������������������������������Ĵ

�       DD      �    1    ��      AA       �   R,S   �

������������������������������������������������������

It is easy to see from the table that the maximum number of transition

conditions per state is three. The behavior of this machine is a Mealy

behavior only when the machine is in state BB (and it's enough to make it a

Mealy machine). When the machine is in state DD it makes an un-conditional

transition (Autonomous) to the AA state.

In This example we will use a full binary encoding of state assignments. For

this kind of encoding we need only tow flip-flops. In the next example we will

use a One-Hot decoded state assignments. In the next example four flip-flops

will be needed.

page 4

For this example ASTATER.EXE will be used with The following command line

parameters:

3 - inputs

3 - outputs

3 - conditions

2 - flip-flops

/2 - Mealy machine

AA - first state

BB - second state

CC - third state

DD - forth state

Here is the command line you have to type:

ASTATER M1 3 3 3 2 /2 AA BB CC DD

Here is the file that will be created by ASTATER.EXE :

M1.ABL

��

�MODULE M1

�TITLE 'M1.ABL created by ASTATER.EXE Version 1.0

�       Time of creation is Sat Aug 20 17:07:13 1994

�      '

�DECLARATIONS

�

�   M1 DEVICE ;

�   input1 PIN ;

�   input2 PIN ;

�   input3 PIN ;

�   CP PIN ; "CLOCK INPUT"

�   MR PIN ; "MASTER RESET (ASYN) INPUT"

�   output1 PIN ISTYPE 'COM' ;

�   output2 PIN ISTYPE 'COM' ;

�   output3 PIN ISTYPE 'COM' ;

�   Q1 NODE ISTYPE 'REG,BUFFER' ;

�   Q0 NODE ISTYPE 'REG,BUFFER' ;

�

�   PS = [Q1,Q0] ; "STATE ASSIGNMENT"

�   AA = [ 0, 0] ;

�   BB = [ 0, 0] ;

�   CC = [ 0, 0] ;

�   DD = [ 0, 0] ;

�

�   C , X , P , Z = .C. , .X. , .P. , .Z. ;

�

page 5

�EQUATIONS

�

�PS.AR = !MR ;

�   PS.CLK = CP ;

�

�STATE_DIAGRAM PS

�

�   STATE AA :

�      [output1,output2,output3]=[0,0,0] ;

�      CASE

�         condition11 : next_state11

�            WITH

�               [output1,output2,output3]=[0,0,0] ;

�            ENDWITH ;

�         condition12 : next_state12

�            WITH

�               [output1,output2,output3]=[0,0,0] ;

�            ENDWITH ;

�         condition13 : next_state13

�            WITH

�               [output1,output2,output3]=[0,0,0] ;

�            ENDWITH ;

�      ENDCASE ;

�   STATE BB :

�      [output1,output2,output3]=[0,0,0] ;

�      CASE

�         condition21 : next_state21

�            WITH

�               [output1,output2,output3]=[0,0,0] ;

�            ENDWITH ;

�         condition22 : next_state22

�            WITH

�               [output1,output2,output3]=[0,0,0] ;

�            ENDWITH ;

�         condition23 : next_state23

�            WITH

�               [output1,output2,output3]=[0,0,0] ;

�            ENDWITH ;

�      ENDCASE ;

�   STATE CC :

pgage 6

�      [output1,output2,output3]=[0,0,0] ;

�      CASE

�         condition31 : next_state31

�            WITH

�               [output1,output2,output3]=[0,0,0] ;

�            ENDWITH ;

�         condition32 : next_state32

�            WITH

�               [output1,output2,output3]=[0,0,0] ;

�            ENDWITH ;

�         condition33 : next_state33

�            WITH

�               [output1,output2,output3]=[0,0,0] ;

�            ENDWITH ;

�      ENDCASE ;

�   STATE DD :

�      [output1,output2,output3]=[0,0,0] ;

�      CASE

�         condition41 : next_state41

�            WITH

�               [output1,output2,output3]=[0,0,0] ;

�            ENDWITH ;

�         condition42 : next_state42

�            WITH

�               [output1,output2,output3]=[0,0,0] ;

�            ENDWITH ;

�         condition43 : next_state43

�            WITH

�               [output1,output2,output3]=[0,0,0] ;

�            ENDWITH ;

�      ENDCASE ;

�

�DECLARATIONS

�

�   NS = [Q1,Q0] ; "NEXT STATE"

�

�TEST_VECTORS

�

�   ([MR,CP,input1,input2,input3] -> [NS,output1,output2,output3])

�

�END

��

All the place holders were generated by ASTATER.EXE in lowercase letters.

Using the replace function of your editor (a global replace) do the folowing

replacements:

page 7

- Replace input1 with U

- Replace input2 with V

- Replace input3 with W

- Replace output1 with R

- Replace output2 with S

- Replace output3 with T

Now do the folowing changes:

- Make state assignments.

for example:

AA = [0,0], BB = [0,1], CC = [1,0], DD = [1,1]

The assignment of combination [0,0] to state AA and the declarations of

attributes 'REG,BUFFER' that were chosen for Q1 and Q0, make the AA state

the reset state of the machine. It is not hard to change it.

- Delete the WITH..ENDWITH blocks from first, third and last state blocks.

These blocks desribe Mealy type behavior, and these blocks are Moore type

blocks (only the second block describes a Mealy behavior).

- Replace dummy conditions (condition1,condition2...) and next_states

(next_state1, next_state2...) , with real conditions and real next_states

and delete lines that are not needed.

It is possible to describe an unconditional transition with ABEL's CASE

statements in the following way:

STATE DD :

[R,S,T]=[1,1,0] ;

CASE

1 : AA                   <- this is the unconditional transition

ENDCASE ;

it is equivalent to the folowing statements:

STATE DD :

[R,S,T]=[1,1,0] ;

GOTO AA

page 8

- Replace zero output assignments (Mealy and Moore type) with real (0 or 1)

output assignments and delete output assignments that are not needed.

- Add test vector values.

Now the file will look like this:

M1.ABL

��

�MODULE M1

�TITLE 'M1.ABL created by ASTATER.EXE Version 1.0

�       Time of creation is Sat Aug 20 17:07:13 1994

�      '

�DECLARATIONS

�

�   M1 DEVICE ;

�   U PIN ;

�   V PIN ;

�   W PIN ;

�   CP PIN ; "CLOCK INPUT"

�   MR PIN ; "MASTER RESET (ASYN) INPUT"

�   R PIN ISTYPE 'COM' ;

�   S PIN ISTYPE 'COM' ;

�   T PIN ISTYPE 'COM' ;

�   Q1 NODE ISTYPE 'REG,BUFFER' ;

�   Q0 NODE ISTYPE 'REG,BUFFER' ;

�

�   PS = [Q1,Q0] ; "STATE ASSIGNMENT"

�   AA = [ 0, 0] ;

�   BB = [ 0, 1] ;

�   CC = [ 1, 0] ;

�   DD = [ 1, 1] ;

�

�   C , X , P , Z = .C. , .X. , .P. , .Z. ;

�

�EQUATIONS

�

�   PS.AR = !MR ;

�   PS.CLK = CP ;

�

page 9

�STATE_DIAGRAM PS

�

�   STATE AA :

�      [R,S,T]=[1,0,0] ;

�      CASE

�         !U : AA

�          U : BB

�      ENDCASE ;

�   STATE BB :

�      [R,S]=[0,1] ;

�      CASE

�         !V : CC

�            WITH

�               T = 1 ;

�            ENDWITH ;

�          V : DD

�            WITH

�               T = 0 ;

�            ENDWITH ;

�      ENDCASE ;

�   STATE CC :

�      [R,S,T]=[0,0,0] ;

�      CASE

�         !V & W : CC

�          V & W : DD

�             !W : AA

�      ENDCASE ;

�   STATE DD :

�      [R,S,T]=[1,1,0] ;

�      CASE

�         1 : AA

�      ENDCASE ;

�

�DECLARATIONS

�

�   NS = [Q1,Q0] ; "NEXT STATE"

�

page 10

�TEST_VECTORS

�

�  ([MR,CP,U,V,W] -> [PS,R,S,T])

�   [ 0, X,X,X,X] -> [AA,1,0,0]; "RESET"

�   [ 1, C,0,X,X] -> [AA,1,0,0]; "AA->AA"

�   [ 1, C,1,X,X] -> [BB,0,1,X]; "AA->BB (T IS X)"

�   [ 1, 0,X,0,X] -> [BB,0,1,1]; "BB/T"

�   [ 1, 0,X,1,X] -> [BB,0,1,0]; "BB/!T"

�   [ 1, C,X,0,X] -> [CC,0,0,0]; "BB->CC"

�   [ 1, C,X,0,1] -> [CC,0,0,0]; "CC->CC"

�   [ 1, C,X,X,0] -> [AA,1,0,0]; "CC->AA"

�   [ 1, C,1,X,X] -> [BB,0,1,X]; "AA->BB (T IS X)"

�   [ 1, C,X,1,X] -> [DD,1,1,0]; "BB->DD"

�   [ 1, C,X,X,X] -> [AA,1,0,0]; "DD->AA"

�   [ 1, C,1,X,X] -> [BB,0,1,X]; "AA->BB (T IS X)"

�   [ 1, C,X,0,X] -> [CC,0,0,0]; "BB->CC"

�   [ 1, C,X,1,1] -> [DD,1,1,0]; "CC->DD"

�   [ 1, C,X,X,X] -> [AA,1,0,0]; "DD->AA"

�

�END

��

Now compile, simulate and generate a JEDEC file using ABEL.

page 11

Example 2 : A Mealy machine with One-Hot state assignment

---------------------------------------------------------

In this example we will design the same state machine that we designed before,

but we will use the One-Hot state assignment. for this kind of assignment

we need 4 flip-flops. This time we will run ASTATER.EXE using the same command

line parameters except the number of flip-flops that is 4 and the name of the

machine that will be changed to M2. Here is how the command line looks:

ASTATER M2 3 3 3 2 /2 AA BB CC DD

The file that will be generated looks like the file we got earlier but with

more flip-flops. All the replacements we did before will be done here too:

- Replace input1 with U

- Replace input2 with V

- Replace input3 with W

- Replace output1 with R

- Replace output2 with S

- Replace output3 with T

The main difference between this example and the previous example, is in the

state assignments. Write the following one-hot state assignments:

AA = [1,0,0,0], BB = [0,1,0,0], CC = [0,0,1,0] DD = [0,0,0,1]

Other important change that must be done (that differ from the previous

example) is to change the declaration of node Q0 from ISTYPE 'REG,BUFFER' to

ISTYPE 'REG,INVERT'. This change is done because we want the reset state to

be AA [1,0,0,0]. When MR is invoked (MR is active-low) we want the machine to

be in the AA state. In this state Q0 (the flip-flop of state AA) must be in a

High (1) state.

All the other changes we had to make in this example to the original ASTATER

generated source file, are the same changes we had to do in the previous

example. Here are all the changes that need to be done:

- Delete WITH..ENDWITH blocks from first, third and last state blocks. These

blocks describe Moore type behavior.

- Replace dummy conditions (condition1,condition2...) and next_states

(next_state1, next_state2...) with real conditions and real next_states and

delete lines that are not needed.

page 12

- Replace zero output assignments (Mealy and Moore type) with real output

assignments and delete output assignments that are not needed.

- Add test vector values.

Now the file will look like this:

M2.ABL

��

�MODULE M2

�TITLE 'M2.ABL created by ASTATER.EXE Version 1.0

�       Time of creation is Sat Aug 20 17:07:13 1994

�      '

�DECLARATIONS

�

�   M2 DEVICE ;

�   U PIN ;

�   V PIN ;

�   W PIN ;

�   CP PIN ; "CLOCK INPUT"

�   MR PIN ; "MASTER RESET (ASYN) INPUT"

�   R PIN ISTYPE 'COM' ;

�   S PIN ISTYPE 'COM' ;

�   T PIN ISTYPE 'COM' ;

�   Q3 NODE ISTYPE 'REG,BUFFER' ;

�   Q2 NODE ISTYPE 'REG,BUFFER' ;

�   Q1 NODE ISTYPE 'REG,BUFFER' ;

�   Q0 NODE ISTYPE 'REG,INVERT' ;

�

�   PS = [Q3,Q2,Q1,Q0] ; "STATE ASSIGNMENT"

�   AA = [ 0, 0, 0, 1] ;

�   BB = [ 0, 0, 1, 0] ;

�   CC = [ 0, 1, 0, 0] ;

�   DD = [ 1, 0, 0, 0] ;

�

�   C , X , P , Z = .C. , .X. , .P. , .Z. ;

�

�EQUATIONS

�

�   PS.AR = !MR ;

�   PS.CLK = CP ;

�

page 13

�STATE_DIAGRAM PS

�

�   STATE AA :

�      [R,S,T]=[1,0,0] ;

�      CASE

�         !U : AA

�          U : BB

�      ENDCASE ;

�   STATE BB :

�      [R,S]=[0,1] ;

�      CASE

�         !V : CC

�            WITH

�               T = 1 ;

�            ENDWITH ;

�          V : DD

�            WITH

�               T = 0 ;

�            ENDWITH ;

�      ENDCASE ;

�   STATE CC :

�      [R,S,T]=[0,0,0] ;

�      CASE

�         !V & W : CC

�          V & W : DD

�             !W : AA

�      ENDCASE ;

�   STATE DD :

�      [R,S,T]=[1,1,0] ;

�      CASE

�         1 : AA

�      ENDCASE ;

�

�DECLARATIONS

�

�   NS = [Q1,Q0] ; "NEXT STATE"

�

page 14

�TEST_VECTORS

�

�  ([MR,CP,U,V,W] -> [PS,R,S,T])

�   [ 0, X,X,X,X] -> [AA,1,0,0]; "RESET"

�   [ 1, C,0,X,X] -> [AA,1,0,0]; "AA->AA"

�   [ 1, C,1,X,X] -> [BB,0,1,X]; "AA->BB (T IS X)"

�   [ 1, 0,X,0,X] -> [BB,0,1,1]; "BB/T"

�   [ 1, 0,X,1,X] -> [BB,0,1,0]; "BB/!T"

�   [ 1, C,X,0,X] -> [CC,0,0,0]; "BB->CC"

�   [ 1, C,X,0,1] -> [CC,0,0,0]; "CC->CC"

�   [ 1, C,X,X,0] -> [AA,1,0,0]; "CC->AA"

�   [ 1, C,1,X,X] -> [BB,0,1,X]; "AA->BB (T IS X)"

�   [ 1, C,X,1,X] -> [DD,1,1,0]; "BB->DD"

�   [ 1, C,X,X,X] -> [AA,1,0,0]; "DD->AA"

�   [ 1, C,1,X,X] -> [BB,0,1,X]; "AA->BB (T IS X)"

�   [ 1, C,X,0,X] -> [CC,0,0,0]; "BB->CC"

�   [ 1, C,X,1,1] -> [DD,1,1,0]; "CC->DD"

�   [ 1, C,X,X,X] -> [AA,1,0,0]; "DD->AA"

�

�END

��

Now compile, simulate and generate a JEDEC file using ABEL.

page 15

Example 3 : A Moore machine with direct outputs

-----------------------------------------------

In this example we will be designing the following Moore machine:

����������������������������������������������������Ŀ

� Present State �  Input  ��  Next State   � Output  �

����������������������������������������������������Ĵ

�       AA      �   !U    ��      AA       �    R    �

�               �    U    ��      BB       �    R    �

����������������������������������������������������Ĵ

�       BB      �   !V    ��      CC       �   S,T   �

�               �    V    ��      DD       �   S,T   �

����������������������������������������������������Ĵ

�       CC      � !V & W  ��      CC       �    -    �

�               �  V & W  ��      DD       �    -    �

�               �     !W  ��      AA       �    -    �

����������������������������������������������������Ĵ

�       DD      �    1    ��      AA       �   R,S   �

������������������������������������������������������

This machine has 3 inputs an 3 outputs and behaves almost exactly like the

machine from the previous examples. The only difference is that the T output

dose not change as a function of V, when the machine is in the BB state (it

used to change on the first and second examples).

In the previous examples the outputs of the machine were not directly the

outputs of the flip-flops. Combinatorial logic was used to decode the

flip-flop outputs combinations to machine output combinations (a waste of

hardware resources).

CP �>Ŀ

����������������Ŀ �  ������Ŀ

�  Cmbinational  ����>�D1    ���� Q1

�                � ��Ĵ> FF  � �

� Sum of product � �  �������� �

�                ��������<������ foldback

U ������>�     Matrix     � �  ������Ŀ

�                ����>�D1    ���� Q0

V ������>�                � ��Ĵ> FF  � �

�                �    �������� �

W ������>�                ��������<������ foldback

�              d �

�              e ��������>������� R

�              c �

�              o ��������>������� S

�              d �

�              e ��������>������� T

������������������

page 16

When a design of this type is implemented with a flip-flop that has no buried

registers, it wastes pins (ABEL will automatically exchange NODEs to PINs if

the device has no nodes).

When designing a Moore machine, one can assign flip-flops so the outputs of

machine will be directly the outputs of filp-flops.

CP �>Ŀ

����������������Ŀ �  ������Ŀ

�  Cmbinational  ����>�DR    ���� R

�                � ��Ĵ> FF  � �

� Sum of product � �  �������� �

�                ��������<������

U ������>�     Matrix     � �  ������Ŀ

�                ����>�DS    ���� S

V ������>�                � ��Ĵ> FF  � �

�                � �  �������� �

W ������>�                ��������<������

�                � �  ������Ŀ

�                ����>�DT    ���� T

�                � ��Ĵ> FF  � �

�                �    �������� �

�                ��������<������

������������������

This will save pins of the PAL, reduce tpd (from the clock signal to the

outputs), and produce clean outputs with no spikes. Not all outputs have to be

produced this way. Some times a mixture of direct and indirect outputs is also

chosen.

In this example we want all outputs to come directly from flip-flop outputs.

To do this we choose to define zero outputs in the command line of ASTATER.

The number of outputs defined for ASTATER is the number of indirect outputs !

We also chose three flip-flops (a flip-flop for every output). This is the

command and command line parameters used:

ASTATER M3 3 0 3 3 /1 AA BB CC DD

page 17

Here is the file that will be created by ASTATER.EXE :

M3.ABL

��

�MODULE M3

�TITLE 'M3.ABL created by ASTATER.EXE Version 1.0

�       Time of creation is Sat Aug 20 17:07:13 1994

�      '

�DECLARATIONS

�

�   M3 DEVICE ;

�   input1 PIN ;

�   input2 PIN ;

�   input3 PIN ;

�   CP PIN ; "CLOCK INPUT"

�   MR PIN ; "MASTER RESET (ASYN) INPUT"

�   Q2 NODE ISTYPE 'REG,BUFFER' ;

�   Q1 NODE ISTYPE 'REG,BUFFER' ;

�   Q0 NODE ISTYPE 'REG,BUFFER' ;

�

�   PS = [Q2,Q1,Q0] ; "STATE ASSIGNMENT"

�   AA = [ 0, 0, 0] ;

�   BB = [ 0, 0, 0] ;

�   CC = [ 0, 0, 0] ;

�   DD = [ 0, 0, 0] ;

�

�   C , X , P , Z = .C. , .X. , .P. , .Z. ;

�

�EQUATIONS

�

�   PS.AR = !MR ;

�   PS.CLK = CP ;

�

page 18

�STATE_DIAGRAM PS

�

�   STATE AA :

�      CASE

�         condition11 : next_state11

�         condition12 : next_state12

�         condition13 : next_state13

�      ENDCASE ;

�   STATE BB :

�      CASE

�         condition21 : next_state21

�         condition22 : next_state22

�         condition23 : next_state23

�      ENDCASE ;

�   STATE CC :

�      CASE

�         condition31 : next_state31

�         condition32 : next_state32

�         condition33 : next_state33

�      ENDCASE ;

�   STATE DD :

�      CASE

�         condition41 : next_state41

�         condition42 : next_state42

�         condition43 : next_state43

�      ENDCASE ;

�

�DECLARATIONS

�

�   NS = [Q2,Q1,Q0] ; "NEXT STATE"

�

�TEST_VECTORS

�

�   ([MR,CP,input1,input2,input3] -> NS ;

�

�END

��

Notice that the Mealy output assignments are missing now from the file.

page 19

All the replacements we did in previous examples are done here in the same

way:

- Replace input1 with U

- Replace input2 with V

- Replace input3 with W

- There are no replacements for output1/2/3 because they were not generated in

file

The main difference between this implementation and the previous ones is in

the state assignments. Write the following state assignments:

AA = [1,0,0], BB = [0,1,1], CC = [0,0,0], DD = [1,1,0]

In case a unique assignment for each state could not be found more flip-flops

could be appended to the state register (we do not need to add flip-flops in

this example).

Other important change that must be made is to change node Q0 from ISTYPE

'REG,BUFFER'to ISTYPE 'REG,INVERT'. This is because we want the reset state to

be AA. When MR is invoked (low) we want the machine to be in the AA state. In

this state Q0 (the flip-flop that is a direct output implementing the R output

of the machine) must be in a High (1) state.

All the other changes we have to make to the original ASTATER generated file

are the same changes we did in the previous examples. Here are all the changes

that need to be done:

- Replace dummy conditions (condition1, condition2...) and next_states

(next_state1, next_state2...) with real conditions and next_states and

delete lines that are not needed.

- Add test vector values.

page 20

Now the file will look like this:

M3.ABL

��

�MODULE M3

�TITLE 'M3.ABL created by ASTATER.EXE Version 1.0

�       Time of creation is Sat Aug 20 17:07:13 1994

�      '

�DECLARATIONS

�

�   M3 DEVICE ;

�   U PIN ;

�   V PIN ;

�   W PIN ;

�   CP PIN ; "CLOCK INPUT"

�   MR PIN ; "MASTER RESET (ASYN) INPUT"

�   Q2 NODE ISTYPE 'REG,INVERT' ;

�   Q1 NODE ISTYPE 'REG,BUFFER' ;

�   Q0 NODE ISTYPE 'REG,BUFFER' ;

�

�   PS = [Q2,Q1,Q0] ; "STATE ASSIGNMENT"

�   AA = [ 1, 0, 0] ;

�   BB = [ 0, 1, 1] ;

�   CC = [ 0, 0, 0] ;

�   DD = [ 1, 1, 0] ;

�

�   C , X , P , Z = .C. , .X. , .P. , .Z. ;

�

�EQUATIONS

�

�   PS.AR = !MR ;

�   PS.CLK = CP ;

�

�STATE_DIAGRAM PS

�

page 21

�   STATE AA :

�      CASE

�         !U : AA

�          U : BB

�      ENDCASE ;

�   STATE BB :

�      CASE

�         !V : CC

�          V : DD

�      ENDCASE ;

�   STATE CC :

�      CASE

�         !V & W : CC

�          V & W : DD

�             !W : AA

�      ENDCASE ;

�   STATE DD :

�      CASE

�         1 : AA

�      ENDCASE ;

�

�DECLARATIONS

�

�   NS = [Q2,Q1,Q0] ; "NEXT STATE"

�

�TEST_VECTORS

�

�   ([MR,CP,U,V,W] -> NS)

�    [ 0, X,X,X,X] -> AA; "RESET"

�    [ 1, C,0,X,X] -> AA; "AA->AA"

�    [ 1, C,1,X,X] -> BB; "AA->BB"

�    [ 1, C,X,0,X] -> CC; "BB->CC"

�    [ 1, C,X,0,1] -> CC; "CC->CC"

�    [ 1, C,X,X,0] -> AA; "CC->AA"

�    [ 1, C,1,X,X] -> BB; "AA->BB"

�    [ 1, C,X,1,X] -> DD; "BB->DD"

�    [ 1, C,X,X,X] -> AA; "DD->AA"

�    [ 1, C,1,X,X] -> BB; "AA->BB"

�    [ 1, C,X,0,X] -> CC; "BB->CC"

�    [ 1, C,X,1,1] -> DD; "CC->DD"

�    [ 1, C,X,X,X] -> AA; "DD->AA"

�

�END

��

Now compile, simulate and generate a JEDEC file using ABEL.

page 21

Example 4 : A Moore mixed example

---------------------------------

In this example we will be designing the following Moore machine:

���������Ŀ

I ���>�         ���>� P

J ���>�         ���>� Q

Inputs     K ���>�    M4   ���>� R     Outputs

L ���>�         ���>� S

M ���>�         ���>� T

�����������

�

Cp ��>���

����������������������������������������������������Ŀ

� Present State �  Input  ��  Next State   � Output  �

����������������������������������������������������Ĵ

�       AA      �   !I    ��      AA       �    P    �

�               �    I    ��      BB       �    P    �

����������������������������������������������������Ĵ

�       BB      �   !J    ��      BB       �    Q    �

�               �    J    ��      CC       �    Q    �

����������������������������������������������������Ĵ

�       CC      �  I $ J  ��      DD       �    -    �

�               �  I!$ J  ��      CC       �    -    �

����������������������������������������������������Ĵ

�       DD      �  K # L  ��      EE       �   R,T   �

�               � !K & !L ��      DD       �   R,T   �

����������������������������������������������������Ĵ

�       EE      �   !M    ��      EE       �   S,T   �

�               �    M    ��      FF       �   S,T   �

����������������������������������������������������Ĵ

�       FF      �    1    ��      AA       �  R,S,T  �

������������������������������������������������������

page 22

This machine has 5 inputs an 5 outputs and behaves as a moore machine.

Suppose we need that the outputs P,Q and T be clean outputs that do not

produce decoding spikes (because they drive for example an asynchronous reset

of a flip-flop or a write-enable input of a RAM) It is recommended to use a

One-Hot state assignment for state A and B because the P and Q outputs are

active only on state AA and BB. We will assign state AA to flip-flop Q2 and

state BB to flip flop Q3. The outputs of the flip-flops Q2 and Q3 will be P

and Q outputs respectively. The T output is not unique to only one state, so

we will not be able to use a one hot assignment in order to produce it. We

will just add a special flip-flop that will produce this output directly. This

will be flip-flop Q4. The output of Q4 is the output T (T is a direct output).

Suppose that the other outputs are allowed to have decoding spikes (because

they drive for example other synchronous devices that share the same clock).

Also suppose for example, that target device has unused buried nodes. We will

want to use the buried nodes and not waste any more output macrocells of the

device. We can choose to fully encode the other state combinations and assign

them to a pair of flip-flops called Q1 and Q0. We will assign CC to

[Q1,Q0] = [0,0], DD to [Q1,Q0] = [0,1], EE to [Q1,Q0] = [1,0] and FF to

[Q1,Q0] = [1,1]

When we use ASTATER.EXE we need only to declare to outputs (R and S), which

are indirect outputs. All other outputs (P,Q and T) are direct flip-flop

outputs. The total number of flip-flops will be 1 + 1 + 1 + 2 = 5. The number

of states is 6 and number of transitions is 2. The number of inputs is 5.

The command and command line parameters used in this example are:

ASTATER M4 5 2 2 5 /1 AA BB CC DD EE FF

page 23

Here is the file that will be created by ASTATER.EXE :

M4.ABL

��

�MODULE M4

�TITLE 'M4.ABL created by ASTATER.EXE Version 1.0

�       Time of creation is Sat Aug 20 17:07:13 1994

�      '

�DECLARATIONS

�

�   M4 DEVICE ;

�   input1 PIN ;

�   input2 PIN ;

�   input3 PIN ;

�   input4 PIN ;

�   input5 PIN ;

�   CP PIN ; "CLOCK INPUT"

�   MR PIN ; "MASTER RESET (ASYN) INPUT"

�   output1 PIN ISTYPE 'COM' ;

�   output2 PIN ISTYPE 'COM' ;

�   Q4 NODE ISTYPE 'REG,BUFFER' ;

�   Q3 NODE ISTYPE 'REG,BUFFER' ;

�   Q2 NODE ISTYPE 'REG,BUFFER' ;

�   Q1 NODE ISTYPE 'REG,BUFFER' ;

�   Q0 NODE ISTYPE 'REG,BUFFER' ;

�

�   PS = [Q4,Q3,Q2,Q1,Q0] ; "STATE ASSIGNMENT"

�   AA = [ 0, 0, 0, 0, 0] ;

�   BB = [ 0, 0, 0, 0, 0] ;

�   CC = [ 0, 0, 0, 0, 0] ;

�   DD = [ 0, 0, 0, 0, 0] ;

�   EE = [ 0, 0, 0, 0, 0] ;

�   FF = [ 0, 0, 0, 0, 0] ;

�

�   C , X , P , Z = .C. , .X. , .P. , .Z. ;

�

�EQUATIONS

�

�   PS.AR = !MR ;

�   PS.CLK = CP ;

�

page 24

�STATE_DIAGRAM PS

�

�   STATE AA :

�      [output1,output2] = [0,0] ;

�      CASE

�         condition11 : next_state11

�         condition12 : next_state12

�      ENDCASE ;

�   STATE BB :

�      [output1,output2] = [0,0] ;

�      CASE

�         condition21 : next_state21

�         condition22 : next_state22

�      ENDCASE ;

�   STATE CC :

�      [output1,output2] = [0,0] ;

�      CASE

�         condition31 : next_state31

�         condition32 : next_state32

�      ENDCASE ;

�   STATE DD :

�      [output1,output2] = [0,0] ;

�      CASE

�         condition41 : next_state41

�         condition42 : next_state42

�      ENDCASE ;

�   STATE EE :

�      [output1,output2] = [0,0] ;

�      CASE

�         condition51 : next_state51

�         condition52 : next_state52

�      ENDCASE ;

�   STATE FF :

�      [output1,output2] = [0,0] ;

�      CASE

�         condition61 : next_state61

�         condition62 : next_state62

�      ENDCASE ;

�

�DECLARATIONS

�

�   NS = [Q4,Q3,Q2,Q1,Q0] ; "NEXT STATE"

�

�TEST_VECTORS

�

�   ([MR,CP,input1,input2,input3,input4,input5] -> [NS,output1,output2])

�

�END

��

page 25

Here is the file after making changes with an editor:

M4.ABL

��

�MODULE M4

�TITLE 'M4.ABL created by ASTATER.EXE Version 1.0

�       Time of creation is Sat Aug 20 17:07:13 1994

�      '

�DECLARATIONS

�

�   M4 DEVICE ;

�   I PIN ;

�   J PIN ;

�   K PIN ;

�   L PIN ;

�   M PIN ;

�   CP PIN ; "CLOCK INPUT"

�   MR PIN ; "MASTER RESET (ASYN) INPUT"

�   R PIN ISTYPE 'COM' ;

�   S PIN ISTYPE 'COM' ;

�   Q4 PIN  ISTYPE 'REG,BUFFER' ;

�   Q3 PIN ISTYPE 'REG,BUFFER' ;

�   Q2 PIN ISTYPE 'REG,INVERT' ;

�   Q1 NODE ISTYPE 'REG,BUFFER' ;

�   Q0 NODE ISTYPE 'REG,BUFFER' ;

�

�   PS = [Q4,Q3,Q2,Q1,Q0] ; "STATE ASSIGNMENT"

�   AA = [ 0, 0, 1, 0, 0] ;

�   BB = [ 0, 1, 0, 0, 0] ;

�   CC = [ 0, 0, 0, 0, 0] ;

�   DD = [ 1, 0, 0, 0, 1] ;

�   EE = [ 1, 0, 0, 1, 0] ;

�   FF = [ 1, 0, 0, 1, 1] ;

�

�   C , X , P , Z = .C. , .X. , .P. , .Z. ;

�

�EQUATIONS

�

�   PS.AR = !MR ;

�   PS.CLK = CP ;

�

page 26

�STATE_DIAGRAM PS

�

�   STATE AA :

�      [R,S] = [0,0] ;

�      CASE

�         !I : AA

�          I : BB

�      ENDCASE ;

�   STATE BB :

�      [R,S] = [0,0] ;

�      CASE

�         !J : BB

�          J : CC

�      ENDCASE ;

�   STATE CC :

�      [R,S] = [0,0] ;

�      CASE

�         I $ J  : DD

�         I !$ J : CC

�      ENDCASE ;

�   STATE DD :

�      [R,S] = [1,0] ;

�      CASE

�          K #  L : EE

�         !K & !L : DD

�      ENDCASE ;

�    STATE EE :

�       [R,S] = [0,1] ;

�       CASE

�         !M : EE

�          M : FF

�      ENDCASE ;

�   STATE FF :

�      [R,S] = [1,1] ;

�      CASE

�         1 : AA

�      ENDCASE ;

�

�DECLARATIONS

�

�   NS = [Q4,Q3,Q2,Q1,Q0] ; "NEXT STATE"

�

page 27

�TEST_VECTORS

�

�   ([MR,CP,I,J,K,L,M] -> [NS,R,S])

�    [ 0, X,X,X,X,X,X] -> [AA,0,0]; "P IS 1"

�    [ 1, C,0,X,X,X,X] -> [AA,0,0]; "P IS 1"

�    [ 1, C,1,X,X,X,X] -> [BB,0,0]; "Q IS 1"

�    [ 1, C,X,0,X,X,X] -> [BB,0,0]; "Q IS 1"

�    [ 1, C,X,1,X,X,X] -> [CC,0,0];

�    [ 1, C,0,0,X,X,X] -> [CC,0,0];

�    [ 1, C,1,1,X,X,X] -> [CC,0,0];

�    [ 1, C,0,1,X,X,X] -> [DD,1,0];

�    [ 1, C,X,X,0,0,X] -> [DD,1,0]; "T IS 1"

�    [ 1, C,X,X,1,1,X] -> [EE,0,1]; "T IS 1"

�    [ 1, C,X,X,0,0,0] -> [EE,0,1]; "T IS 1"

�    [ 1, C,X,X,0,0,1] -> [FF,1,1]; "T IS 1"

�    [ 1, C,X,X,X,X,X] -> [AA,0,0]; "P IS 1"

�END

��

The following changes were made:

input1 -> I

input2 -> J

input3 -> K

input4 -> L

input5 -> M

output1 -> R

output2 -> S

State Assignment:

AA = [ 0, 0, 1, 0, 0] ;

BB = [ 0, 1, 0, 0, 0] ;

CC = [ 0, 0, 0, 0, 0] ;

DD = [ 1, 0, 0, 0, 1] ;

EE = [ 1, 0, 0, 1, 0] ;

FF = [ 1, 0, 0, 1, 1] ;

Inserting the true outputs of R & S

Inserting transition conditions

Changing attribute of Q2 from 'REG,BUFFER' to 'REG,INVERT'

Changing signal type of Q4,Q3 and Q2 from NODE to PIN

Adding test vector values

page 28

Errors, Warning & Note Messages

-------------------------------

If any error occurs the program will abort execution. Some errors will

generate a help screen. this is the help screen:

------------------------------------usage-------------------------------------

ASTATER <module> <in> <out> <con> <ff> /1|2 <state_name> <state_name>...

---------------------------------parameters-----------------------------------

<module>.........................Module name \(file_name = module_name.ABL

<in>...............................................Number of input signals

<out>..............Number of output signals that are not flip-flop outputs

<cond>......................Maximum number of relevant condition per state

<ff>..................................................Number of flip-flops

/1|2|3...................Machine type : /1 - Moore , /2 - Mealy , /3 Dummy

<state><state>.........................A list of state names (two or more)

------------------------------------------------------------------------------

Here is a the list of errors that the program can report:

Error (1) : Define number of inputs.

The number of inputs parameter is not defined correctly. It has to

be an integer.

Error (2) : Define number of outputs.

The number of outputs parameter is not defined correctly. It has

to be an integer.

Error (3) : Define maximum number of transition conditions.

The maximum number of outputs parameter is not defined correctly.

It has to be an integer.

Error (4) : Define number of flip flops.

The number of flip-flops parameter is not defined correctly. It

has to be an integer.

Error (5) : Define machine type - moore (/1) or Mealy.

The type of the machine is not defined correctly. It

has to be a foreslash (/) followed by an integer.

page 29

Error (6) : Machine type 'n' is an unknown machine_type.

The type of the machine number is not defined correctly. It

has to be a /1 (Moore) or /2 (Mealy).

Error (7) : Can't create file 'file_name' for writing.

Error (8) : Autonomous state machine is not supported.

Autonomous transitions are supported, but only if they are not

global. Read on previous text how to describe Autonomous

transitions using the CASE..ENDCASE syntax.

Error (9) : Too many inputs.

The maximum number of allowed inputs is 30.

Error (10) : Too many outputs.

The maximum number of allowed outputs is 30.

Error (11) : Calculated number of transition conditions is too big.

Defined number of transition conditions by user was zero. In a

situation like this the software normally chooses a number

that is equal to (2)^(number_of_inputs). In this case the

calculated number is too big.

Error (12) : Too many transition conditions.

The maximum number of allowed conditions is 4096.

Error (13) : No flip-flops in machine.

A state machine with no Flip-Flops is not a state machine. It is

a combinatorial system.

Error (14) : Too many flip-flops.

A maximum of 30 flip-flops is allowed.

page 30

Error (15) : 'n' states can not be assigned to only 'm' flip-flop(s).

If for example you tried to declare tow flip-flops and six states

it will not be possible to assign all six states to only four

possible flip-flop combinations. This is because tow flip flops

can decode only four states.

Error (16): Missing command line parameter(s).

The minimum number of command line parameters must be at least 7:

- number of inputs

- number of outputs

- maximum number of input conditions

- number of flip-flops

- machine type

- a first state name

- a second state name

Error (17): A Mealy type machine with no indirect outputs.

Declaration of a mealy machine with no outputs is impossible

because a mealy machine must have at least one output that depends

on an input and not only on flip-flops.

Here is a list of Errorlevel that are set when exiting the program:

0 - Successful execution

1 - Bad numeric values of parameters

2 - Bad definition of parameters

3 - File Overwrite not allowed

4 - File write problems

Warnings & Notes will not be followed by aborting of execution. Warnings are

for making you pay attention because some thing bad might be going on. Notes

are only for informing you.

page 31

Here is a list of Warnings that the program can report:

Warning (1) : Output file 'file_name' already exists.

The user has an option to Overwrite it (by hitting the <Y> key)

or to abort (hitting any other key)

Warning (2) : Zero outputs.

Zero outputs is possible because all outputs of the machine can

come directly from flip-flop outputs. In these cases it is

possible for the designer to enter zero as the number of outputs

(using the 0 command line parameter).

Warning (3) : The number of transition conditions was changed

from 'm' to 'n'.

If the number that defined transition conditions was zero, and

the user defined a number of inputs that is greater then zero,

the software will generate a canonical number of transition

conditions = (2)^(number_of_inputs). This can be a large number.

Other cases when this warning is reported are when the maximum

number of defined state transitions is higher than it is

possible with the defined number of inputs. For example if the

the user defined tow inputs and five conditions a warning

will be reported. It will tell you that the maximum number of

conditions was decreased, to four conditions (the maximum

possible number of conditions when the machine has tow inputs).

Here is a list of Notes that the program can report:

Note (1) : Machine type is Moore.

Note (2) : Machine type is Mealy.

Note (3) : Number of un-used states is: 'n'.

The program calculates the difference between the largest number

off possible flip-flop combinations and the actual number of states

that was defined. The designer has to pay attention to these

Hang-Up states. The machine can enter a Hang-Up state during

power-up or as a result of noise.

page 32

Limitations & Legal details

---------------------------

The software is written by and Copyrighted to - Amos Zaslavsky. ASTATER is a

shareware. This means that you are allowed to copy it freely, and give copies to

other users.

You are encouraged to do so, but with the following limitations:

- You must distribute ASTATER.EXE and ASTATER.DOC together and unmodified.

- You may not charge more than a minimal distribution fee (maximum $5.00)

for covering disk price & postage of a diskette containing the files.

- You may not distribute ASTATER with or as part of other commercially sold

software without written permission of the author.

- You are not permitted to sell this program for profit in any way, shape or

form.

- If you like this program and use it regularly, the author would appreciate a

contribution of $25.00, sent to the address below.

- As with all shareware, these contributions are the incentives for the

author to continue developing this utility and other utilities.

- Contributors suggestions for enhancements will be considered in future

versions.

- The author would appreciate being informed of any bugs !

- Future enhancements may include improovments like ABEL Symbolic-state style,

entering input & output names, an interactive version, and other HDL's like

PALASM and CUPUL or even C.

My address is:

Amos Zaslavsky

28 Berel Katzenelson st.

Haifa

Israel 32768

Telephone (voice):

972-4-230219

Thank you.