Fundamentals of FOSSIL implementation and use

Draft Version 3   March 7, 1987

Vincent E. Perriello, VEP Software

IFNA Address: Network 141 Node 491 (141/491)

Usenet address: ...decvax!envore!vaxine!spark!141!491!Vince_Perriello

Copyright (C) 1987, VEP Software, Naugatuck, CT 06770. All rights reserved.

This document may be freely used or copied by anyone interested in the data

contained herein. No fees may be charged for distribution of this document.

You will be held  accountable for all such charges,  and expected to either

reimburse those persons or organizations so charged,  or to make a donation

in the exact amount of those fees to the International FidoNet Association,

to  assist  them in their  efforts to  advance the  technology of  personal

computer telecommunications.

Fundamentals of FOSSIL implementation and use                         Page 1

A. Objectives of this document

This document is directed at implementors or intellectuals.  It is meant

for use in implementing applications that can use FOSSIL drivers, or for

details needed to implement a new FOSSIL. As such it won't always go out

of its way to explain itself to the neophyte.

This document will have served its purpose to you if you are able to use

the data contained within to perform either of the above tasks.   If you

feel that necessary data has been omitted please contact Vince Perriello

at the above listed address so that the appropriate changes can be made.

B. Historical perspective

For those people who were not lucky enough to have an IBM PC or a system

nearly completely compatible, the world has not been very friendly. With

his implementation of the Generic Fido(tm) driver,  Tom Jennings made it

possible for systems that had nothing in common with an IBM PC except an

808x-class processor, and the ability to run MS-DOS Version 2 and above,

to run his Fido(tm) software. That was a lot to ask, and a lot of people

thought it was enough.

But not everyone.  While Thom Henderson was debugging Version 4.0 of his

SEAdog(tm) mail package,  an "extended" Generic driver was designed  (in

cooperation with Bob Hartman)   as a quick kludge to help him get past a

problem with certain UART chips.The new hook was quickly pounced upon by

Vince Perriello,  who, with almost DAILY prodding (ouch! it still hurts)

by Ken Kaplan,had been working with Henderson to get DEC Rainbow support

into SEAdog. Vince then coded a driver to use this hook and - Voila! -

SEAdog 4.0 started working like a champ on the Rainbow.

Then something WONDERFUL happened. Wynn Wagner started having a bad time

getting the Greenleaf(tm) Communication Libraries to work in exactly the

way he wanted them to. Enter Bob Hartman. Having already enjoyed success

in the effort with Thom Henderson, he suggested to Wynn that with a very

few extensions, the driver that SEAdog(tm) 4.0 liked could drive Opus as

well. Then Vince called Wynn, Wynn called Bob, Bob called Vince, and the

FOSSIL driver came into existence.

FOSSIL, by the way, is an acronym that Vince dreamed up. It took a while

for everyone to stomach it,but it now seems to be loved by all. It is an

acronym for "Fido/Opus/SEAdog Standard Interface Layer". And the concept

is catching on.   Henk Wevers is reportedly coding a terminal program in

Turbo Pascal that uses FOSSIL services to do all the work.   This should

mean that the result will play elsewhere.   And there are already FOSSIL

implementations for the Tandy 2000, Sanyo 555, Z100 and others,   so the

potential of a properly coded FOSSIL application is very great.

Fundamentals of FOSSIL implementation and use                         Page 2

C. Basic principles of a FOSSIL driver

1)  Interrupt 14H.

The one basic rule that the driver depends upon,  is the ability for ANY

target machine to allow the vector for INT 14H (absolute locations 50 to

53 in segment 0) to be "stolen" by the driver. In a system where the INT

14H vector is used already, it must be possible to replace the "builtin"

functionality with that of a FOSSIL,  when an application that wants the

use of a FOSSIL is to be run on the target machine.

2)  How to install a FOSSIL driver in a system

There's no hard and fast way to do this. The FOSSIL might be implemented

as part of a device driver (like Ray Gwinn's X00.SYS) and therefore gets

loaded using a line in CONFIG.SYS at bootup time.  It might be done as a

TSR (terminate and stay resident) program, in which event you install it

by running the program  (DECCOMM by Vince Perriello and Opus!Comm by Bob

Hartman work this way, for example).

3)  How an application can detect the presence of a FOSSIL

The driver has a "signature" that can be used to determine whether it is

present in memory. At offset 6 in the INT 14H service routine is a word,

1954 hex,  followed by a byte that specifies the maximum function number

supported by the driver. This is to make it possible to determine when a

driver is present and what level of functionality it provides. Also, the

Init call (see below) returns a 1954 Hex in AX.  SEAdog(tm) looks at the

signature and Opus just goes for the Init. Fido doesn't do either.

4)  How to call a FOSSIL function

The FOSSIL driver is entered by issuing a software Interrupt 14 Hex from

the application  program. The code corresponding to the desired function

should be in 8-bit register AH. For calls that relate to communications,

the port number will be passed from the application in register DX. When

DX contains a zero (0) it signifies use of COM1, or whatever the "first"

serial port on your machine is called. A one (1) in DX points the driver

at COM2, and so on.

Fundamentals of FOSSIL implementation and use                         Page 3

D. Functions currently defined for FOSSILs

AH = 0	Set baud rate

Input:	AL = baud rate code

DX = port number

This works the same as the  equivalent IBM PC BIOS call,  except that it

ONLY selects a baud rate.  This is passed in the high order 3 bits of AL

as follows:

000 =  110 baud

001 =  150  ''

010 =  300  ''

011 =  600  ''

100 = 1200  ''

101 = 2400  ''

110 = 4800  ''

111 = 9600  ''

The low order 3 bits can be implemented or not by the FOSSIL, but in all

cases, if the low order bits of AL are 00011,  the result should be that

the communications device should be set to eight data bits, one stop bit

and no parity. This setting is a  MINIMUM REQUIREMENT  of Fido, Opus and

SEAdog.

AH = 1	Transmit character

Input:	AL = character

DX = port number

Output: AX contains status bits (see function 3)

AL contains the character to be sent.   If there is room in the transmit

buffer the return will be immediate,  otherwise it will wait until there

is room to store the character in the transmit buffer.  On return, AX is

set as in a status request (see function 3).

AH = 2	Receive a character

Input:	DX = port number

Output:	AL = input character

If there is a character  available in the  receive buffer,  returns with

the next character in AL.  It will wait until a character is received if

none is available.

Fundamentals of FOSSIL implementation and use                         Page 4

AH = 3	Request status

Input:	DX = port number

Output:	AX = status bit mask (see below)

Returns with the line and modem status in AX.  Status bits returned are:

In AH:

Bit 0 =	RDA  - input data is available in buffer

Bit 5 = THRE - room is available in output buffer

Bit 6 = TSRE - output buffer is empty

In AL:

Bit 7 =	DCD  - carrier detect

This can be used by the application to determine  whether carrier detect

(CD) is set,  signifying the presence/absence of a remote connection, as

well as monitoring both the input and output buffer status.

AH = 4	Initialize driver

Input:	DX = port number

( BX = 4F50

CX = ^C flag address --- optional )

Output:	AX = 1954H if successful

This is used to tell the driver to begin  operations,  and to check that

the driver is installed. This function should be called before any other

communications calls are made.  At this point all interrupts involved in

supporting the comm port (specified in DX) should be set up for handling

by the FOSSIL, then enabled.  If BX contains 4F50 hex,  then the address

specified in DS:CX is that of a ^C flag byte in the application program,

to be incremented when  ^C is detected in the keyboard service routines.

This is an optional service and only need be supported on machines where

the keyboard service can't (or won't) perform an INT 1B or INT 23 when a

Control-C is entered.

AH = 5	Deinitialize driver

Input:	DX = port number

This is used to tell the driver that comm port operations are ended. The

function should be called  when no more comm port functions will be used

on the port specified in DX.

AH = 6	Raise/lower DTR

Input:	DX = port number

AL = DTR state to be set (1 = Raise, 0 = Lower)

This function is used to control the DTR line to the modem.	AL = 0 means

lower DTR (disable the modem), and AL = 1 means to raise DTR (enable the

modem).  No other function (except Init) should alter DTR.

Fundamentals of FOSSIL implementation and use                         Page 5

AH = 7	Return timer tick parameters

Output:	AL = timer tick interrupt number

AH = ticks per second on interrupt number in AL

DX = approximate number of milliseconds per tick

This is used to  determine the parameters of the timer tick on any given

machine.  Three numbers are returned:

AL =   Timer tick interrupt number

AH =   Ticks per second on interrupt number shown in AL

DX =   Milliseconds per tick (approximate)

Applications can use this for critical timing  (granularity of less than

one second) or to set up code  (such as a watchdog)  that is executed on

every timer tick. See function 22  (add/delete function from timer tick)

for the preferred way of actually installing such code.

AH = 8	Flush output buffer

Input:	DX = port number

This is used to force any pending output.   It does not return until all

pending output has been sent.  You should use this call with care.  Flow

control  (documented below)  can make your system hang on this call in a

tight uninterruptible loop under the right circumstances.

AH = 9	Purge output buffer

Input:	DX = port number

This is used to purge any pending output.   Any output data remaining in

the output buffer (not transmitted yet) is discarded.

AH = 10	Purge input buffer

Input:	DX = port number

This is used to purge any pending input.   Any input data which is still

in the buffer is discarded.

AH = 11	Transmit no wait

Input:	DX = port number

Output:	AX = 1 if character was accepted and 0 if not.

This is exactly the same as the "regular"  transmit call, except that if

the driver is  unable to  buffer the character  (the buffer is full),  a

value of zero (0) is returned in AX. If the driver accepts the character

(room is available),  a one (1) is returned in AX.

Fundamentals of FOSSIL implementation and use                         Page 6

AH = 12	Read no wait

Input:	DX = port number

Output:	AX = next character or FFFF if none available

Return in  AX  the next character in the receive buffer.  If the receive

buffer is empty return FFFF.

AH = 13	Keyboard read no wait

Output:	AX = IBM-style keyboard scan code or FFFF if

no keyboard character available

Return in  AX the next character  (non-destructive read ahead)  from the

keyboard;  if nothing there  (in the type-ahead buffer),  return FFFF in

AX.   Use IBM-style  function key mapping  in the high order byte.  Scan

codes for non-"function" keys are not specifically required,  but may be

included.

AH = 14	Keyboard input with wait

Output:	AX = IBM-style keyboard scan code

Return in AX the next character from the keyboard;  wait if no character

is available. Keyboard mapping should be the same as function 13.

AH = 15	Enable or Disable flow control on transmit

Input:	AL = bit mask describing requested flow control

DX = port number

This is used to stop output when the "other end" is overwhelmed, or when

a BBS user wants to stop a screen.

Two kinds of flow control are supported:

Bit 0 = 1	XON/XOFF

Bit 1 = 1	CTS/RTS

Flow control is enabled, or disabled, by setting the appropriate bits in

AL  for the types of flow control we want to ENABLE (value = 1),  and/or

DISABLE  (value = 0),  and calling this function.  Bit 2 is reserved for

DSR/DTR but is not currently supported in the DECCOMM implementation.

Applications  using this  function  should set all bits  ON  in the high

nibble of AL as well.  There is a compatible  (but not identical) FOSSIL

driver implementation that uses the  high nibble as a control mask.   If

your application sets the high nibble to all ones,  it will always work,

regardless of the method used by any given driver.

Fundamentals of FOSSIL implementation and use                         Page 7

AH = 16	Extended Control-C / Control-K checking and transmit on/off

Input:	AL = flags byte

DX = port number

Output:	AX = status (see below)

This is used for BBS  operation,  primarily.  A bit mask is passed in AL

with the following flags:

Bit 0	enable/disable Control-C / Control-K checking

Bit 1	disable/enable the transmitter

The Enable (bit 0 = 1) and Disable (Bit 0 = 0) Control-C/Control-K check

function is meant primarily for BBS use. When the checking is enabled, a

Control-C or Control-K received  from the communications port will set a

flag internal to the FOSSIL driver,  but will not be stored in the input

buffer.  The next use of the function will return the value of this flag

in register AX then clear the flag for the next occurrence. The returned

value is used by the BBS  software to determine whether output should be

halted or not.

The Enable (Bit 1 = 1) and Disable (Bit 1 = 0) Transmitter function lets

the application restrain the asynchronous driver from output in much the

same way as XON/XOFF would.

AH = 17	Set current cursor location.

Input:	DH = row (line)

DL = column

This function looks exactly like like INT 10H, subfunction 2, on the IBM

PC. The cursor location is passed in DX: row in DH and column in DL. The

function treats the screen as a coordinate  system whose origin (0,0) is

the upper left hand corner of the screen.

AH = 18	Read current cursor location.

Output:	DH = row (line)

DL = column

Looks exactly like INT 10H,  subfunction 3,  on the IBM PC.  The current

cursor location  (using the same coordinate  system as  function 17)  is

passed back in DX.

AH = 19	Single character ANSI write to screen.

Input: AL = character to display

The character in AL is sent to the screen by the fastest method possible

that allows ANSI processing to occur (if available). This routine should

not be used in such a way that DOS output  (which is not re-entrant) can

not be employed by some FOSSIL driver to perform the function  (in fact,

on the IBM PC that is likely to be how it's done).  On some systems such

as the DEC Rainbow this will be a very fast method of screen writing.

Fundamentals of FOSSIL implementation and use                         Page 8

AH = 20	Enable or disable watchdog processing

Input:	AL = 1 to enable or 0 to disable watchdog

DX = port number

When watchdog is enabled,   the state of the carrier detect (CD) line on

the comm port specified in DX should be constantly monitored. Should the

state of that line become FALSE (carrier lost), the system should be re-

booted, to enable the BBS (or other application) to start up again.

AH = 21	Write character to screen using BIOS support routines

Input:	AL = character to display

The character in AL is sent to the screen using  BIOS-level Input/Output

routines. This differs from function 19 in that DOS I/O CAN	NOT be used,

as this function might be called from driver level.

AH = 22	Insert or Delete a function from the timer tick chain

Input:	AL    = 1 to add a function or 0 to delete one

DS:DX = address of function

Output:	AX    = FFFF if unsuccessful

This function is used to allow a  central authority  to manage the timer

interrupts, so that as code is loaded and unloaded, the integrity of the

"chain" is not compromised.  Rather than using the traditional method of

saving the old contents of the timer vector, storing the address of your

routine there,  and executing a far call to the "old" routine when yours

is done, instead you call this function. It manages a list of such entry

points and calls them on a timer tick (interrupt) using a FAR call.  All

the usual cautions about making DOS calls apply (that is, DON'T!).

This makes it possible for a program to get in and out of the tick chain

without having to know whether another program has also done so since it

first insinuated itself.

AH = 23	Reboot system

Input:	AL = 0 for "cold boot" or 1 for "warm boot"

Perform the old 3-finger salute.  Used in extreme emergency by code that

can't seem to find a "clean" way out of the trouble it has gotten itself

into.  Hopefully it won't happen while you're computing something in the

other half of a DoubleDOS system. If your machine can make a distinction

between a "cold" (power-up, self-test and boot) and a "warm" (just boot)

bootstrap,  your FOSSIL should support the flag in AL. Otherwise just do

whatever bootstrap is possible.

Fundamentals of FOSSIL implementation and use                         Page 9

E.  Validation Suite.

Well, there is one, but it's involved.  Run FIDO_GEN V11w on your FOSSIL

for about a week with  reasonable caller activity  in the message areas,

file uploads and downloads,  and FidoNet(tm) mail.  Run SEAdog for a few

days,  sending and receiving a reasonable  number of messages.  Then run

Opus for the  rest of your natural life.  Why not?  It's a good package.

Anyway,  some reasonable  compromise with the  above steps should ensure

that your FOSSIL will work with most everything coded for one.

F.  Distribution.

The good folks who  distribute Opus will be more than happy to take note

of your new FOSSIL driver and to add it to their distribution.  And,  of

course, you can upload it to any system you want, but your work will get

more publicity in the Opus community than anywhere else.

G.  Technical Discussion.

A FOSSIL echomail conference is in the process of starting up.   It will

be coordinated by Vince Perriello at IFNA node 141/491.  Contact him for

details on how to join. Keep in mind though, this conference is intended

SPECIFICALLY  for implementors of  FOSSIL  software and not as a general

Q&A conference,  for people who think  FOSSILs have something to do with

paleontology.