Lens Evaluation Software (LES)

Ver 4.0  Aug. 11, 1995

LES is a geometrical raytrace program which can be used to evalu-

ate optical systems.  It accommodates systems with tilted and decentered

components  and with rotationally symmetric aspheric surfaces.   It will

trace real and paraxial rays, produce ray fan tables, and plot spot dia-

grams.   Extensive context sensitive help is provided so that  no user's

manual or detailed external documentation is needed.  Version 4.0 of LES

is FreeWare and may be distributed without restriction.

Computation is in double precision and an  8087/80X87 math copro-

cessor is optional but recommended if processing speed is desired.   LES

was written in Pascal and has been compiled with Microsoft's Pascal Com-

piler version 3.32.  Graphics are provided for spot diagrams only.

Each lens in the  library uses 4342 bytes.   A minimum of 320k of

RAM is required  with which  twelve lenses  can be accommodated.  If RAM

resident  programs  are installed,  this number  may vary.   Each 64k of

additional RAM permits  storage of fourteen more lenses.   If an attempt

is made  to keep  a lens when  there is  insufficient  room in RAM,  the

program will exit to DOS without saving the current library to disk.

Optical path differences  calculated in this program are with re-

spect to the chief ray.   Except in special cases,  therefore,  programs

which compute  these differences  with respect  to some other  reference

will produce different values.

Run time errors  will generally result in an exit to  DOS without

saving the  current library.   Operations which could  cause such errors

(analyzing systems at abnormally large field angles, for example) should

be avoided unless the working lens library is safely ensconced on a non-

volatile disk.

None of the run time errors discussed above will lose the library

on your disk, at worst, you will lose only the additions made during the

current session.

If you are using DOS 4.0 or later, you may print graphics from an

EGA/VGA display to an  IBM or Epson  compatible printer by first loading

"GRAPHICS.COM"  and then printing  with the  "<PrintScreen>"  key.   For

earlier versions of  DOS,  third party  software such as  "GRAFPLUS"  by

Jewell Technologies, Inc. can be used.  If you use an EGA/VGA video card

without such  software and wish to print spot diagrams,  select the  "C"

option when prompted.  This will put the display into the  CGA  mode for

graphics  from which  you can print  with the  "<Shift>  <Print Screen>"

keys.  In this case you must also have previously loaded "GRAPHICS.COM".

The executable program is loaded with the command:

les [d:][path]filename[.ext]

[d:] to specify the drive that contains the lens library file.

[path] to specify the directory path that contains the lens library

file.

filename[.ext] to specify the name of the lens library file.

(The parameters enclosed in square brackets are optional.  If an

extension has been included as part of the filename, however, it

must be included with the filename in the command to load.)

This command may be executed from the  DOS command line or from a

batch file.  If the library filename is omitted from the initial call to

LES, the user will be prompted for a library filename.   It is possible,

therefore, to have a number of lens library files, each with a different

filename.  To use a different library file, however,  it is necessary to

return to DOS and reload.

A lens library called  "LENSLIB"  is included with this  release.

It contains  several sample  lenses and can  be used to  build a working

lens library.

You can create an empty  lens library file by using the  DOS sub-

command REM with redirection.  To do this type:

REM > [d:] [path] FILENAME [.EXT]    (Either upper or lower case,

and where the parameters in

brackets ( [] ) are optional.)

This will  create a zero byte  file called  FILENAME.   You you can then

load the  program with this new,  but empty,  file and use it  to create

another library file called FILENAME.

A first time user may wish to become familiar with the program by

going through this exercise.   First enter the L)ibrary path,  L)ist the

lenses there,  F)etch the lens called  SINGLET, go to the top level path

using /)Up,  then begin the  D)escribe path.   Here the user could check

the R)adii,  spacings and indices;  then the M)aterials and P)arameters.

A next step would be to return to the top level using /)Up,  then to the

A)nalyze path where a  P)araxial trace is done followed by a single R)ay

or F)an.  Then /)Up to the top level and Q)uit the program.

For those users  who are not familiar with many of the  terms and

techniques used in optical system evaluation, a file,  WALKTHRU.TXT,  is

included for their benefit.   Remember, however, to make use of the help

options that may be accessed from within the program.

The program authors are  Don Nicholson and Nelson Wallace who may

reached be through:

Don Nicholson

2124 Linda Flora Dr.

Los Angeles, CA 90077-1409

(310) 476-4413  (10:00 - 17:00 Pacific Time)

(310) 476-9914 (fax)

dnich@alumni.caltech.edu