Northern Lights FrontEnd v1.0 (11/03/92)

(c) Copyright 1992 by J.C. Kilday Associates

Developed at the Northern Lights BBS

207-761-4782   v.32bis/VHST

Table of Contents

-----------------

Introduction ....................  1

Description .....................  2

Other Answering Modes ...........  2

Installation ....................  3

Distribution and Restrictions ...  5

After You Register NLFE .........  6

Support .........................  6

NLFE v1.0                         Page 1

Introduction

------------

The Northern Lights FrontEnd is a host communications program intended

for use with PCBoard BBS software  (see Note 1, below).  It's  purpose

is to  frontend that  code and  to allow  selection of  an alternative

program to take  the call.   It was developed  at the Northern  Lights

specifically to  accommodate UUCP  calls.   (The UUCP  term is derived

from "Unix to Unix Copy Program" and denotes the facility available on

many Unix systems for the exchange of files among systems.)

UUCP is the means by which many sites are "connected" to the  Internet

allowing  the   exchange  of   e-mail  (private   messages)  and   the

participation  in  Usenet  newsgroups  (public  forums  consisting  of

articles  posted  for  all  to  read  and  with  various  options  for

accommodating responses, both public and  private).  UUCP also may  be

used for  the transfer  of text  and binary  files among  sites.   One

interesting and very  convenient way this  may be accomplished  is via

e-mail.

UUCP implementations provide  minimal scripting capability  with which

to establish a connection and effect the exchange of information.  For

this reason, it is not practical to attempt support of UUCP via a door

which  is  the  common  way  PCBoard  and many other systems might run

alternative  applications.    There  would  be  too many "expect/send"

sequences for UUCP-style scripts  to reliably handle, and  usually too

much  intervening  text  between  these  sequences  from  the  initial

"CONNECT" to the point of entry of a door-based UUCP application.  The

Northern  Lights  FrontEnd  (abbreviated  as NLFE, pronounced "nelfy")

provides a very simple interface for handling UUCP calls while  adding

only one additional  step for regular  callers (or those  automated by

sophisticated scripts) desiring normal access to the BBS.

A UUCP script needed for login at the Northern Lights is as simple  as

this:

"" "" "" gin:--gin: uucp NEW:--NEW: uucpID word: uucpPASSWORD

where  uucpID  and  uucpPASSWORD  would  vary  with  the  site that is

calling.  The "uucp" string following "gin:"  is the string to send to

invoke the UUCP facility (and is a configurable setting within  NLFE).

The Northern Lights, by the way, is using the Waffle BBS package  (see

Note 2, below) to implement its UUCP service.

Regular callers respond to NLFE's "Login:"  prompt with virtually  any

keystroke(s) followed by <ENTER> to login into the normal BBS.

NLFE is designed for operation on an IBM-compatible computer connected

to  a  Hayes-compatible  modem,  including  most  of  today's v.32 and

v.32bis modems.

---

Note 1: PCBoard is a product of Clark Development Co., Inc.

Note 2: Waffle is a product of Darkside International and Thomas Dell.

NLFE v1.0                         Page 2

Introduction (Cont'd)

---------------------

The complete NLFE v1.0 package includes the following files:

NLFE.DOC     - this file

NLFE.COM     - executable module

NLFE.OVR     - NLFE "overlay" file

CONFIG.HLP   - context-sensitive help file

BOARD.BAT    - sample PCBoard BOARD.BAT file using NLFE

UUCP.BAT     - an example of the "alternate commmand"

FEORDER.FRM  - order form for registering NLFE

READ.ME      - quick review information for SYSOPS

Description

-----------

The Northern Lights FrontEnd is normally the active program when  your

system is awaiting a call.  Control of the program is by means of  the

function keys  as defined  in the  legend at  the bottom  of it's call

waiting and status screen.

When NLFE is awaiting a call,  a display including the date and  time,

some key setup information, and present front-end status is shown.  If

you are running a multi-node PCBoard system, and have provided for the

PCBNODE environment variable in each node, your screen also  indicates

the node number.  A screen blanker turns off the screen if there is no

keyboard activity within a 2-minute  span.  Striking any key  restores

the display.  The  display also is toggled  on and off by  hitting F9.

F10 presents information about the  program.  F3 allows access  to the

configuration screen where various  NLFE setup options are  specified.

F1 or F2 cause the Northern  Lights FrontEnd to terminate and to  take

the BBS offline.  F1 leaves the telephone line in a BUSY state  (modem

offhook) while F2 leaves the modem onhook.

When  the  front-end  is  operational,  you  no longer have continuous

access  to  your  board  software's  waiting  screen  and  the sysop's

functions/displays that  it provides.   However,  you may conveniently

access your board software through use of NLFE's F8 key to "Force next

call to  BBS".   Hitting F8  brings up  the board  at the call waiting

screen with the normal keys and displays available.  Should a call  be

received at this point, however, the regular BBS code would handle  it

(and handle it inappropriately should it  be a UUCP call).  NLFE's  F8

key is intended primarily to bring up the board's call waiting  screen

in the  event you  wish to  do a  local login.   The  normal keys  for

exiting  the  main  BBS  code  will  return  control  to  NLFE if your

BOARD.BAT is set up correctly.  (See the sample BOARD.BAT included.)

Instead of  initial messages  from your  BBS software,  a caller first

sees text produced by the  front-end upon connecting.  The  registered

version of the Northern Lights FrontEnd displays a line of text (up to

65 characters  which you  have specified),  followed by  "Login:"   on

another  line.    The  caller  may  enter any transmittable characters

followed by  <ENTER> (so  long as  it is  not a  null entry,  that is,

<ENTER> alone) in order to access  the BBS per normal.  Having  a part

of  the  65-character  text  line  contain "Hit <SPACE><ENTER> for the

BBS." seems to have caused minimal confusion at the Northern Lights.

NLFE v1.0                         Page 3

Description (Cont'd)

--------------------

Entry of a sysop-configurable text string at the login prompt  invokes

the alternate facility (UUCP process, for example).  If a null  string

is entered (<ENTER> only)  no more then twice,  the "Login:"  will  be

displayed again on a new line.  Upon the third entry of a null  string

the front-end will disconnect and recycle.  (No response to the  login

prompt  also  results  in  a  disconnect  after an appropriate waiting

period.)

The unregistered version of NLFE displays additional text and  inserts

a  6-second  delay  before  the  65-character  greeting line and login

prompt are displayed.   The additional  text and delay  are eliminated

after completing the the self-registering procedure described later in

this document.

Other Answering Modes

---------------------

The Northern Lights FrontEnd  normally is set up  to answer a call  on

the first  ring.   But where  there is  need for  the host computer to

share  a  phone  line  and  not  intrude  on  voice  calls, or perhaps

accommodate  Caller-ID,  special  answering  modes  may be configured.

"Long Ring" may  be configured such  that NLFE answers  only after the

specified number of  rings have been  received.  Alternatively,  "Ring

Back" may be chosen.  This mode requires a caller to dial twice before

NLFE answers.  The first call consists of a specified number of  rings

(or NLFE can be configured for  a non-specific number of rings on  the

first call).  NLFE answers a second call if received within one minute

of the first that lasted the proscribed number of rings.

Installation

------------

The Northern Lights FrontEnd is designed to operate under DOS 3.0, and

up, on  IBM compatibles  equipped with  Hayes 1200-compatible  modems,

which includes most of  today's crop of v.32  and v.32bis modems.   If

NLFE has  been configured  to answer  on true  ring detect,  the modem

cable used must have  pin 22 (ring indicate)  connected.  Also pin  20

(Data Transmitter Ready signal) must  be connected and the modem  must

be  capable  of  providing  result  messages  indicating  baud rate at

connect time (e.g., CONNECT 1200).  Recommended Hayes switch  settings

are:

SW 1 - Up, DTR normal (pin 20 controlled by program)

SW 2 - Up, Verbal result messages sent (not numeric result codes)

SW 3 - Dn, Result messages sent (not quiet mode)

SW 4 - Dn, Commands not echoed back

SW 5 - Dn, Auto Answer suppressed (controlled by program)

SW 6 - Up, Carrier detect indicated by modem (not always on)

SW 7 - Up, Single phone line connection

SW 8 - Dn, Normal "AT" command set enabled

If you are using  a modem other than  the Hayes standard (and  most of

you will be), make sure that its basic configurable functions are  set

up the same as above.  If your modem does not have switches for one or

NLFE v1.0                         Page 4

Installation (Cont'd)

---------------------

more of the above functions,  there should be appropriate AT  commands

to achieve the same state.  Please consult your modem owner's manual.

(Note: Sorry to drag experienced sysops through the foregoing....but

that text is necessary for the sake of completeness.)

To install the  software, copy NLFE.EXE,  NLFE.OVR, and CONFIG.HLP  to

any directory in your DOS PATH (or to the default directory when  NLFE

is invoked).  The package is designed to support multi-node  operation

so, only one copy is needed  on your hard drive (or in  your operating

RAM disk).

To  configure  NLFE,  run  it  once  in  each  of  your  node  working

directories.   This is  to create  a configuration  file, NLFE.CFG, in

each directory which is the current working directory for each of your

nodes.  You may start it up  in LOCAL mode to accomplish this so  that

it won't look  for your modem(s),  send intitialization strings,  etc.

Local mode is invoked by entering an "L" (ignore quotes) as a  program

parameter (Example:  NLFE L ).

Initially, the NLFE.CFG file cannot be found, you are so informed, and

the  configuration/setup  screen  is  presented with default settings.

Use the F1  key to display  the available help  for each configuration

item as you proceed through  the various settings.  Some  items should

have the  same settings  as in  your board  setup for  the given node.

Items that should  be the same  are:  modem  initialization, comm port

specification, opening  speed and  whether port  is locked  or free to

autobaud, time of your daily event, number of rings before  answering,

and answer on true ring detect (Y/N).

When you  have finished,  hit the  F10 key  to save  the configuration

information.  After NLFE returns to its status screen, hit F1 or F2 to

quit operations in LOCAL mode.

Next, you must configure each BBS node on which you will be running

the front-end.  There are 4 things you must do:

1. Force  the board  code to  recycle to  DOS at  the end of each call

(allowing  your  BOARD.BAT  to  take  certain  actions  and invoke the

front-end for  the next  call).   PCBoard users  should run  PCBSETUP,

select F - Options 1, and answer Y to "Exit to DOS After Each Call".

2. Place  a copy  of a  file named  RET_CMD.SYS in  each of  your node

directories (or place in only one directory on your system but provide

a fully pathed  reference to it  in your BOARD.BAT).   The RET_CMD.SYS

file  should  include  one  line  containing  the same text string you

specified in the NLFE configuration screen for "BBS Command:".

3. Next, modify  the BOARD.BAT file  for each node  to operate in  the

manner documented in the sample BOARD.BAT supplied with this  package.

This is the  most critical part  of the installation  process.  Errors

here can cause some very puzzling  results.  It is suggested that  you

use the logic exactly as documented in the sample.

NLFE v1.0                         Page 5

Installation (Continued)

------------------------

4. Lastly, set up the alternate  process.  It is recommended that  you

specify in the NLFE config screen at the "Alt Command:"  an expression

such as:  "CALL UUCP" where UUCP is the name of a .BAT file (any  name

following  "CALL "  in  the  config  screen  will  do,  so long  as it

corresponds to the batch file to be CALLed).  NLFE writes out a  batch

file called FE_CMD.BAT containing that statement if the caller selects

such  action  at  the  "Login:"    prompt.    The caller "selects" the

alternate function by  entering the string  you have specified  as the

"Alternate Command Select String:".

At  the  Northern  Lights,  for  example, the alternate command select

string is "UUCP" (ignore the quotes -- this is not case sensitive,  by

the  way).    The  the  alternate  command  is  "CALL UUCP".  A sample

alternate command batch  file is provided  in UUCP.BAT.   The %1 shown

carries the speed of the caller connection (even though your comm port

may be locked).

After completing the above for all BBS on nodes on which the front-end

is to operate, the installation is finished.

Distribution  and  Restrictions

-------------------------------

The  Northern  Lights  FrontEnd  is  made  available to you as a fully

functional  package.    It  is  distributed  as SHAREWARE.  Should you

decide to use it,  you are obliged to  pay a registration fee  of $20.

Registered users are provided support via the BBS mentioned at the end

of this documentation.  Use the form FEORDER.FRM for registering  your

copy.

You are welcome to distribute this package to others so long as it  is

unmodified and distributed in its entirety, and it is not included  or

bundled  with  other  goods  or  services  for which a fee is charged.

Exceptions are  that NLFE  may be  distributed by  bulletin boards  or

other information  services even  though they  receive fees  to access

their downloadable files,  or by library  services so long  as the fee

for the diskette on which this  package is contained is not more  than

$6.50.

Use of the Northern Lights  FrontEnd beyond a brief evaluation  period

by  individuals,  business  entities,  corporations,  and   government

agencies is prohibited  without payment of  a registration fee  of $20

per   copy   (volume   purchasing   and  site  licensing  arrangements

available).

NLFE v1.0                         Page 6

After You Register NLFE

-----------------------

Once  your  registration  has  been  received,  you  will  be  sent  a

registration  number.    You  may  use  this  number  to  convert to a

registered copy of  NLFE which will  remove the display  of EVALUATION

COPY, disable the SHAREWARE screen  with its brief delay presented  to

each caller, and insert your name  as the registered user.  To  make a

registered  copy,  invoke  NLFE  with  the  command  line parameter of

REGISTER (Example:   NLFE REGISTER ).  You are prompted  to enter your

name and your registration number.  That's all there is to it.

Support

-------

Support is  available to  registered users  of NLFE  through the  main

message base of Northern Lights BBS which can be reached at the number

given below.   Please note that  support is provided  only for use  of

NLFE  on  equipment/software  for  which  it  was  designed (described

earlier).

Registration fees may be made payable to:

J.C. Kilday Associates

P.O. Box 1961

Portland, ME 04104

The author may be contacted at the above address, or via Internet  (as

jkilday@nlbbs), or through the  Northern Lights BBS (address  messages

to Jack Kilday) at  207-761-4782.  The BBS  operates 24 hours, 7  days

per week.  Calls accepted up to 14400 bps (v.32bis or VHST).