HDDCHECK V2.00, copyright (c) 1997, 1998 by Frank Berreth.

Syntax: hddcheck [/?] [/h] [/q] [/o] [/c] [/b buffer_size] [drive numbers ...]

/? or /h prints this text.

/q excludes verbose information in the logfile.

/o uses the old INT13 method.

/c read only one track at once.

/b specifies the buffer size used to read the drive.

'driver number' is a drive number (1-4) of the drive that should be

checked. You can specify more than one value.

You can try using the /o or /c or both options if you encounter problems with

this program. Do not use these options when it is not really necessary, they

are only for supporting a VERY old BIOS.

Use option /b to change the buffer size for reading. Each read operation will

be monitored, so the buffer size is a very important parameter. Normally the

program tries to determine the right buffer size, but just in case ... The

buffer size is specified in kilobytes and should be a power of 2 (like 32).

Don't be confused when the program shows a CHS (0/0/0) ... information. This

is wrong, but some SCSI BIOS just report that, the prg. will work correctly.

WHAT DOES THIS PROGRAM

======================

'HDDCHECK' reads your complete hard disk structure, regardless of installed

operating systems, and displays any known errors. In addition to this,

'HDDCHECK' analyzes the damage status of the magnetic surface on your hard

disk and forecast a possibly drop out within the next few weeks.

HOW DOES IT WORK

================

'HDDCHECK' reads any sector of your hard disk. If a sector appears to be

erroneous, an intelligent IDE drive tries to compensate the fault using

internally stored error correction code (ECC) and copies this sector to a

previously reserved area, which is (hopefully) error free. This operation is

completely transparent to all operating systems like WinNT or Linux and is

called remapping. Note that all SCSI drives I know are a little 'silly' in

that point, cause *you* have to start a surface scan from the menu of your SCSI

controller BIOS that will do a remapping when there is a erroneous sector.

Because of this automatic remapping, there'll be a good chance to keep data

undamaged or at least to spot erroneous sectors if you check your hard disk

periodically with this program.

Is that all?

------------

That's what many programs do (like the surface scan from 'scandisk'), although

they can't check multiple partitions at once, they can't check unknown file

systems, they can't check 'between' partitions etc. etc. ppp.

But this program goes further! It tries to spot 'weak' sectors (or sector

blocks) that are not reported to the operating system. With that information,

the prg. should be able to warn you when your drive is standing on the cliff.

Tomorrow, it could be one step further ;)

Into deep ...

-------------

Usually, the hard disk's firmware reserves a few sectors for the remapping of

faulty sectors. No operating system is aware of this extra space, because

the remapping is transparently handled by the firmware. Of course, if these

special sectors are all in use, no attempts will be launched to ECC-correct

the sector and remap it afterwards. A disk with a lot remaps is also slower

than a less erroneous drive because of extra reload operations.

Things get worse if there is actually a 'weakened' sector that should be

remapped, but all reserved sectors are used. Your hard disk could only try

to read this sector until it can be corrected with ECC. Have in mind that

information is still stored analogous, so this makes sense (Note: This

re-read operations are time consuming, the context with HDDCHECK will be ex-

plained later). If your hard disk has read this sector several times but

could not correct it, an error will be reported to the OS. If a hard error

is reported, there'll be only a very slim chance to get intact data of this

sector with repeated read operations. Fact: All disk utilities like 'scandisk'

can/will only display hard errors. At that time it's already to late and your

data is lost.

This is where HDDCHECK kicks in

-------------------------------

This theory has lead to a program which reads your hdd in handy blocks and

measures the time for each read operation. Those time values are stored in a

diagram that is written into a log file.

The time units used are timer ticks, where one tick is about 0.84 microsec.

The diagram first shows the time and than how many read operations have been

accomplished in this interval. The protocolled time varies from 3000 ticks to

50000 ticks. Each hdd has it's individual distribution of these values and

it'll vary in each test. Normally, the peak should be at 5000-7000 ticks for

your drive. In the best case, there are no values in the >50000 time slot.

Any read operation that takes more than 50000 ticks is considered abnormally

long and the sector block will be stored as a 'weak' sector in your logfile

if you didn't used the /q option.

When it comes to judge your drive, only this weak sector count is used.

You have told me a lot, what about some test examples

-----------------------------------------------------

Enough on theory, some real world tests showed that this program is genius.

In my first case I tested a hdd which behaves suspiciously. The used

operating system (WinNT) sometimes made entries in the event log indicating

drive problems, but mostly WinNT crashed without even a warning or a blue

screen. The log file after the test reviewed _many_ values above 50000 ticks

(about 800 read operations took more than 50000 ticks). Note that the hdd

only sometimes reported one or more bad sectors (read errors) on a regular

basis to the OS. This explains why NT only sometimes reported an error into

the event log. Note also, that the position of the bad sector reported by the

hdd varied which seems to be senseless. In conclusion, normal test programs

like 'scandisk' mostly don't report errors. You also have trouble to test

more than one partition or a whole hdd regardless of the used file system.

Think about it: The traditionally scanned hdd could have a partially damaged

surface and you will never know for sure. Have in mind that 'scandisk' cannot

help you, because reported errors by scandisk are typically real, uncorrect-

able errors as stated above.

After this initial test there were many other checks following, but the hdd's

I have had for testing seemed to work OK over half a year. Note, that there

must be proofed, that hdd's first show some hard errors and than more and more

hard errors and that HDDCHECK tells you that there will be errors just before

the first one shows up.

OK, I must apologize, but until now I only have one another example which is

not the best you can get. We have bought two equal hdd's at the same time,

where one of them was build in a WinNT server and the other one was stored

for future use. After 3 month the server behaves suspiciously. Once again,

HDDCHECK, which was first run on that hdd, tells that this hdd is destined to

fill our trash can. The second hdd, which was unused till now and checked by

HDDCHECK got the same judgment, although this hdd does not report any

errors. I'm sorry that I did not check the WinNT server hdd before it was

build in.

I know, obviously this test does not tell much because the surface need not

to be of the same quality, but there is a good chance.

Is there something I should know

--------------------------------

OK, I say that we can try to classify between a good hdd (with intact surface)

and a hdd with defecting surface. Note that the test does not tell you any-

thing about a head crash or motor loss, that means these are dangerous

failures now and forever. Although, 'HDDCHECK' does not check the logical

structure of your file systems, use whatever prg. you normally use for this.

If you start 'HDDCHECK', the program scans all hdd's in your system. After

the scan of one hdd, a protocol file is created, named 'hddX.chk', where X is

the number of your hdd. If there exists a log file, the protocol is appended.

If your hard disk really reports an error, the error text is immediately

printed on screen, but not appended to the logfile. After the test, a quick

result is printed. Note that this classification of your drive is done with

really no great use of artificial intelligence. It is recommended that you

personally view the log and make your own opinion. Have in mind that there

should not be more than 30-50 read operations beyond 30000 ticks on a normal

drive. Because your old log is not deleted, you can compare these logs to see

if something gets worse. Check your drive periodically (let's say every week)

and backup all your critical data (think of head crash or crazy programs).

GOOD LUCK

LEGAL STUFF

===========

This program was written by Frank Berreth. All rights are reserved.

You have permission to use and distribute 'HDDCHECK' (executable format)

for free. You cannot sell it, but you can demand a handling charge for

distributing it on a media.

This software is provided 'as is' without express or implied warranty.

The author is not responsible for any damage caused by this program.

The idea and the algorithm, as described above, is a copyright of Frank

Berreth. You are not allowed to implement this algorithm in any of your

programs. If someone wants to port this application to e.g. Linux, email

me!

e-mail: fberreth@altavista.net

A FEW LAST WORDS ...

====================

The first version of the prg. had some kind of really severe judgment. If

there showed up one hard error the prg. said you should trash your HDD, but

on SCSI disks this could be 'normal', just remap (manually).

Looking at the weak sector count, the judgment there was far to strict too.

It's always a good idea to look at the logfile and think about what's standing

in this document, so you can make your own opinion about the test. A good help

will be looking at the logfiles that are provided with this program, read the

'readme.txt' there!

But thinking of tests, you can really help me improving the prg. This is not a

usual give-me-money-plz-plz-plz begging, keep your money, I need your logfile!

I'm in a need of logfiles for removable disks, for various HDD's etc. etc.

If you want to help me, then email me your logfile *once* of all of your drives

and every time the judgment of the program changes create a new logfile (best

would be to rename the HDDx.CHK files to something else and then start the

scan) and email it to me.

THANKS

If you find this program useful or brain dead, drop me a mail in both cases!

(fberreth@altavista.net)