Forthplay and Romplay Documentation and Comments

Date:  09Jul89

If you have this file, you should also have copies of

ROMPLAY.BAS, ROMPLAY2.BAS, ROMPLAY3.BAS, and 4THPLAY.COM,

CD-Audio players for the Hitachi CDR1503S or CDR3500 CD-ROM

Drives and PC bus interface cards.

Use Romplay & Romplay3 for keyboards with function keys across

the top (enhanced layout).  Romplay2 is better for the original

keyboard.  Romplay is intended for those times you want to do

something other than compute.  Forthplay is better for starting

play or checking a drive.

Required:

Hitachi CDR1503S, CDR3500, or 100% compatible CD-ROM

drive.

Hitachi PC bus interface card: CDIFI2, CDIFI3A, CDIFI4, CDIFI4A,

CDIF25A, CDIF25A2, or CDIF35A

300H base address for Forthplay, Romplay base is changeable (see

line 10010).

For Romplay, GWBASIC (Non-IBM, about 80K file size), or the

BASICA version for IBM, (if you really have an IBM), or with

Tandy & Compaq, the BASICA version with Tandy or Compaq MS-DOS.

On a monochrome system, you will need to edit the program colors

and may need to change some color indicators and graphics

chracters.  First try changing the colors defined around 10000.

Blue/green paper must black, tip colors set to white/lwhite.  It

may be less trouble to use Forthplay and auto.

Romplay Functions:

E     Eject Disc

A     Accept Disc - Close Tray when present and open

S     Sample first 17 seconds of each track

P     Play - Start to End, Next Que, or Section Repeat

<   >   Reverse/Forward 1 Track    -  ,  .

<<<   >>> Reverse/Forward 1 minute   -  {  }

<<   >>  Reverse/Forward 10 seconds -  [  ]

N     Number Play - Enter Start & End track numbers

------   Spacebar toggles Pause On/Off - clears modes

C     Cue to start of current track or release cue/pause

Q     Quit - Immediately go to DOS with clear screen

1-9    First digit of number play entry sequence

R     Repeat Play from Starting to Ending Track

T     Toggle Que On/Off for use at next Play or end

of current play

B     Begin - Mark currently played location for Section

Play

F     Finish - Section Play end - enable mode for play

(Must be greater than Begin location) (Press Play

to start Section Play - Space Bar to exit)

M     Manage Que - set mode for Que functions

/     Shuffle Play (Makes new random shuffled Que and plays)

D     Drive (0-3) - Select CD-ROM drive for program

Manage Que Functions:

< >   StepQue - Rotate stack of Que entries

F    Fill Que with all tracks in reverse order

C    Clear Que to 0 entries

S    Shuffle all Que entries

I    Insert number into current bottom of Que  (prompt)

1-9   First digit of insert  - before prompt

D    Delete number from Que (prompt)

T    Toggle Que Mode - Manage or normal modes

Notes: Spacebar or Return/Enter accepts digits when valid

Enter 0 to cancel number delete or insert

When digits expected - only digits accepted

Spacebar or other keys will exit Manage mode

< > StepQue operates from keyboard when Que is on

Slide Cue Time Scale Operation and Use:

The time scale shows the current minute position and relative

time remaining.

It can be used to cue directly to any minute of the disc using

the cursor keys.

Cursor left/right - Slide reverse/forward 1 minute

Cursor up         - Move laser to selected minute and read

time/track code

Cursor down       - Play to end of disc from selected location

The normal program mode is automatic repeat play of the entire

disc from start to finish.  This mode is active when other modes

are clear.  When another mode is selected, prior modes are

cleared.  Pause, or skip functions clear other modes, and restore

play to end operation.

When the program is run, it checks to see if a I/F card is

installed at the address. If card/drive isn't there, it will list

line 10010 for editing.  PRTA=&H300 is the base address for Port

A on the card.  If something else uses the address, it should not

answer drive commands in the next step.  Then the program trys a

drive command to see if a drive answers.  If no answer, changes

the drive number and trys again.  When no drive answers, decides

the user forgot to turn it on, and waits to rerun.

The program updates the screen times, watches for end of play, a

keypress,  or change in drive status, until something happens.

If a CD is playing when the program is started, or when the drive

number is changed, it obtains the current play location, and

returns to it after finding the maximum times.

Since basic is too slow to read the Disc Table of Contents (TOC)

which lists the track times, it finds the maximum Minutes &

Seconds through use of a clever "hack", by attempting play at 99

minutes, then trying lower numbers until it sees the drive in

play mode, then counts up until it goes to pause again.  At that

point, it backs up two seconds and reads the track number on the

disc.  Doing it this way allows you to have a player in basic

with Source code.  The Forth program just reads the TOC and gets

the times, but Forthplay doesn't have all of the Romplay

features.

Times are used for display and skip wrap-around, while the

maximum track number is needed for other operations.

If your host is very fast, you may need to use slow mode or add

some delay loops to let the max time routine operate properly.

On ROM tracks the times change to yellow or light red.

The drive remembers only the last start/end time given to it by

the program.  It will continue to play when the program is

exited.

The drive is intended for data transfer and it performs audio

functions only through host control and monitoring of its

operation.

Forthplay Commands:

Play       ( n n Play ) ( n Play ) ( play )

n is starting track n is ending track, ie:   2 5 Play

n is starting and ending track,  ie:  3 play

Play plays tracks 1-99, or to disc end

Tune       Shows current Track and Index, if playing

Time       Time in Track and Time within entire disc

Mode       Reports Drive Status, except busy and lock

TOC        Generally shows Table of Contents for installed disc

List of tracks, start times, and end time of disc

Channels   Reports status of mute bits to be used at next play

Stereo     Enable both channels at next play

Left       Enable left channel at next play

Right      Enable right channel at next play

Mute       Mute channels at next play

Eject      Ejects disc when not locked

Accept     Closetray

CloseTray  Accepts disc - CDR1503S only

Lock       Disables front panel Open/Eject button

Unlock     Enables Open/Eject button on drive

Drive      ( n Drive )  Sets Drive (0-3) for commands, ie: 3

Drive

Drives     Reports active CD-ROM drives

Romdrive   Reports current CD-ROM drive used for commands

Xplay      ( n n n n n n xplay ) ( n n n xplay ) ( xplay)

Xplay     Extended play by min, sec, frame, to min, sec, frame

0 0 0 99 59 74 is default

0-6 arguments when stack is empty - if works strange

retry

Pause occurs when times are invalid

Rplay      Replay last xplay, Stime & Etime Setting

Stime      Set start location for repeat, must be playing disc

Etime      Set end location for repeat

Menu       Shows Command menu

Bye        Exits immediately to DOS on active drive

Credits    Resets Drive and I/O card to inital setting - shows

opening

Auto       Starts automatic play like Romplay - partial

implementation

Forthplay consists of words added to the F83 Forth language.

With Forthplay & F83 forth, the contents of the current line to

the left of the cursor is read when Enter is pressed.  Correct

typing errors with the Backspace key and retype the remainder of

the line.  The editor you will be using is the one provided as

part of this version of the language.

Type one of the commands on the Menu screen and press

Enter/Return.

Commands which require numbers need the numbers before the

command, with the numbers separated from each other and the

command with a space.  This is rather similar to some calculators

which use "reverse Polish" number entry.  (ie: number and += and

second number and += or -  etc.)

Forth accepts numbers using a "stack" to hold the numbers.  Words

which need numbers expect to find them on the stack.  Words which

obtain numbers, leave the numbers on the stack for other words to

use.  This is why numbers must precede command words which use

them.  Entering the required numbers separately from the command

is possible, so that entering:

1

10

PLAY

works the same as:

1 10 PLAY

which will cause the drive to play tracks 1 to 10, and then

pause.

2 PLAY

will play only track 2.

1 0 32 2 59 74 XPLAY

will play from minute 1, second 0, frame 32, to minute 2, second

59, frame 74.

20 XPLAY

will play the remainder of the disc from 20 minutes.

Xplay defaults to 0 0 0 99 59 74, limiting is done, but no error

checks.

TOC provides a list of track start times by track number.

Once read for a disc, .TOC will show it again.  If TOC seems to

be showing times for a prior disc, do DRIVES which will clear the

old disc status.

When Forthplay is first started, use  DRIVES  then  MODE to see

if everything is working.

Forthplay doesn't error check.  Once it starts, you're in a live

programming language which assumes that you know what you're

doing.

There are many words not on the Menu screen, so if you type a

word forth doesn't know, it will print the word followed by ?

and clear any numbers from the stack.  Some of the command words

were selected to use alternate names because Forth already used

the first word chosen for another function. If you type a word

Forth knows without the arguments it needs, you may lock up the

system or leave invalid values for the next word used.  If you

bomb out, power down for 15 seconds, and cold start to be sure

your system is clean.

You can get a list of words used to make the program by entering

Words

Press spacebar to stop listing.  The last words defined are

listed first.  All words before EMPTY are part of Forthplay.

Enter  SEE wordname  to see the contents of a word.  Try:

SEE ACCEPT

Always Exit Forthplay with Bye and Romplay with Quit to reset the

Interface Card and make sure DMA channel 3 is clear.  Although my

programs don't use it, the interface card may be enabled if you

exit improperly, causing panic when the Hard Drive on your XT

doesn't work.

These programs talk only to the Hitachi I/F Card and Drive.

Everything else is handled by F83 or basic.

Forthplay and Romplay are private works not written for hire, but

rather for my own use.  They are not supported by any Hitachi

company or division, and the code they use may not be supported

on future models.  You use the programs or code at your own risk,

they represent my understanding of hardware commands.

Background:

My first audio player software for CD-ROM drives was Forthplay.

It came alive on the evening of December 6, 1987, based on the

hardware docs for program developers from Hitachi Sales Corp. of

America.   Most of Forthplay was done on a Compaq portable the

following weekend.  Without the fine work of Laxen and Perry on

F83, neither Forthplay or Romplay would exist.

The availability of Forthplay later led to use of several

CDR1503S drives by Knotts Berry Farm (a southern California

amusement attraction), when they decided to use Forth and Hitachi

drives for audio applications.

A version of Forthplay was also done for the original Atari 800

computer, with interfacing being handled through joystick ports

1-3.  Worked nicely.

Romplay started as a lark during a three day weekend in Feb. 88.

It was begun to see what could be done with basic.  Suprised me

that anything was possible, let alone Romplay.  I consider it a

"good hack".

Romplay met a few CD-ROM insiders at the 1988 Microsft CD-ROM

conference in Seattle, Wa, and one of the Nimbus people got the

first copy.  Along the way, a few others received copies to use

in demos.  Afterwards, I added features every couple of months,

until it reached its present state.  Romplay3 was a search for a

better screen layout, with the use of character graphics for a

BIG time display.  At 4.47MHz, it skips a second display every

now and then.  A full graphics diplay and mouse is needed to do a

Mac-like picture of a CD audio player with all the buttons

working.

As part of the distribution disk, you may have versions of

Allplay and Romlock, a set of small executeable utilities.

Allplay issues a play all tracks command to drives 0-3, and

LOCK300.EXE and KCOL300.EXE which lock and unlock the drives at

various addresses.  Allplay may be named PLAY300.EXE or have a

valid base address as part of its name.

Forthplay and Romplay versions below 1.0 are Freeware and may be

copied as needed.  They are dedicated to all the people who have

created free software for any computer system.  Thanks...  Roscoe

Tomcat Softworks

Contact:  71777,2564