PrintGL 1.28 HP-GL Commands

(C) Copyright Ravitz Software Inc. 1990,1992

Ravitz Software Inc.                              BBS/FAX 606-268-0577

P.O. Box 25068

Lexington, KY  40524-5068

USA                                  Compuserve Cary Ravitz [70431,32]

Introduction ....................................................... 2

HP-GL Commands                                                       3

HP-GL Character Sets .............................................. 13

p-2

Introduction ---------------------------------------------------------

This   document  is   a   description  of  the   subset   of   HP-GL

(Hewlett-Packard  Graphics  Language)  that is supported by PrintGL.

This  information was included in  PRINTGL.DOC with PrintGL 1.24 and

earlier shareware packages.  It is  now in a separate  file to  make

printing the PrintGL reference manual easier.

If  you are using PrintGL to work with commercial graphics software,

then you will probably  not  need this manual.  If  you are  writing

HP-GL code you will find here  the information  needed to  make your

HP-GL compatible with PrintGL.

p-3

HP-GL Commands -------------------------------------------------------

PrintGL's HP-GL specification is taken from the  programming manuals

for the HP 7475A, ColorPro (7440/17440), and 7550A.  The 7550 manual

is  the  best reference for HP-GL - HP  manual 07550-90001.  PrintGL

supports  the 7475, and 7440/17440 command sets except for character

set 8 (Katakana), digitize commands, output commands, the error mask

command, and some device control  commands.  Supported commands  are

listed below.  Commands that are not applicable to printing, such as

pen velocity, are  considered supported  but are completely ignored.

Commands that are not supported are flagged and then ignored.

In addition, PG,  AF, ES, LO, the enhanced IW command, character set

5, and proportional spaced fonts, from the 7550, are supported.  And

forty pens are allowed instead of eight.

In reading a plotfile, PrintGL ignores carriage returns, line feeds,

and nulls  except where  they may be valid characters (LB,  SM,  DT,

esc.), and ASCII EOF (026) is interpreted as an end of file mark.

AA x_center , y_center , arc_angle [, chord_angle]

arc absolute

AA moves the pen through an arc, starting at the current position,

with the pen in the current up/down state.

AF

advance full page (7550 extension)

AF starts a new plot. PrintGL starts every page with the pen up at

0,0.

AP

automatic pen pickup - IGNORED

AR x_center_delta , y_center_delta , arc_angle [, chord_angle]

arc relative

AR moves the pen through an arc, starting at the current position,

with the pen in the current up/down state.

CA character_set

designate alternate character set

See HP-GL Character Sets for a list of character sets.

CI radius [, chord_angle]

circle

CI draws a  circle centered at the current position.  The  current

up/down state and position are unchanged by a CI command.

p-4

CP [spaces , lines]

character plot

CP  moves  the  pen by character spaces (+ is  along the text) and

lines  (+  is  up),  using  the current up/down  state.  Using  no

parameters is the same as a carriage return/line feed.

CS character_set

designate standard character set

See HP-GL Character Sets for a list of character sets.

DF

default values

This sets default values for the following commands:  CA,  CS, DR,

DT, ES, FT, IW, LO, LT, PA, PT, SC, SM, SL, SR, SS, TL.

DI [run , rise]

absolute character direction

DI sets the baseline direction for labels.  The default is 1,0.

DR [run , rise]

relative character direction

DR  sets the baseline  direction  for labels.  Run  and rise are a

percentage of P2X-P1X and P2Y-P1Y. The default is 1,0.

DTt

define label terminator

The label terminator ends an LB command. The default is ASCII 003.

EA x_opposite_corner , y_opposite_corner

edge rectangle absolute

EA  draws a  rectangle defined  by  the  current position  and the

parameters. The pen position and up/down state are unchanged.

EC

enable cutter - IGNORED

EP

edge polygon

EP draws  the  current  polygon  edge.  Any  pen  up/down used  in

creating the polygon is respected in drawing the edge.

p-5

ER x_opposite_corner_delta , y_opposite_corner_delta

edge rectangle relative

ER draws a rectangle with corners defined  by the current position

and the command  parameters.  The current pen position and up/down

state are unchanged.

ES [ space [, line]]

extra space (7550 extension)

ES adds to the space between characters and  lines.  The units are

in character cells (1.5*width, 2*height).  The allowed range is -1

to 1. -.3333 causes characters to touch.

EW radius , start_angle , sweep_angle [, chord_angle]

edge wedge

EW draws a wedge centered at the current position. The current pen

position and up/down state are unchanged.

FP

fill polygon

FP  fills  the  current  polygon.  Pen up/down  commands  used  in

creating the polygon are ignored.

There is a limit of 2000 fill line segments per polygon.

FT [type [, spacing [, angle]]]

fill type

These are  the  fill types.  Type  2  is identical to  type 1  for

PrintGL.

1  solid lines spaced at half the pen thickness (see PT command)

2  unidirectional solid lines spaced at half the pen thickness

3  lines at the specified spacing

4  cross hatched lines at the specified spacing

5  type ignored

A spacing  parameter  of 0 sets  the  spacing  to 1%  of the P1-P2

diagonal. Spacing is in x axis units (this is only relevant if the

axes have different scaling).  PrintGL supports any  integer angle

for fill. Cross hatched lines are at angle + 90.

IN

initialize

p-6

IP [p1x , p1y [, p2x , p2y]]

sets scaling points P1 and P2

IP sets the scaling points (P1 and P2) in plotter units. P1 and P2

are used by several commands to help with size independent plots.

IW [x_lower_left , y_lower_left , x_upper_right , y_upper_right]

input window

IW sets a  clipping window in plotter units.  With  no parameters,

the clipping window is turned off.

If  PrintGL's  /YE switch is  set and user  scaling  is on then IW

parameters are in user units.  In this case the window is rescaled

with an IP command, but any SC command locks the current window.

LB..text..terminator

label

Labels use  the current character set, based on CA, CS, SA, and SS

commands. See HP-GL Character Sets for font specifics.

LO [ position_number ]

label origin (7550 extension)

LO sets the label origin.  The label origin relative to the  label

is  shown in the diagram below.  For positions 4..9 and 14..19 the

label buffer is limited to 150 characters. The default is 1.

13                     16                      19

3......  .......  ...6...  .......  ......9

.     .  .     .  .     .  .     .  .     .

12 2     .  .     .  .  5  .  .     .  .     8 18

.     .  .     .  .     .  .     .  .     .

1......  .......  ...4...  .......  ......7

11                     14                      17

LT [pattern_number [, pattern_length]]

line type

PrintGL's line type  patterns  are close to, but  not identical to

HP's.  Pattern length is a percentage of the P1-P2 diagonal with a

default  of 4%  and a minimum  of 4 dots for types  1 and 2 and 16

dots for other types.

no parameter  solid            1  �               �

2  ��������        ��������

0             dots at          3  ������������    ������������

endpoints        4  ������������  � ������������  �

only             5  ����������  ��  ����������  ��

6  ������  ��  ��  ������  ��  ��

p-7

PA [x , y] [, x , y] ...

plot absolute

PA sets absolute mode and optionally moves the  pen in the current

up/down state.

PD [x , y] [, x , y] ...

pen down

PD sets  the pen  down and optionally  moves  it with  the current

absolute/relative mode.

PG [i]

new page (7550 extension)

PG starts  a new  plot.  The parameter is ignored.  PrintGL starts

every page with the pen up at 0,0.

PM i

polygon mode

PM 0 starts polygon mode.  The current position is the first point

in the polygon.

PM 1 starts  a new  subpolygon.  The  position after the next  pen

movement is the first point in the subpolygon.  The first pen move

of a subpolygon is done with the pen up, regardless of the current

up/down status.

PM 2 ends polygon mode.

PM  1 and PM 2 close  the current subpolygon with  the current pen

up/down state. If used without first starting polygon mode (PM 0),

PrintGL adds an implied PM 0 before processing the PM 1 or PM 2.

The following  commands are accepted in polygon  mode: PM, PA, PR,

PU, PD,  AA,  AR,  CI  (circles  are  automatically  prefixed  and

suffixed by PM 1), and IN (ends polygon mode).

The polygon buffer is limited to 500 data points.

PR [x_delta , y_delta] [, x_delta , y_delta] ...

plot relative

PR sets relative mode and optionally moves the  pen in the current

up/down state.

PS

paper size - IGNORED

PrintGL ignores PS. The paper size must be set with the /L option.

p-8

PT [thickness]

pen thickness

PT specifies twice the line spacing used in solid fills and may be

from 0 to 5.0 mm. It is reset to .3 mm with each SP command.

PU [x,y] [,x,y] ...

pen up

PU  raises  the  pen  and  optionally  moves it with  the  current

absolute/relative mode.

RA x_opposite_corner , y_opposite_corner

shade rectangle absolute

RA  fills a rectangle with corners defined by the current position

and the command parameters.  The current pen  position and up/down

state are unchanged. The rectangle edge is not drawn.

RO [angle]

rotated coordinate system

RO or RO0 sets up a normal coordinate system. RO90, the only other

allowed  value, sets up a system that is rotated  90 degrees.  The

type of rotation depends on the paper size.  This command does not

change the scaling points, so an IP command is useful after an RO.

The HP 7475 specifies two different meanings for RO90 depending on

the  paper  size (A/A4 or B/A3).  PrintGL  decides  which  type of

rotation to do based on the sum of  the  print  window  width  and

height.  If this sum  is  greater than 24 inches  then large paper

rotation is used. Otherwise, small paper rotation is used.

Small paper rotation:              Large paper rotation:

RO 0             RO 90             RO 0             RO 90

y��������Ŀ   0,0��������Ŀy     0,0��������Ŀy    y��������Ŀ

�        �      �        �         �        �      �        �

�        �      �        �         �        �      �        �

0,0����������x    x����������         �        �      �        �

�        �      �        �

�        �      �        �

�        �      �        �

x����������   0,0����������x

RR x_opposite_corner_delta , y_opposite_corner_delta

shade rectangle relative

RR  fills a rectangle with corners defined by the current position

and the command parameters.  The current pen  position and up/down

state are unchanged. The rectangle edge is not drawn.

p-9

SA

select alternate character set

SA specifies the alternate character set as current. The alternate

character set is chosen with CA.

SC [xmin , xmax , ymin , ymax]

scaling

SC  sets user units for subsequent drawing.  The units are defined

by  user values for P1 (xmin,ymin)  and  P2 (xmax,ymax).  Using no

parameters  turns  scaling off, and  subsequent drawing is done in

plotter units (1/1016 inch).

SI [width , height]

character size absolute

SI sets the capital letter  box  width and height  in centimeters.

Letter  spacing  is  1.5  *  width and line spacing is 2 * height.

Using no parameters with B or A3 paper is the  same as SI.285,.375

and any other paper is the same as SI.187,.269.

SL [tan_angle]

character slant

SL sets the character slant.  The parameter is  interpreted as the

tangent  of  the angle from vertical.  Using no parameter sets the

slant to 0 degrees.

SM[c]

symbol mode

SM  sets symbol  mode, which centers  the symbol mode character at

any plotted point.  Any character from ASCII 033 to 127 except ";"

sets symbol mode. Any other character turns symbol mode off.

SP pen_number

select pen

SP selects a pen.  Using 0 or no parameter stores the pen  without

selecting a new pen.  This is usually done at the  end of a  plot.

PrintGL ignores any drawing with pen 0.

SR [width , height]

character size relative

SR sets the capital letter box width and height as a percentage of

P2X-P1X  and P2Y-P1Y.  Letter  spacing is 1.5  *  width  and  line

spacing  is  2  *  height.  Using no  parameters  is the  same  as

SR.75,1.5.

p-10

SS

select standard character set

SS specifies the standard character  set as current.  The standard

character set is chosen with CS.

TL [positive_length [, negative length]]

tick length

TL sets the tick lengths  used by XT and YT.  The  parameters  are

lengths above  and  below the current position as a  percentage of

P2Y-P1Y for x-axis ticks and  a percentage of  P2X-P1X  for y-axis

ticks.

UC [[pen_control ,] x_delta , y_delta] ...

user character

UC specifies  a user defined character.  The pen control number is

99 for pen down  or -99  for pen  up.  Pen movements are relative,

with x_delta in  units  of 1/4 the character  width and y_delta in

units of 1/8 the character height.

VA

adaptive velocity - IGNORED

VN

normal velocity - IGNORED

VS

pen velocity - IGNORED

WG radius , start_angle , sweep_angle [, chord_angle]

shade wedge

WG fills a wedge centered at the current position. The current pen

position and up/down  state are unchanged.  The wedge  edge is not

drawn.

XT

x-axis tick

XT draws a vertical line, with length specified by the TL command,

at  the current position.  The pen position  and up/down state are

unchanged.

p-11

YT

y-axis tick

YT draws  a  horizontal  line,  with length  specified  by  the TL

command, at the  current position.  The  pen position  and up/down

state are unchanged.

esc.(

esc.Y

plotter on

Esc.(  and esc.Y turn the plotter on.  This is only relevant  with

the /YY option.

esc.)

esc.Z

plotter off

Esc.) and esc.Z turn the plotter off. They are only effective with

the /YY option.  The plotter on  and off commands let you  include

comments  or  data  to  pass through  to  another  device  in  the

plotfile. These are flagged as unsupported commands if /YD is set,

as an indication that you should set /YY.

esc.@, esc.H, esc.I, esc.M, esc.N, esc.R

RS-232 device control commands - IGNORED

p-12

HP-GL Character Sets -------------------------------------------------

PrintGL  supports all of the character sets that  are included  with

the HP 7475 and 7440/17440 except Katakana, and adds character set 5

and  the  proportional  spaced  fonts   from  the  7550.   PrintGL's

characters are similar to but not identical to HP's.

0  ANSI ASCII

1  9825 character set

2  French/German

3  Scandinavian

4  Spanish/Latin American

5  Special Symbols (7550 extension)

6  JIS ASCII

7  Roman Extensions

8  Katakana - NOT SUPPORTED BY PRINTGL

9  ISO IRV (International Reference Version)

30  ISO Swedish

31  ISO Swedish For Names

32  ISO Norway Version 1

33  ISO German

34  ISO French

35  ISO United Kingdom

36  ISO Italian

37  ISO Spanish

38  ISO Portuguese

39  ISO Norway Version 2

The character sets specify a font for codes 033..127. Codes 000..032

and 128..255 are independent of the character set and are all no ops

except these.

003  default label terminator

008  backspace

009  half backspace

010  line feed

011  reverse line feed

013  carriage return

014  select alternate character set

015  select standard character set

032  space

Each of  the  character sets  is available as a proportional font by

adding 10 to the set number.  The proportional  fonts  are kerned to

improve  appearance (the character spacing is adjusted to compensate

for  character shape).  Use the LO  command to simplify working with

proportional fonts. To improve the appearance of large text, use the

ES command to reduce the character spacing.

Codes 65 to 81 in character set 5 are symbols designed to be used in

symbol mode (SM command) or as one character labels.  When used in a

label (LB command) these characters  are centered at the current pen

position and leave the pen position unchanged.

p-13

The  HP-GL  characters cannot  be printed  with  the  standard ASCII

character  set.  The file HPGL.CHR is an HP-GL file that can be used

to print a  replacement  for this page, showing  the character sets.

Use  the command PRINTGL  HPGL.CHR  /M1  /AO0,0  with  a  /F  option

specific to your printer to do this.