CMS Communication System

User's Manual

V2.05

Copyright (C) 1993 by MTH Software

Contents

1.    Installation

1.1   Hardware installation

1.2   Software installation

1.3   Configuration

1.4   Activate Communication Manager

2.    Send

2.1   Send file

3.    Download

3.1   Download file

4.    Receive

4.1   Receive file

4.2   Show received files

5.    Utilities

5.1   Show outgoing files

5.2   Stop outgoing files

5.3   Restart outgoing file

5.4   Delete file

5.5   Sendlog

5.6   Mailboxlog

5.7   Mailboxdirectory

5.8   Phonedirectory

5.9   Create text files

Appendix

A:    User defined modem configuration

B:    Compatibility

C:    Protocol

1.   Installation

1.1  Hardware installation

Connect  your  modem  with  a  modem  cable  to  the  first free

communication port of your computer.

CMS for DOS can use one of four predefined port addresses:

COM1      0x3F8     INT 4

COM2      0x2F8     INT 3

COM3      0x3E8     INT 5

COM4      0x2E8     INT 5

CMS  for  Windows  uses  the  Windows  communication driver. The

addresses  can  be  selected at the Windows system configuration

panel.

If  you  are  using  an  internal  modem  card,  be  sure  it is

configurated to one of these port addresses and interrupt number

and there is no other device using the same address or interrupt

level.

Connect your modem to the phone line and switch your modem on.

Internal modem cards do not need to be switched on.

1.2  Software installation

If  you are using a harddisk, we recommend to copy all the files

from the CMS masterdisk to a subdirectory of your harddisk.

Insert  the CMS floppy disk into drive a: and type the following

commands:

C:          (Enter)

CD\         (Enter)

MD CMS      (Enter)

CD CMS      (Enter)

COPY A:*.*  (Enter)

1.3  Configuration

Here  you  will  give  CMS  informations  about  your  modem and

computer hardware.

Start CMS by typing:

CMS   (Enter)

Select  "Configuration"  in the "Utilities" menu and fill in the

following fields.

Modemtype

---------

Press (F1) and select your modem in the listbox.

If you don't find your modem in the modem list, you have to make

a user defined modem installation as described in Appendix A:

Communication port

------------------

The number of the communication port used by your modem.

1 =   COM1

2 =   COM2

3 =   COM3

4 =   COM4

DOS: CMS will used predefined port addresses and interrupts.

WINDOWS : CMS will use the windows communication ports.

Incoming rings

--------------

After this number of rings CMS will answer the phone.

e.g.  If  you  put  a 2 here CMS will answer the phone after the

second ring.

WINDOWS: CMS will always answer after the first ring.

Dialprefix

----------

Here you put the dialprefix commands, your modem needs to

get a dialtone and to dial.

e.g.

T       Tonedial.

P       Pulsedial.

T0W     Tonedial, dial a 0, wait for dialtone.

P0W     Pulsedial, dial a 0, wait for dialtone.

Number of retries

-----------------

This is, how many times CMS will try to send the file in case of

line busy or interrupted transmission.

Printertype

-----------

Put  a  1  if  you  use  a  dot matrix printer, a 2 if you use a

HP-Laserjet compatible laser printer.

Editor

------

The name of your ASCII text editor you want to use to prepare

textfiles.

1.4  Activate Communication Manager

The  CMS  Communication  Manager is called once and stays in the

background to wait for incoming calls or transmission orders.

DOS: Type CMSR (Enter) at the DOS command line.

WINDOWS: Start CMSW.EXE by clicking on it from the file manager.

Note: CMSR.EXE may not be used under Windows.

If  a  message  shows  that  the modem setup failed, you have to

check again your modem configuration.

CMS  is now ready to receive or transfer files.

You  can  test  the  configuration  by being called at the phone

number  where  your modem is connected. After the first ring the

CMS  Communication Manager window will appear and the modem will

answer the phone.

DOS: If necessary you can remove CMSR from memory by typing

CMSR -R  (Enter)

2.   Send

2.1  Send file

Start CMS by typing CMS (Enter) at the DOS prompt.

WINDOWS:  When sending files CMS.EXE should be used in a DOS Box

set to window mode instead of fullscreen mode to avoid

transmission delays.

Usually  DOS  boxes  can  be used always in fullscreen

mode,  if  the modem receives an incoming call Windows

will  switch  out of the DOS box into Windows and CMSW

will answer the call.

Select  SEND FILE and enter the name of the file in the FILENAME

field.

e.g. C:\LETTERS\TEST.DOC  (Enter)

You  can  send  any  kind  of file, including binary and graphic

files.

If you want to send textfiles as electronic mail, you should use

the  file extension .DOC for files using the IBM-ASCII character

set, and .TXT for files using the ANSI character set.

These  are  the standard file extensions for CMS electronic mail

text files.

Future  versions  of  CMS will use this extensions for automatic

conversion of character sets.

In  the  next  field  you  have  to type the phone number of the

destination CMS computer.

If  you  have  already  created a phone directory you can select

phone numbers by pressing F1.

How to create a phone directory is described in chapter 5.8. You

can  also  select , without using F1, by typing the shortname of

the CMS destination.

The field REMARKS should be completed with the filename and the

name of the sending station.

e.g.  TEST.DOC FROM FASTWORK INC.

Now  you set the time when the file should be transmitted. (e.g.

20.30) or type (Enter) to transmit immediately.

The  speed menu is used to adapt the transmission speed to known

bad line conditions.

You reduce the speed if you want two highspeed modems to connect

at  a  lower  speed.  This  can  be  necessary  at long distance

connections.

If  you would not reduce the speed of a highspeed modem on a bad

phone  line  the  modems  would try to connect always at maximum

speed which would always result in an interrupted transmission.

Select the desired transmission speed and press (Enter).

Now  you  will  be asked if you want to use high compression. If

you press (Y) the file will be compacted using high compression.

If  you  press  (N)  the  file will be compacted in shorter time

using normal compression.

Now  your  file  has  been  transfered  to the CMS Communication

Manager.  After  5-20 seconds the CMS window will appear and the

modem will start dialing.

If  it  was  not  possible  to  send the file, the Communication

Manager  will  try  to  send the file after 30-70 seconds again.

The   maximum   number  of  retries  has  been  defined  in  the

configuration screen.

If  you  do  not  want  CMS  trying to send the file select STOP

OUTGOING FILES at the UTILITIES menu.

Using  the option RESTART OUTGOING FILE you can activate sending

of the selected file again.

3.   Download

3.1  Download file

Downloading a file means to copy a file from another computer to

the own computer.

To download a file you select DOWNLOAD FILE and type the name of

the desired file.

Every  CMS Mailbox should have a file named INFO.DOC which shows

filenames and other information.

Now you type or select the phone number of the CMS mailbox.

If  the  mailbox  requires a password for downloading files type

your password into the field PASSWORD, otherwise leave it empty.

Select transmission time and speed as described in chapter 2.1.

New: Our Service Mailbox

Download  the  file  INFO.DOC (without password) for more infor-

mation.

CMS Service Mailbox: (++49) 69-648 257 68     (300-14400bps)

4.   Receive

4.1  Receive files

Receiving  of  files  is done automatic by the CMS Communication

Manager (CMSR.EXE or CMSW.EXE).

If  loaded,  calls will be answered and incoming files stored on

your disk.

4.2  Show received files

To see received files select RECEIVED FILES at the CMS menu.

You  will see all received or downloaded files, where downloaded

files are marked with <-.

All  files  are  sorted  in order of reception and new files are

marked with *.

This * stays until you view or uncompress the file.

New files always stay at the end of the received file list.

You select a file by using the cursor keys and press (Enter).

Now  you  will be asked, if you want to view the file. Press (Y)

to  view  the  file. Only textfiles with max. 80 chars. per line

will be shown in a useful manner.

After this you have the possibility to print the file.

To  uncompress  and  copy the file into a directory of your disk

type  the  desired  filename.  If  a file with this name already

exists,  a  warning  appears and you can decide to overwrite the

old file.

After  this you will be asked if you want to delete the received

file, press (Y) if you are sure you don't need the file anymore.

5.   Utilities

The utility functions can be found at the UTILITIES menu.

5.1  Show outgoing files

This function shows send and download orders which have not been

processed.

The  listbox  shows  date,  time,  remarks  and  the  number  of

remaining dial retries.

5.2  Stop outgoing files

If  you  want to set the retry count of outgoing files to 0, use

this function.

This function sets the dial retry count of all outgoing files to

0 and no more dialing occurs.

If  the Communication Manager window already controls the screen

press (ESC) to interrupt the transmission.

WINDOWS:  If  using  CMSW.EXE the transmission can not be inter-

rupted by pressing (ESC). Mouse and keyboard are disabled during

transmission.

5.3  Restart outgoing file

If  you  want  the Communication Manager to start sending a file

again, select the file with the RESTART OUTGOING FILE option and

press  (Enter).

The  dial retry count will be set to the number given in the CMS

configuration  and  the Communication Manager will start dialing

again.

5.4  Delete file

With  this  selection  you  can  delete received files, outgoing

files and download orders.

5.5  Sendlog

The  sendlog  shows  you  previous send and download activities.

Send  activities  are  marked  with  ->, download activities are

marked  with  <-.

It  shows  you  date,  time,  phone  number,  transmission time,

filename and OK or ERR.

You can also print or delete the sendlog file.

5.6  Mailboxlog

The  mailboxlog  shows you which files have been downloaded from

your CMS mailbox by other users.

It shows date, time, password and filename.

You can also print or delete the mailboxlog file.

5.7  Mailboxdirectory

If  you  want to use your computer as a CMS mailbox, you can use

this  functions  to  add  files to your mailbox, to delete files

from your mailbox and to view a list of your mailbox files.

SHOW  FILES  shows the files in your mailbox, listing name, text

and size.

ADD  FILE  asks  you  for  the  file to add to your mailbox, the

compression type and copies the file to the MAILBOX directory.

To  give  other  users  access  to  your CMS mailbox you have to

create  a  file named USER in the CMS directory. It has to be an

ASCII file with the passwords to use for downloading files.

A  password line in this file consists of 8 characters or spaces

and (;)

e.g.

GRISWOLD;

FREDTEST;

ROGER   ;

If  you want your CMS mailbox to be open for all callers without

password, create the USER file with 8 spaces and (;)

e.g.

;

In  any  case  you  have  to  create  a USER file for permitting

download of files from your mailbox.

Without this file no download is possible from your mailbox.

Note:  In any case callers have only access to the files in your

MAILBOX   directory.  All  other  files  of  your  computer  are

inaccessible  to callers. This assures complete security of your

computer.

All  files  in  your  mailbox directory are compressed CMS files

which use the orginal filename.

5.8  Phone directory

Using  an  ASCII text editor you can create a file with the name

PHONE  in  your  CMS  directory.  In this file you put CMS phone

numbers with a shortname.

With  this  shortname you can select the number or use (F1) when

sending files.

Each line has to have the following format:

shortname,phone number

e.g.

griswold,2034568793

WebbInc,4042345743

5.9  Create text files

If   you  have  specified  an  ASCII  text  editor  in  the  CMS

configuration,  you  can  create text files by pressing (F10) at

the CMS menu.

Please  use  the  file  extensions .DOC and .TXT as described in

appendix B: for electronic mail files.

Appendix A: User defined modem installation

A  user defined modem installation is necessary, when your modem

does not work with any of the modemtypes in the modemlist.

Press (F1) to call the modemlist and select USERDEFINED.

Now you can input your individual modem configuration.

Speed

-----

Here  you  put  the maximum speed your modem can use to transfer

data between modem and computer.

e.g

2400      for V.22bis modems without error correction.

9600      for V.22bis modems with error correction (MNP4/V.42).

19200     for V.32bis modems with error correction.

Modem setup string

------------------

To  work  with  CMS  your  modem  has to be set to the following

configuration by typing commands into the modem setup string.

Transmission mode V.21, V.22, V.22bis, V.32, V.32bis

Responses in verbose form

CMS will understand the following modem responses:

CONNECT

CONNECT 300

CONNECT 1200

CONNECT 2400

CONNECT 4800

CONNECT 9600

CONNECT 14400

IMPORTANT:  The  CONNECT response has to show the modem to modem

speed  (Link  speed),  not the modem to computer speed, which is

already   known.  Extended  response  codes  and  call  progress

responses  (CARRIER,  PROTOCOL,  COMPRESSION) will create errors

and should not be activated.

RING

RINGING

BUSY

DIALTONE

DIAL TONE

NO DIALTONE

DIAL LOCKED

Modem speed independent of line speed (speed conversion ON)

Error correction (MNP4/V.42) activated

CTS/DTS flow control

DTR OFF causes hangup  (&D2)

No auto answer         (S0=0)

Wait time until CONNECT 60 seconds   (S7=60)

As  this  configuration  is  already  used  by many modems it is

possibly  you  do  not  have to enter any command into the modem

setup string.

Most of the modems have only to be configurated not to show call

progress  responses (CARRIER, PROTOCOL, COMPRESSION) and to show

the  link  speed  (modem  to  modem  speed)  after  the  CONNECT

response.

If  you want to view modem commands and responses during dialing

and transmission, use the option -D as described below.

Note: If your modem has no speed conversion and error correction

it is also possible to use it with CMS. You only have to use the

options  -B  (always)  and -N (seldom) as described below in the

options line.

Using   build-in  error  correction  and  data  compression  CMS

garanties  fast  and  reliable  transmission also with non-error

correction modems.

If you still have problems configurating your modem, call us, we

will try to help you.

If  you  have created successfully a modem setup for a new modem

send us the configuration, we will add it to our modem list.

Answercommand

-------------

The  command to answer an incoming call is usually A, but it can

be necessary to use another command.

If you leave this field empty, CMS will use A to answer calls.

Limit2400-9600 command

----------------------

These  are  commands  used  to  direct  the modem to establish a

connection at a limited speed.

They are only necessary when using V.32 or V.32bis modems if you

have problems connecting at high speed and you want to limit the

connection speed.

The  commands  are  sent to the modem before dialing if you have

selected a lower than the maximum speed when sending the file.

Maximum speed command

---------------------

This  command  is  sent  to the modem after each transmission to

reset the modem to the maximum speed.

Options

-------

Some  options  may  be  necessary to configurate CMS to use your

modem efficiently.

An option command consists of a (-) and a character to type into

the options line.

By giving an option command you activate the option.

Possible options are:

-B        Baud adjust ON

If  your  modem  has no speed conversion, type -B into

the  options  line  and  CMS  will automatic adapt the

computer  port  speed  to  the  speed indicated by the

CONNECT response.

This   option   is  usually  necessary  for  non-error

correction modems.

-N        This option is only necessary for non-error correction

modems.

Usually  CMS  filters  MNP/V.42  protocol  requests of

calling   error   correcting   modems.  But  if  error

correcting  modems  have  problems  connecting to your

non-error correcting modem, use this option and during

the first 10 seconds all requests will be ignored.

-D        This  option  is  very  helpful if you want to see the

commands  and  responses to and from the modem. If you

type -D into the options line all normally not visible

commands will be shown on the screen.

Appendix B: Compatibility

1.)  To asure compatibility of electronic mail messages, you should

use the following file extensions for E-Mail text files.

.DOC      for text files  using the IBM-Extended ASCII character

set.  Text files  created  under  DOS usually use this

character set.

.TXT      for  text  files  using  the  ANSI character set used

under  Windows. Text files created with Notepad under

Windows use this character set.

Future,  CMS  based, E-Mail systems will use this extensions for

automatic conversion of character sets.

2.)  If you are using CMS as a mailbox system, put always an INFO.DOC

file with some information into your mailbox directory.

Somebody  who  has  only  the  phonenumber of your computer will

always try first to download this file.

Appendix C: Protocol

Starting  with  Version  2.05,  CMS  uses a new, faster transfer

protocol.  When you see the messages "Framesize" and "Blocksize"

at  the  beginning of the transmission the new protocol has been

activated.  Fastest  transmission  speed is possible between two

Windows  programms, because CMSW can use a larger blocksize than

CMSR.

The  new  protocol  is 100% downward compatible, so there are no

problems to communicate with previous versions of CMS.