CD-LINK USER'S GUIDE

--------------------

CD-Link is a product of:

Intellectual Products Corporation

4665 Lower Roswell Road #169

Marietta, GA  30068

Phone: (404)992-5327   FAX: (404)992-2906   BBS: (404)640-5017

Compuserve ID: 73747,66

Internet Mail: 73747.66@compuserve.com

Copyright 1992, 1993 Intellectual Products Corporation

All rights reserved worldwide

INTRODUCTION

------------

CD-Link is a device driver that allows DOS applications to access CD-ROM disk

readers as if they were "standard" DOS-compatible disk drives.  For DOS users,

it is an alternative to Microsoft's CD-ROM Extensions.  CD-Link is also

available separately for The Software Link's PC-MOS operating system.

Supporting High Sierra and ISO 9660 disk formats, the driver will work with

most popular CD-ROM drives and controllers.

Some significant features of the product are:

* Ability to use the hard disk as a large cache, for greatly

improved performance without sacrificing memory.

* Redirection of DOS function calls at the INT 21 level for

easy interfacing to networks and alternative operating

systems.

* RAM overhead approximately 8K for PC-MOS, 10K for DOS.

* Works with vendors' existing low-level drivers, for

compatibility with most popular CD-ROM hardware.

* CDSTAT utility reports usage statistics, to simplify fine-

tuning of the installation.

INSTALLATION

------------

YOUR USE OF THIS PRODUCT IS SUBJECT TO THE LICENSE AGREEMENT THAT IS CONTAINED

IN THE FILE LICENSE.DOC, PROVIDED WITH THIS SOFTWARE.  BE SURE TO PRINT THIS

AGREEMENT AND DO NOT PROCEED FURTHER UNLESS YOU HAVE READ IT AND AGREE TO ITS

TERMS AND CONDITIONS.

Installation of the product consists of copying CDLINK.SYS and CDSTAT.COM from

the installation diskette to your hard disk, adding a line to your CONFIG.SYS

file, and rebooting the system.

The config.sys statement to be added has the following format:

For DOS:

DEVICE=CDLINK.SYS [D=devicename]

[C=cachefilespec]

[K=cachefilesize]

[N=numberofdrives]

[O=offsetvalue]

[H=numberofhandles]

[L=longreadsectors]

for PC-MOS:

DEVICE=CDLINK.SYS [D=devicename]

[C=cachefilespec]

[K=cachefilesize]

[N=numberofdrives]

[O=offsetvalue]

[H=numberofhandles]

[L=longreadsectors]

[T=numberoftasks]

Parameters must be all entered on one line, and are separated from each other

by one or more spaces.  No spaces should be entered within a parameter, nor

immediately before or after an "equals" sign.

More specifically, the optional parameters are as follows:

<devicename> is the system name assigned to the hardware-

dependent CD-ROM device driver.  This name will either be

specified on the DEVICE= command line for that driver, or

will have a default value in that driver.  Default is

"MSCD001".

<cachefilespec> is the fully qualified file specification

for the work file that CD-Link creates and uses for cacheing

of CD-ROM data.  "Fully qualified" means that a drive letter

and complete path specification must be supplied - e.g.,

"C:\WORK\CACHE.DAT".  Default is "C:\CDLINKCA.CHE".  This

cache file is typically maintained on a hard disk, but users

with RAM to spare may want to implement it on a RAM-disk.

<cachefilesize> is the number of kilobytes of disk space to

be allocated to the cache file.  Default is 122.  The

specified number may be rounded down by as much as 41K,

since the cache file is allocated in increments of

approximately this size.  This parameter is used only when

creating a new cache file, and is ignored if the file

already exists.

<longreadsectors> is a number from 2 to 65,535 which specifies

the minimum number of whole CD-ROM sectors in a single read

request that will cause cacheing to NOT occur.  The theory is

that if there is a "long" read request for some large number

of contiguous sectors, then it may be better to always read

them directly from the CD-ROM as opposed to consuming large

portions of the cache file.  You use L= to specify this minimum

number.  Default is 16.

<numberofdrives> is the number of physical CD-ROM drives

that are to be supported.  Default is 1.

<offsetvalue> does not normally need to be specified.  It is

used to tell the system how to identify which file handles

are for CD-ROM files.  A byte value in the PSP handle table

that is greater than or equal to <offsetvalue>, but less

than <offsetvalue+numberofhandles>, is deemed to specify a

CD-ROM file handle.  This parameter is provided as a way to

resolve possible conflicts with other products, such as LAN

operating systems, that also apply offsets to file handle

table entries.  BEWARE THAT <OFFSETVALUE+NUMBEROFHANDLES>

MUST NEVER BE GREATER THAN 255.  Default is 200.  Allowable

range is 20-250.

<numberofhandles> indicates the maximum number of CD-ROM

file handles that may be open at one time.  Default value is

20.  Allowable range is 5-235.  If you specify a value

greater than 63, then you will also have to specify

offsetvalue (see above).

<numberoftasks> applies only to PC-MOS installations.  It

specifies the maximum number of system tasks that may access

CD-ROMs.  Default is 5 (1 if DOS is used).

No separate utilities are required to create or maintain the cache file.  The

driver will at boot time automatically recognize and use an already-existing

cache file with the name specified with the "C=" parameter, or if one does not

exist a new one will be created.  Therefore if you want to create a new cache

file (e.g., to change its size), all you have to do is delete the old one and

re-boot.

You are encouraged to make the cache file as large as practical.  Any size from

roughly 128K to several megabytes is reasonable.  Cached data remains in the

file and will be re-used even after you power off your computer; this will

translate to a significant performance benefit as the system "learns" your

CD-ROMs.

IMPORTANT NOTE: Cacheing performance is closely tied to the performance of your

hard disk.  We strongly recommend that you use a cacheing utility for your hard

disk - or if you use PC-MOS, take the time to optimize your "cache=" parameters

(write cacheing IS recommended).

THE CDSTAT UTILITY

------------------

CDSTAT.COM is a program that you run to display statistical information about

CD-Link.  Simply type CDSTAT at the command prompt, and the following

information is displayed:

Short (cached) reads  - The total number of CD-ROM reads processed since

the last time the computer was booted, where

the number of sectors requested was less than

the value of the "L=" parameter (see below); this

is regardless of whether the read was satisfied

from the cache.

Cache hits            - The number of read requests satisfied from the

cache since the last boot.

Cache index rewrites  - The number of times that a cache index record has

been modified since the last boot.  Frequent index

rewrites are an indication that performance can

be improved by making the cache file bigger.

Long (uncached) reads - The number of reads since the last boot where the

number of sectors requested was greater than or

equal to the "L=" parameter.

Cache file opens      - The number of times since the last boot that the

cache file has been opened.  Normally, these opens

are infrequent.

Cache file errors     - The number of times since the last boot that an

I/O error occurred while processing the cache

file.  A nonzero value here indicates a problem.

Capacity (sectors)    - The number of 2K CD-ROM sectors that may be held

in the cache file at any given time.

Sectors cached        - The number of CD-ROM sectors currently stored in

the cache file.  Under normal use, this number

will reach the capacity fairly quickly.

DOS 5/6 AND WINDOWS

-------------------

The ability of DOS 5 and 6 to put itself, device drivers, and TSRs into upper

memory is a big plus.  CDLINK (for DOS) appears to work well when loaded with

the "DEVICEHIGH=" config.sys statement.

As noted above, use of a disk cacheing utility is strongly recommended.  We

have had good results with SMARTDRV.EXE, which can also be loaded into upper

memory with DOS 5 and greater.

If you are using EMM386.EXE or a similar memory manager, be sure to tell it to

avoid using the memory address space occupied by your SCSI controller.  For

example, if your SCSI card uses 16K of address space starting at C8000, then

the appropriate parameter for EMM386 is X=C800-CBFF.

Similarly, if you are using Windows 3.x in enhanced mode, you will need to add

an "emmexclude" statement to the [386Enh] section of your SYSTEM.INI file.  In

the above situation, the statement would be "emmexclude=C800-CBFF".

The following are the relevant CONFIG.SYS statements for a sample system using

a Corel LS2000 SCSI controller set for memory address C8000 with a NEC CD-ROM

drive (this is only an example; your system will probably require different

parameters):

DEVICE=C:\DOS\HIMEM.SYS

DEVICE=C:\DOS\EMM386.EXE X=C800-CBFF I=CC00-EFFF NOEMS

DOS=HIGH,UMB

DEVICEHIGH=C:\CORELDRV\NEC_LS2.SYS /ID:0 /MEM:C800 /N:1 /D:MSCD001

DEVICEHIGH=C:\CDLINK.DOS C=C:\$CACHE$ D=MSCD001 K=1024 L=8

Some SCSI drivers may not let you specify the memory address of the board,

instead looking for boards at all of the possible addresses.  This will cause a

problem if you have already loaded EMM386.  In such a case we suggest using a

"DEVICE=" statement to load the hardware driver BEFORE the "DEVICE=" statement

for EMM386.

CONSIDERATIONS FOR PC-MOS USERS

-------------------------------

Note that CD-Link for DOS and MOS are DIFFERENT products.  The DOS version will

not work with PC-MOS, and vice versa.

To minimize CPU and RAM overhead, CD-ROM controllers using DMA are NOT

recommended for use with PC-MOS.

Be sure to FREEMEM around the memory address space occupied by your SCSI

controller!  For example, if your SCSI card uses 16K of address space starting

at C8000, then your PC-MOS config.sys parameter might be "FREEMEM=CC000,F0000"

(assuming the given range is otherwise not occupied by ROM or other hardware

devices).

The following are the relevant CONFIG.SYS statements for a sample system using

a Corel LS2000 SCSI controller set for memory address C8000 with a NEC CD-ROM

drive (this is only an example; your system will probably require different

parameters):

MEMDEV=C:\PCMOS\$386.SYS

FREEMEM=CC000,F0000

CACHE=1024,2,10,2,A,B,C

SMPSIZE=64K

DEVICE=C:\PCMOS\UPDAT501.SYS

DEVICE=C:\CORELDRV\NEC_LS2.SYS /ID:0 /MEM:C800 /N:1 /D:MSCD001

DEVICE=C:\CDLINK.MOS C=C:\$CACHE$ D=MSCD001 K=2048 L=8

PC-MOS USERS: IF YOUR CD-ROM CONTROLLER USES DMA

------------------------------------------------

Your best bet is to avoid DMA with PC-MOS.  However if this is not an option,

do the following in your CONFIG.SYS file:

1. Set up a "split SMP" by using a SMPSIZE statement of the form

"SMPSIZE=nnnK,nnnK", specifying enough memory in the second parameter

to contain the CD-ROM hardware driver and the CDLINK.MOS driver

(this requirement is usually less than 32K).

2. Use "LDEVICE=" (not "DEVICE=") to load both the CD-ROM hardware driver

and CDLINK.MOS into the lower segment of the SMP.  In some cases this

may not be necessary with the hardware driver.

3. Specify "L=500" in the CDLINK.MOS command line.  This will ensure that no

reads are performed directly into an application's task space.

WARNING - IF YOU ARE UPGRADING FROM 1.00

----------------------------------------

CD-Link release 1.00 had a different cache file format from subsequent

releases.  Therefore if you are converting from 1.00 (or a beta version) be

sure to delete your old cache file and reboot!

WARNING FOR HITACHI CD-ROM DRIVE USERS

--------------------------------------

A pecularity was found with the driver HITACHIA.SYS version 2.20, and may also

exist in other versions.  This driver requires that it be the LAST device in

your config.sys file (let's hope this does not become a common requirement from

developers of device drivers!).

While we normally ask that CD-Link be installed after the associated hardware

driver, it does appear to work correctly if the Hitachi driver comes last.

- End of Document -