HAMCOM

Author: Dan Diehlman AE6G

5478 N. Bond

Fresno, CA 93710    209-439-5520

Note: The author of this program has expended considerable time and energy

in writing, testing and debugging of this program. Each copy is

customized for one call sign. If you would like to buy a registered copy,

contact me at the above address. You will then receive also any

corrections/updates.

It has, I believe, a couple of unique features, one being the QSO

card file which enables you to do away with any card file and gives

practically instantaneous response on Call Sign searches. The other

feature is the low RFI(noise) generated. This is very noticeable on

HF. During inactivity, the processor chip is put into a full halt

condition, thus eliminating birdies caused by instruction loops.

This was made possible by the fact that the program is completely

interrupt driven, therefore it can be put to sleep, then awakened by

a keystroke or communications activity. This was written to take

advantage of a PK232 -IBM/clone configuration, with 256K or more.

RUNNING THE PROGRAM

To run the program, copy all the files onto a

diskette which already has Dos on it.

Then type "Hamcom"

The first time you run the program, it will ask for information necessary

for customization as follows:

prompt                       reply

What type TNC are you using?     Appropiate number.

What is your Call?               Your call

What is your Selcall?            Your Selcall (if you have not determined your

selcall, see 232 ref manual.)

Is this a PCJR? y/n             y or n

Which Comport 1 or 2 ?          1 or 2

Select type of Display Adapter  1-4

Are you using a Color Display?  y or n       n for monochrome monitors

y for any color monitors

Enter your CTEXT.Use > for CR   Your connect message. If you wish to imbed any

Carriage Returns, use the ">" symbol.

Enter your BTEXT.Use > for CR   Your beacon text.

Which Drive for default         Enter A,B,C or whatever drive you wish to be

the default drive for logging, receiving, and

sending files. It can be overridden in any

event when specifying a file i.e. B:xxx.xxx

Baud rate between TNC and computer.  Select any of the listed speeds. 1200 is

best. (You should use 1200 baud with the PK232)

Customized Packet Ending        This allows you to enter a special characters

which will be appended to your packets when

you release it with the "]" key. I set mine for

"==>" but set it for BK or whatever you like.

Special Parms:                   Opportunity to set additional/different

parameters. Separate them with a slash.

such as:    monitor on/txd 50

Do you have a 440-940 connected. Discussed later.

Is your primary output a Voice Synthesizer Y/N? Normally select "N". If "Y"

is selected then the screen is written thru

BIOS calls so that voice synthesizers can

intercept. Also assists the synthesizers in

pronouncing words instead of spelling the

words.

On subsequent runs of the program, you will not have to enter this information.

It will be preserved in a file.  If, in the future, you desire to change these

parameters, select "Alt F6" (F36) from the main screen.

The program will then proceed and stop on the main display, with the heading

"The following data was stored in the TNC". Below it will be listed any

connects you received, while your computer was off.

This data ends with the message "1: Proceed with TNC reset" and "2:Proceed

without TNC reset". The first time you run the program, use "1". This will

set certain parms in the TNC. On subsequent runs, if the TNC has not been

powered off, you can use "2".

After you depress a key the program will load it's other files , ending up

back on the Main Screen.

The Main Screen displays any text from the TNC above the horizontal line, and

any text that you type is displayed below the line. If you would like to

change the line position, use the "Alt D" to raise or lower the divider.

The lower left hand corner displays the current Mode of the TNC.

The lower right of the screen displays the status of whether you are also

logging to diskette, the data received from the TNC. At the far right on the

bottom of the screen, a number (0-99) will appear on some TNC's. This number is

an indication of how many connects you have had. It is reset to 0 by going to

the Scroll Back Buffer.

Also on the bottom line you can display Help in any mode by depress "Alt H". It

can be turned off in the same manner.

The Main Screen has the feature of being able to scroll back thru the received

text. To use this feature: use the "Page Up" key to scroll back further. If

you reach the top of the buffer, a message at the top of the screen will say

"Top of Buffer:". You can at any point, scroll back down through the buffer

using the "Page Down" key, or the "Home" key to get to the top of the buffer,

or you can return to the Main Screen by depressing the "Esc". At any

time, while in the Scroll Back Buffer, you can depress "Alt M" for a menu.

While in the Scroll Back Buffer, you can "Mark" the beginning and "End" of

a block of text, and then either "Send" the block or "Write" it to a file, or

"Print" it. You may also load any Text File into the buffer. You can also

edit the text in this buffer. You may also "Search" for particular words.

Notepad: You can reach the Notepad screen from the Main Screen by depressing

"Alt N". This screen is useful for any notes. I use it for recording tuner

settings etc. The only keys in effect on this screen are the regular alpha/

numeric keys and the Cursor positioning keys. Anything written onto this

screen will be preserved in a file and read back in automatically whenever

you rerun the program. You may exit this screen with the "ESC" key.

Menu: You can recall the Menu screen from the Main Screen by depressing "Alt M"

On this screen are listed all the functions available from the Main Screen.

F11 through F20 are invoked by using the regular function keys along with

the Shift Key. i.e. F12 is called by using "Shift F2". Similarly F21 is called

by using the Ctrl Key along with F1, additionally F31 is called by using the

Alt key long with F1.

F1-F30: This is the User Macro function

Depressing F1 to F30 will execute Macros 1 to 30.

Initially all the macros will be empty. To create a macro depress

"Alt F". This will take you to a screen which displays the macros,

and gives you the opportunity to enter any macro. The screen is

self explanatory.

examples: "c w1aw>"  will initiate a connect to w1aw.

"m 4> mfrom yes all> mto yes all>" will set monitor to 4 etc.

F31: This will pull down a window and allow you to select different modes

on PK232  TNC's.

F32:Log to Disk. Initially the received data is being logged to "Logdat.1"

as indicated in the lower right hand part of the Main Screen.

This key will pull down a small window, allowing you to stop/restart a

new log, or to display the previous log, or to view any ASCII file.

F33: This will set the Date and Time in the TNC from the Dos clock.

F34: Allows you to Create/Send a Brag tape, the size is limited only by

Available disk space.

F35:File Transfer. This will send/receive any Ascii or Binary file. using

Hamcom's own protocol or Yapp protocol.

You will be prompted for the name of the file to send. A "File Xfer in

progress" message will be posted and at the completion a "File Xfer

Complete" message will be posted when all the data has been sent from

the computer to the TNC. Note: There may be remaining packets yet to be

sent and acknowledged in the TNC before the xfer is fully complete.

NOTE: To do Binary Transfers, you must have a Computer to TNC cable

that has pins 1-8 and 20. You cannot have just (TD,RD and ground).

(N O T E   T H I S   I S   V E R Y    I M P O R T A N T)

F37:CW Output. Causes your computer to output received text in CW using the

computer speaker. You can vary the speed up or down. This feature is for

the Diehard CW man who would rather copy CW even while working packet or

RTTY.

F39:Tone. This key toggles a tone on/off to help the visually impaired to

tune in data modes on hf. It sounds a 2100 hz note in all modes except

for Morse, in which mode, it sounds a 800 hz note.

F40:This will cause the received data to also go to the printer. Successive

depressions of this key toggles the feature On/Off.

Alt A:This will initiate a quick search of a call sign. In addition, it will

enter the callsign so that subsequent depressions of the "[" left bracket

will cause that callsign to be transmitted.

Alt B:Sound. This pulls down a menu which gives you control over the bell etc.

The soft bell can be selected or the regular bell, which is the default.

The soft bell is rather nice when you are in the room, and don't require

the harshness of the loud regular bell.

Alt X:End Program. Always use this to exit the Program. It will insure that all

necessary files are updated, and that the tnc is put in the proper state

to receive connect messages, and also set monitoring to 0.

Alt M: Menu.  Takes you to the Menu Screen from the Main Screen.

Alt N: Notepad. Takes you to the Notepad Screen, already discussed

Home: Clears the Main Screen.

Del: When using a PK232, this clears the transmit buffer in all modes.

Alt F: Setup Macros....previously discussed

Alt C: Sends CQ-automtically adjusts to whatever mode is in effect.

Alt G: This is a Special Function being used in development  work.

Not for general use.

Alt H: Toggles On/Off a summary of commands at the bottom of the Main Screen.

Alt K: This clears the Scroll Back Buffer.

The program is set up so that the Esc key duplicates the Ctrl C

function, thus lessening the number of keystrokes necessary therefore

whenever you want to enter a Ctrl C, just hit the ESC key.

The Tab key is set up in packet to do Channel Switching more easily.

For instance, if you are in converse mode, in packet, and you wish to

switch to another channel, instead of typing Ctrl C,Chs X, K simply

type "Tab" and the number of the channel you are going to. It will

switch you to that channel and return you to converse mode.

The program is set up so that the "Enter Key" does send the packet.

Or you may use the "right bracket" key to send the packet with your

customized ending. (substitute for BK or K or whatever).

Alt S:  Shows the current state of all packet streams.

Incidentally, when in a multiple connection situation, any change in

the received data stream, will automatically insert a solid line

accross the screen to alert you to the change.

Alt Z:  Allows you to create messages for individual callers. Allows

personalized connect messages.

Ctrl D: If in packet, sends a disconnect.

Baudot: self explanatory

Ascii:  self explanatory

Morse:  self explanatory

Amtor:  normal operation except for turnaround, type only the "]". It is not

necessary to type the "?" .

The "Tab" key has been setup to force Turnaround. It is the equivalent

of going to control mode, typing ACHG, enter, k, enter. Again, we are

trying to save unnecessary keystrokes.

FAX:    selects Fax mode, and turns Prcon On.

Alt L      This is a very usefull feature in that it is not necessary to

maintain a card file on your contacts. You can quickly go from

the Main Screen to search on Calls, and return, knowing the

calling party's name,qth, etc.

To implement this feature, type "Alt L".

You will be given several choices, however, until you have

created a database using the "Create" option, none of the other

functions will actually do anything. So intially choose.

"Scratch and Create DATABASE". It will then ask you to type the

word "CONFIRM" (must be in upper case), to proceed. The reason

for this added entry, is to prevent any accidental erasure. I

have a 128K file, and would hate to lose mine accidentally.

Enter a couple calls and the other requested information and then

do some searches to become familiar with this feature. A search

on a call is almost instantaneous with 1500 entries in the file.

Searches on QTH's, Activity(dates), takes longer as it must

sequentially go through the file to search for matches.

An "*" asterisk, limits the field searches, such as searching on

QTH thusly "TO*" will list all contacts with Tokyo and Toledo  and

Toocumcari if Toocumcari were spelled that way.

An Activity search such as "12/*/86" will list all qso's in Dec of

86. etc.

A Call Search such as "W3*" will list all qso's with W3's.

Alt P:     DOS SHELL. This feature allows you to temporarily leave the

program and go to DOS and run whatever you like. Whenever you

are finished, and have the Dos prompt, type "EXIT" and you will

return right back into the ham.com program, as though you had never

left. During your absence, the TNC will store any incoming messages

and give them to you on your return.

Alt T:     This is only active if you have stipulated in the configuration

the presence of a Kenwood 440 or 940 and the Kenwood inteface.

Selecting this function, will take you to a screen which

has a simulated transceiver dial, displaying the frequency, offset

modes, etc. The options are listed at the bottom of the screen.

To return to the Main Screen, depress Escape. If you are using a

440 you can build several databases using F14, and select any of

them to be loaded into the 440 memories.

Naturally, it would be wise to backup your files occassionally.

If you have "snow" on the screen, redo the Hamparms and answer "Y" to the

question "Is your Primary Output a Voice Synthesizer".

If for some reason you need to include an ESC character ($1B or dec 27),

the reverse apostrophe will send that character.

You can store the other fellow's call by typing a "Alt A ", (This also

gives a quick search of the log), and recall it by typing the Tilda "~".

This is handy for modes other than packet where you must identify periodically.

If you are transferring a physical card file into the program's card file,

it is best not to enter them in a sorted order, but rather in a harem/scarem

order. Because of the search algorithm, search times will be reduced.

When using a PK232 in Morse, Baudot, Ascii modes, you can type ahead the

text you want to send, even while receiving, and when you actually want

transmission to begin, enter a "ctrl X".

Additionally, when using a PK232, you are far better off to remove the

batteries. Sometimes the PK232 can get into a state wherefrom it is hard

to exit as long as the batteries are inserted.

There are 3 parameters that can be used when calling the program.

If you call the program thusly "Hamcom/1", it will load,restart then tnc,

and continue without intervention.

If you call the program thusly "Hamcom/2", it will load, and continue

without manual intervention.

You may also eliminate the beginning audible cw greeting by using the "/q"

parameter.

You may also add any parameter you would like set in the tnc, such as /'mon on'.

You may bring the program up in a mode other than Packet by including that mode

in the commandline thusly  "hamcom/Amtor".

For illustration: I bring up my PK232 in Amtor mode, without the opening CW

message, and without reinitializing the tnc thusly: "Hamcom/2/q/Amtor".

I would appreciate any criticism . Thank You

Hope you enjoy using the program.

The program assumes the following addresses for the Com Ports:

Com1      3F8       IRQ 4

Com2      2F8       IRQ 3

Com3      3E8       IRQ 4

Com4      2E8       IRQ 3

***** NOTE: PLEASE DO NOT USE WRITE PROTECT TAB ON YOUR WORKING DISK

HAMCOM NEEDS TO WRITE SEVERAL FILES TO THIS DISK

-------------------------------------------------

Dan Diehlman AE6G

5478 N. Bond

Fresno, CA 93710

209-439-5520

COMMAND LINE PARAMETERS

The following options, if included in the commandline which calls the

program, will produce specific results.

Q:         Causes a reduction in some of the sounds.

D:         Special

CWXMITOK:  Allows PTT to be operative in Morse mode

DUALSPEED: Indicates to the program that a special 1200/9600 combo modem is

present in the TNC.

NOANSI:    Forces the program to ignore ANSI sequences.

KFS:       Special

NOCLICK:   Eliminates  click which normally accompanies received text lines.

CALLBOOK:  Indicates the host computer has a remote activated

Callbook lookup.

CW:        Allows computer to be used as a CW keyboard thru the printer

connector, without using a TNC.

TRAP:      Sets up specific trap sequence to flag specific received data.

WWV:       Will warn the operator at 17 mins past each hour.

NO1200:    Will prevent remote requested downloads at 1200 baud.

NOFILE:    Will prevent remote requested downloads.

/1:        Will automatically initiate a "1" startup, resetting the TNC.

/2:        Will bypass resetting the TNC.

DIAGNOSTIC:Special parm not generally useable.

NOMODECHG: Starts the TNC without changing it's mode.

AMTOR:     Brings up the PK232 in Amtor mode.

MORSE:     Brings up the PK232 in Morse mode.

BAUDOT:    Brings up the PK232 in Baudot mode.

ASCII:     Brings up the PK232 in Ascii mode.

PACTOR:    Brings up the PK232 in Pactor mode.

FAST:      Causes the default Morse speed to be 30 wpm

IGNORE:    Causes the TNC to ignore a particular callsign.

DISABLEMSG:Causes the TNC to not check for stored messages.