T E L E M

version 1.4e

ARSENE satellite telemetry decoding package

by Frederic Rible fc1oat @ f6abj.frpa.fra.eu

(documentation for version 1.4e by Bernard Pidoux, f6bvp)

This software is distributed free of charge by:

ATEPRA

Association Technique pour l'Experimentation du Packet RadioAmateur

23, rue de Provins

F-77520 Mons-en-Montois

France.

If you want to support ARSENE project you can send your donation to ATEPRA

or RACE association.

ARSENE is a mini satellite built by french RACE radio amateur organization.

Its mass is 158 Kg and its orbit 20 000 - 36 000 Km with zero degrees

inclination. The output power is 15 Watts on VHF.

It has three AX25 standard 1200 bauds AFSK FM packet transponders with three

uplink frequencies on 435.050-435.100-435.150 Mhz.

The frequency of the downlink and of the AX25 beacon is 145.975 Mhz.

This beacon will send telemetry giving the status of radio modules

and electrical equipments aboard the satellite.

Hardware requirements for copying the telemetry in real time:

A VHF FM receiver, a standard packet Terminal Node Controler with no special

modem (i.e. bell 202 standard) a PC with serial port connected to the TNC

using 8 wires ( wires 2 to 8 plus wire 20). Signal strengh should make

ARSENE easy to copy with a 145 MHz yagi antenna with 13 dB gain. Although

signal is transmitted with vertical linear polarisation, a circular

polarisation receiving antenna could be better.

This real time telemetry decoding program will run with any color graphic

adaptator. The monitor can be black and white but, of course, full colors

effects will not be performed unless a CGA, EGA or VGA monitor is used.

A- Distributed file catalog

===========================

SIMU.BAT       Batch for ARSENE reception simulation (no TNC required)

TELEM.EXE      The telemetry decoding program

TELEM.CFG      Configuration file with telemetry channels equations

TELEM.COL      Color configuration file

TELEMCOL.COL   Color configuration file

TELEMB&W.COL   Black and white color configuration file

CONFIG.TNC     TNC configuration file

ESS.COM        Interrupt driven Serial I/O driver version 1.11

written by Gerard FC1EBN (also author of TPK);

Its performance are such that you can run at up to 19,2 Kbd.

TELEMENG.DOC   This file

B- Installation

===============

First, protect your original floppy and make a backup copy.

TELEM.EXE will run with a TNC-2 Prom 1.1.5 or higher version or compatible.

The configuration file CONFIG.TNC was programmed for a TNC-2 connected to

serial port COM1 with a 9600 bauds speed, 8 bits, no parity. In order to

modify  COM number, speed or format, you only need to change the first three

lines in CONFIG.TNC file. The third line that represents the format is coded

like the following:

XYZ with :      X = N   no parity

X = E   parity even

X = O   parity odd

Y = 8   8 bits data

Y = 7   7 bits data

Z = 1   1 stop bit

Z = 2   2 stop bits

The next three lines in CONFIG.TNC initializes the TNC serial i/o parameters.

AWLEN 8 tells the TNC to run 8 bits serial communication

PAR   0 tells the TNC to run 0 parity serial communication

RESTART make the previous instruction active.

These instructions are for a TNC2. You can modify it for your particular case.

For example they will not work with a TNC-20 that use E71 serial parameters

only.

LCALLS parameter is set for ARSENE-1 callsign. If ARSENE satellite callsign

is changed to FX0ARS-1, change here the corresponding callsign.

You can choose color display or black and white display by copying either

TELEMB&W.COL (for black and white) or TELEMCOL.COL (for color) to TELEM.COL

file.

Before running TELEM.EXE program, you must install the serial driver ESS.COM.

For example, if your TNC is connected to port COM 1, you type ESS 1 or include

a line with "ESS 1" (without quotes) in the AUTOEXEC.BAT

TELEM.EXE program must be run from the same directory where the files

TELEM.COL, CONFIG.TNC and TELEM.CFG are. When TELEM is run without argument

it looks for a TNC connected to the serial port and make the initialization

of the TNC.

C- Usage

==========

To run TELEM.EXE, you only need to enter command "TELEM".

If you would like to simulate satellite telemetry reception you can run

the program in simulation mode, even without connecting it to a TNC. You

should use the batch file SIMU.BAT or type TELEM SIMU at the DOS prompt.

In that case the serial driver ESS is not necessary. Random number are

generated inside the program.

In the present version there are only 5 possible commands when TELEM is

running, using the function keys:

- F1 : To acknowledge a new alarm (there is a beep warning and the values

that triggered the alarm by outpassing the normal values are

blinking).

After pressing the F1 key, the beep is stopped and the figures that

start the alarm stay in red except if the values return to normal.

- F2 : To toggle the display from real values to rough figures send by the

satellite and vice versa.

- F3 : To toggle the display from real time telemetry to archived values.

You can move through the records using Page UP and Page DOWN keys.

- F4 : To toggle the display from analog telemetry to STATUS channels.

You can toggle screens back and forth. A change in any STATUS

channel makes the corresponding item blink. F1 key will stop the

blinking.

- F10 : To quit the program.

D- In case of difficulty

=========================

1/ The program displays "The TNC-2 is not responding" :

Possible causes are :

- there is no TNC connected to the serial port !

- the serial port configuration is not compatible with your TNC:

check carefully the first three lines of CONFIG.TNC file;

- ESS driver is not loaded: load it typing ESS 1

(1 for COM1: depending on the port number you are using)

- the ESS driver is locked (for example after using a communication

program like Procom or Telix) : you should unload it and reload it.

Use the command ESS S and then ESS 1 (for COM1: !).

- the TNC you are using is not compatible with TAPR prom 1.1.5

or higher.

=> change your TNC or modify the CONFIG.TNC file.

For any information about ARSENE satellite please contact:

Radio Amateur Club de l'Espace

5, Place du Mont-Vallier

F-31280 Dremil-Lafage, France.

RACE can also be reached via its INTERNET mailbox:

race@amsat.org

f6bvp Paris, March 25, 1993.

---------------------------END OF DOCUMENTATION --------