TRAN, version 0.4

September  1990

TRAN is a text-to-speech program for the I.B.M.-P.C..  It can read ASCII

(askey) text files, translate normal English spelling to phones (basic

building blocks of speech sounds.  TRAN can also sound out each phone

through the internal speaker of the P.C., to an 8-bit DAC on one of the

P.C.'s I/O ports, or to a binary sound file.

usage: tran [ +|- flags] [-options] [file-name]

The file-name is an ASCII text file (with no word processor formatting

codes).  If no file-name is given TRAN reads input from the keyboard; in

this case the program can be terminated with CTRL-C.

The TRAN program contains two independent sets of 47 phones.  For the

internal speaker, TRAN uses a set of phones encoded as a sequence of

bits which determine the position (in or out) of the P.C.  speaker at a

rate of about 16 kilohertz.  When outputting to an I/O port, TRAN uses a

set of phones with 8-bit values for generating the speech wave-forms

with a digital-to-analog convertor (DAC) at a rate of 10 kilohertz.

COMMAND LINE OPTIONS

Command line flags and options control various features of the program.

A '+' turns the flag on and a '-' turns the flag off.  Options can use

either '+' or '-'.  The square brackets below indicate default values.

flags: + = on, - = off

i   use hardware interrupts to time the output [off]

P   type output of phone translation [off]

r   type rule number and phone translation [off]

s   say output (make sounds) [on]

t   echo input to console [off]

T   do phone translation [on]

v   type other internal information [off]

options:

c        say the time of day once

C        say the time of day every 10 seconds

D N      make an 8-bit waveform file for phone N

d1 N      use N for space delay [5]

d2 N      use N for voice pitch [1]

f N      output 8-bit wave-form to binary file N

R        print all pronunciation rules

p N      output 8-bit wave-forms to DAC at port N

where N = LPT1, LPT2, or LPT3 for a printer port

or N = VMK1, VMK2, VMK3, or VMK4 for a COVOX VMK board

or N = INT to use the internal speaker

or N = the hexidecimal port number

z        print duration and loudness of each 8-bit phone

Z        make 8-bit waveform file for each phone

?        type this summary of TRAN usage

USING TRAN

The following are examples of ways to use the TRAN program.  The

simplest way to use TRAN is just to type "tran" without any command line

arguments.  In this case TRAN will interactively read lines you type and

attempt to speak these lines as English text.

You can have TRAN type out and read this file with the command:

tran +t tran.doc

If you also want to see the phone translation add the +P flag:

tran +P +t tran.doc

You can save the phone translation in a file by typing

tran -s +P tran.doc > tran.phn

and listen to the phone translation at some other time file by typing

tran -T tran.phn

You can save the sound output in a binary file by typing

tran -f tran.trn tran.doc

The sound file format is a 128 byte header followed by 8-bit (unsigned)

waveform values.

The TRAN program will say (and type) the time of day once if you type

tran +t +c

or will continue to announce the time every 10 seconds if you type

tran +t +C

ADJUSTING THE OUTPUT RATE

When TRAN outputs to the internal speaker of the I.B.M.-P.C., there are

two timing parameters, d1 and d2, that control the rate that TRAN

speaks.  These are set automatically, but can be adjusted if necessary.

Making d1 larger increases the pauses between words and making d2 larger

lowers the pitch of the voice during phones.  Both d1 and d2 must have a

value of 1 or greater.  On a 4.77-MHz, I.B.M.-P.C./X.T., the best values

for the timing parameters are d1=2 and d2=1.  Setting these values

explicitly, by-passes the automatic setting, which saves a second or two

starting the program.  These values can be set on the command line

tran -d1 2 -d2 1 ...

or by using the environment variable TRAN to pass these values

set TRAN= -d1 2 -d2 1

Any of the other command line flags and options may also be set using

this environment variable.

When TRAN outputs to an I/O port, the rate may be determined either by a

programmed delay loop (as it is for the internal speaker) or by

borrowing hardware interrupts from the timer normally used by DOS to

keep trak of the time of day.  The default is to use the programmed

delay loop and the loop count can be adjust using -d1 as described

above.  The +i flag enables the hardware interrupt timing method.

REFERENCES

The speech-to-text rules used in the TRAN program come mostly from an

article in an IEEE journal:

Elovitz, H.S., Johnson, R., McHugh, A., and Shore, J.E.  (1976).

"Letter-to-Sound Rules for Automatic Translation of English Text to

Phonetics," IEEE Transactions on Acoustics, Speech, and Signal

Processing, Vol. ASSP-24(6), pp 446-458.

See the file RULES.TXT for a complete list of the speech-to-text rules

used by the TRAN program.

The 1-bit phones were extracted from a public domain program called

SPEECH by Andy McGuire.

The 8-bit phones were recorded from my voice with an 8-bit

analog-to-digital convertor (COVOX Voice Master) and can be output to

any 8-bit (unsigned) digital-to-analog convertor (DAC) attached to any

P.C.  I/O port (for example, the Speech Thing, also from COVOX, which

attaches externally to the printer port).  For further information

regarding these products, please contact:

COVOX inc

675-D Conger Street

Eugene, OR 97402