PCroute SLIP Interface

July 28, 1989

David Johnson

University of Mississippi

Introduction

The Serial line Internet Protocol interface for PCroute is designed

to allow the connection of two broadcast bus routers with a point-

to-point serial line.  These two routers appear to the

interconnected Local Area Networks as a single router but allow

remote LANs to be connected for little expense.  A SLIP line may

also serve as backup to a broadcast bus line.  This SLIP interface

follows the RFC1055 specification and its suggested enhancement and

therefore should be compatible with other SLIP implementations.

Specification

The SLIP interface may be configured for support of up to four

lines each with differing data rates.  In practice, four lines,

each running at 19.2K baud, cannot be supported unless the usage

of the lines is low.  Any of the data link interfaces of

PCroute may be designated to any of the standard COM ports (1-4).

The PCroute program provides flexible configuration of interfaces.

The router supporting the SLIP lines need not contain only one

Ethernet interface, but may contain several or an AppleTalk

interface as well.  Each additional interface increases PCroute's

overhead and thus lowers the per/line bandwidth which can be

supported.

Implementation

The PCroute SLIP implementation provides a standard data link

interface to the upper level routines and interacts with them only

through the DL_IP level interface.  Thus no modifications were made

to any of the upper level software.  The user configuration

program, config.c, was enhanced to provide configuration of data

rate for each SLIP line.  The config.inc file also had to be

modified to support this addition.  Other than the addition of a

slip.inc module, only declare.inc had to be modified to add the

SLIP protocol.

Many possibilities exist for implementing a serial port handler.

I could just poll each port every time the coroutine switch enters

the data link routines.  This is obviously not possible since one

cannot predict the polling rate and characters might be lost.  I

could have an interrupt routine for each port for read and write

interrupts, yielding eight interrupt routines.  This is the common

method, but if multiple high-speed lines are used, the interrupt

overhead would completely halt the system.  One interrupt handler

for each port which supports the read and write interrupts together

could be used but would not significantly reduce interrupt

overhead.  I could even have one routine poll each of the ports for

input and output which is called by the timer clock of the PC every

0.5ms or so.  This method is the best on its own merits, but if no

activity is occurring on the SLIP lines, the system would be

drained for no reason.

A hybrid of the last two choices was selected.  Each of the ports'

interrupts is tied to one handler which handles both read and write

interrupts for all ports.  This handler is called on any event from

any port but it polls each port for input or output.

The hybrid choice was made because it does not consume CPU

bandwidth with useless polling.  I assumed that it is quite

possible for someone to place two or more Ethernet boards and

several SLIP lines in one router.  If this were to happen, having

a constant interrupt when no input or output is necessary on the

SLIP lines would degrade the performance of the Ethernet routing.

Also, there should be ample time for upper level routines to do

processing between event interrupts from the COM ports.

The data link routines of SLIP do not read from or write to the COM

ports.  Serial I/O is done exclusively by the interrupt handler.

The DL routines merely allocate a write buffer and enqueue it on

the write queue.  The interrupt handler will check the write queue

for each line for a character and send it.  The DL write routine

checks to see if the interrupt handler is currently writing any

frame to the interface.  If not, the DL routine sets up the send

pointers and sends the start of frame character.  The interrupt

routine will take care of the rest of the frame.  This is done

because the interrupt handler is triggered on either read or write

events from any COM port.  If no frames are coming in and the last

write event has already occurred, another interrupt will not occur

until a frame arrives.  Once a frame has been completely written

the frame is dequeued and its buffer is deallocated.

When a new frame arrives on a SLIP interface, a frame buffer is

allocated and the handler fills this buffer with the frame.  Upon

completing this activity the handler enqueues this frame on the

read queue.  The read routine is called frequently and checks the

queue for available frames.  If a frame is available, it calls the

upper level routines to process it.  When they are through with it,

they call DL_IP_R_RETURN which dequeues and deallocates the buffer.

This process is possible because only one frame may be processed

by the upper levels for each interface.

A minor optimization was made on the read side of the interrupt

handler.  A check is made, if a character is available, to see if

an overrun has occurred.  When this occurs a flag is set to signal

this condition.  After the end of frame character has been read

this flag is checked and if set the bad frame is discarded.

Configuration

To configure a SLIP interface into PCroute, edit the declare.inc

file to include only the interfaces in your machine and include a

SLIP_DECLARE line for each SLIP line you wish to support.  The

parameters are <name> <task> <port> <wqueuesize> <wbuffsize>

<rqueuesize> <rbuffsize>.  <port> is the number of the COM port to

which this logical interface is attached (1-4).  The <wqueuesize>

and <rqueuesize> are the number of frames allowed on the write and

read queues, respectively, and should be set to at least 10 for

write and 3 for read.  The <wbuffsize> and <rbuffsize> are the

total size of the buffer memory pool from which to allocate a

buffer for each frame.  The write buffer should be at least 10000

and read at least 4000.

When configuring the executable, using config.exe, the SLIP

interface should be assigned the IP address of the remote router's

ethernet interface and vice-versa.  This makes the two routers

behave as one unit.

130.74.128.1                   130.74.96.1

/----------------------/               /----------------------/

Ethernet 1        ^                       ^        Ethernet 2

|                       |

+-------+               +-------+

|PCroute|               |PCroute|

+-------+               +-------+

130.74.96.1 |                       | 130.74.128.1

|                       |

+----------SLIP---------+

If the routers exchange RIP information with Internet connected

routers, it is wise to turn off RIP on the SLIP lines.  If this is

not done, the SLIP line will become tied up with a large number of

RIP packets.

Enhancements

Studies will be conducted to see if error detection will provide

the extra capabilities to support the loss in performance.

Modifications may be made to allow software and hardware flow

control between these SLIP routers to allow more high-speed lines

to be serviced.  Lastly, enhancements will be made to allow

multiple SLIP lines to be grouped together to provide better

throughput and capacity to a department.

Conclusion

The SLIP interface for PCroute will allow inexpensive

interconnection between buildings on campus and yet provide

sufficient throughput for most jobs.  Support of multiple lines on

one router allows the sharing of cost between several departments

but allows them connection to campus wide services.  Overall, the

SLIP implementation has provided a means of interconnection without

a campus-wide high-speed bus network.