gen_fft - generate inline code to perform a given size FFT

USAGE

gen_fft log_fft_size < instruction_formats

DESCRIPTION

Reads the log base two of the fft size from the command line,

then reads seven template lines from stdin;

builds a section of inline code from the given templates

that implements a decimation-in-time FFT of the given size.

Can generate FFT's of length 8, 16, 32... up to some arbitrary limit.

See Oppenheim & Schafer, "Digital Signal Processing", 1975, p. 318;

the particular dataflow graph implemented is that of figure 6.10,

which uses bit-reversed input, and accesses coefficients in normal order.

See the application note "Implementation of the Fast Fourier Transform

Algorithms with the TMS32020," in

"Digital Signal Processing Applications with the TMS320 Family",

Texas Instruments Inc., for macros to use with this program.

The routines in that note are available online from the Texas Instruments

DSP Bulletin Board, 300-2400 baud, telephone 713-274-2323,

in the archive FFT32020.ARC.

This program was tested by generating a 256-point FFT and comparing

with the routine FFT256, "A 256-POINT RADIX-2 DIT COMPLEX FFT FOR THE

TMS32020", given in that application note.

The twiddle factors are represented as fixed-point numbers with

12 bits of fraction, where unity is 4096, to accomodate the TMS320's

MPYK instruction.  Of course, 4096 can't be represented in a 13-bit

field, but that's okay, because only the special-case butterflies

need to use unity, and they don't use MPYK to do it.

Although this was written to generate code for the TMS320x0,

the fact that it is template based should make it very easy to

modify to generate code for other architectures.

The resulting code expects the input array to be in bit-reversed

order; see the file 'gen_brev.doc' for info on generating bit-reversal

routines.

INPUT

Reads seven lines from stdin giving the format of six macros:

1. radix-2 butterfly for any twiddle factor

2. radix-2 butterfly for twiddle factor of 1              (k / N = 0)

3. radix-2 butterfly for twiddle factor of (i+1)/sqrt(2)  (k / N = 1/8)

4. radix-2 butterfly for twiddle factor of i              (k / N = 1/4)

5. radix-2 butterfly for twiddle factor of (i-1)/sqrt(2)  (k / N = 3/8)

6. radix-4 butterfly for stages 1 and 2; twiddle factors are all trivial

7. comment macro used to explain the purpose of the following code

In all butterfly templates, the first 'radix' %d's are expanded to the

index of the locations the butterfly is to operate upon.

In the radix-2 butterflies, the third and fourth %d's are expanded to

the real and imaginary componants of the twiddle factor, expressed

as 12-bit fixed-point fractions for use with the TMS320's MPYK instruction.

In the comment template, %s is expanded to the text of a comment.

The expansion of %d's is done by the C function printf(), so you can

use fancier formatting (for example, %03d) if you like.

OUTPUT