SEESAT4.DOC

SEESAT earth satellite tracking program - documentation

Paul Hirose - 1992 Jan 5

Introduction

SEESAT is an earth satellite orbit prediction program for visual

observers.  It computes the information needed to plot a satellite

in a star atlas and observe it with binoculars.

SEESAT uses the SGP4 orbit prediction model, which is the primary

NORAD model for near-earth satellites.  (I can provide the SGP and

SGP8 models on request.)  SEESAT is compatible with the NORAD 2-Line

element set format.

SEESAT is written in the C language.  For maximum portability, it

assumes only a teleprinter terminal.  Since it is a text-only,

command-driven program, SEESAT can be ported to any system with a C

compiler.  All source code is public domain without restrictions.  You

are welcome to use my code in your own projects, compile and

distribute SEESAT executables, create custom versions of SEESAT, etc.

I distribute SEESAT as C source code only, relying on others to create

the executables.

Primary distribution points for SEESAT are the Celestial BBS in Dayton

(513-427-0674) and the CSS BBS in Toronto (416-458-5907).  Both

systems are open 24 hours.  These are also good places to reach me if

you have questions about SEESAT.

Warnings

Users of former SEESAT versions should be aware of some changes.

The program no longer prompts for observer's position at startup.

Height, latitude, longitude, and time zone must be entered via

commands.  The program will NOT warn if you've forgotten to set any of

these values!

Observer's height is now measured in KILOMETERS, not feet.

Time zone is entered in the same format used to enter time in the rest

of the program, instead of the former decimal hours.  In addition, the

sign has reversed, so negative (west) longitudes now have negative

time zones.

These changes should not cause any inconvenience, since SEESAT now

provides for an automatic initialization file.

New Features

These are all new with this release:

A command to select display of all predictions, not just the ones

above the horizon.

A command to select display of satellite longitude with respect to

your meridian instead of Greenwich.

Satellite longitude and latitude are displayed to the nearest .1

degree.

The prediction table has a new column for estimated magnitude, which

is computed from absolute magnitude, range, and phase angle.  Absolute

magnitude input is automatic if SEESAT senses a Molczan-format element

set, or it may be entered with a command.

Latitude, longitude, height, and time zone are input by commands

instead of by answering questions at startup.  This lets you change

them without exiting the program.

A new command, NEXT, loads the next satellite in the elements file.

You can tell SEESAT how many characters to use from the name line in

the orbital element file.  This can prevent it from taking Molczan's

size data as part of the satellite name.

The + character in element sets has a checksum value of 0, in

accordance with the latest NORAD policy.

All orbital elements may be entered manually.  The old SEESAT didn't

provide for satellite name or epoch

Automatic adjustment of a prelaunch element set, if you input the

assumed and actual launch times.

RUN executes a prediction run.  (Old SEESAT didn't have an explicit

run command.)

A prediction run may be specified in terms of START and SPAN, in

addition to the old START/STOP and CENTER/SPAN modes.

I have added provision to interrupt a prediction run and return

control to the keyboard.  Routines are included for the Laser C (Atari

ST) and Turbo C (IBM clone) compilers.

SEESAT is now bilingual:  CENTER and CENTRE are equivalent commands.

Commands REPEAT, PRINT?, and SKIP allow you to create an iterating

command line.

A new command, EX <filename>, will execute the specified batch file.

This causes the command interpreter to get its lines from the file

instead of the keyboard.

RET transfers control from batch file to keyboard or vice versa.

At startup, SEESAT looks for file SEESAT.BAT.  If present, it will be

executed as a batch file.  You may have this file set SEESAT to your

location and perform any other initialization tasks.

Sample Session

This is from an actual SEESAT session.  It may be used as a quick

check for proper functioning of your program.

A>seesat4

SEESAT (SGP4) 1992 Jan 4 by Paul Hirose

>lat 34.915 lon -117.933 height .73 zone -800

>open b:noaa9.ele

complete; 1 satellites found

>load "noaa 9"

NOAA 9

1 15427U 84123  A 91289.49913910  .00000538  00000-0  30856-3 0  8532

2 15427  99.1650 305.9266 0016081  84.5452 275.7530 14.13148923352652

>start 1991 oct 27 1750 span 136 step 12

NOAA 9        1991 OCT 27

time   az el   R.A.    dec    range sun   mag    alt  lat    lon

1850:00 286 38  17h 7m  31 50'   1265  13 * 1.2    841 36.6N 127.5W

>center 1850 span 18 step 4

NOAA 9        1991 OCT 27

time   az el   R.A.    dec    range sun   mag    alt  lat    lon

1842:00 185  2  20h43m -53  0'   3181  12 * 2.6    848  8.7N 120.0W

1846:00 203 22  19h39m -29  0'   1755  12 * 1.5    845 22.6N 123.5W

1850:00 286 38  17h 7m  31 50'   1265  13 * 1.2    841 36.6N 127.5W

1854:00 330 11  13h11m  53 20'   2399  15 * 2.4    839 50.4N 132.9W

>para 1850

parallactic angle = 68

>sun 1850

az=270 el=-23

>moon 1850

az=44 el=-16

75% illuminated

>exit

Entering Commands

SEESAT asks no questions and has no menus.  It is a command-driven

program.  You enter a command at the > prompt and press RETURN.

SEESAT executes the command, possibly generating output, then returns

to the > prompt.

Multiple commands may be entered on one line.  They will be executed

from left to right.  Use one or more spaces to separate items on the

command line.  E.g., to load satellite "Mir" from file 929.ELE, you

could do:

OPEN 929.ELE

LOAD MIR

or enter both commands on one line:

OPEN 929.ELE LOAD MIR

Up to 80 characters may be typed on a single line.  Backspacing is

allowed to correct mistakes.  SEESAT does not erase characters from

the screen as you backspace over them, but they ARE erased inside the

program.  SEESAT will only execute the characters up to (but not

including) the cursor.

You may use lower or upper case.  SEESAT converts the command line to

all upper case before executing it.  I will use upper case in this

documentation.

Command Arguments

In the description of SEESAT commands, you'll see that most commands

must be followed by an "argument", i.e., an item for the command to

act on.  The three most common types of argument are:

<number>

<time>

<date time>

I will now describe the format of these arguments.

The <number> argument is a just a single number.  The following

examples should all be safe formats to use for <number>:

.001  13.78299201  35.  35  3.5e1  .256e-3

For example, the LAT command sets your latitude:

LAT 34.915

A <time> argument is in terms of minutes.  If more than 2 digits are

present, it is considered hours and minutes.  If a colon is present,

anything on the right of the colon is considered seconds.  Leading

zeros are allowed but not required.  E.g.:

1811:15    = 18h 11m 15s

1811:15.65 = 18h 11m 15.65s  (decimal seconds are allowed)

1811       = 18h 11m  0s     (seconds are optional)

0300       =  3h  0m  0s

300        =  3h  0m  0s     (leading zeros are optional)

59:50      =  0h 59m 50s

30         =  0h 30m  0s

5          =  0h  5m  0s

:10        =  0h  0m 10s

:01        =  0h  0m  1s

:1         =  0h  0m  1s     (leading zeros are optional)

:00.1      =  0h  0m  .1s

:.1        =  0h  0m  .1s    (leading zeros are optional)

0          =  0h  0m  0s

The fields in a <time> are allowed to exceed "normal" bounds.  Any

number up to 99 is legal for hours, minutes, or seconds.  In addition,

seconds may have as many decimal places as desired.  E.g., 460 (4h

60m) is a legal <time>.  It is identical to 500.  A more extreme

example is 9999:99, which is 99h 99m 99s.

The STEP command is an example of a command which requires a <time>

argument.  For example,

STEP 12

sets the prediction run time step to 12 minutes.

Other SEESAT commands require a <date time> argument, which is

composed of year, month, day, and time.  All digits of the year must

be given.  Use the first 3 letters of the month name.  The time part

of <date time> follows the same format as the <time> argument

described earlier.

The MOON command is an example of a command which takes a <date time>

argument.  E.g.,

MOON 1991 DEC 28 1730

will cause SEESAT to display the moon's azimuth and elevation at the

given time.

The day part of a <date time> may be ANY integer.  E.g.,

MOON 1991 JAN 0 1200

MOON 1990 DEC 31 1200

will give identical results, as will these commands:

MOON 1991 JAN 33 1200

MOON 1991 FEB 2 1200

You may omit year, year/month, or year/month/day, e.g.,

MOON 1991 JUN 12 1800

MOON JUN 12 1800

MOON 12 1800

MOON 1800

are all legal commands.  The omitted part of the date will default

to the last value(s) you entered in a <date time>.  All commands share

the same default date.  For example, if you enter the commands

MOON JUN 11 2200

SUN 12 2200

MOON 2200

you will get the moon's position on June 11 at 2200, the sun's

position on June 12 at 2200, and the moon's position on June 12 at

2200.  Since the year has not been specified, in all three cases it

will default to the last year you entered.

Note that you may omit the entire date from <date time>, but you

must always include the time!

There is no default date at program startup, so your first <date time>

must include the complete date.  Remember to use the full year, not

just the last two digits.

Setup Commands

LAT <number>          [degrees; negative if south]

LON <number>          [degrees; negative if west]

HEIGHT <number>       [kilometers; negative if below sea level]

These commands set SEESAT to your location.

At 1000 km (a typical viewing distance), a 10 mile error in your

position (16 km or .14 deg) causes a 1 deg error in the satellite

position predicted by SEESAT.  That is for worst-case geometry, i.e.,

the error is at right angles to your line of sight.  Therefore, it is

safe to omit the HEIGHT command if you're unsure of the correct value.

(The default value at program startup is zero.)  There will be an

appreciable error only if you're at an unusually high altitude.  Note

that height is KILOMETERS, not meters!

ZONE <time>

<time> argument equals (local time) - UTC.  A negative sign is

permitted.  E.g., for Pacific Standard Time:

ZONE -800  or

ZONE -0800

The ZONE value need not be an integral number of hours, e.g.,

Newfoundland standard time is 3h 30m behind UTC:

ZONE -330

Default ZONE at program startup is Greenwich time.

PRECESS <date time>

Controls the correction of Right Ascension and declination for

precession.  PRECESS sets the final epoch.  The epoch of the elements

is always used as the initial epoch.  For 1950.0 or 2000.0

coordinates, respectively:

PRECESS 1950 JAN 0 2210

PRECESS 2000 JAN 1 1200

These are Greenwich times, so, strictly speaking, the PRECESS command

should be given before setting ZONE.  But for all practical purposes

it doesn't matter.  Precession is so slow there will be virtually no

error even if you miss by a full year.  Over several decades, though,

it will build up to a significant level.  For example, if your atlas

is 1950.0 and you neglect the PRECESS command, an error of up to 42

arc minutes can occur in your plot of a satellite's track.  This is

perhaps four or five times worse than SEESAT's prediction accuracy

under good conditions!

The PRECESS value remains until you change it or exit SEESAT.

Default setting is 2000.0 at program startup.

MERIDIAN

The satellite longitudes in the prediction table may be computed

with respect to either Greenwich or your local meridian.  MERIDIAN

toggles this mode, and informs you of the current mode.  Default is

Greenwich.

ALL

Toggles SEESAT between normal mode (predictions below horizon

suppressed) and a mode which displays all predictions.

Element Set Loading Commands

OPEN <filename>

Opens the orbital element file.  If an element file is already

open, that file will be closed first.

OPEN builds an index of the satellites in the file, using linked

blocks in RAM.  Each block holds 50 satellites.  Storage is requested

as needed at run time, so the size of the element file is limited only

by available memory.  Assuming your system uses 4-byte longs and

2-byte pointers, each 50-satellite block uses 1352 bytes.

The index only contains the name of the satellite and its location

in the file.  The elements are not read from the disk until you issue

the LOAD or NEXT command.

INDEX

Lists the satellites in the currently open file.  If there is

more than one screenful, it will pause with a "more>" prompt.  At this

prompt you may either press RETURN to continue the listing, or enter a

command (or commands) just as you would at the normal command prompt.

In that case, the listing is aborted and your commands are executed.

LOAD <name>

Loads the named satellite from the file you opened with the OPEN

command.  If the name has spaces, enclose the name in quotes.  A

closing quote is not required if the name is the last item on the

line, e.g., you can do

LOAD "COSMOS 1383"  or

LOAD "COSMOS 1383

If the elements fail checksum, you'll get a warning but SEESAT will

let you use the elements.

Setting Run Parameters

START <date time>

STOP <date time>

SPAN <time>

CENTER <date time>

CENTRE <date time>

Set the parameters of the prediction run.  CENTRE is identical to

CENTER.  Any of these commands begins a prediction run automatically

if it's the last one on the command line.

STEP <time>

Controls size of time steps in the prediction run.  A run begins

automatically if STEP is the last command on the line.

There are three modes for setting the time limits of a prediction run:

1.  a start time and stop time

2.  a start time and time span

3.  a center time and time span

For example, these command lines generate indentical prediction runs:

START 1900 STOP 2100 STEP 10

START 1900 SPAN 200 STEP 10

CENTER 2000 SPAN 200 STEP 10

The order of the commands in the line doesn't matter.  E.g.,

STEP 10 STOP 2100 START 1900

START 1900 STOP 2100 STEP 10

have the same effect.

You don't have to re-issue a command if its value hasn't changed.  For

example, suppose you enter:

CENTER 2000 STEP 2 SPAN 8

and decide that the resulting table is satisfactory except that the

span should have been 12 minutes  Enter:

SPAN 12

and a run will occur with the same CENTER and STEP values as before.

To change modes you must give both commands associated with the new

mode.  For example, you do a run with these commands:

START 1730 STOP 1900 STEP 10

Then suppose you want a run starting at 1730, of 12-minute span, with

2-minute time steps.  Enter:

START 1730 SPAN 12 STEP 2

You did START 1730 for the first run, so why do it again for the

second?  Because you are changing modes.  The first run was in

start/stop mode.  You are changing to start/span for the second run,

so you must give both START and SPAN.  If you had said:

SPAN 12 STEP 2

the program would have remained in start/stop mode.  (Again, the order

of the commands on the line doesn't matter, as long as the required

commands are present.)

In center/span mode, the first prediction is always 1/2 SPAN before

the CENTER time, and of course the remaining predictions are spaced at

the STEP value you specified.  This can cause unexpected results if

SPAN is not an even multiple of STEP.  E.g., for

CENTER 1800 SPAN 5 STEP 1

the predictions will be for 1757:30, 1758:30, etc.

Since decimal seconds are allowed in <time> or <date time>, it is

possible to specify the time parameters of a run as finely as you

desire.  I discuss SEESAT's time reckoning accuracy in the section

describing the prediction table.

Some Additional Orbital Element Commands

NEXT

Load the next satellite in the file, i.e., the one following the

satellite currently loaded.  If you haven't yet loaded a satellite

from the open file, NEXT will load the first one.  The file must be

opened with the OPEN command before NEXT can access it.

LENGTH <integer>

Sets maximum number of characters the OPEN command will consider

significant in the satellite name when building the index.  The LENGTH

command must therefore be issued before OPEN, to have any effect.  Any

number from 1 - 22 is allowed.  Default is 22, and may be left alone

unless you're using a file such as Molczan's N2L series.  In that

case, you'll want to reduce LENGTH to 15 to prevent SEESAT from using

the extra data as part of the satellite name.  LENGTH is set to 22 if

you enter a number larger than 22.

AOP <number>

B <number>

E <number>

I <number>

MA <number>

MM <number>

MMDOT <number>

MMDOTDOT <number>

NAME <satellite name>

RAAN <number>

Allow you to manually enter orbital elements or modify elements

already loaded.  Respectivly:  argument of the perigee, BSTAR,

eccentricity, inclination, mean anomaly, mean motion, mean motion dot,

mean motion dot dot, name, Right Ascension of the ascending node.

Mean Motion dot and mean motion dot dot are used only by the SGP

model.  The newer SGP4 model is the one I include with the standard

SEESAT package, so normally the MMDOT and MMDOTDOT commands have no

effect.

EPOCH <epoch>

Manually enter epoch of the orbital elements.  Must be in NORAD

format:  YYDDD.DDD... (use any number of decimal places).  Unused

digits in the integer part of day number must be padded with spaces or

zeros.  If spaces are used for padding, the number must be enclosed in

quotes.  E.g.,

EPOCH 91003.52029891    or

EPOCH "91  3.52029891"

NOMINAL <date time>

ACTUAL <date time>

These commands adjust the epoch and RAAN of the currently

loaded elements for the difference between the nominal and actual

launch times.  They are useful for correcting a prelaunch element

set.  E.g.,

NOMINAL 19 1851 ACTUAL 1918

tells SEESAT that the currently loaded elements assume a launch on the

19th at 1851, but the launch actually occurred at 1918.

You can't use NOMINAL or ACTUAL by itself!  If you use one, you

must also use the other or you'll get crazy results.  The order of the

commands does not matter, and they don't have to be on the same line.

Just be sure that both commands have been given before starting a

prediction run.

The entered values are remembered.  So you may, for example,

use NOMINAL just once, then experiment with different ACTUAL values.

Loading an element set (even reloading the same one) disables the

effect of NOMINAL and ACTUAL.  Their values are still remembered,

however, so you may re-enable the adjustment by giving one or both

commands.

The NOMINAL and ACTUAL arguments may be for any time zone, as

SEESAT cares only about their time difference.

MAG <number>

For entering the absolute magnitude of a satellite.  It will be

adjusted for range and illumination angle to generate the "mag" value

in the prediction table.  Absolute magnitude is its magnitude at 1000

km and 50% illuminated (i.e., 90 degree phase angle).

Absolute magnitude input can be automatic during loading of the

elements from the file.  If the first line of the element set (the

satellite name line) is longer than 32 characters, SEESAT assumes it's

a Molczan format line, and reads the magnitude.  You can use the MAG

command to override the value if necessary.

MAGBIAS <number>

Bias to be applied to SEESAT's computed magnitude before display.

A negative sign is allowed.  The default is zero.

If your absolute magnitudes assume a different range and/or

illumination than 1000 km and 50%, the MAGBIAS command will bring your

scale into coincidence with SEESAT's.  If r and k are your assumed

standard conditions (in km and percent, respectively), set MAGBIAS to:

2.5 * log10 ((1000/r)^2 * k/50)

For example, if your absolute magnitude is for 1000 km range and 100%

illuminated, enter:

MAGBIAS .8

OFFSET <time>

Applies an offset to the epoch of the satellite elements, thereby

making the satellite come early or late in the predictions.  Useful

for putting a satellite ahead of or behind schedule, to evaluate the

resulting track drift with respect to the stars.  Also can be

used to adjust for any discrepancy noted between predicted and actual

times of passes.  A negative sign is allowed on <time>.  A negative

<time> will make the effective epoch EARLIER, and make the satellite

come EARLIER in your predictions.

If OFFSET is nonzero, an advisory of its value is printed at the

top of each prediction table.

OFFSET is reset to zero when an element set is loaded.

Flow Control Commands

The next three commands only work within the same command line.

PRINT?

If the last prediction run resulted in a line of data being

printed, execute the command to the right of PRINT?.  Otherwise,

skip it.  There must be at least one command after PRINT?.

REPEAT

Jump back to beginning of command line.

SKIP

Skip the command to the immediate right of SKIP.  To be used

following PRINT?, to reverse the test.  There must be at least one

command after SKIP.

Other Commands

SUN <date time>

MOON <date time>

Print the azimuth & elevation of the body at the given time.  For

the moon, percentage of illumination is also given.

PARA <date time>

Print the parallactic angle at the predicted position of the

satellite for the given time.  Parallactic angle is the direction of

celestial north, as seen in a binocular field of view.  E.g., 0 =

straight up, 90 = 3 o'clock.  This command allows you to examine your

star atlas plot and visualize the star field orientation you'll see

when you go outside.

RUN

Begin a prediction run, using the current time parameters.  The

START, STOP, CENTER, SPAN, or STEP command automatically begins a

run if it is at the end of the command line.  That is the normal way

to get a run.  The RUN command is convenient if, for example, you load

a new element set and want a run without changing time parameters.

EX <filename>

Execute a batch file of commands.  Any SEESAT command may appear

in a batch file.  Multiple commands per line are allowed, just as if

you were entering the command line manually.  EX itself may be in a

batch file.  If encountered, it will close the current batch file and

begin executing the specified file.  Control will not return to the

preceding file.  I.e., you can chain batch files but not nest them.

RET

If encountered in a batch file, returns control to user.  If

entered manually, resumes execution of the batch file.

HELP

Displays a help screen.

EXIT

Exit from SEESAT.

I have two final points to explain about the command interpreter.

First, it is legal to give the same command more than once in a line.

If you have entered a long command line, it is sometimes easier to

repeat a command than to backspace to correct an incorrect argument.

Second, your copy of SEESAT may have the ability to abort a prediction

run or execution of a batch file.  I say "may" because there is no

portable way to implement this feature.  There is provision to install

it in the program, and I've supplied routines for Turbo C (for IBM

clones) and Laser C (for the Atari ST).  If your SEESAT was created

with either of these compilers, you can press any key during a

prediction run, execution of a command line (even an iterating one),

or execution of a batch file, and get the > prompt back.

Prediction Table

A table of predicted satellite positions is the main output of SEESAT:

NOAA 9        1991 OCT 27

time   az el   R.A.    dec    range sun   mag    alt  lat    lon

1845:00 196 15  19h59m -37 20'   2081  12 * 1.8    846 19.1N 122.6W

1846:00 203 22  19h39m -29  0'   1755  12 * 1.5    845 22.6N 123.5W

1847:00 215 29  19h14m -17 50'   1476  12 * 1.2    844 26.1N 124.4W

Time is the time of the prediction.  The table value is rounded to the

nearest second but, internally, time is maintained to full precision.

For an IEEE double precision value (53-bit significand), this is about

30 microsecond precision in the context of SEESAT.  Therefore, it's

practical to use split-second time steps in the table if you can keep

track of what entry goes with what time.

To reduce cumulative error, the time variable that is incremented

during prediction runs is based on the epoch of the element set, not

on 4713 BC as is the rest of SEESAT's time reckoning.  This variable

would typically have a (IEEE double) precision of .2 or .3 nanosec.

Azimuth and elevation are rounded to the nearest degree.  There is no

adjustment of elevation for refraction.

Right Ascension is rounded to the nearest minute, declination to the

nearest multiple of 10 minutes.  Both are adjusted for precession.

Range is the straight-line distance in kilometers.  It is truncated to

an integer.  (Truncation drops the fractional part, thereby creating

an error of up to 1 in the last place.  On the other hand, rounding

guarantees an error of 1/2 the rounding precision or less.)

"Sun" is the elevation of the sun above the satellite's geometrical

horizon (i.e., the horizon adjusted for dip due to satellite height

but not for refraction), rounded to the nearest degree.  A positive

number indicates that the satellite is in sunlight.  Since the horizon

dip in low earth orbit may be 30 degrees or more, the "sun" value can

considerably exceed 90 degrees.

The magnitude is an estimate based on the satellite absolute

magnitude, the phase (illumination) angle, and range.  If SEESAT

senses a Molczan-format element file, it automatically extracts the

absolute magnitude.  Otherwise, you may enter it via the MAG command.

If no absolute magnitude has been input, the displayed magnitude is

prefixed with a *, indicating that this is a "relative magnitude" with

respect to the standard conditions of 1000 km range and 90 degrees

phase angle.  E.g., "* -.6" means that the satellite will be about .6

mag brighter than it would be at 1000 km and 90 degrees phase angle.

You can change the range and phase angle corresponding to relative

magnitude 0.  Use the MAGBIAS command and the formula in its

description.  But note that the MAGBIAS value will still be in effect

if you enter an absolute magnitude.

Altitude is in km, truncated to an integer.  It assumes a spherical

earth 6371 km in diameter.

Latitude and longitude are rounded to the nearest .1 degree.  Latitude

is the geocentric latitude.  Longitude may be made to read with

respect to Greenwich or your local meridian, toggled with the

MERIDIAN command.

8.  Batch Files

A batch file is an ASCII file containing commands for SEESAT.  Lines

may be up to 80 characters long.  Any command which is legal to enter

at the > prompt may be placed in a batch file.  The file may be any

length.  A batch file is executed by typing EX <filename>.

If SEESAT finds EX in a batch file, any remaining commands on the

line are executed normally, the current batch file is closed, the new

one is opened, and it begins execution.  Control will NOT return to

the old batch file.

At startup, SEESAT looks for file SEESAT.BAT.  If present, it is

executed automatically as a normal batch file.  For example, my

SEESAT.BAT contains:

height .73 lat 34.915 lon -117.933 zone -800

start 1992 jan 1 0 length 15

meridian

This begins by setting my location and time zone.  Then the START

command establishes a default year and month.  (Any command that

requires a <date time> argument would do the same.)  The day and time

(1st of month at midnight) are dummy values; I only have them there

because it's illegal to omit them.  They could be any value.  To

prevent START from triggering a prediction run, I follow it with a

command to set the max name length to 15.  Finally, I prefer to have

satellite longitudes with respect to my local meridian, so "meridian"

toggles SEESAT to the correct mode.

This version of SEESAT introduces the PRINT?, REPEAT, and SKIP

commands for controlled iteration.  For example, the following

commands will step through a file, returning control to you if 1) a

prediction above the horizon occurs, or 2) end of element file is

reached.

START 1991 OCT 20 1900 SPAN 200 STEP 8 OPEN 897.ELE

NEXT RUN PRINT? SKIP REPEAT

The first line sets up our prediction run parameters.  Since any of

the parameter-setting commands will trigger a run if it is at the end

of the line, I place the OPEN command there.  The second line does the

iterating.  It uses NEXT to load the next element set, performs a

prediction run, then executes PRINT?.  PRINT? checks if a line of data

has been printed.  If true, the next command executes.  If false,

execution jumps over the next command.

Iterating command lines may also be placed in batch files.  For

example, you could check a file for visible satellites with the

following batch file:

NEXT START 1900 SPAN 200 STEP 8 RUN PRINT? RETURN REPEAT

Suppose that line is a batch file named A. You'd invoke it as follows:

START 1991 OCT 20 0 OPEN 897.ELE EX A

The START command on this line is just to set up the default date.

That allows you to omit date from the batch file so you can use the

same one day after day.  The batch file will keep loading and running

predictions until something comes over the horizon.  It then does a

RETURN, at which point you can manually explore the pass in more

detail.  To resume checking the file just type EX A.  (You can't

use RET, since the single line in the batch file has already been

executed.)

A batch file can also check a specific list of satellites.  For

example,

LOAD "NADEZHDA 1"

START 1900 STOP 2100 STEP 10 RUN PRINT? RETURN

LOAD "NADEZHDA 2"

START 1900 STOP 2100 STEP 10 RUN PRINT? RETURN

LOAD "NADEZHDA 3"

START 1900 STOP 2100 STEP 10 RUN PRINT? RETURN

will, with proper lead-in commands, check the evening for passes of

these satellites.  In this case, you'd use RET to go back to the

batch file after exploring each visible pass.

Element File Format

Each orbital element set comprises three lines in the following format.  (The

fourth line here is a ruler to help you count characters and is not present

in an actual element set.)  A file may contain any number of these element

sets.  Blank lines are allowed, but any other extraneous lines (titles,

comments, etc.) are NOT PERMITTED!

CCCCCCCCCCCCCCCCCCCCCC         -N.N

1                 NNNNN.NNNNNNNN -.NNNNNNNN          -NNNNN-N       N

2       NNN.NNNN NNN.NNNN NNNNNNN NNN.NNNN NNN.NNNN NN.NNNNNNNN     N

123456789 123456789 123456789 123456789 123456789 123456789 123456789

C can be any character

N is any digit.

- represents either a hyphen or a space.

The first line is the name of the satellite.  The line may any length,

but only the first 22 characters are recognized as the name.  (22 is

the default; it may be changed to anything from 1 to 22 via the LENGTH

command.)  Trailing spaces are removed from the name and it is

converted to upper case before it is placed in the index.

During loading of an element set, SEESAT checks the length of the name

line.  If it is longer than 31 characters, it is assumed to be a

Molczan-format name line, and the field beginning at character 31 must

contain the satellite absolute magnitude.  See the diagram above.

The second and third lines are the standard Two-Line Orbital Element Set

Format used by NASA and NORAD.  The format description is:

Line 2

Column		Description

1		Must be digit 1

19 - 20		Epoch Year (Last two digits of year)

21 - 32		Epoch (Julian Day and fractional portion of the day)

34 - 43		First Time Derivative of the Mean Motion

54 - 59		BSTAR drag term (decimal point assumed before 54)

60 - 61		power of 10 to multiply BSTAR by

69		Check Sum (Modulo 10)

Line 3

1		Must be digit 2

9 - 16		Inclination [Degrees]

18 - 25		Right Ascension of the Ascending Node [Degrees]

27 - 33		Eccentricity (decimal point assumed before 27)

35 - 42		Argument of Perigee [Degrees]

44 - 51		Mean Anomaly [Degrees]

53 - 63		Mean Motion [Revs per day]

69		Check Sum (Modulo 10)

The NORAD 2-Line element sets contain other fields, but the

above are the only ones used by SEESAT.  First derivative of mean

motion is not used if BSTAR is present.   Except for BSTAR, leading

and trailing zeros are not required.

Compiling SEESAT

The source code is in seven files:

SEESAT.H

SEESAT.C (only 2 lines)

DRIVER.C

UTIL.C

SGP4.C

ASTRO.C

READEL.C

Check the "Misc Dependencies" section in SEESAT.H.  You may have to

make changes.  Every C file has a #include for SEESAT.H; you may have

change my driver specifier so your compiler will find SEESAT.H.

References

Robert M. L. Baker, Jr. and Maud W. Makemson, "An Introduction to

Astrodynamics", Academic Press Inc., New York, 1960

W. Gellert, H. Kustner, M. Hellwich, H. Kastner, "The VNR Concise

Encyclopedia of Mathematics", Van Nostrand Reinhold Co., 1975

Robin M. Green, "Spherical Astronomy", Cambridge University Press, 1985

Jean Meeus, "Astronomical Formulae for Calculators", 3rd ed.,

Willmann-Bell Inc, Richmond, Va., 1985

"American Practical Navigator", Vol. I & II, Defense Mapping Agency

Hydrographic/Topographic Center, Washington, D.C., 1984, 1981 respectively

Felix R. Hoots and Ronald L. Roehrich, "Spacetrack Report No. 3", Office of

Astrodynamics, Peterson AFB, 1980

U.S. Naval Observatory, "Nautical Almanac 1992", Washington, D.C., 1991

Finally, thanks to the people who contributed suggestions, reference

materials, and testing time to SEESAT:

Rocky Harper

TS Kelso

Ted Molczan

Rick Welch

John Williams

Walt Witherspoon

#o your compiler will find SEESAT.H.