ORBIT

A Spaceflight Simulator

Charles Goldberg

RD 2 Box 415

Titusville, New Jersey 08560

ORBIT is a working orbital simulator similar to those used by

NASA to plan and monitor spaceflights in near-earth and lunar

trajectories.  Based on thrust and vector data which you pro-

vide, the program calculates and displays the resulting orbital

trajectory.

The program is based on a time-driven numerical integration

of the Newtonian three-body problem (Euler's Method).  It pro-

vides for the entry of an initial launch vector and any number

of subsequent burns.  It is capable of reproducing a full range

of orbital trajectories, including:

o Suborbital flights

o Parking orbits

o Polar orbits

o Deburn & reentry

o Transfer trajectories

o Lunar bypasses

o Lunar orbits

o Lunar landings

o Geosynchronous orbits

and more.

In fact, the simulator will reproduce almost any real-life tra-

jectory within a half-million miles of earth with sufficient

accuracy to permit serious research.

However, ...

BE PREPARED FOR A VERY SLOW PROGRAM!

Depending on the speed of your PC, ORBIT can take as much as two

hours to complete a six-day lunar mission.  Longer missions

(e.g. bisynchronous orbits) may take substantially longer.  For

the sake of your monitor:

IF YOU PLAN TO LET ORBIT RUN FOR MORE THAN A FEW HOURS, REDUCE

CONTRAST ON YOUR MONITOR OR EVEN SWITCH IT OFF TO PREVENT THE

IMAGE FROM BURNING IN.

STARTUP SEQUENCE

At the end of this document, a number of "recipes" are given to

get you started.  I recommend that you follow these recipes

carefully, saving your launch files.  You will find that these

launch files will save you an enormous amount of time in later

simulations.

After following a recipe, you might want to repeat the recipe with

slight differences in your vectors.  Observing the results of

minor variations will give you a feel for your spacecraft very

quickly.

When you start the program, you will be asked for a launch file.

If you enter a <Return>, the program will place you at Kennedy

Space Flight Center in Florida.  Later, as you build your launch

file library, you will be able to choose other places (and times)

for your starting point.

Next, the program asks for a launch date.  If you enter <Return>

it will set the launch date for July 1st at 00:00 UT.  Until

you are fully familiar with the program, the default setting

will be just fine.

Next, the program asks for a "Differential Velocity" in kilometers

per second (i.e. the increase in the spacecraft's velocity

as a result of the "burn").  Use your recipes initially, then

modify this value to produce alternate trajectories.

When you are making a ground launch from KSFC, the program will ask

for an azimuth (follow your recipes at first).  If you are executing

a "burn" in space, the program will ask for a polar vector and an

equatorial vector.  Once again, follow your recipes.

At this point, you are ready to launch.  Watch your telemetry (the data

printing out on the right hand side of your screen).  As you become

more familiar with these readouts, you will begin to "feel" the

spacecraft.  Also, in following the recipes, note your telemetry

readings at each orbital maneuver.  Particularly watch for perigee

and apogee, and keep an eye on your velocity.

When executing an orbital maneuver, check your telemetry to identify

where some of the recipe values are coming from.  Learn to recognize

direction-of-motion vectors, tangent vectors and compensating vec-

tors.  Note how they are used.

IN FLIGHT

Your control keys will be displayed at the bottom of the screen.

F2   PLOT    This key switches your display to geocentric plot.

There are two plot options, polar or equatorial.

The polar plot will usually be more useful, but

you should get familiar with both.

F3   VIEW    Switches among the various geocentric plot options:

E: earth-centered plotting

S: spacecraft-centered plotting

M: moon-centered plotting

+: zoom in

-: zoom out

D: data switch (for full screen graphics)

F4   MAP     This is the Mercator plot, showing the spacecraft's

position in latitude and longitude.  This is the

display shown at launch.

These keys can be used at your option, at any time, to change the

display.  Changing these keys does not alter the simulation, only

the display.

F5   EPHM    This key provides an on-line lunar ephemeris for use

in planning lunar trajectories.  When you finally

understand how to use this table, you will be ready

to select alternate dates for your launch!

F6   TIMER   This key works just like an alarm clock.  You set it

for a specific time.  When the simulation reaches

the set time, the timer "beeps" and the simulation

pauses for a manual restart.  Use the timer for

wakeup calls on long, boring sections of your missions.

F7   LOAD    Use this key to read in a previously saved launch file.

After reading in your launch file, the program will

give you the option of entering a burn.  If you

<<return>> through the vector inputs, the program

will pick up at the time and direction vector at which

the flight was initially saved.

F8   SAVE    Use this key to save your current position in a launch

file.  The program will ask for a file name, which

can be any legal DOS name (including path and prefix

if you wish).

F9   SLEEP   This is an important key.  Use it to switch between

the simulators two time modes:

Regular time (40-100 simulation minutes per minute of

real time)

Sleep mode (150-400 simulation minutes per minute of

real time)

Use the regular mode for near-earth orbits or at other

times when you need minute-by-minute readouts.  Use

sleep mode during less critical periods.  In sleep mode

readouts are only displayed every 4 simulation hours.

F10   OMS    This is the orbital maneuvering system, the spacecraft

engines.  They can be engaged at any time.  The simu-

lation will pause, asking for differental velocity,

polar vector and equatorial vector.  It will then

execute the burn and continue the simulation.  Follow

your recipes to start.  Then add your own variations.

In addition to the program keys, the following keys can be used at

any time:

P: resets the perigee gauges

A: resets the apogeee gauges

+: zoom in

-: zoom out

THE RECIPE BOOK

1. SUBORBITAL FLIGHTS

Using a standard launch velocity of 4 km/sec, complete a variety of

launches from Kennedy Space Flight Center, with varying azimuths

(0, 90, 180, and 270 degrees).  Observe the effects of earth rotation

on apogee and flight duration.

2. STANDARD ORBIT

Launch from KSFC at a launch velocity of 3 km/sec and 90 degree

azimuth.  At t-plus-5 minutes, tap F10 and enter a differential

velocity of 5.7 km/sec, polar vector 6.8228 degrees, equatorial

vector 95.5907 degrees (check your telemetry to see where these

numbers are coming from).

SAVE (F8) this orbit at t-plus-15 minutes.

Observe the orbit for several revolutions in each of your display

modes.

3. POLAR ORBIT

Launch from KSFC at 4 km/sec and 0 degree azimuth.  At t-plus-5,

enter a 5.5 km/sec burn at 127.7023 & 0.0000 vectors.

Observe how earth rotation (in Map mode) makes polar orbits use-

ful for mapping projects.  Observe difference in energy require-

ments as compared with a standard eastern launch.

4. EQUATORIAL ORBIT

Start from your standard launch file at t-plus-15 minutes.  At

t-plus-31 minutes, execute a 3 km/sec burn at 71 & 196.1490.

Save for later use in geosynchronous transfers and lunar

trajectories.

5. GEOSYNCHRONOUS ORBIT & APOGEE KICK

From equatorial orbit, place the spacecraft into a Hohmann Transfer

Orbit to Geosynchronous Apogee (at Radius Vector = 26270 miles).

Find the correct velocity by trial and error, using direction

of motion readouts for polar and equatorial vectors.

The objective of this part of the project is to use just enough

thrust to get your spacecraft to the proper RV.  In the process

you will be getting a feel for transfer orbital mechanics.

Once you have your spacecraft at the right RV, execute another

burn at apogee to bring your spacecraft into uniform circular

motion (UCM).  Calculate your velocity by comparing the UCM

readout with your velocity readout.  Execute your burn in the

direction of motion.

Confirm that you have achieved geosynchronous orbit by extended

tracking in map format (the spacecraft should move in a small

figure-8 VERY SLOWLY).

6. LUNAR TRAJECTORY

By now, you should have learned that you produce a transfer

orbit by burning (usually in the direction of motion) at the

point in the orbit where you want to place your perigee.

Apogee will then be located 180 degrees from perigee.  In other

words, you go where you want to go by aiming your apogee.  Then,

when you reach apogee, you kick your spacecraft into whatever

local trajectory you want.

Now, if you provide an additional 3.1 km/sec to your spacecraft

from equatorial orbit, you will produce a transfer orbit with

an apogee at about 238000 miles (i.e. at the distance of the

lunar orbit).  It takes about three days for the spacecraft

to make that journey.

Now all you have to do is aim your apogee at the place where

the moon will be in three days and you will get a close en-

counter.  The lunar ephemerides should help you to pick your

lunar window.

If at first you don't succeed...

7. THE ADVANCED STUFF

If you have run enough lunar trajectories, you will have dis-

covered rubber-banding, retrograde bypasses, lunar orbits

and lunar landings.   Now, you're ready for the interesting

stuff.

For example, in 1975, two Soviet astrophysicists, V. Ivashkin and N.

Tupitsyn published an article pointing out that (especially at

higher earth latitudes) a geosynchronous orbit could be achieved

with less energy by using a lunar bypass as an accelerator into

geosynchronous orbit.  In other words, instead

of firing a standard transfer orbit and apogee kick (like you

did in Project 5), they recommend a lunar trajectory which will

loop behind the moon and fall back toward earth.  When the returning

spacecraft reaches geosynchronous distance, you can execute a

burn to change the orbit into geosynchronous uniform circular

motion. They claim that such a procedure will require less energy

than the more traditional method.

Are they right?

Or try this one.

Suppose that we want to colonize the moon.  Large quantities of

materials and manpower need to be shuttled to the moon.  Now,

of course, we could transfer it all in individual spacecraft,

but that's expensive and unnecessary.

Suppose instead that we placed a cargo transport in an elongated

orbit with its perigee close to earth and its apogee at lunar

distance.  Can we develop a trajectory which will shuttle back

and forth between the earth and moon in a stable bisynchronous orbit?

If we could set up that type of trajectory, we could load the

transport here and offload it there with minimum energy.