***************************************************************************

PC PURSUIT automatic scanning dialer for the ST

SHAREWARE (distribute freely/contributions welcome) by

Tom Zerucha

24075 Lee Baker

Southfield, MI 48075

Special features of this program include a simple list scanner, instead of

programming a custom script for other communication programs.  It also will

automatically break out of the Racal-Vadic command submode if the outdial

modem is in it (you normally see INVALID COMMAND/MANUAL ANSWER, etc. to

normal Hayes commands).  It also stops trying non-operative area codes

and will loop until a connect is made.  If you specify neither -E or -I

you can run this in background under MT C Shell - the only messages will

be the DIALING TELENET and [bbsname] CONNECTED.

***************************************************************************

Options (specify as first parameters on line -A -B same as -AB, uc/lc same)

-C	reconnect with telenet before proceeding (noise on your end)

-E	echo what is going on to console

-R	redial last accessed number

-1...9	repeat count within areacode (default is 3 - number of tries)

-I	keyboard input control - the following keys are active

esc	redials local TELENET

space	go on to next area code

return	hang up on telenet and exit

any other key	exit, but preserve connection

***************************************************************************

Reads a reperetory file (that you can read too and modify with a standard

editor) with phone numbers, a flag to activate it, and a comment field.

Also within the file are the userid and password.  The exact format is

<local_phone_number>		- number preceeded with T(one) or P(ulse)

<userid>			- to access PC Pursuit (DIALxxx/12,________)

<password>			- corresponding password (PASSWORD = ________)

aaa-ppp-nnnn-ffff-BBSname-comments...

- a blank line may be inserted anywhere

000 000 0000 0			- last line, only the 000 area code required

(any info following will be ignored, move unused numbers here if you don't

want to delete them, since the default file will be bigger and take longer

to access.)

the local phone, user id and password must be on the first lines in the order

given.

aaa - area code (PC Pursuit access code in DIAL___/12)

ppp - prefix (seperate for later prefix verify)

nnnn - number

f - 1-4 digits, flag even means don't access, odd means try access.

comments - at least 1 character, can use numbers, any non control char.

The dashes can be any nonnumeric character (or left out completely, but the

field must be the proper length, i.e. flag should be 0001).

The first 21 (or less) characters of the comment field are displayed

when a connect is made, usually as the BBS name.

The file defaults to "AUTODIAL.REP" if no parameter is specified.  It will

also ignore the local dialing and assume Telenet is already connected.  If

you desire local dialing first, specify autodial.rep in the command line.

This was designed to be used with shells, (and with a communication program

that chains to this that I am still developing,) so change to .TTP to use

from the desktop.

Local Phone Number is sent to modem verbatim (preceeded w/ ATD and

with a cr afterword - up to 19 chars max hayes compatible).

The area codes are collected, and when a city is reached, each number

in that area is tried.  They do not have to be grouped in the file.

The area codes are tried in the order they first occur in the

list.  Any message except BUSY or CONNECTED as a response to

an area code flags it and is not tried again (like NOT OPERATING).

Break is supported, Telenet is set so that <cr>@<cr> will no longer work

(important when uploading), and Break will instead.

The numbers within the area code are tried in the order they were entered.

if all are busy, that area is disconnected and the next area code

is tried.

The login dialog is displayed while the program is active (all the prompts

and messages appear on the screen).  In addition, some messages

may appear (like when the password is being entered).

Preset the RS232 config call for your application. The program does not

change port characteristics.

This program should be compatible with most shells.

When a connect is made, the program exits after writing the reperetory with

modified flags to autodial.rep.  The bell is rung twice and two

returns are sent (to set baud rate for some BBSs).

In the prototype repertory, information can be placed after the 000 000 line

such as phone numbers rarely dialed.  This will speed things as the

working repertory will not be as large.

The program will exit if a very serious error occurs - with a hangup command

to TELENET.

The program uses a Malloc to get a buffer for the file and doesn't make

sure the file is shorter than the buffer.  It requests all memory

minus 32K.  It only reads in as much of the file as will fit.

Duplicate phone numbers will be attempted twice (or as many times they appear

in the list).

Keys supported:

[esc] - redial telenet (if there is noise on your connection)

[return] - exit to telenet and hang up

[space] - skip to next area code

any other key exits, but does not alter the connection

If the connection is successful, a file called autodial.rep will be written

out (which is the default if no parameter is specified), in format where

it can be read again.  The flag will have modified bits:

0000 0ABC -

A will be set as a connected flag.  This will allow bit C to

be selectively set to restore the number (for the

next day's pursuit).

B will be set as a last number dialed. - will be cleared by

the next run of this program if another number is reached.

later the program will resume at this number, or to

recatalog the number (Singly vs Daily access).

C will be zero for the attempted number (to prevent reaccess)

(since a 1 in this position will cause the number to be

tried).  This is so you can enter every number into a

master repertory, with the main

Additional bits will allow for specifying wether the number is

to be single access, daily access, limited hours, etc.

Extra fields may be added (try count, access count, hours, etc.)

The program doesn't catch area codes not part of PC Pursuit.  They will be

attempted but will give the unknown address message.  No conversion

is performed either (703's are accessed from 202)

If the modem at the outdialing city is badly jammed (I have taken care of

the Racal-Vadic submode, but apparently other things can happen)

it will give no messages but continue with the next area code.

It seems that after a number of successful connects (and subsequent

disconnects), Telenet hangs up.  In this case, the local number

will be redialed.

EXAMPLE FILE (use your own local number, IDs and passwords)

***************************************************************************

P5551212

USERIDXX

PASSWORD

408 745 5308 1 Atari Corp Line 1              active

408 745 5970 0 Atari Corp Line 2              inactive

000 000 0000 0