Programming a Math Co-Processor-Part 1.

Now that you have purchased your TT and are amazed at its awesome

possibilities, what do you do next.  As you probably realize, there is

an excellent math co-processor, the Motorola M68882, sitting on the

mother board, waiting to quickly multiply numbers together, in a

blazing burst of speed.  What you may not have realized is that the co-

processor not only improves floating point mathematics, but also

integer and long integer multiplication.  The chip also supports all

three of the IEEE standards for binary representation of floating point

numbers.  What this means is that a floating point number can have

either eight, sixteen, or twenty four significant digits, an accuracy

that is astounding.

But how do you use the M68882 without purchasing a new, expensive

compiler, with idiosyncrasies that you are unaware of.  Also what do

you do if your favorite compiler is no longer being produced, or your

favorite language is an Esperanto version of Snobol, and you are one of

the three people in the world that is still using it.  Well, this is

simple enough.  Almost all high level languages allow for extensions to

them; this is a significant advantage of them.  If you are writing in

Assembly code, just skip over the next few paragraphs, flagellate

yourself several times to get in the proper mood, and just start

putting code down on paper.

My favorite language is Modula-2, which was written by Nikolas

Wirth, the author of Pascal.  He wrote the language as a programming

tool, while he considered Pascal a teaching tool.  The major

differences are in the modularity that is possible in the language.

Modula-2 uses the concepts of libraries that are to be specified at the

beginning of a program (similar to the <include>  statements in C).

Each library consists of two programs, a Definition section and an

Implementation section.

The definition module just shows what functions and variables can

be used from the implementation library.  The procedure definitions are

easy to write, and quickly specify the form and usage of procedures and

functions.  The implementation module is the actual heart and guts of

the procedure, performing the action.  When a Definition module is

compiled, a number is written in the file to give it a certain version.

When the Implementation module is compiled, then this same number is

written into the file.  Thus if you change the Implementation file, the

same Definition file can be used.

Writing in Modula-2 is quite similar to writing in C.  You first

type in the program using an Editor.  Then a compiler produces source

code, and finally a linker stitches it altogether to make a runable

program.  There are some differences in the actions however.  First,

when the compiler runs, it checks all of the Definition files as to

proper usage of the various procedures.  Modula-2 is very strong on

type checking.  You must use the same sort of number for operations.

Thus you cannot multiply a floating point number with an integer.  You

would get an error message.  If a procedure expects two integers and

returns a character, then it must be sent two integers, and the

resultant answer be stored in a CHAR variable.  It also writes the code

number into the source code for the linker to find.

The linker then looks into each library, and makes sure that the

proper code is present.  Since it is an optimizing compile, it then

takes each procedure that is needed, and links only those together to

make a program.  This is a nice quick efficient method.

Thus to use the math co-processor, all that is needed is a math

co-processor library to be linked with the rest of your procedures.

For each of the mathematical functions, write the proper procedure to

use the processor.  Similarly, write a side stream if a co-processor is

not present.  (Never discriminate against people without math co-

processors.  Until recently you too were one of those unfortunate

souls.)  Write procedures for trigonometric functions, the normal

mathematics ones, and even for the various types of variables.  Quite

simple.

Now is it as easy as it seems.  Yes it is, if you have the proper

language.  Modula-2 is a working language and has many features that

Pascal did not have.  These include bit functions, the ability to

interact with the various registers, and even the ability to put

assembler code directly into the program.  Each of the eight Data

registers (D registers) can be manipulated, as well as the eight

Address registers (A Registers).  To put information into the Data

register the command is PUTREG(value,#) where the data registers

correspond to 0 to 7.  The Address registers are called 8-15.  To

retrieve information the command is REGISTER (#) where # is 0-15

corresponding to either the Data or Address register.

The command for inline Assembly coding is CODE() so that you can

hand assembly anything you wish into a program.  I can hear the screams

of anguish now.  "Hand assembly of code went out 25 years ago.  If I

could write assembly, I would."  I also can hear the snickering between

the whip strokes.  It is not as difficult as you think, all you need is

a combination of commands between high level and low level with a

smattering of intellectual effort,

Let us start with the most important part of the library, the

determination of the presence of the math co-processor.  This needs to

be done in Supervisor mode.  Put a value of 0 into floatflag.  Put a

value of 0 into the D0 register.  Perform a float no operation command

(FNOP), and then move the value in D0 into floatflag.  If the value is

2, then the M68882 math co-processor is present and if the value is 1

or 0, then it is not present.  See how easy it was to do. Two CODE

commands, and rest in a high level language. (Don't forget to put a

return at the end of any procedure that is called by the Supervisor as

well as disable the stack checking and always generate 16 bit relative

calls.)  Note that FNOP had been set up as a constant before it was

used.

PROCEDURE CheckChip;

BEGIN

SETREG(0,0);

CODE(FNOP);

floatflag:=BYTE(REGISTER(0));

CODE(04e75H);   (*RETURN*)

END CheckChip;

Performing a mathematical operation is just as easy.  There are

two types of operations, diadic and monadic.  This means that one

number is needed for the operation vs two numbers.  An example would be

the addition function.  You need two values if you are going to add

them.  Conversely, the SINE function only needs one number.  (There are

even functions that need no numbers, like the return of a constant like

pi or e.)

Monadic functions are the simplest so I will try to explain the

usage first.  Let us use the SINE function since this is one of my

favorites.  Get the number and put it into the D0 register.  (The D0

register is never reserved.)  Tell the M68882 chip that you want to

find the SINE of the value found in the D0 register and put that value

in the floating point register #0.  Now move that value back to the D0

register and return the value from the function.  To find the COSINE of

a value, tell the M68882 the Cosine function, and everything else is

the same.  Note that F68882, REALOP, and FSIN are constants.

PROCEDURE Fsin(x:REAL):REAL;

BEGIN

IF INTEGER(floatflag)=2 THEN

SETREG(0,x);

CODE(F68882);CODE(REALOP+FSIN);

CODE (0F200H  );CODE(6400H);

RETURN (REAL(REGISTER(0)));

ELSE

RETURN sin(x);

END;

END Fsin;

Diadic functions are only slightly more difficult.  Let us add

two numbers together. Get the first number and put it into the D0

register.  Move that value to the FF0 register.  Get the second number

and put it into the D0 register. Tell the M68882 chip that you want to

add the value found in the D0 register with the number in FF0 register

and put that value in the floating point register #0.  Now move that

value back to the D0 register and return the value from the function.

To subtract two numbers, tell the M68882 the FSUB function, and

everything else is the same.

PROCEDURE Fadd(x,y:REAL):REAL;

BEGIN

IF INTEGER(floatflag)=2 THEN

SETREG(0,x);

CODE (0F200H  );CODE(REALOP);

SETREG(0,y);

CODE(F68882);CODE(REALOP+FADD);

CODE (0F200H  );CODE(6400H);

RETURN (REAL(REGISTER(0)));

ELSE

RETURN (x+y);

END;

END Fadd;

Is it really this easy?  Well almost!  There are two slight

modifications that are needed.  First of all, a number is not a number

is not a number.  There is a great deal of difference in how a long

integer is stored as compared to a BYTE and there is even more

differences between the various types of floating point numbers. You

must tell the M68882 what the format of the number is.  This is also

where a major modification needs to be made.

When a long integer or a REAL is put into the 32 bit long D0

register, it fits well.  When a LONGREAL which is 64 bits long is put

into a 32 bit register, who knows what will happen.  Luckily, the

M68882 can use indirect addressing, and this will be explained when the

next module comes out.

The second difference is in sending the number out of the FPP.

All of the calculations use the complete 96 Bit complexity that it

can, but system can send out numbers in any format that is desired.

This must also be specified and will be in the next installment - LONG

FLOATING POINT NUMBERS.

Otherwise, it is as simple as I am making it to be from the

inclosed Modula-2 code.