Cold Hard Cache 4.0

Cold Hard Cache 4.0

for the Atari ST and MEGA ST Computers

with or without a Hard Disk

Copyright 1989, Robert E. Owens III

Cold Hard Cache is an easy to use disk caching  program  that

works  with both hard drives and floppies.  Disk caching  improves

disk  I/O  speed  by  keeping  the  sectors  that  are  used  most

frequently in RAM. The I/O speed of a cached drive will be between

that of a RAM disk (very fast) and the uncached drive (much slower

than a RAM disk).

Benefits:

o  Faster disk operation.

o  Longer life for your disk drives.

o  Longer life for your media.

o  Less waiting on programs that grind your disks.

Drawbacks:

o    The  cache program and the sector buffers take  up  some

RAM memory (about 20K minimum).

Cold Hard Cache was designed to be as small, reliable, and as

fast as possible.  The program buffers reads only.  All writes  go

directly to disk unless the sector is already in the cache and the

data  has not changed.  This ensures that the disk's contents  are

current  in  case  of a  crash,  power  failure,  or  accidentally

powering off the computer.

The size of the code may surprise you.  Good things  come  in

small packages. Version 1.0 was about 8K long while Version 3.5 is

just over 2K long.  The code has been carefully refined and tested

for  about  two years on several models of hard  drives  including

Atari,  Supra,  and  ICD models and should be compatible with  any

other models.

Included Files:

cachev4.prg    The caching program.

chcstat.prg    The   GEM   application   status/configuration

program.

chcstat.acc    The    GEM   accessory    status/configuration

program.

chcstat.rsc    The GEM resource file for both chcstat.prg and

chcstat.acc.

chcache.doc    This file.

- 1 -

Cold Hard Cache 4.0

Installation:

1.   Place the following files in the root directory of your  boot

disk:

chcstat.prg

chcstat.acc

chcstat.rsc

2.   Place cachev3.prg in the AUTO folder of your boot disk.  Make

sure it is placed after your hard disk driver (like  AHDI.PRG

on  Atari drives).  Also,  if you are  networked,  make  sure

cachev3.prg is placed BEFORE your network driver or there  is

a very good chance of scrambled files.

Configuration/Status Reporting:

General:

There are two versions of the  configuration/status  program.

Chcstat.prg and Chcstat.acc. They both operate the same except one

is a stand-alone program and the other is a desk accessory. I will

refer to these as the Chcstat from here on.

Chcstat first tests whether cachev3.prg is installed.  If  it

is not Chcstat will complain. Then a dialog is displayed.

The  top half of the dialog contains performance  and  status

info. The bottom half contains configuration settings.

Configuration:

You may choose the size of the cache and the drives you  wish

to be cached.  The size of the cache is measured in buffers.  Each

buffer requires 524 bytes (512 for the sector,  12 overhead).  The

cache should be at least 32 buffers but not too big. I usually use

256  buffers (about 128K).  You can try various sized  caches  and

watch the statistics to find a good size.

You may choose the maximum sized multi-sector transfer to  be

considered  cachable.  The  default  setting is  4  (512  bytes  *

4 sectors = 2K). You may specify any value from 1 to 9999 sectors.

Large  values  will  cause  a  loss  of  performance  since  large

transfers  (like  program  loads) will fill the  cache  and  flush

potentially useful sectors (like directory and FAT  sectors).  You

may  tune  the  cache for your particular  needs.  If  you  use  a

database  with large records you may wish to specify a larger  max

size to accomodate the size of your records.

You may also choose the drives you wish to be cached.  Select

the  drives  with the mouse.  Caching is enabled on  the  Selected

drive letters. There is no harm if you enable a drive that doesn't

exist.  If  you enable caching on a ramdisk you will make it  work

SLOWER than if it was not cached.

- 2 -

Cold Hard Cache 4.0

Status/Performance:

The status and performance info is useful for tuning the size

of  the cache for optimum performance for  your  hardware/software

environment.  The performance is based on cachable sectors.

An  average  access time is  given  (in  milliseconds)  which

represents the average time it takes to locate and read a cachable

sector  from disk.  The average is for all of the enabled  drives,

not just one drive.

The approximate read time is displayed which  represents  the

total amount of time it took to read all of the cachable sectors.

The approximate time savings is calculated from  the  average

access  time and the number of sector requests satisfied from  the

cache (Hits).

The read status shows the number for cachable sector requests

(tries),  the  number of those requests satisfied from  the  cache

(hits), and the percentage of hits.

The  number  of write optimizations  is  displayed.  A  write

optimization  occurs  when  a sector write is  requested  and  the

sector  is  in the cache AND the contents of the sector  does  not

change so no write takes place.

Exiting:

There are three ways of exiting the dialog.

Cancel    The Cancel button will exit without changing any of

the settings.

OK        The  OK button will enable the selected drives  and

then exit. Any changes to the size will be ignored.

If   the  Reset  Stats  button  is   selected   the

statistics will be reset.

Save      The  Save button will save the current settings  in

the cache.inf file (so they will take effect  after

the next boot), enable the selected drives and then

exit.  If  the Reset Stats button is  selected  the

statistics will be reset.

NOTE:     The cache program cachev3.prg must be positioned in  the

AUTO  folder after your hard disk driver  (ahdi.prg  for

Atari drives) and before any network drivers.

NOTE:     If  the alternate key is held down when  cachev3.prg  is

run (from the \auto folder during the boot process)  the

cache will NOT be installed.

NOTE:     Users  of  TURBODOS  may  see  lower  performance  since

TURBODOS  does some caching  internally.  TURBODOS  will

disable  caching on floppies since it  bypasses  RWABS()

and the ROM routines that handle floppies.  Cachev3 when

- 3 -

Cold Hard Cache 4.0

used with TURBODOS can still give good results if  there

is lots of disk activity. Try it and see what you get.

NOTE:     Cachev4   should  work  with  TOS  1.4   with   Extended

partitions (> 16 Meg) however only the first 16 Meg will

be cachable.  THIS HAS NOT BEEN TESTED!  TRY IT AT  YOUR

OWN  RISK!  MAKE  SURE YOU HAVE A BACKUP OF  YOUR  DRIVE

BEFORE YOU TRY!

NOTE:     Cachev4  prevents  writes to the boot sector (#  0)  for

enabled  drives.  This will prevent utilities that  make

partitions  bootable (or viruses) from working.

NOTE:     Some  Hard  Disk  utilities  (like  markbad)  will   not

function correctly when the caching is  enabled.  Simply

disable   the  drives  with   the  chcstat  program   or

accessory.

BUG ALERT:

Cold   Hard  Cache  3.2  did  NOT  recognize  Media   Changes

correctly.  This  may have affected Removable Hard Drives  as

well as Floppies.  This has been corrected in version 3.3.

Cold Hard Cache 3.3 would crash (Address error) if a read  or

write request specified a transfer to or from an odd  address

(very rare). This has been corrected in version 3.4.

Cold Hard Cache versions before 3.5 would not allow  floppies

to  format  correctly.  Writes to the Boot  Sector  were  not

permitted.  They are now allowed for floppies but NOT allowed

for drives C through P.

Cold  Hard  Cache  was  written using  the  Mark  Williams  C

Compiler. I highly recommend this product.

Cold Hard Cache is released as Shareware.

If  you use this program please make a donation ($15  or  so)

to:

Robert E. Owens III

34 Pinecrest Dr.

Covington, LA 70433

USA

This  program was designed to save time and has proved to  be

valuable. I usually save 20 to 40 minutes a day!

If you have any suggestions or comments about this program  I

would like to here from you. I can be reached by Stamp Net (at the

above address) or on Genie (at R.OWENS3).

- 4 -