FSCK - Atari ST File System ChecKer and repairer

Version 1.1

February 6, 1988

Copyright Barry Locklear, 1988

All rights reserved.

This program may be freely redistributed only for

non-profit purposes.  For any other uses, please

contact the author for permission to use.

Author: Barry Locklear

Genie: xmg21417

CompuServe: 76327,2102

UseNet: ihnp4!druhi!lbl

This program was developed because I occasionaly have problems with the file

system on my hard disk.  There were hard disk checking programs in the public

domain, but they didn't fix the errors that they found.  So I wrote this

program.  If you find any bugs (heaven forbid!), then please contact me at

the above address.  Also, if you have any ideas for improvement please contact

me.

What FSCK Does For You

----------------------

FSCK does a thorough check on the file system to make sure that each

file is in a consistent state.  It makes sure that the correct number of

clusters are allocated for each file, that the cluster chain is intact, and

that the . directories really do point to themselves.  The only thing it

doesn't check for is that .. really points to the parent directory.  Maybe

in the next release.

FSCK doesn't correct problems concerning a specific file because there are

many ways to deal with the problem.  For the next release, I may include some

file specific repairs for conditions such as premature end-of-file, too many

clusters, and free blocks in cluster list. This would probably mean that

information in the file would be lost, but at least you may be able to

salvage something with the program.  Let me know if you want this feature,

but I think that a sector editor is a better way to solve these particular

types of problems.

After checking the file and directory structure, fsck then checks the FATs to

make sure that there are no blocks that say that they are allocated, but

really aren't connected to a file.  If you have specified the '-f' option,

after each error is found, FSCK will ask if you wish it to fix the error.

At the end, if you have told it to fix at least one error, it will ask if

you wish to write the corrected FAT to disk.  The FAT consistency feature is

particularly helpful because most of the file system errors that I have

seen deal with blocks that are left dangling by GEMDOS.

Usage

-----

fsck [-f] <drive specifier>

-f means to repair the file system as much as possible.  FSCK asks for

permission before it takes any action.

<drive specifier> : if empty means check all drives.  Otherwise indicates

the drive to check.  The drive specification is of the

form a or a:.  Legal drives range from A to P.

NOTES -

The hexadecimal numbers printed out by this program may look somewhat

different from what you are used to.  That's because this program is

written in Jefferson Software Modula-2 and the hex-number format for

Modula-2 is <number>H where the H means HEX format.  I may change this in

future versions of the program if people request it.

What the error notifications mean

---------------------------------

File Specific Errors:  To try and fix most of these you'll need a disk sector

editor program like Disk Doctor from ANTIC.

"File <filename> has bad first cluster value of <number> " :

<number> is usually 0 in this case.  It usually means that a file was

opened for writing, but the program terminated without writing anything

to it.  I don't know if this is caused by an error in GEMDOS or not,

but the easiest way to fix it is to delete the file.

"File <filename> has more Clusters allocated than should be used!" :

This means that according to the length specifier for the file,

FSCK didn't reach end of file when it should have.  This could be

caused by having a block on 2 file chains. Solution: You might try

copying the file, but it probably won't help.

"File <filename> doesn't have enough clusters assigned to it!" :

This means FSCK reached end-of-file before it should have.

Again, this usually is caused by a file cluster being on two chains.

Again, there really isn't a good solution.

"File <filename> has INVALID FAT value of <number> in cluster <number>" :

"Found free cluster <number> in middle of <filename>!!!":

"Found bad sector <number> in middle of <filename>!!!":

These messages are bad news because they indicate a bogus FAT entry

which means part of your file is probably unrecoverable.  You MAY be

lucky if this FAT entry was supposed to indicate end-of-file, but

according to Murphy that never happens.  You DID back up your files

didn't you???

Non-file-specific errors

"Internal write error...." :

"Internal read error..." :

FSCK had an internal error.  This should NOT have any effect on your

hard disk.

"Cluster <number> should be FREE but indicates <some condition>>:

If you didn't get any file specific errors, or if you did and

they were internal errors or "first cluster was <number>" errors,

then you should normally type yes if you are fixing the disk.

This will free up those sectors so they can be used again.

Otherwise, you may wish to take some other action.

"Cluster <number> should have value <number> but indicates

"Cluster <number> should be END OF FILE but indicates

These errors should rarely if ever come up.  Type yes ONLY if the

conditions for the previous message are true.

"Cluster <number> should have illegal value of <number> but indicates.. "

You should never see this error! Answer no to it.

Other errors

"Out of memory...." :

ran out of memory. This should NOT affect your disk.

"Read error while checking ... " :

Couldn't read the disk.  If you are checking a floppy it may not be

in the drive.  If it is a hard disk, then it may be damaged or have a

bad sector.

"Write of <first | second> FAT failed" : hardware error ocurred while trying

to write the FATs to disk.  Disk may be corrupted in this case.  Run FSCK

without the fix option to make sure the disk is OK.