SIO-Driver vers. 1.0

by James C. McCormick

SIO-Driver is a 'stay resident' program that enables the Atari ST to

use an 8-bit disk drive. The 8-bit drive can be accessed from the desktop

or from an application just as any other drive would be. The program uses

the ST's printer port to connect to the drive and emulates the 8-bit Serial

Input/Output (SIO).

What you need:

_ Atari 8-bit disk drive (810,1050,Rana, etc.)

_ XformerII (or similar) cable.

Contents of SDRIVER.ARC file include:

_ SDRIVER.DOC - This file!

_ SDRIVER.PRG - to be placed in auto folder and must be run

to enable access to 8-bit drives.

_ SCONFIG.TOS - Format and configure utility for the 8-bit

disk drive.

1.	How to use SIO-Driver

This program is very easy to use, and once the program is loaded

(usually from the auto folder), you won't need to run any other programs

to access the 8-bit drive.  (except to format!) You can store ST files to

your 8-bit drives just as you would your ST drive.

I have written this program in hopes that some people, such as myself,

have one or two 8-bit drives around that aren't being used, as well as a

mountain of 5 1/4" blank floppies. This program will allow you to put

your old drive to good use.

1.1	Hooking up the system

You must plug the XformerII (or similar) cable between the Atari ST's

printer port and the 8-bit disk drive. You can use the cable available for

Darek Mihocka's XformerII or purchase a similar cable from me.

(ordering instructions later) You may connect additional 8-bit drives by

daisy chaining them through the standard SIO cables.

1.2	SDRIVER.PRG

This is the actual interfacing software. When placed in the auto

folder (it can be run from anywhere at anytime though) it will run on

bootup and look for a file named 'SDRIVER.DAT'. This file is created

using the configure utility and tells the SDRIVER program which desktop

drive letter, C through P, belongs to which 8-bit drive unit, 1 through 4.

You can have up to 4 drives, each with it's own letter (or icon).

When SDRIVER.PRG runs, it will tell you which drives are online and which

are offline. This is for your own information only, you can turn on the 8-bit

drive anytime after the program is run and access it.

You must note though, that of your auto-boot programs that might

use any of the operating system disk vectors, SDRIVER.PRG must be run LAST.

This is so that the configure utility will be able to sense that the SDRIVER

program is installed. Ramdisks are the main offenders here. To insure that

SDRIVER is loaded last, use one of the public domain auto folder file sort

programs.

This version of SIO-Driver only supports single-sided single-density

disks, but future versions will take better advantage of higher capacity

drives.

1.3	SCONFIG.TOS

This program is used to format disks in your 8-bit disk drive and

create the SDRIVER.DAT file. Note that this is the only way to format a

disk since the desktop format command will not work.

The program is menu driven and fairly self explanatory. The main menu

provides two options, formatting and configuring.

1.3.1	Formatting

The format menu asks which drive unit to format, 1-4. This has no

connection with what drive letter is associated with a drive, it is just the

unit number. (the number selected with the switches on the drive). After

formatting the disk, you will be returned to the main menu.

1.3.2	Configuring

When entering the configure menu, the program will attempt to read

an existing SDRIVER.DAT file in the current path, if one is not found,

default configurations will be chosen for each drive. The menu options

include modifying the configuration of drives 1-4. The current configuration

will be displayed. Either a drive letter will be shown, meaning that this

drive unit will respond when you access this letter of drive icon, or an

asterik will be shown, indicating that the drive is not in the configuration.

After choosing which unit numbers will correspond to which drive, you

must select the 'save configuration' option in order to create the new

SDRIVER.DAT file. It will be written in the current path.

1.3.3	Adding the new drive icon

After you have created the SDRIVER.DAT file using SCONFIG.TOS

program, and you have placed SDRIVER.PRG and the new SDRIVER.DAT file in

the auto folder, you must create a drive icon for your 'new' 8-bit drive.

This process is the same for adding a ram disk.  Just click once on an

existing drive to highlight it, then select 'Install Drive' from the

options menu.  Type in the letter of your new drive in the space

provided and click on 'install'.  Now that your icon is installed, you

might wish to make it permanent by 'saving desktop' from the options

menu.

1.3.4   Setup Summary

_ Run the configure/format utility (sconfig.tos) to configure

your 8-bit drives. Be sure to 'save configuration'.

_ copy the SDRIVER.DAT file (just created with sconfig.tos) as well

as SDRIVER.PRG to your auto folder and reboot.

_ install a drive icon as the drive letter you specified in the

configure program and then 'save desktop' from the GEM options

menu.

_ Run the configure program again to format a disk(s).

_ Use your new drive!

2	What SIO-Driver will (and won't) do

With SIO-Driver installed, you can read/write/copy/delete files

on the 8-bit disk drive, just like any other drive. You can do anything

that a regular drive can do except boot from it. The file system used on

the drive is the ST's file system. IMPORTANT: You can NOT read and write

8-bit Atari files. The disks used by this program become ST disks. Of

course, no harm will be done by trying, but I don't want anyone to have

any misconceptions about the function of this program. The file structure

that the 8-bit drive will be using is ST only.

Now the bad news. As you may have guessed, these drives are SLOW.

Atari SIO runs at 19.2k baud. This didn't seem so bad on the old 8-bit

machines but now that you have them up against the regular ST drives, the

slow pace is more noticable. Also, not only can you not use the desktop

format option, but whole disk copies won't work either. You can copy all the

files from one drive to the other, but you can't drag the disk icon onto

another one.

You will also notice that while access to the 8-bit drive is active,

your mouse and keyboard is inactive. This is required because the timing

needed to interface to the 8-bit drives is software controlled and must be

very precise, therefore while accessing, all interrupts are disabled. This

means no mouse and keyboard input. I believe that this is a small price to

pay since you the mouse is useless in the 'busy bee' state anyway.

With my 8-bit drives I have successfully copied/deleted files.

I've run programs from these drives, loaded pictures and text files, and

even ARC'ed files to and from them. In all cases they have performed just

as any other ST drive (except the speed of course). One thing I haven't tried

that is certain to fail, is downloading to these drives. With the

computer's interrupts disabled on the drive accesses, I feel certain that

the modem would no longer receive data; try it if you must but don't say

I didn't warn you!

3.	Future plans?

Of course future plans depend on the interest in this program. I

would like to have a desk accessory to configure and format the disks. Also,

a 'whole disk' copy would make fast backups of both your ST floppies AND

your 8-bit disks. (one pass copier!) And certainly the ability to use the

Atari XF551 disk drive is on the horizon, as well as any enhanced or

double density drives.

4.	Shareware

This program is indeed shareware. Upload it everywhere. But if

you find this program usefull, please send a donation. I have listed

two options for becoming a registered user.

1 _ Send $10 (or whatever you feel adequate) to the address below

and you will receive a disk containing the source code to

this program and a new version of the configure utility.

As a registerd user, you will be notified of any new versions

and utilities which will be available for the price of the disk

and postage.

2 _ Send $25 dollars to become a registered user, receiving

everything mentioned above, and I will send you a SDRIVER cable

as well.

Be sure and let me know what kind of disk you want it on. 3.5"

single or double sided, (or even a 5.25")

5.	Feedback

Please contact me at the address below with any questions,

suggestions, or (shudder) bugs. I can't fix it unless I know it's broken!

6.	Many Thanks

Thanks to John Stanley and Doug W. on GEnie for coming to the rescue

and solving a very tricky problem with the mouse.

Many thanks to Jim Gross of the Huntsville Atari Users Group for

lending me equipment and pointing out the many 'features' of earlier versions

for me (some might call them bugs). Thanks to Larry Mears who before running

my code for the first time on his machine, looked from his hard drive, to his

ST, and then to me and said 'Are you sure?'

7.	Legal

This document and software are Copyright 1989 by James C. McCormick.

SDRIVER may not be sold. It may be freely distributed however as long as this

document and the accompanying program is not modified.

I have made every effort to assure that this software runs bug-free,

however I cannot be responsible for any loss of data caused by this

software.

James C. McCormick

P.O. Box 6333

Huntsville, Alabama

35824-0333

GEnie: J.MCCORMICK4

H.A.U.G. BBS

(205)-461-7893

as James McCormick