-------------------------------------------------------------------------------

._____.  __     __ .______.  .______. ._.

/ .___. \ \ \   / / | .___. \ | .____! | |

| |   !_!  \ \_/ /  | !___! | | !__.   | |

| |   ._.   \   /   | .__. ./ | .__!   | |

| !___! |    | |    | |  \ \  | !____. | !____.

\._____./    !_!    !_!   \_\ !______! !______!

.__   ._. ._.      ._. ._. .__    __. ._.            .____.

\ \  | | !_!      | | !_! |  \  /  | !_! __    __  / .__. \

\ \ | | ._.  .___| | ._. | . \/ . | ._. \ \  / /  | !__! |

\ \| | | | / .__. | | | | |\  /| | | |  \ \/ /   | .__. |

\ ! | | ! | !__! | | ! | | \/ | | | !  / /\ \   | !__! |

\__| !_! \______! !_! !_!    !_! !_! /_/  \_\  \.____./

-------------------------------------------------------------------------------

CyReL VIDIMIX-8 Features for SUNRISE M16-1280

Version: 1.2b   Date:   02/93

-------------------------------------------------------------------------------

CyReL VIDIMIX-8

The Integrated Desk-Top Video Module

for the CyReL SUNRISE M16-1280

for the ATARI TT030 and MEGA STE

*  3 composite video inputs.

Software allows to instantly switch between several video sources.

*  Software controllable 75 Ohm termination or high impedance for

loop-through on each input. Enables the operation of a control

monitor on each input or signal looping to the next video

processing station.

*  Inputs CCIR compatible. Almost any tape recorder, video disk,

Camcorder, Camera or video effects equipment can be used as a

video source.

*  1 composite video output. Accepted by most VCR and TV equipment.

*  1 S-VHS output ( MINIDIN-4 ) for better output quality and

signal fidelity.

*  Simultaneous S-VHS ( Y/C ) and NTSC / PAL output. Record the S-VHS

output while controlling the composite output on a monitor.

*  Programmable sync output ( high / low, composite, H & V ).

Flexible enough to match any equipment requirements.

*  12 different NTSC & PAL output standards. Many different PAL and NTSC

standards and options supported.

*  Interlaced / non-interlaced output possible. Captured interlaced video

frames/clips can be played back in non-interlace mode for use on a

computer monitor. Also de-interlaces incoming TV signals.

*  Pixel output configurable to 1:1 or 4:3 ratio. Ideal for switching

between computer graphics (1:1) and video (4:3) applications.

*  Programmable 24-bit gamma correction. Allows many color correction

functions as well as a host of special effects like solarisation.

Software controlled color look-up table bypass feature.

*  8 different YUV / RGB input formats. Process and save captured

video frames and clips in many different formats including the

most widely used YUV modes.

*  Video keying. Allows a wide range of 'graphics-over-video' effects,

including video titling, GEN-locking and image keying.

*  Software controlled chrominance phase. Adjust the color quality of

the output signal through software controlled slider functions.

*  Chrominace bandwidth enhancer. Boosts the output quality or reduces

the signal strength to match any recording equipment specifications.

*  Software controlled color sub carrier adjust. Prevents loss of

color due to weak or mis-aligned color sub carrier frequencies.

*  Four operating modes ( including : Gen-lock, master / slave ).

The VidiMix8 can either receive or generate all necessary control

signals to drive a variety of video equipment.

*  Selectable fast tracking mode for VCR operation ( in Gen-lock mode ).

Enables very stable gen-lock operation even when operating with

various VCR playback modes.

*  Color or B&W operation. Introduce color to B&W pictures or convert

color signals to black and white.

*  Background color generator. Flash fills the entire screen. Fades to

or from any desired shade or black.

*  Real time true color digitizing. Digitizer can perform on-the-fly

picture scaling. Scaling function and amount is software controllable.

*  Possible output resolutions :

- NTSC  640 x 525  60Hz  interlaced

- NTSC  720 x 525  60Hz  interlaced

- PAL   720 x 625  50Hz  interlaced

- PAL   768 x 625  50Hz  interlaced

- NTSC  640 x 524  60Hz  non-interlaced

- NTSC  720 x 524  60Hz  non-interlaced

- PAL   720 x 624  50Hz  non-interlaced

- PAL   768 x 624  50Hz  non-interlaced

-------------------------------------------------------------------------------

Mail : Cybercube Research Ltd, 126 Grenadier Cres,

Thornhill, Ontario, Canada L4J 7V7

Voice : (416) 882-0294

Fax : (416) 886-3261

BBS : (416) 882-5895

GEnie : CYBERCUBE

Specifications subject to change without notice. All brand names

and trademarks are the property of their respective owners.

Copyright (C)1993 by CYBERCUBE Research Limited

-------------------------------------------------------------------------------

<EOF>