Smodem protocol overview

------------------------

Smodem is a new generation file transfer protocol. Unlike older protocols,

like Zmodem, it has a separate low level multiplexed transfer layer (MSLP)

and a high level file transfer layer. This new design simplifies protocol

design and will give a maximum efficiency of as high as 99,5%.

Multiplexing has made it possible to send one or more files in one

continuous stream without any breaks between files. This will boost

multiple file transfers dramatically.

Multiplexing has also made it possible to add a full screen ANSI-BBS

terminal emulator with a local full featured line editor and colored

back scroll buffer of unlimited size to Smodem. Terminal emulation

allows you to chat during file transfer.

The protocol has a symmetrical nature. You can connect Smodem with itself

by looping transmitted characters back to the receiver. There is no reason

to limit transfers in one direction. Smodem will transfer files in both

directions at the same time without noticeable performance loss.

Smodem detects DESQview and DPMI (Windows, OS/2) and will free all unused

processor capacity to the host system.

Smodem is *totally* written in the C language and compiled with Microsoft

Visual C++ v1.0 compiler using full optimizations for the i286 CPU. It

needs over 200KB free memory to function properly and more if you want

to shell from Smodem. There is no magical multitasking to continue file

transfer if you drop to DOS!

Smodem is published as a freeware product. If you feel guilty to use

software you have not paid for, please send me as many empty 1.44MB 3,5"

diskettes as you like as a gift to the following address:

AriSoft Oy (Inc)

Lenkkitie 5

79100 Leppavirta

FINLAND

Multiplexed Serial Link Protocol (MSLP)

---------------------------------------

MSLP has the capability to manage 32 independent channels simultaneously.

Every single channel uses a secured two phased ack/nak type subprotocol.

Maximum efficiency is achieved by using multiple parallel channels

simultaneously to keep the transmitter busy.

Maximum packet size floats between 16 to 1024 bytes. Transfer errors will

reduce packet size depending on error frequency. One important detail is

that MSLP needs to resend only failed packets. The transmitter window size

(size of transmitted packets which have not been acknowledged) can be

limited to get faster response.

The packet frame contains one byte for channel number and two bytes

for a 16-bit CCITT-CRC. (Same algorithm, which is used inside error

corrected modems.) Acknowledgements and enquiries will be included

in the packet frame when needed.

Packet separators contain two bytes. If those two bytes are found in

transferred data, one byte is inserted for capsulation. The possibility

to find those separator bytes in random data is so small, that it will

not affect protocol efficiency like old style one byte separators do.

Maximum efficiency in one-direction error free transfers will be:

max packetsize           1024

--------------------------- = -------- = 0.9951 = 99.5%

max packetsize + frame size   1024 + 5

If packet size is limited to 256 bytes, efficiency will be 98.1% and

still better than original Zmodem!

If the data link can not transfer some special characters, then a

traditional character encapsulation mode could be activated. This mode

encapsulates selected character codes, which inflicts performance about

0.4% + additional 0.4% for every selected code.

If the data link is limited to 7 data bits, then a special bit collection

mode could be activated. The highest bits for every seven bytes are

collected to one byte. This collection inflicts performance about 12.4%

and doubles character encapsulation possibility.

Maximum transfer line correction mode of MSLP includes both the character

capsulation mode for every control character and the bit collection mode

in both directions. The maximum mode works at almost every possible data

link, which can transfer all visible ASCII characters from space to '}'.

This mode has very low efficiency - only 68.5% - and it should be used

only for test purposes if default settings does not work.

Command line options

--------------------

/p:port

Selects com port number (1..4). This setting is only mandatory setting.

Without this or corresponding settings you will only get internal help

information. See also /a: and /i: options.

/s:speed

Changes dte baud rate (50..115200) using 8,N,1 configuration. If this

parameter is omitted, then baud rate and line parameters are reserved

in their current values.

/a:addr

Uses special uart chip address (100..3ff). You need this parameter if

you are not using one of the standard communication ports COM1-COM4.

See also /p: and /i: options.

/i:irq

Uses special uart chip irq (2..7). You need this parameter if your

communication port irq is changed from default setting or you are

using special uart chip address. See also /p: and /a: options.

/h

Disables rts/cts flow control. Normally rts/cts handshakes are used

to control modem buffering. Use this option if your modem does not

support this kind of action. See also /H and /n options.

/H

Enables xon/xoff flow control. Use this option only if your modem

does not support rts/cts handshakes because it reduces maximum transfer

speed. See also /h option.

/C

Enables maximum corrections for non transparent transfer line. This

option is only for test purposes. Use it only if you have problems to

initiate file transfer or transfer hangs. If this option removes your

problem, then it is possible to configure Smodem to work properly.

See also /I, /I:, /O, /O:, /h and /H options.

/I

Select 7-bit data input mode. Use this option if you can receive

only 7-bit data. See also /O and /O: options.

/O

Select 7-bit data output mode. Use this option if you can transmit

only 7-bit data. Use this option for telnet connections. See also

/O:, /I and /I: options.

/I:hex

Encapsulates specified input characters. For telnet connections

you should specify /I:0. See also /I, /O and /O: options.

/O:hex

Encapsulates specified output characters. For telnet connections

you should specify /I:11 /I:13 /I:1d. For rlogin connections you should

specify at least /I:7e depending on your terminal settings. See also

/O, /I and /I: options.

/c

Disables carrier detection. You need this option if you are dialing out

from terminal mode (/t) or if your modem does not inform carrier status

properly. Some modems drops the carrier signal occasionally. Use this

option if you get erroneous "Carrier lost" messages.

/n

Disables slow RTS handshakes from disk operations. This option may give

slightly better performance or it can ruin the whole thing depending

on your hardware performance. See also /h option.

/v:mode

Changes default packet and transmitting window size. Possible values

are T(iny), S(mall), C(ompact), M(edium), L(arge) and H(uge) (default).

Larger values will give slightly better throughput. Smaller values will

give much faster terminal response.

/E:sec

Exits after delay (0..600 sec) when all file transfers done. This option

is useful in BBS environment to stop chatting when file transfer is done.

There is a fixed delay of 120 seconds at the beginning of transfer to

allow user to select files to upload. See also /e: option.

/e:sec

Exits after inactivity (0..600 sec) when all file transfers done. This

option is useful in BBS environment to stop chatting when file transfer

is done and sysop is not chatting with user any more. There is a fixed

delay of 120 seconds at the beginning of transfer to allow user to select

files to upload. See also /e: option.

/t

Enables terminal mode. In this mode Smodem does not send initialisation

strings until it receives one. You may need ansi.sys or some replacement

for full ANSI support.

/u[:[@]dir] {files}

Uploads subsequent files and selects default upload path(s). You can

enter as many upload directories as you need. You can also specify

directory list file name after 'at' (@) sign. All upload directories

will be scanned when uploading file without pathname. After /u

parameter you can enter files or list of files with 'at' (@) sign.

Example: /u:c:@updirs.lst /u:c:\temp bbs*.zip @upload.lst.

See also /D:, /f, /k and /y options.

/d[:dir] {files}

Downloads subsequent files and selects default download path.

You can enter only one download directory. After /d parameter you can

enter files or list of files with at (@) sign.

Example: /d:c:\download /d:c:\temp bbs*.zip @download.lst.

See also /U:, /f, /o, /j, /w, /x and /X options.

/z:file

Enables file transfer log file. Created log file is DSZLOG compatible.

File transfer information is appended to the end of this file.

See also /X option.

/f

Frees up/download path restrictions. If you specify this option, then

user from other end of connection can download from and upload into any

directory in your computer. Use with care! You have to use this option

if you want to select separate download directory from download dialog.

/r

Enables auto upload. If you specify this option, then user from other

end of connection can download files from your computer. See also /f

option.

/o

Allows to overwrite files. Without this option you can not download

files which already exists in your disk. You can still resume aborted

downloads. See also /j and /w options.

/j

Disables auto download. This option prevents downloads totally.

It should be used in BBS systems which does not support bidirectional

file transfers when downloading from BBS system. See also /r option.

/w

Disables resumed downloads. Normally file transfer will continue

if it has broken in some reason. This option can be used in BBS systems

to prevent users to resume uploads in some reason. See also /x option.

/k

Disables wild cards. This option can be used in BBS systems to prevent

users to download files from upload directory without specifying full

filename. See also /j option.

/x

Removes aborted downloads. This option can be used in BBS system which

does not support resumed uploading. See also /w and /X options.

/X

Enables aborted download entries to transfer log file. This option

is added to make possible to resume uploads to some BBS system.

If upload is aborted, then BBS system will not be informed to remove

aborted file. See also /z: option.

/y

Removes successfully uploaded files. This option can be used in some

special situations to keep upload directory automatically clean.

/-

Disables MSLP initialization messages. This option prevents user

information to scroll out from screen in some situations. See also

/l and /L options.

/+

Displays transfer statistics screen. You can add this option for your

convenience. Without this option you can still open statistics screen

with F2 key.

/b

Uses black and white colors. This is normally detected automatically.

/q

Quiets alarm bell. This option prevents you to hear warning message

beeps and yelling noise. You can change the quiet status from menu.

See also /m option.

/m

Disables error messages. In BBS system there is no need to show error

messages locally. This option hides them. See also /q option.

/g

Disables default terminal log file. Default name is smodem.$$$.

Log is used for coloured back scroll buffer. This log is not in ASCII

format. You can not read it with editor. See also /g: and /G options.

/G

Removes terminal log file after use. Normally log file is saved for

later use. With this option log file will be removed after use.

See also /g and /g: options.

/g:file

Selects terminal log file name. This option changes default log file

name SMODEM.$$$. You can also specify different drive or directory for

the log file. See also /G and /g options.

/@:prompt

Displays user defined prompt. With this option you can enter 1-39

characters long prompt to your local line editor.

/l:lines

Selects number of rows (20-50) from bottom of the screen. If your BBS

system displays user information at the top of the screen, then you

can reserve part of the screen from this information. See also /L: option.

/L:lines

Selects number of rows (20-50) from top of the screen. If your BBS

system displays user information at the bottom of the screen, then you

can reserve part of the screen from this information. See also /l: option.

Menu options

------------

File/About          Shows information about program version.

File/Download       Displays download selection dialog.

File/Upload         Displays upload selection dialog.

File/Abort/Download Aborts downloading.

File/Abort/Upload   Aborts uploading.

File/Yell           Send calling noise to other end of connect.

File/Erase Log      Erases backscroll buffer file contents.

File/DOS Shell      Drops to dos temporarily.

File/Exit           Aborts all file transfers and terminates protocol.

Set/Packet size     Selects one of the possible packet sizes.

Set/Exit when done  Terminates protocol after file transfer if checked.

Set/Quiet           Quiets alarm sounds if checked.

View/Statistics     Displays transfer statistics.

View/Split          Displays split screen.

View/Log File       Displays back scroll buffer.

Use esc to hide popup windows and dialogs. Use cursor keys to edit

your entry or to move position in back scroll window.