1

CwDrill 1.4                                            Page

-----------------------------------------------------------------

CwDrill 1.4

(c) Copyright AYMON & CO

1376 Goumoens-la-Ville

Switzerland

1. Introduction  . . . . . . . . . . . . . . . . . . . . . .    2

2. Installation  . . . . . . . . . . . . . . . . . . . . . .    2

3. Paddle Connection . . . . . . . . . . . . . . . . . . . .    2

4. Starting CwDrill  . . . . . . . . . . . . . . . . . . . .    3

5. Keying with the paddle  . . . . . . . . . . . . . . . . .    4

5.1. Menus  . . . . . . . . . . . . . . . . . . . . . .    4

5.2. Keying of numerical parameters . . . . . . . . . .    4

5.3. Choice of a file . . . . . . . . . . . . . . . . .    4

6. Main Menu . . . . . . . . . . . . . . . . . . . . . . . .    5

6.1. Skill Drill  . . . . . . . . . . . . . . . . . . .    5

6.1.1. Paddle  . . . . . . . . . . . . . . . . . .    6

6.1.2. "World" parameters  . . . . . . . . . . . .    7

6.2. Keying practice  . . . . . . . . . . . . . . . . .    8

6.3. Copying practice . . . . . . . . . . . . . . . . .    9

6.4. Default values . . . . . . . . . . . . . . . . . .    9

6.5. Colour, Black-White  . . . . . . . . . . . . . . .    9

6.6. Fran ais, English, Deutsch . . . . . . . . . . . .    9

7. Miscellaneous . . . . . . . . . . . . . . . . . . . . . .    9

7.1. Several configuration files  . . . . . . . . . . .    9

2

CwDrill 1.4                                            Page

-----------------------------------------------------------------

1. Introduction

CwDrill  is  an  interactive  program  which  allows to connect a

paddle directly to a  serial port  of a  PC, the  user thus being

able to exercise and improve both keying and copying.

Its  Menu-oriented  style  makes CwDrill especially userfriendly.

CwDrill does not consider any  keyboard  input  (except  for Ctrl

Break), it is controlled by the paddle only.

Each copy is personalized by its owners callsign.

2. Installation

CwDrill is supplied on a 3 1/2'' (720KB) disk containing at least

the following files:

CWDRILL.EXE         the program as such

PADDLE.EXE          a test  program,  which  controls  the paddle

connection (see below)

CWDRILL.QSO         a file containing a list of callsigns

xxxx.CFG            a configuration  file which  contains all the

program  parameters.   It   is  automatically

rewritten  by  CwDrill  after  each  use  and

therefore contains the last used values.

CWDRILL.DOC         this file

CwDrill does not need  any  special  installation.  On  hard disk

systems,  the  supplied  files  can  be  copied  into a directory

specially created for this purpose. A  backup copy  should always

be made. CwDrill is not copy protected.

3. Paddle Connection

The paddle  is to  be connected directly to a serial port (port 1

or port 2) of the PC. The cable for this  purpose has  to be made

by  the  user.  For  this,  two  10K  resistances are needed. The

following connections have to be made:

1)   RTS and DATA CARRIER DETECT by a 10K resistance.

2)   RTS and RING INDICATOR by a 10K resistance.

3)   the "dot"-contact of the paddle to RING INDICATOR.

4)   the "dash"-contact of the paddle to DATA CARRIER DETECT.

5)   the paddle common to TD.

The keyed morse code is output to DTR (RS-232 levels, abt. +- 10V)

The pins of a 9 (25) pole plug are as follows:

- RTS pin 7 (4)

- DATA CARRIER DETECT pin 1 (8)

3

CwDrill 1.4                                            Page

-----------------------------------------------------------------

- RING INDICATOR pin 9 (22)

- TD pin 3 (3)

- DTR pin 4 (20)

- GROUND pin 5 (7).

The cable is then connected to port 1 of the PC to be tested with

the PADDLE program which indicates the state of the paddle:

Dash      : contact "dash" activated

Dot       : contact "dot" activated

Dot Dash  : both contacts activated

If problems  occur, the tensions of the "dot" and "dash" contacts

of the paddle against the computer  ground must  be checked. Open

contact: more than 3 Volt (generally 3.5 V); closed contact: less

than -3 V (generally -9 V).  If necessary,  the resistance values

have to be reduced.

Warning:  paddle common is about -10 V to computer ground.

CwDrill can  only be  started if the paddle is properly wired and

plugged in.

4. Starting CwDrill

Before starting CwDrill:

-    set  the  directory  containing   CwDrill  as   the  current

directory

-    plug the paddle into port 1 or port 2.

Then start CwDrill with the command:

CWDRILL config port

config:   name of  the configuration  file to  be used (extention

.CFG). This file has to be in the current directory.

port:     number of port into which paddle is plugged.  It can be

1 or 2. This parameter can be left out if it is port 1.

Examples:

CWDRILL HB9DBC

cwdrill hb9dbc 2

During  the  starting  phase  CwDrill  checks  if  the  paddle is

properly plugged in. It is  important  not  to  close  the paddle

contacts  during  this  phase,  otherwise  the message "Keyer not

connected (port x) will appear.

The message "Illegal configuration file" appears if the file name

is not  correct, if  the file has been modified or rewritten by a

program other than CwDrill (using a text editor  e.g.) or  if the

file date is a later one than the system date. In such cases, the

4

CwDrill 1.4                                            Page

-----------------------------------------------------------------

original configuration file supplied with the  program has  to be

used or the system date must be corrected.

When CwDrill is started correctly it shows its identification. To

go to the Main Menu, key anything.

5. Keying with the paddle

5.1. Menus

The Menus are presented in a  list of  elements. Each  element is

identified by a number. One element is highlighted.

By keying "E", the highlight moves downwards.

By keying "T", the highlight moves upwards.

To select the highlighted element, "K" has to be keyed.

Disregarding the  highlight, each element can also be selected by

keying its number.

To leave a menu without selecting anything, "I" has to be keyed.

5.2. Keying of numerical parameters

If a numerical parameter  needs to  be changed,  CwDrill opens an

editing window  and shows  the highest and lowest possible values

as well as the current parameter value.

To edit new values, it is quite sufficient to key  these and then

"K".  The  new  value  is  only  accepted if contained within the

allowed range.

The letters "O" and "T" can replace "0".

Mistakes can be corrected  using  the  "Error"  sign  (8  dots or

more).

To leave the editing window without modifying the value, "I" must

be keyed.

5.3. Choice of a file

For file choice CwDrill opens a window in which all or a  part of

all possible files appear.

One file  is highlighted. To move the highlight forwards, "E" has

to be keyed, to move it backwards, "T" must be keyed.

To chose a file, the highlight must be placed on its name and the

"K" must be keyed.

To leave the window without chosing, "I" has to be keyed.

5

CwDrill 1.4                                            Page

-----------------------------------------------------------------

Should the  window be  empty in  spite of the fact that there are

files in the directory, "T" has to be keyed several times to move

the highlight backwards.

6. Main Menu

The Main Menu contains the following choices:

1    Skill Drill

2    Keying practice

3    Reading practice

4    Set default values

5    Colour/Black-White

6    Fran ais, Deutsch

7    Back to DOS

6.1. Skill Drill

This  is  the  main  CwDrill  exercise.  It  is not only the most

difficult one,  but  also  the  one  that  shows  the interactive

possibilities of CwDrill most clearly.

This exercise contains the following menu:

1    Begin

2    Paddle

3    "world" parameters

4    Duration (Min)

5    QSO File

6    Main menu

The exercise starts by chosing "1". It runs as follows:

A callsign  randomly chosen from a given file (QSO file parameter

of the menu) is "transmitted" by the loudspeaker at random speed,

pitch  and  style.  The  user  has  to  immediately  repeat  this

callsign. If he waits too long or makes  a mistake,  the callsign

is repeated  up to  3 times.  If he repeats correctly, his answer

time is taken and stored in  a statistic.  The exercise continues

with a  new callsign.  The duration of the exercise is determined

by the user.

The main window shows the exercise log. After each callsign a new

entry is  recorded in  the exercise log. It contains the callsign

and the keying speed as well  as  the  given  answers.  A missing

answer is  symbolised by  "~~~". If  the answer has been correct,

time needed and number  of  attempts  are  noted.  If  no correct

answer is  given, the  exercise is temporarily interrupted so the

callsign that was not copied can be read on the screen.

A mistake can be corrected (as is done in normal transmission) by

keying a  numer of  "E" (at least 4) and then the callsign again.

As long as the delay between  two  signs  keyed    (including the

series  of  "E"  in  case  of  mistakes)  is  not longer than the

"patience"  parameter  (see  6.1.2),  the  correspondent  will be

waiting.

6

CwDrill 1.4                                            Page

-----------------------------------------------------------------

Instead of  a callsign,  QRS or  QRQ can  be keyed  to change the

speed of the correspondent.

Also, QSD can be  keyed  to  improve  the  correspondent's keying

style.

To temporarily interrupt the exercise QRX or "EB" can be keyed. A

menu appears:

1    Continue

2    Paddle

3    Abort

"2" changes the paddle parameters (see 6.1.1).

"3" or "I" return to exercise menu.

The "statistics" window indicates how much time is left, how many

callsigns  have  been  transmitted,  how many callsigns have been

copied correctly, the average copying time as well  as the number

of callsigns copied per hour.

After  the  set  duration  ("Duration" parameter, default value 3

minutes) or  by keying  QRT the  exercise is  interrupted and the

exercise menu  displayed. If  the log is not empty, there are two

more choices:

6    Explore log file

7    Save log file

Keying "6",  a log  window appears.  To move  forwards within the

log, "E" is keyed, backwards with "T" and return to the beginning

with "B".

The "7" saves the log into a file the name  of which  should be a

number  between  1  and  9999  with  the  extention  ".LOG".  The

statistics appear in the log files as well.

Note:          The QSO files (extension  ".QSO")  are  text files

with one callsign per line and contain up to 3'000

lines. The callsigns can be replaced by figures or

words.   The   file   taken  as  default  file  is

CWDRILL.QSO. It contains callsigns of OM contacted

by the author during contests.

6.1.1. Paddle

The  choice  of  this  option  allows  the modification of paddle

parameters. The following menu appears:

1    Speed

2    Pitch

3    Dash-Dot ratio (x1000)

4    Interval-Dot ratio (x1000)

7

CwDrill 1.4                                            Page

-----------------------------------------------------------------

5    Connection

6    Memory

The default speed is 120  signs/minute  (based  on  the reference

word PARIS).

The pitch default value is 800 Hz.

The dash-dot ratio (x1000) default value is 3000.

The interval-dot ratio (x1000) default value is 1000.

The choice  of option  5 (connection) changes the polarity of the

paddle contacts. The default value is "Normal".

The  choice  of  option  6  (memory)  enables  (or  disables) the

memorisation of  an element  (dash or dot) during the sending out

of a dot or dash. The default value is "Active".

Warning:       A modification of paddle  parameters may  create a

situation where nothing can be keyed correctly any

longer. In a case like this,  nothing else remains

to  be  done  except  pressing  down the keys Ctrl

Break and then to key anything  in order  to leave

the program without the parameters being saved.

6.1.2. "World" parameters

The   choice   of   this   option  allows  the  changing  of  the

correspondent's characteristics. The following menu appears:

1    Maximum speed

2    Minimum speed

3    Highest pitch

4    Lowest pitch

5    Delay between calls

6    Patience

7    Category 1

8    Category 2

9    Category 1 maximum speed

As explained above, the callsigns of the correspondent are chosen

randomly from the QSO file.

The correspondent's speed is chosen randomly between "max. speed"

(default 250) and "min. speed" (default 50).

If the  chosen  speed  exceeds  the  "Category  1  maximum speed"

(default 100),  the correspondent  is automatically considered as

belonging to category 2  (rather fast),  otherwise as  category 1

(rather slow).

The categories  1 and 2 allow to classify the keying style of the

correspondent randomly but according to certain percentages.

8

CwDrill 1.4                                            Page

-----------------------------------------------------------------

There are  four possible  styles: electronic,  bug, straight good

and straight  poor. The  percentages of  each style can vary from

one  category  to  the  other.  The  default  percentages  are as

follows:

Category 1:    Electronic 30%

Bug 10%

Straight (good) 30%

Straight (poor) 30%

Category 2:    Electronic 60%

Bug 30%

Straight (good) 10%

Straight (poor) 0%

This  means  that  in  category  1  there is a 30% probability of

encountering a correspondent with  an electronic  keying style, a

10% probability of a bug style etc... These percentages have been

chosen in such a way that in category  1 (rather  slow), there is

more  of  straight  keying  to  be encountered than in category 2

(rather fast).

If  one  would  like  e.g.  that   all  correspondents   use  the

"electronic style", the following parameters have to be chosen:

"Maximum  speed  category  1  =  same  value  as "Maximum speed".

Therefore all  correspondents will  be classified  in category 1.

One then fixes the "Electronic" percentage of category 1 to 100%.

If one  desires that  all correspondents  should have one and the

same speed one fixes "Max. speed" = "Min. speed".

The parameter "Delay between  calls" (default  3000 milliseconds)

fixes the time interval between two consecutive calls of the same

correspondent in case there should be no anwer, i.e. it fixes the

maximum  amount  of  time  available  until  the beginning of the

answer.

The "patience"  parameter (default  1000 milliseconds) determines

the  time  a  correspondent  will wait after having "received" at

least a sign. This time is therefore identical  with the interval

between signs without the correspondent supposing that the answer

is finished.

6.2. Keying practice

In this exercise the keyed signs appear directly in a window. The

menu allows the changing of the paddle parameters (see 6.1.1):

CwDrill is  very tolerant  towards the  keying style of its user.

His style does not need to be perfect  in order  to be recognised

correctly.  To  describe  this  tolerance with words is, however,

impossible. Only the actual trying out can show this clearly.

9

CwDrill 1.4                                            Page

-----------------------------------------------------------------

CwDrill knows the shortcuts commonly used  like e.g.  "QR", "QS",

"CQ".

The sign  last keyed  can be deleted by keying "error" (8 dots or

more).

By keying "SK" one returns to the exercise  menu. There  option 7

allows to return to the main menu.

6.3. Copying practice

In this exercise CwDrill transmits text read from a file.

The menu offers the following choices:

1    Begin

2    Speed

3    Pitch

4    From the file

5    Duration (Min)

6    Main menu

The  speed  is  defined  by  option 2 (default 150), the pitch by

option 3 (default  800),  the  text  file  (extention  ".TXT") by

option 4  (no default) and the duration by option 5. To return to

the main menu one  can either  chose option  6 or  key "I". Begin

exercise with option 1.

During  the  "transmission",  the  "statistics" window shows time

left and number of signs  transmitted.  The  transmission  can be

interrupted by  keying any  sign. A menu permitting to change the

transmission parameters or to abort the exercise then appears.

6.4. Default values

This choice sets the parameters to their default  values. Default

values are explained in the appropriate places of this document.

6.5. Colour, Black-White

This option  allows to  chose a colour or black and white screen.

The type of screen is saved in the configuration file.

6.6. Fran ais, English, Deutsch

CwDrill speaks 3 languages. The choice of this option

allows the changing into another of the specified languages.

7. Miscellaneous

7.1. Several configuration files

10

CwDrill 1.4                                            Page

-----------------------------------------------------------------

If severals people use  the same  program, each  one of  them can

have  his  (or  her)  own  configuration  file.  To  create a new

configuration, simply make a  copy of  the original configuration

file.