Execute VIDEOID to identify the type of video adapter(s). The information is

written both to the screen and also to a text file named VIDEOID.TXT

The first six steps, identifying the installed video adapter(s) are based on

routines from Richard Wilton's "Programmer's Guide to PC and PS/2 Video

Systems" (Microsoft Press, ISBN 1-55615-103-9) with minor modifications to

identify 64k/256k EGAs. An improved Hercules detect routine is based on

David B. Doty's "Programmer's Guide to the Hercules Graphics Cards"

(Addison-Wesley Publishing, ISBN 0-201-11885-8).

If an EGA or VGA is detected, a seventh step dumps the first 216 bytes of

video RAM to show the manufacturer's copyright notice.

If a VGA is detected, step 8 attempts to identify the chipset manufacturer

and the amount of video memory, if possible. These routines are primarily

based on John Bridges' VGAKIT with modifications and additions. VideoId adds

detect routines for DGIS, the IBM 8514/a, the IBM XGA, the Hercules Graphics

Station and additional memory tests, where available.

Much thanks to Bill Hinkle for the Tseng 4000 memory detection and to

Bert Tyler for the IBM XGA routines!

detect  memory  memory test results

------- ------- ---------------------

AcuMos                     vgakit  vgakit       256k, 512k, 1meg

Ahead Systems type A       vgakit          ???k

Ahead Systems type B       vgakit          ???k

ATI                        vgakit  VIDEOID      256k, 512k, 1meg

Cirrus                     vgakit  vgakit  ???k

Compaq                     vgakit  vgakit             512k

Chips & Technologies       vgakit  vgakit       256k, 512k

DGIS                       VIDEOID         ???k

Everex                     vgakit  vgakit       256k, 512k, 1meg

Genoa                      vgakit          ???k

Hercules Graphics Station  VIDEOID VIDEOID                  1meg

IBM 8514/a                 VIDEOID         ???k

IBM XGA                    VIDEOID VIDEOID            512k, 1meg

NCR                        vgakit  vgakit  ???k

Oak Technologies           vgakit  vgakit       256k, 512k, 1meg

Paradise                   vgakit  VIDEOID      256k, 512k

Trident                    vgakit          ???k

Trident 8900               vgakit  vgakit       256k, 512k, 1meg

Tseng 3000                 vgakit          ???k

Tseng 4000                 vgakit  VIDEOID      256k, 512k, 1meg

VBE (VESA)                 vgakit          ???k

Video7                     vgakit  vgakit       256k, 512k, 1meg

I'll try to update this as more chipsets surface, and WELCOME hearing from

anyone who can fill in the gaps. As of this writing (12/16/1991), we need

video memory detection for:

- Ahead Systems (A & B)

- Genoa GVGA

- IBM 8514/a

(The AFI HQMode - Query Current Mode returns the number of bit

planes, which should indicate 4 planes=512k and 8 planes=1meg, but

SEEMS to return 8 planes even when the 1meg memory expansion isn't

installed.)

- NCR

- Trident, pre-8900

- Tseng 3000

Bob Berry