V P L O T

Version  1.00

U S E R ' S   M A N U A L

by

Adam A. Mentes, N6QR

P.O. Box 950

Pine Grove, CA 95665

Copyright (c) 1991 Adam A. Mentes. All rights reserved.

VPlot V1.00 User's Manual

____________________________________________________________

TABLE OF CONTENTS

QUICK START . . . . . . . . . . . . . . . . . . . . . .    4

INTRODUCTION  . . . . . . . . . . . . . . . . . . . . .    5

What is VPlot? . . . . . . . . . . . . . . . . . .    5

Why should you use VPlot?  . . . . . . . . . . . .    5

MS  Windows, CUA  and  the  search  for  the  Holy

Grail.  . . . . . . . . . . . . . . . . . . .    7

System Requirements. . . . . . . . . . . . . . . .    9

Fabulous Future Features.  . . . . . . . . . . . .    9

Credits and Acknowledgements.  . . . . . . . . . .    9

GETTING STARTED . . . . . . . . . . . . . . . . . . . .   10

Installation.  . . . . . . . . . . . . . . . . . .   10

Program Files. . . . . . . . . . . . . . . . . . .   10

Slope Determination. . . . . . . . . . . . . . . .   11

THE WINDOWS . . . . . . . . . . . . . . . . . . . . . .   12

The Menu.  . . . . . . . . . . . . . . . . . . . .   12

File  . . . . . . . . . . . . . . . . . . . .   12

Clear . . . . . . . . . . . . . . . . . . . .   12

Run . . . . . . . . . . . . . . . . . . . . .   13

Callsign  . . . . . . . . . . . . . . . . . .   13

Setup . . . . . . . . . . . . . . . . . . . .   13

DataLog . . . . . . . . . . . . . . . . . . .   13

About . . . . . . . . . . . . . . . . . . . .   13

Commercial  . . . . . . . . . . . . . . . . .   13

The Main window. . . . . . . . . . . . . . . . . .   13

Antenna Height Icons  . . . . . . . . . . . .   13

Setup Information . . . . . . . . . . . . . .   14

Results Box . . . . . . . . . . . . . . . . .   14

Main Lobe Width  . . . . . . . . . . . .   14

Reflection Zone  . . . . . . . . . . . .   14

Max. Reflection Gain . . . . . . . . . .   14

Max. Total Gain  . . . . . . . . . . . .   14

Previous Run . . . . . . . . . . . . . .   15

Plot Area . . . . . . . . . . . . . . . . . .   15

Plot . . . . . . . . . . . . . . . . . .   15

Tricks with the Mouse  . . . . . . . . .   15

Station Configuration Setup window.  . . . . . . .   15

Frequency Band  . . . . . . . . . . . . . . .   15

Number of Antennas  . . . . . . . . . . . . .   15

Type of Antenna . . . . . . . . . . . . . . .   16

Antenna Height  . . . . . . . . . . . . . . .   16

Ground Characteristics  . . . . . . . . . . .   17

Ground Slope  . . . . . . . . . . . . . . . .   18

Select Direction . . . . . . . . . . . .   18

Direction Name . . . . . . . . . . . . .   18

Slope Angle  . . . . . . . . . . . . . .   18

Slope Scroll Bar . . . . . . . . . . . .   19

DONE button . . . . . . . . . . . . . . . . .   19

Station Callsign entry window. . . . . . . . . . .   19

2

VPlot V1.00 User's Manual

____________________________________________________________

About VPlot window.  . . . . . . . . . . . . . . .   19

VPlot Commercial window. . . . . . . . . . . . . .   19

PRODUCT SUPPORT . . . . . . . . . . . . . . . . . . . .   20

LEGAL STUFF . . . . . . . . . . . . . . . . . . . . . .   21

License. . . . . . . . . . . . . . . . . . . . . .   21

Warranty.  . . . . . . . . . . . . . . . . . . . .   21

APPENDIX  . . . . . . . . . . . . . . . . . . . . . . .   22

Uploading to your local BBS. . . . . . . . . . . .   22

About the Author.  . . . . . . . . . . . . . . . .   23

REGISTRATION AND ORDER FORM . . . . . . . . . . . . . .   24

3

VPlot V1.00 User's Manual

____________________________________________________________

QUICK START

I never fully understood  those who lack enough patience  to

read the manual  before trying a program. But  then again, I

do it  myself all the  time. Generally, I offer  the excuse:

"Just want to see how intuitive the program really is..".

I hope you'll  find this program  intuitive. To start,  just

type "WIN VPLOT", the same way  as you would start any other

Windows program. Click  on the menu items and  see what they

do. SETUP enables you to configure the data for your station

layout. RUN will plot your  "take off angle" pattern. If you

are an experienced Windows user,  you'll be drawing plots in

no time. If not, then it might take a minute or two more.

After a few runs, you may -   or may not - get the  feeling:

"Hey! This thing may be of use to me!".  If you do, make the

investment in  time, and read  the rest of these  pages. The

program may be intuitive enough  to run without the  manual,

but the results may not be as correct or useful as they will

be with a fuller understanding.

Enough said! Go  to it, and  have fun! I  hope you will  get

interested enough to read the rest...

4

VPlot V1.00 User's Manual

____________________________________________________________

INTRODUCTION

What is VPlot?

Vplot is a computer program designed to serve as  a tool for

amateur radio  operators, particularly those  with interests

in DXing  and DX Contesting.   It helps to  design effective

antenna systems, and to  evaluate prospective antenna sites.

The program runs on IBM  PC/AT and compatible systems, under

the Microsoft Windows V3.0 operating environment.

Specifically, VPlot calculates  and draws  the vertical  (H-

plane) radiation patterns for typical horizontally polarized

antennas over level, or  sloping ground of real  and varying

characteristics.

Why should you use VPlot?

One of the  oldest subjects for argument among  DXers is the

question of  the "ideal" QTH,  or antenna site.   Most DXers

fall into one of two groups: those that believe the ideal to

be a tiny island  in the middle of a giant  salt water lake,

and those  for whom  the ideal is  a mountain top,  with the

land sloping off in all directions, and smog-free visibility

for a hundred miles or more.

For me,  the  subject acquired  practical  dimensions  when,

after  decades of apartment  dwelling and using  antennas of

the indoor dipole  class, a move to the  "country" was made.

Few believe me  when I insist that the selection  of a moun-

tain top QTH was made for reasons other than the interest in

DXing.  In any case, the land slopes down in  all directions

from the top, and the view can be spectacular after a winter

storm  clears the air...   Before committing much effort and

money into  tower and  antenna systems, I  sought to  have a

better understanding of antenna site engineering.

As is the case in all engineering efforts, the first step in

finding the  solution is to  define the desired outcome.   A

cursory study of  any textbook  on antennas  makes it  clear

that  the  vertical  ("take-off")  angle  of a  horizontally

polarized antenna system is a  function of the height of the

antenna above ground.   The books  explain that the  antenna

pattern is  the result  of direct  waves combining  with re-

flected  ones either in  a reinforcing, or  in a destructive

manner.   The reflected  wave  arrives delayed,  due to  the

extra  distance it  had to  travel from  the antenna  to the

ground surface  before getting  reflected.   This delay  ac-

counts for the two waves being out of phase with each other.

By  the repetition of many calculations for each incremental

angle, a pattern  can be calculated and plotted.   Computers

are really good  at this kind of task.   The first question,

5

VPlot V1.00 User's Manual

____________________________________________________________

however, still is: what are the desired angles of radiation?

There are other good  books in the radio amateur  literature

explaining  the fundamentals of ionospheric propagation.  We

do not have  to dwell on that  here, except to say  that our

generally simplistic models of  common HF propagation  modes

are vague enough  to preclude useful prediction  of specific

arriving  wave angles between two  points at any given time.

There  are of course more accurate prediction methods, using

techniques such as ionospheric sounding, etc., but these are

not commonly available to the  amateur community.  So how do

we  arrive at  the answer  to the  question of  desired wave

angles?   In  the first  place, the  answer may  be somewhat

different for the DXer and the Contester.

Most DXers  have noted, that  if they listen  to a  given DX

station for a long period of time, the signal will build  to

a peak,  and then  decline.  The  peak will  not necessarily

occur at the same time as it does for their neighbor  on the

other side  of the town.   It is reasonable to  assume, that

the peak in signal is  dependent, among other things, on the

vertical lobe pattern of the receiving antenna.  The amateur

with  an antenna system exhibiting sharp multiple lobes (for

instance a 28  MHZ beam at 120 feet) will often observe what

has  been described  as the  "travelling  footprint".   In a

period of an hour or two, the  signal from a DX station will

repeatedly go  up and  down by  as much  as 20-25  dB.   One

explanation is that with the  passage of time, the  changing

height  of the ionosphere  results in the  changing arriving

wave angle going through the  peaks and nulls of the receiv-

ing antenna.  The wise DXer  saves his lungs and finals, and

waits until the  DX peaks on his  antenna.  Then he  can im-

press  friend and  foe  alike  by "bagging"  the  DX in  one

call...

The Contester on  the other hand, can seldom wait.  If he is

to run  up a good  score, he has  to work  them as he  finds

them.   As the  late Dr. James  Lawson, W2PV  concluded, the

contester  can sacrifice a  dB or two  in peak lobe  gain in

order to avoid the sharp multiple lobes in favor of a single

"fat" lobe.  My  personal preference, even for  casual DXing

is one big fat lobe, and for the same reasons.

The  other  desired  pattern  characteristics for  effective

DXing and DX Contesting is substantial radiation at very low

angles.  It has been shown time after time, that the station

capable of very low angles  of radiation may not always pre-

vail, but will  do so in  the majority  of situations.   And

this  is true even on the  low frequency bands.  The "Kings"

of 80  m will inevitably  have patterns with very  low angle

lobes.

A long standing myth among DXers  is the old axiom: "when it

6

VPlot V1.00 User's Manual

____________________________________________________________

comes to antennas, the higher, the better..".   As discussed

above, it is   a shaky proposition even over flat lands, but

it certainly is not true over a down slope.  In fact, over a

down slope, with the antenna at certain heights, you can end

up with  a major "null"  just where  you want  your lobe  to

peak.   And while the maximum  theoretical gain from reflec-

tion is 6 dB, the  null from cancellation can exceed  -20 dB

in real practice!

Two conclusions from all this.   First, perhaps one ought to

worry less about  the magnitude of peak gain  than about the

angle  at which that gain occurs.  Second, if the antenna is

not over flat ground, the height  of the antenna can have  a

far greater impact on the effectiveness of the antenna, than

the two or three dBs of additional free space gain for which

DXers choose  long boom  Yagis.  With  so many  variables to

deal with  simultaneously, computer  simulation is  a useful

aid in design.  Ergo VPlot.  I used an early version, creat-

ed for my  personal use only (with an  atrocious user inter-

face) to design my own antenna system.

So what is the "ideal" QTH?  If  you want one big "fat" lobe

from  zero to 20 degrees, a hilltop with a gentle down slope

is about  the only  way to  get it  with  a single  antenna.

Furthermore, you'll get that pattern with a surprisingly low

(and therefore less expensive) mast  or tower.  On the other

hand, the flatlander over the salt water marsh with his tall

tower may get several dBs  over the hilltopper with the same

antenna.   And if the signals arrive at the right angles for

him, he wins!  Like most other things in life, the choice is

a trade-off..   The more important question is,  what is the

right QTH for you, or  in more realistic terms, what is  the

right antenna setup for your QTH?  Only you can  answer that

one.  A few hundred "runs" with VPlot ought to help.

Of MS Windows, CUA and the search for the Holy Grail.

The  IBM PC  was brought  into  our world  by the  boys (and

girls) at Boca Raton as  an "open system".  This  meant that

specifications were made public, and the IBM system could be

enhanced or actually "cloned" by third  parties (competitors

to  IBM in  many cases).   This  open system  approach, very

different  from  say  that of  Apple's  Macintosh  which was

originally a closely guarded  closed system, was responsible

for the truly incredible success of a  technically less than

outstanding  computer design.  The open system approach also

resulted  in  an  unbelievable  proliferation  of  expansion

hardware.  In a classic  dilemma of standards versus innova-

tion,  in  the  final accord,  the  marketplace  established

standards in  some areas, but  by and  large innovation  and

proliferation still rages  on.  The area of  word processing

software serves  well as an  example.  Any top  quality word

7

VPlot V1.00 User's Manual

____________________________________________________________

processor for  DOS supports literally hundreds  of different

interfaces  to legions  of printers.   This means  that each

application developer  has written  specific code  (drivers)

for all of those printers.  The cost, and the duplication of

effort  is immense.  Perhaps Microsoft or Wordperfect Corpo-

ration can afford that.   The small developer with the small

application  certainly can  not.   The  result is  that word

processing  software  from  the  smaller developers  support

fewer printers.

Another fairly obvious problem with IBM PC Software, and one

that  Apple people wasted  no time pointing  out repeatedly,

was the fact that every  application developer had their own

idea (or in  some cases no idea) of what  the user interface

should be like.   This resulted  in at  least two major  ef-

fects.   One,  some otherwise  fine  programs sport  a  user

interface that could be characterized only as atrocious, and

two,  with each  new application  the  user had  to learn  a

unique  interface. In  Buzzwordese this  is  described as  a

"steep learning curve"; in plain English - a waste of time.

Enter GUI, CUA, and MS WINDOWS! Is nirvana here?   Well, not

quite.   But perhaps we  are moving in the  right direction.

MS WINDOWS' Graphic  User Interface (GUI) and  most applica-

tions  running under it  are compliant with  the IBM Systems

Application  Architecture  (SAA)  Common  User Access  (CUA)

specification.   (Just  listen  to  all  that  acronym-laden

techno-babble!).  What  all that means, is that  the PC with

MS Windows will  look more like Apple's Macintosh.   This is

not the end of the open system architecture either.  Innova-

tion can proceed  almost unimpeded.  Only  now, the designer

of the hardware  device is also expected to  supply the Win-

dows  interface software  (driver) for  his  hardware.   CUA

compliance means  that any application running under Windows

will look and feel a lot like any other Windows application.

Therefore, once  you learn to  use one application,  some of

what you learned will directly apply to others.

So,  is there  a fly in  the ointment?   (Would I  raise the

question if there was not  one or more?).  Two  are obvious.

First, MS  Windows is  an evolving product.   While  V3.0 is

vastly superior to earlier versions, it is far  from perfec-

tion.   As this  is being  written, V3.1  is just  below the

horizon,  with significant  improvements.   Second,  all the

Windows goodies come at a price.  The environment gobbles up

system resources and performance at a rate that shocks those

of us who were around when 16K RAM  was a lot of memory in a

Mini computer.   Then again, with a MegaByte  of RAM selling

for  less than  $50, why  not throw  brute resources  at the

problem?.  So, if you  want to keep  up with the  times (and

WINDOWS), obtain enough  high performance hardware to  do it

in style. Whether you or I like it or not, the trend is set.

8

VPlot V1.00 User's Manual

____________________________________________________________

System Requirements.

This program requires, and runs under the MS Windows operat-

ing environment,  V3.0 or later.   There are no  software or

hardware prerequisites other  than those imposed by  MS Win-

dows itself.   The  program should run  fine on  any machine

capable of running  applications under Windows 3.0  in stan-

dard or in enhanced mode.  The  program will not work on ma-

chines running Windows in "real"  mode.  In practical terms,

this means you need a machine with an 80286 or better CPU.

Fabulous Future Features.

Future enhancements  to VPlot  will be  highly dependent  on

user interest and  support.  I am interested  in suggestions

for improvement from you.  As of this writing, the following

features  have  been identified  for  probable inclusion  in

future versions:

Configuration File Save/Load.

Data logging of runs.

Selectable metric unit measurements.

Context sensitive Help System.

Antenna stacking.

Vertically polarized antennas.

Credits and Acknowledgements.

I  would  like to  express my  gratitude to  two individuals

whose help contributed greatly to this program.

First, Tom Mandey,  N2AG, who was there when  I first caught

the DX bug at the tender  age of thirteen, and who acted  as

an unpaid consultant  during the development of  an earlier,

DOS version of VPlot.  Tom has one of the best minds I know.

I am  also indebted to Dr. Morris Brown, N6DJM, who acted as

most of the  testing facility throughout the  development of

VPlot.  He found the bugs I overlooked, but even more impor-

tant,  his enthusiasm and encouragement got the project over

the humps to completion.

IBM, PC/AT are trademarks of International Business Machines

Corporation.

MS-DOS, MS WINDOWS are trademarks of Microsoft Corporation..

9

VPlot V1.00 User's Manual

____________________________________________________________

GETTING STARTED

Installation.

VPlot can be installed to and loaded from either a floppy or

a hard disk. However, if you are running MS Windows -  which

is   a prerequisite - then  the likelihood is  high that you

have a  hard disk...   Loading the  program takes  time, and

from a floppy that time may be even more annoying.

I suggest you make a  subdirectory on your hard disk, named,

say, "VPLOT"  (isn't that  imaginative?), and  copy all  the

files there.   Assuming that  Windows is on your  "path", to

run the program, while  in the VP subdirectory,  simply type

"WIN VPLOT".  If  you are already on friendly terms  with MS

Windows, then you  know how to set  it up so you  can launch

VPlot from an Icon in the Program Manager.

Program Files.

VPLOT100.ZIP

This is the  ZIP file  that actually  contains all  the

other files in a compressed form.  Any dissemination of

the  program should be done only by copying (and/or up-

loading to BBSs) this single file.

VPLOT.EXE

This is the  main executable file.   Remember, it  will

not run under DOS,  it will only tell you that it needs

MS Windows...

VBRUN100.DLL

This  is a Dynamic Load Library (DLL) that is essential

to  run the program.   Windows loads routines from this

library, as needed, when the program is running.

VPLOT.DOC

VPlot User's  Manual.  This  is a pure ASCII  file con-

taining the  documentation you  are now  reading.   The

formatting of the  file is such,  that it should  print

correctly  on practically any  printer by simply typing

the simple DOS command: COPY VPLOT.DOC PRN:

If you are short of disk space, the only two files  that you

must have to run the program are VPLOT.EXE and VBRUN100.DLL.

The other files can  be deleted or kept elsewhere.   As long

as you have  kept the original VPLOT100.ZIP,  the others can

be unZIPped from it again.

10

VPlot V1.00 User's Manual

____________________________________________________________

Slope Determination.

The eye  is a  notoriously poor  judge of  slope angles.   I

strongly recommend that after deciding which five directions

are the important ones for  your activities, go out and make

actual measurements  of the slopes  in each of  those direc-

tions.  There  are many ways of measuring  the slope angles.

Using surveying tools,  like a transit level is certainly an

elegant way  of doing it,  if available.  When  measuring my

slopes,  I used a  Rube Goldberg transit,  improvised on the

spot from a carpenter's level and a camera tripod.  Once you

have determined the elevation  loss (gain) for a given  dis-

tance along  the ground, the  slope angle can be  found with

your  handy-dandy scientific pocket  calculator (or - horror

of horrors -  trig tables).

For those who remember  their trig as well as I  do, that is

to say, just barely, the  angle is the ArcSin of  the eleva-

tion loss over the ground distance.  For example, let us say

your ground loses  3.5 feet in elevation over  a ground dis-

tance of 20 feet.  Then the angle (= ArcSin (3.5/20)), works

out to be  slightly over 10 degrees.   Because it is  a down

slope, for  VPlot we express  it as a  negative angle,   -10

degrees.   If the slopes are  very uneven, make measurements

at  various distances  from the  proposed  tower site.   The

knowledge gained will be of much use later.

Of course  one of the ways to measure  your slope is to have

the professionals do it.  While hiring a survey crew is very

expensive, chances are you can buy a topographic map of your

neighborhood very  cheaply.   Chances are  your slopes  have

been measured by the pros years ago.

11

VPlot V1.00 User's Manual

____________________________________________________________

THE WINDOWS

The Menu.

For users new  to MS Windows: All menu  items are accessible

either by clicking the mouse (after moving the mouse pointer

to the menu item); or for those without a mouse (get one; it

will coexist in peace with  pussycats in the house) from the

keyboard by  pressing the  Alt key  simultaneously with  the

underlined character in the menu item name.  Some more often

used items down  in the menu hierarchy are  accessible by so

called "accelerator" keys as well.  The accelerator keys are

noted on  the menus, and  are noted below  where applicable.

For VPlot,  an effort was made  to keep the menu  simple and

access  to the  most used  features a  single click  or key-

stroke.

File

New

Clears internal and externally displayed

data.   Resets station  setup parameters

to default values.

Save

Not implemented in Version 1.00. Intend-

ed to save configuration data on disk to

eliminate  the need  to reconfigure  the

setup each time the program is loaded.

Load

Not implemented  in Version 1.00.   Com-

plimentary to  "Save" above.   Will load

configuration setup from disk file.

Print

Screen

As implied, this  will print the  screen

with the  plot  and  all  textual  data.

This can take a long time with  Windows.

Have patience, and a cup of coffee...

Note: Using the Windows "Control Panel",

you may be able alter  your printer con-

figuration  for  optimum  print quality.

May take some experimentation...

Accelerator: Ctl + P

Log

Not implemented in Version 1.00.

Exit

This is the preferred method of  exiting

the program once finished for the day.

Accelerator: Ctl + X

Clear

Clears the  plot, and the text  data re-

lated to the run from the results box.

12

VPlot V1.00 User's Manual

____________________________________________________________

Run

Calculates and plots  the antenna system

vertical  radiation  pattern,  using the

station  configuration  setup   data  as

entered and  displayed.   Station design

is an  iterative process.   It may  take

hundreds  of runs  to  decide the  final

placement of the tower  and antenna at a

given QTH.

Callsign

Entering the station callsign serves two

purposes: if the results are printed out

it associates the data with the station;

when File Save/Load  and DataLog is  im-

plemented  in a  future  version of  the

program, the data  will be identified by

the  station  callsign.    Do not  worry

about case, the program will convert  it

to uppercase automatically.

Setup

Brings up and turns control over  to the

Station Configuration Setup window.  For

details, see below.

DataLog

The data log features are not implement-

ed in Version  1.00.  When  implemented,

they will provide a capability to keep a

log of all relevant data from successive

runs, including  setup and  results data

for viewing and/or printing.

About

Brings up an information  window.  Tells

you  the version  number,  and not  much

else.  Then again, no extra charge...

Commercial

Brings up the VPlot "Commercial" window.

This  is as close  as you get  to a hard

sell while running the  program...  Once

registered,  feel  free to  just  ignore

this menu item.

The Main window.

Antenna Height Icons

All station configuration setup is entered on

the window provided  for that purpose.   From

the main window, the only setup item accessi-

ble is antenna height.  This facility is pro-

vided, since in most situations, many succes-

sive  runs  are  made  varying  the   antenna

height,  to find the best compromise.  The up

and down arrow icons represent antenna height

13

VPlot V1.00 User's Manual

____________________________________________________________

adjustment up or  down.  Each click  with the

mouse  or  keyboard  accelerator  input  will

alter the antenna height  by the Stepsize (as

preset in the Setup window).

Accelerators: Ctl + U (up) and Ctl + D (down)

Setup Information

The information entered on the Station  Configura-

tion Setup window, is displayed on the main window

for  visual reference, and  so that it  will print

out along with the plot and the run results.

Often,  once an  antenna height  is  selected, you

want to check out  the pattern in the  various di-

rections.  To  avoid the necessity of  bringing up

the setup  window to select  another direction, an

accelerator key,  Ctl S  (for slope)  can be  used

from the main  window to "cycle through"  the five

directions.

Results Box

Main Lobe Width

The main lobe  is the lobe with  the greatest

reflection  gain at its  peak.  The  width of

the lobe is defined by the -3 dB (half power)

points. As discussed earlier,  this is criti-

cally important  information for the  HF DXer

and Contester.

Reflection Zone

For  the purposes  of  VPlot, the  reflection

zone is defined as the distance from the base

of the tower where the downward radiated rays

reflect from the ground.  The "from" and "to"

distances  define the  points where  the rays

corresponding to  the main  lobe -3dB  points

reflect.   The  important consideration  here

is: when you specified the slope angle,  this

is  the zone where the accuracy of that angle

is significant.

Max. Reflection Gain

The theoretical maximum reflection gain (with

an isotropic radiator over perfectly reflect-

ing  ground, etc.) is 6 dB.  Reality will al-

ways  yield something  less.   The  kicker of

course is, that while  the reflection gain is

limited  to 6  dB, reflection loss  (from ray

cancellation effects) can  be much more  (al-

though the  same factors  that limit gain  to

less than 6 dB, also limit reflection loss in

the cancellation "notch")

Max. Total Gain

This  value is the gain of the system over an

isotropic  source in free space, at the given

14

VPlot V1.00 User's Manual

____________________________________________________________

radiation angle.

Previous Run

Provided for quick comparison, the Max. Total

Gain from the previous run.

Plot Area

Plot

When  you click on  the menu item  "Run", the

program  calculates  and  plots  the  antenna

vertical radiation  pattern in  increments of

one degree.   There are  many complex  calcu-

lations involved, and they can bring a power-

ful machine  (what a  relative term!),  let's

say an 8  MHz 286 with no  math co-processor,

to it's  knees in  no time.   The one  degree

step increment  was chosen to keep  the plot-

ting time reasonable  on reasonable machines.

Also, it makes  little sense to have  resolu-

tion much beyond accuracy.

Tricks with the Mouse

After  a plot  is drawn,  you  can place  the

mouse cursor anywhere on  the plot (actually,

anywhere within the square bound on two sides

by  the X and  Y baselines of  the graph) and

click  and hold down  the mouse button.   You

will see  a little  window light up,  showing

the gain  at the  angle the  mouse cursor  is

pointing to.  The left mouse button will show

the  total gain  in dBi,  the  right one  the

reflection gain in dB.   It's fun to find out

how  deep those  nulls in the  pattern really

are.  This feature is not accessible from the

keyboard.  It is your reward for keeping mice

in the house.

Station Configuration Setup window.

When you  bring up the Setup window for the first time, each

item is set at a default value.  These defaults are indicat-

ed below.

Frequency Band

You have  the choice  of one  of the nine  HF

bands.   Chose by clicking on the option but-

ton.

Default = 20 m

Number of Antennas

In  VPlot V1.00 you have no choice.  Plotting

for stacked antennas is planned for a  future

version.

Default = Single

15

VPlot V1.00 User's Manual

____________________________________________________________

Type of Antenna

There are four choices provided.  Some expla-

nation may be useful  here.  The "free-space"

radiation  patterns used  by the  program are

actually  simple  mathematical approximations

of real radiation patterns.  They are arrived

at  by raising the  Cosine of the  angle to a

power dependent  on which  antenna we're  ap-

proximating.  For those interested, the actu-

al  values  used,  the  corresponding  "free-

space"  H-plane  half-power lobe  widths  and

gains over an isotropic source are:

ANT  Pwr of COS     H-plane Width   dBi

Dip       1              90 deg     2.14

2el       1.8            70 deg     7.3

3el       2.5            60 deg     9.6

6el       5.6            40 deg    12.6

Some might  challenge those  values, particu-

larly for the 6 el Yagi, as being optimistic.

Remember, the  purpose here is  to illustrate

concepts, not to promote particular antennas.

Think  of that 6 el Yagi  as an ultimate long

boom HF antenna of giant proportions.  Often,

you learn that  it does not pay  enough divi-

dends to put up giant antennas...

Default = 3 el Yagi

Antenna Height

The  meaning of antenna  height in the  context of

VPlot  needs  explanation.   It  is the  effective

height  of the antenna.  For  an antenna over flat

land, that is  the same as the physical  height of

the  antenna.  Not  necessarily so for  an antenna

aimed in the direction of a down slope.  Visualize

the following.  Most hilltops have in fact, a flat

top.  This is where, often, the house is built and

the antenna  tower  is  erected.    The  effective

height of the tower over  the down slope is depen-

dent  on its  distance from  the  point where  the

terrain begins  to slope down.   You can  find the

effective height of the tower by drawing an imagi-

nary  line to extend the  slope to the point where

it  intersects the tower.  The effective height of

the tower is the part that is  above the intersec-

tion.

This  brings up  an  important consideration  that

many hilltop  dwellers forget  to consider  in the

design  of their antenna systems.  Most real hill-

top QTHs  have a  flat area on  the top,  with the

ground  sloping  off at  DIFFERING  angles in  the

16

VPlot V1.00 User's Manual

____________________________________________________________

different directions.   The LOCATION of  the tower

may give different  effective antenna heights  to-

ward different  directions over  different slopes.

In  that situation,  the careful placement  of the

tower or mast may enable you to achieve an optimum

compromise of  effective antenna  height over  the

slopes in most directions.  Of course, if you have

a  30 to  200 foot  motorized  crank-up mast  with

remote control from  the shack, then this  is of a

lesser concern.

This is probably a good place to debunk a favorite

myth among DXers.  This  has to do with the effec-

tive  height of antennas depending on the depth of

the  water table  or aquifer,  or  God only  knows

what.   Forget it!  The characteristics of the top

foot or less determine the reflective characteris-

tics of  your ground.   The HF radio waves  do not

penetrate  your  desert-like soil  in search  of a

vast underground pool of salt water.  Sorry!

Initial Scroll Bar

The Initial  Scroll Bar sets  the initial an-

tenna height.

Default = 60 feet

Step Scroll Bar

The  Step Scroll  Bar sets  the  size of  the

height change step  each time the up  or down

arrow icons are clicked on the main window.

Default = 5 feet

Ground Characteristics

The  characteristics  of  the  reflective  surface

determine  the  character of  the  reflected wave.

The  two characteristics that are used in the cal-

culations  are Conductivity  and dielectric  Perm-

ittivity.  Of course the values vary in real life,

but I believe the four choices provided cover most

situations adequately.

For those who  care, the actual values  of conduc-

tivity and  permittivity used in  the calculations

are:

Ground         Cond.               Perm.

Salt Water     4.000               80

Good           0.030               30

Average        0.010               10

Poor           0.001                4

A fresh water lake  has characteristics similar to

Good  earth.  Urban landscape is  Poor.  Don't get

too concerned.  It does make a difference, but not

all that much.  When  you think of all the commer-

17

VPlot V1.00 User's Manual

____________________________________________________________

cial  HF  communications  outfits  locating  their

antenna farms  on salt ponds,  remember they  were

concerned mostly about vertically polarized anten-

nas. For those, ground characteristics make a much

more significant difference.

Ground Slope

In most situations, except over flat ground, the slope,

whatever  it is,  varies  considerably over  relatively

short distances.  Consideration  was given to providing

a  means for  you to enter  actual topographic  data of

elevation vs. distance in the five selected directions,

but the idea  was rejected.  As  discussed earlier, the

purpose  of  the program  is to  give you  guidance for

antenna  site engineering, rather than to nail down the

exact pattern over precisely defined locations.  In the

game  of DXing  and Contesting,  there  are other  very

important  variables (such  as  propagation) which  are

even  more  vague in  our understanding  and scientific

modeling.

The ground slope  you should enter for VPlot  to use in

the calculations is the slope of the ground in the area

of the Reflection Zone.  (Which in turn is dependent on

the slope... Experiment!).

Select Direction

Select one of the five directions by clicking

on their option buttons.   From the keyboard,

just use TAB,  as usual, to shift  the focus.

You will know when  the direction option but-

tons have focus by a color change to red.

Direction Name

This field is for your convenience only.  The

computations  don't  care  about geographical

directions.   The  preset directions  are the

five that are the  primary directions of con-

cern  for DXers and Contesters on the US West

Coast.   For  others,  living elsewhere,  the

directions  of  interest will  be  different.

These  fields are editing  text fields.  When

you click on the field (or TAB to it with the

keyboard) the field  color changes to  white,

and  is ready for  keyboard input.   The main

purpose of the five direction setup is so you

can  enter the  directions and  corresponding

slopes just  once, and then switch among them

at will with a single keystroke.

Slope Angle

This  is a  read only  display  of the  slope

angle you set with the scroll bar for each of

18

VPlot V1.00 User's Manual

____________________________________________________________

the five directions.

Slope Scroll Bar

The Slope Scroll Bar is used to set the slope

for the five directions.  Any adjustment with

the scroll  bar applies to the direction cur-

rently selected,  as indicated by  the option

button.  The range is from -45 to 45 degrees.

Negative values indicate a down slope,  posi-

tive values an up slope (hope you do not have

many of those...).

DONE button

When done with making setup changes,  a click

on  the DONE button will get  you back to the

main  window.   From  the keyboard,  just hit

ENTER for the same effect.

Station Callsign entry window.

Just type  in the desired  callsign.  The  program will

automatically convert  your input to  uppercase.   When

done, click on  the OK button.  Clicking  on the CANCEL

button will erase the callsign from the screen, if any.

About VPlot window.

When done, just click OK to make it go away.

VPlot Commercial window.

When  done reading, click on  the button (or hit Enter)

to return to the main window.

19

VPlot V1.00 User's Manual

____________________________________________________________

PRODUCT SUPPORT

I sincerely hope that  VPlot is intuitive enough, and  where

it is not, this documentation is clear enough, that you will

be able to use it without problems.

If you have  questions, or suggestions for  improvement, you

may wish to  write to me at  the address given on  the front

page  of this  manual.   I  will attempt  to be  responsive.

HOWEVER, I SEEK YOUR UNDERSTANDING  FOR THE FACT THAT I WILL

NOT SUPPORT THE PRODUCT TO ANY EXTENT BY TELEPHONE.

If you order  an updated version with  your registration, it

will be mailed as  soon as the update is available.   In any

case, all registered users will be notified by mail when the

new version becomes available.

20

VPlot V1.00 User's Manual

____________________________________________________________

LEGAL STUFF

License.

VPlot is  not public domain  software, nor is it  free soft-

ware. VPlot is  copyright (c) 1991 by Adam  A. Mentes, N6QR.

Non-registered  users are granted  a limited license  to use

VPlot on a trial basis for the purpose of determining wheth-

er VPlot  is suitable for  their needs.   The use  of VPlot,

except for this limited purpose, requires registration.

Users may not modify VPlot in any way.

All users are  granted a limited license to  copy VPlot only

for the  trial use  of others subject  to the  above limita-

tions, and also the following:

VPlot  must be copied in unmodified form, complete with

the  full documentation  file  containing this  license

information.

No fee, charge  or other compensation may  be requested

or accepted by any licensee.

VPlot  may not be  distributed in conjunction  with any

other product.

Operators of  electronic  bulletin board  systems  may  post

VPlot for  downloading by  their users only  as long  as the

above conditions are met.

Distributors of public domain or user supported software may

distribute copies of  VPlot subject to the  above conditions

only  after obtaining  written permission  from  the author.

Please write for details.

Warranty.

The author  makes no  warranty of any  kind, express  or im-

plied,  including  without  limitation,  any  warranties  of

merchantability  and/or fitness  for  a particular  purpose.

The  author shall  not be  liable  for any  damages, whether

direct, indirect, special or consequential arising from  the

failure of this program to  operate in the manner desired by

the user.  The author shall not be liable for any  damage to

data or property which may be caused directly or  indirectly

by the use of the program.

IN NO EVENT WILL THE AUTHOR BE LIABLE TO YOU FOR ANY DAMAGES

ARISING OUT  OF YOUR USE OR INABILITY TO USE THE PROGRAM, OR

FOR ANY CLAIM BY ANOTHER PARTY.

21

VPlot V1.00 User's Manual

____________________________________________________________

APPENDIX

Uploading to your local BBS.

When uploading to  a BBS, generally, a short  description or

synopsis of  the program is  given. I recommend you  use the

following:

"Version  1.00  of N6QR's  VPlot.  Plots

vertical  takeoff  angles  for  antennas

over level  or sloping ground.  Helps to

find the optimum antenna height for your

location.  Requires MS  Windows V3.0  or

later."

22

VPlot V1.00 User's Manual

____________________________________________________________

About the Author.

Having started in Ham Radio and DX chasing some 38 years ago

at  the age of  13, I have  experienced many aspects  of the

hobby.   Over the years,  I have used antennas  ranging from

indoor  dipoles in  ground floor  apartments,  to long  boom

Yagis at height.  I have used VPlot extensively in designing

my current station.  For  what is a relatively modest setup,

I have had great success with the station.

The antenna farm  consist of a single, 65 foot guyed Rohn 45

tower, with a 20 foot long 3" O.D. 1/4" wall chrome-molybde-

num mast.  The mast is supported by two thrust bearings, one

at the top  of the tower, the  other 5 feet down  the tower.

At the very top of the mast is a 4 element 40m Yagi by  KLM.

A foot above the top of the tower is a KLM KT34-XA tribander

Yagi for 10, 15 and 20, 90 degrees offset from the 40m beam,

to reduce the  otherwise severe interaction.   Half way  be-

tween the  two beams is  a Cushcraft WARC band  trap dipole,

covering 12, 17 and 30 meters.  About three feet or so below

the  top of the tower are the support points for separate 80

and  160m half wave dipoles strung  up in drooping (Inverted

Vee?) configuration.  Last, but not least, the QTH is on top

of a ridge, with the ground sloping down in all directions.

This  station enabled me  to work DXCC  on each of  the 9 HF

bands,  and over  200 countries  on all  but  160m.   I have

achieved the DXCC Honor Roll years ago, and I am one country

away from  having worked them all.  I  have a drawer full of

contest  certificates, and enough  plaques to cover  a wall.

But I am proudest of my Milliwatt DXCC trophy, earned at the

very bottom of  the last sunspot cycle, with  a power output

of less than one watt.

Professionally, I  am an Engineering  Consultant, performing

computer system and software/firmware design for clients.  I

enjoy my professional  work as much as I enjoy my hobby.  It

is a special pleasure to combine the two from time to time.

23

REGISTRATION AND ORDER FORM

VPlot Version 1.00

Mail to:

Adam A. Mentes, N6QR                 Date...................

P.O. Box 950

Pine Grove, CA 95665

Name................................... Callsign............

Address.....................................................

City.............................State..........ZIP.........

Registration fee......................................$20.00

Please send next update when available ($7.00)........$.....

Prefer 5.25"....... 3.5"....... diskette.

Total enclosed........................................$.....

(Cash, Check, M.O., IRCs acceptable at 50c ea.)

Where did you obtain your copy of VPlot?....................

............................................................

Comments/Suggestions for improvement:.......................

............................................................

............................................................

............................................................

............................................................

............................................................

____________________________________________________________

THANK YOU FOR REGISTERING!

____________________________________________________________