NEATFIX.DOC

PURPOSE:

--------

The NEATFIX program changes the programming of the Chips & Technologies 286 NEAT chip

set, specifically in the CompuAdd 220 (286/20 Mhz).  The bus, as programmed by the CompuAdd

BIOS, will not allow the Artisoft AE2 adapter (16 bit Ethernet card) to operate correctly in 16 bit

mode, and causes the machine to hang at various times.  This program should ONLY be used

in a CompuAdd 220.  Its effect is to change the bus' mode from asynchronos to synchronous

operation.  NEATFIX is provided as an interim solution, the user is urged to upgrade the 220

BIOS.  CompuAdd Corp. released a new BIOS version in late January, 1991 that programs the

bus controller correctly.

Artisoft, Inc. assumes no liability for any damages resulting from the use or mis-use of this

program.

OPERATION:

----------

NEATFIX is run without any arguments, and should be placed in the computer's AUTOEXEC.BAT

file to be run at boot time.  Since it changes programming which the machine's BIOS puts in

place, it must be run each time the computer is booted or rebooted.  It must be run before any

Network Operating System (NOS) software is installed, and should be run before any TSR

application.  To assure that it is run first, copy the NEATFIX.COM file into the root directory of

boot disk of your computer.  Then add the line:

NEATFIX

to the AUTOEXEC.BAT file, using an ASCII file editor.  This line should be placed somewhere

near the top of the file, preferably near the PATH and/or PROMPT statment(s).  Save the changes

you've made, then reboot the computer.

To return the computer to original bus programming, remove the NEATFIX line from the

AUTOEXEC.BAT (or REM it out) and reboot the computer.

NEATFIX MESSAGES:

-----------------

NEATFIX will return one of 2 messages:

Message: "The Bus has been synchronized."

This message will appear if NEATFIX is successful in changing the bus programming.

Message: "NEAT chipset not installed. Nothing done."

This message will be returned if the bus is not a NEAT chipset or the program was unable to

change the programming for an undertermined reason.