NAME

INTCNT - Interrupt Counter

SYNOPSIS

INTCNT int#

int# = interrupt number

DESCRIPTION

INTCNT is a TSR utility that counts how many time an

interrupt is generated.

After the program is loaded, a four digit counter appears

on the upper right hand corner of the screen.  The counter is

updated every 18 clock ticks (about 0.99 second) by the timer

interrupt handler, reflecting the value of the internal program

counter, which is then reset as the screen counter is updated.

Initially the screen counter only reads 0000, since at this point

the actual interrupt handler that does the counting, though

having been loaded into memory, has yet to be attached to the

interrupt vector of the interrupt to be monitored (hereafter

referred to as the target interrupt).  The hot key ALT-F1 is

needed to make the connection and to start the actual counting.

To enable the use of a hot key with the program, the

keyboard interrupt is hooked by the program keyboard interrupt

handler.  Whenever ALT-F1 is pressed, the program keyboard

interrupt handler determines whether the program target interrupt

handler is attached directly to the target interrupt vector.  If

the connection has not been made, the program keyboard interrupt

handler makes the connection, sends a "beep" to the speaker, and

removes the ALT-F1 key press from the keyboard buffer.  If the

connection has already been made, the program keyboard handler

simply does nothing and allows the ALT-F1 key press to pass

straight through.

After the program target interrupt handler is attached to

the target interrupt vector, it springs into action whenever the

target interrupt is invoked.  The job of the interrupt handler is

to simply increment the internal program counter and then

transfers control over to the original target interrupt handler.

The value of the internal program counter is reflected by the

screen counter, which is updated every second and shows exactly

how many times the target interrupt has been invoked for the past

second prior to the update.

INTCNT does not provide a way to unisntall itself.  It is

recommended that the system be rebooted after the use of INTCNT.

Even though utilities such as MARK/RELEASE might serve to

uninstall the program, there might be circumstances where

interrupt handlers are heavily swapped in which the addresses of

INTCNT's interrupt handlers cannot be entirely removed from

memory.

Also, it is recommended that INTCNT be used in a

relatively clean system, with few other TSRs already in place.

INTCNT functions properly when loaded into UMB using DOS5's

EMM386.EXE and LOADHIGH.  INTCNT has also been shown to run under

OS/2's VDM.  However, with OS/2's visualization of various system

resources, the results generated by INTCNT are not necessary

valid.

HOW TO USE INTCNT TO MONITOR MODEM PORT

INTCNT was originally written to monitor the COM port to

which a modem is attached.  The original plan called for the COM

port interrupt, either interrupt 12 for COM1, or interrupt 11 for

COM2, to be hooked at the program start.  Upon realizing that

most communication programs swap the COM port interrupt handler

with their own interrupt handlers, the author derived the hot key

scheme to allow the COM port interrupt to be hooked after a

communication program's interrupt handler had been put in place.

Since some communication programs change the COM port interrupt

handlers to handle different modem-to-DTE speed and protocol

during the course of a single online session, the hot key scheme

was expanded so that its subsequent use allows "rehooking" of the

COM port interrupt.

By using INTCNT, the user can monitor how often an

internal modem or the COM port to which a modem is attached is

interrupting the CPU.  Typically, the modem or a COM port with a

16450 UART generates one interrupt for every "character" it

receives over the phoneline.  With the use of data compression,

the number of interrupts generated can be considerably more.  On

the other hand, with the use of a 16550 family of buffered UART

chip, the number of interrupts generated can be considerably

less.

These are step-by-step instructions for using INTCNT to

monitor the modem inside a communication program:

- INTCNT writes directly to the video memory, and thus

can only by used with communication programs that run under

"text" mode.

- load INTCNT into memory by issuing the command INTCNT

12 if modem is using COM1 or INTCNT 11 if modem is using COM2.

the screen counter 0000 should then be visable at the upper left

hand corner of the screen.

- load communication program.  At the terminal mode hit

the ALT-F1 key to hook the COM port interrupt.  A "beep" should

emit from the speaker.  "Talk" to the modem by issuing the

command ATZ, AT&V, etc..  and observe the screen counter.  The

counter should change to reflect that the modem is indeed

generating the expected COM port interrupts.  This is just to

confirm that INTCNT works properly, and this step should be

bypassed for subsequent sessions.

- get online.  For maximum entertainment value, find a

modem or BBS that would connect at 14,400bps with v.42bis

compression turned on.  Make sure the modem-to-DTE speed is set

to three to four times the modem-to-modem speed if compression is

used.

- scroll some ASCII file across the terminal or initiate

a file transfer.  If at any point the screen counter seems to be

stuck at 0000, hit ALT-F1 again to rehook the COM port interrupt.

- it is normal for the number of interrupts generated by

the modem to go from zero to three to four times the

modem-to-modem speed and back periodically.  The communication

program is simply exercising flow control over the modem -

telling it to start and stop sending data - so that the modem

does not send data faster then the communication program's

ability to process them.  Obviously, the less often the screen

counter shows a slow down of interrupt level the faster the

system and/or the better the communication program is able to

utilize the full throughput of the modem.  This information is

helpful for evaluating the suitability of a system under Windows,

Desqview, or OS/2, as a modem that cannot be made to sustain full

throughput under native DOS only suffers further loss of

throughput under a multi-tasking environment.

- another reason for the modem interrupt level to fall to

zero periodically is that the host modem is not sending data as

fast as the modem-to-modem connection allows.  This can be

verified by turning off all flow control between the local modem

and the communication program.  If full throughput is thereafter

sustained (even though the text scrolling across the screen turns

to garbage or file transfer shows numerous errors), then any

decrease of throughput occurs as a result of flow control applied

at the local level.

- if at any point the screen counter shows an abnormally

high number of interrupts, such as numbers at the neighborhood of

8000 or 9000, the target interrupt handler of INTCNT is probably

"doubled up" by the communication program's swapping action of

its various interrupt handlers for the target interrupt.  The

session should be stopped at this point and the system rebooted.

BUGS

INTCNT might not work with TSR or command shell that

extend the keyboard buffer, or programs that lock out the ALT-F1

key sequence.  Also, INTCNT should not be used to monitor the DOS

interrupt 21H.  Doing so might cause system lockup.

The author, Fai Lau, can be reached on Compuserve via 76477,1667.