NISTSET v1.4

by Joe Terwilliger

For revision notices see end of document.

There are probably quite a few other programs out there which will

do this same thing as this, but so far I haven't seen anything this compact

with as many features.  What with the Olympics going on and all I figured

I'd might as well set a record too.   (Jt 2/14/92)

PURPOSE:

NISTSET will call the National Institute of Standards and Technology and

get the latest time and update your system clock IMMEDIATELY.  Due to

processor speeds and whatnot you will likely lose a few nanoseconds, but if

you think about it, TECHNICALLY the actual time is different at each minute in

longitude around the world but hey, we have to have SOME consistency, right?

NISTSET does not set the date.  I leave this task up to you.  Buy a calendar.

There are quite a few parameters available for NISTSET, but first there is

one requirement:

You must have a Hayes compatible type modem with at least 300 baud capacity

(ooooh, scary!)  ie, your modem must utilize the AT command set.  The default

speed setting is 1200 baud.  If you ARE using a 300 baud modem be sure to use

the /3 parameter when running the program.  Your modem must return messages

like 'OK' and 'BUSY' to be fully operational with this program.

DEFAULTS:

The default settings are:

1200 baud on COM1: (parity and bits are set automatically)

Touch Tone Dialing

Long Distance, Out-of-State call.

Set to Eastern Standard Time.

PARAMETERS:

There are a few command line parameters for operating NISTSET on different

machines and in different areas.  The command line structure is as follows

(all parameters are optional)

NISTSET COMn: /3 /UTC n /P /L /T /AT initstring

COMn:    Change this only if you are using another com port other than

COM1:

/3       Changes baud rate to 300.

/UTC n   Changes your Universal Time Coordinate.  The time received

from NIST will be in UTC (previously GMT - Greenwich Mean Time)

The adjustments can range from 1 to 10 which will be SUBTRACTED

from the UTC value.  Since this program will only currently run

work in countries using the US area code format (ie, Canada, Mexico,

etc...) I have only accounted for UTC adjustments in that area (1 to

10 - hey, I just said that)  Here are some common UTC adjustments:

Code     Locale

1    Canary Islands

2    Atlantic Ocean

3    Greenland / Rio

4    Dominican Republic / Canadian Maritimes

5    EST USA

6    CST USA

7    MT  USA

8    PST USA

9    Alaska

10   Hawaii

The default setting once again is USA Eastern Standard Time

/P       This sends an ATDP command to the modem before sending the

phone number instead of ATDT.  Non-technically that means it dials

PULSE.  Again, you must have a Hayes compatible modem to do this.

/T       If you live within the 303 area code, use this to dial Toll to

NIST (with a 1- prefix).

/L       Use this if you live in the 303 area code and the /T switch doesn't

seem to be working.  (This dials the number locally)

/H       For use with High Speed modems with locked ports.  Technically all

this does is skip the routine which initializes the modem.  You MUST

have your com port properly initialized or NISTSET will exit with a

time out error.  Use your high speed modem's init program or the DOS

MODE program to do this before using this switch.

/AT initstring

If you wish to Initialize the modem replace initstring with

your commands.  You MUST seperate /AT and the string with a space

New in this version:  the /AT parameter MUST be the LAST switch

on the command line.  All switches set AFTER this switch will be

ignored (and sent to the modem as commands which might cause

problems)  Using this with the /H command will NOT properly

initialize your com port!

For a quick list of parameters type NISTSET /?.

WHAT HAPPENS WHEN YOU RUN NISTSET:

A basic NISTSET screen output would look like this:

NISTSET v1.5

(c) 1992 Joe Terwilliger

CONNECT 1200

[NIST Data will be displayed here as it comes down the line]

COMPUTER TIME: 20:12:43

READ TIME: 00:08:12

Disconnecting...

Offline.

After NISTSET connects it will receive and 'catch' the on-mark time sent from

NIST.  After it receives the time, it will display the time on your computer

(COMPUTER TIME:) and then display the time READ by NISTSET (this is for

verification purposes.)  This time is in UTC format (Universal Time

Coordinate. If this sounds strange to you, it used to be called GMT (Greenwich

Mean Time) but was changed a few years back.)  The READ TIME is NOT adjusted

for your local time, or for DST.  ie.  NISTSET will convert the above 00:08:12

to 20:12:43 if run for EST mode and assuming DST (daylight savings) is

detected.  The formula in this case would be:  24 - 5(est) + 1(dst) = 20.

Obviously this will vary depending on the time of year and the settings you

use.

ABORTING NISTSET:

At any time during the dialing or connection and before the computer has

received the time, you may press ESC to abort the program.  If it has

connected to NIST then NISTSET will attempt to send +++ then ATH to hangup.

Depending on your modem this wait may be up to 6 seconds. If your modem is

still cooking after doing this, then it didn't quite manage to hang up and

you'll have to do it yourself (shut off your computer or just your modem if

it's external).  This is likely because of com port buffering problems,

incompatible modems (unlikely because, like everything else, modems are 99.44

percent pure compatible these days)  This has been a major problem to work in

simply (programming wise) so if you do encounter any disconnect problems,

reports would be appreciated.

TIMEOUT ERROR or `NISTSET WON'T WORK':

There is a problem with the IBM BIOS command INT 14h that it requires a proper

response from the com port before it will send ANY data down the line. If the

Carrier Detect is enabled on your modem or the Data Set Ready is enabled then

you will get a time out error.  There is a way to avoid this -by using direct

IN and OUT statements to the COM port - however in an effort to keep this

program small and to save me some time, I haven't incorporated these methods

within the program.  If you ARE having a time out problem with the program

then you can use the other enclosed program - AT.COM - to set your CD and DSR

off.  For Hayes compatible 2400 style modems enter the following at the DOS

prompt:  AT &C0&D0.  I don't know the commands for 1200 or 300 baud modems;

check your manual.  The easiest way to reset the parameters is to issue an AT

Z and/or AT &F command to your modem.  99% of the time this will work.

FYI, I am not issuing a faulty or 'buggy' program here, these errors are

problems with your MODEM which have to correct separately (if indeed you are

getting a time-out error).

DISCLAIMER:

I have tested this version of NISTSET out as many ways as I could think of,

but still I offer NO guarantees that it will work perfectly or at all.  The

author (that's me) takes NO responsibility for any damage - good or evil -

that this program may inflict on your system.  However, the author wishes to

assure you that he would never do ANYTHING like that intentionally unless you

drive a Yugo.  In keeping with good ShareWare standards the author would also

like to invite any bug reports or suggestions for improvements be sent to him

at the address listed below.

REGISTERING:

If indeed you did find this program useful for setting your VCR clock

accurately or assuring your boss that you were NOT late, but that HIS watch is

off, then please consider registering your version of NISTSET.  It's only $5

and if you do, the author will provide you with a receipt and written

notification of any future releases or improvements to the program which will

be upgradable for FREE.

For $20 I will send a printed copy of the source code for the latest version

of NISTSET AND register you as an owner of NISTSET.

Send anything and everything (except complaints - see below) to:

Joe Terwilliger

RR1 Box 5880

Sebago Lake, ME  04075

If you have a complaint, please write in red crayon on a 3x5 card, and give it

to your local grocer.  He'll know what to do with it.

REVISIONS:

v1.1  2/16/92

Added compensation for DST during the summer months (oops!)  Since this is

now automatically detected depending on codes received from NIST, do NOT

adjust the UTC code to compensate manually!  It's not like the world will

end if you do though, your time will just be an hour off...

Added minor error detection routine.  It's good if you have your modem set

to ignore the DTR (data terminal ready) and CD (carrier detect) and you

won't get this message.

v1.2  2/17/92

Changed /AT command parameters to shorten program length.  The /AT switch must

now be the LAST switch on the command line.

v1.3  4/5/92

Added code to check for OK after sending +++.  If it is not detected within 6

seconds the program exits to DOS, but your modem will still be on-line. If

this happens for some reason your modem has a problem.  I would recommend

shutting the modem off if it's external, or your system if it's internal.

However, since most modems produced now meet the Hayes AT command set

standards, it shouldn't be a problem.

Code was added to detect a BUSY message from the modem.  In this case you will

get a message from NISTSET and it will disconnect your modem and exit.

v1.4  4/13/92

Fixed code I just added.  Now after NISTSET has set the time it sends +++

immediately.  It waits for OK from the modem or for 5 seconds, whichever comes

first.  If it receives OK, then it will wait an additional second (to avoid

com port delays and lost buffering) before sending ATH.

Also added code to display your systems time, before it sets to NIST time.

(so you know how far off the beam your multi-thousand dollar computer actually

was)