4029UTIL for DOS

By:

J. Keith Wedinger

Version:

1.5

Date:

June 6, 1992

©Lexmark International, Inc.

Introduction

4029UTIL is designed to allow the user to send common escape sequences and commands to the 4029 LaserPrinter family without having to remember the complicated PPDS escape sequences.  Like the 4029 is the follow on to the 4019, 4029UTIL is the follow on utility to 4019UTIL.

Installation

Installing 4029UTIL is very easy.  Just use the following steps.

1.

If you got to this point, you successfully found, downloaded and unzipped 4029UTIL.ZIP from the Lexmark Bulletin Board.

2.

4029UTIL.ASC is this file and is the only documentation for 4029UTIL.  Please print on your 4029 and read completely before using 4029UTIL.

3.

Place the 4029UTIL files (4029UTIL.EXE and 4029CP.ASC) in a directory which is part of your PATH statement in your AUTOEXEC.BAT file.  For instance, if your PATH statement looks like this:
PATH C:\;C:\DOS;C:\TOOLS;C:\WINDOWS;
A good place to put the 4029UTIL files would be in your C:\TOOLS directory.

4.

In your AUTOEXEC.BAT file, add the following statement:
     SET 4029PORT=<printer port>
where <printer port> will be replaced by the port where your 4029 LaserPrinter is attached.  For example, if your 4029 is attached to LPT1, the above statement will look like this:
     SET 4029PORT=LPT1

DO NOT place a trailing a trailing colon (:) on the above statement.  A colon will cause 4029UTIL not to recognize the port.

NOTE to AES Users:  If you are running AES, the above statement should be:

SET 4029PORT=AES_PORT

This allows 4029UTIL to send data to your 4029 without AES intervening with a SIC (reset) command.

5.

Re-boot your PC or PS/2 to allow the change in your AUTOEXEC.BAT file to take effect.

You are done and ready to roll.  To see the complete syntax for 4029UTIL, enter 4029UTIL or 4029UTIL ? at the DOS prompt.  This will display a helpful syntax screen for 4029UTIL.

What 4029UTIL Does

Currently, this utility supports the following commands:

1) Select a desired bitmap font based on pitch.  The following pitches are supported:  10, 12, 17, and PS.  PS will select a proportionally spaced font.

2) Select a bitmap font from either font card based on position number.  The position numbers can be determined by looking at the Print Fonts output.  To perform a Print Fonts, hold the Start/Stop (ALT) button and press the Display More (Print Fonts) button on the op panel.

3) Flush print buffer with a form feed.

4) Reset printer to HP emulation mode.

CAUTION:  Switching datastreams causes ALL download fonts and macros to be lost.

5) Set orientation to landscape or reverse landscape.

6) Set lines per inch (values 1 - 72 supported).

7) Set orientation to portrait or reverse portrait.

8) Reset printer to PostScript data stream mode (if the PostScript option is installed...option will be ignored by the printer otherwise).

CAUTION:  Switching datastreams causes ALL download fonts and macros to be lost.

9) Reset printer to IBM ASCII (PPDS) data stream mode.

CAUTION:  Switching datastreams causes ALL download fonts and macros to be lost.

10) Select resident font based on position number.  See #2 above to get help on determining font position numbers.

11) Reset printer to factory default settings.

CAUTION:  Use this option carefully.  This option causes ALL previous printer settings including paper source to be reset to factory defaults.  This SIC does not write to NVRAM and therefore will not 	destroy your stored settings.  To get back to your NVRAM settings, use the NVRAM option on 4029UTIL.  Download fonts and macros will be retained.

11) Reset printer to NVRAM installation defaults.  If you have stored certain settings in NVRAM using either the op-panel or SIC commands, this option will reset the printer to these settings.  Do NOT fear. This option WILL NOT write to NVRAM and destroy your settings.  Download fonts and macros will be retained.

12) Select soft (download) bitmap font based on position number See #2 above to get help on determining font position numbers.

13) Print a test document to test current printer settings.

14) Print a sample of the currently active font in code page format.

15) Reset printer to plotter emulation mode.

CAUTION:  Switching datastreams causes ALL download 	fonts and macros to be lost.

16) Select a paper source (i.e. tray).  1 on the tray option selects tray 1, 2 selects tray 2, E selects the auto envelope feeder, M selects manual sheet feed, ME selects manual envelope feed and A selects the 100 Sheet Auxiliary Feeder.

17) Selection of PPDS scalable fonts.  See Selecting Scalable Fonts section below for more details.

18) Print any file by specifying its COMPLETE Drive:Path\Filename.

For example:  4029UTIL C:\TOOLS\README.TXT

The syntax for each of the above functions can be displayed on the screen by entering:

4029UTIL

or

4029UTIL ?

After a few uses, you will find the syntax very easy to remember.

Selecting Scalable Fonts

To correctly and accurately select any scalable font for first time use, please follow the following syntax:

4029UTIL [FIXED|TYPO] <Font Name> [Code Page] [POINT XXX.XX] [PITCH XXX.XX]

Note:  Parameters following FIXED|TYPO can be in any order.

The parameters show above have the following meaning:

FIXED

Starts a new font selection and limits the selection to fixed pitch (monospaced) scalable fonts.  When FIXED is specified, you can only select Courier, Courier-Bold, Courier-Oblique or CourierBoldOblique during scalable font selection.

TYPO

Starts a new font selection and limits the selection to typographic scalable fonts.  When TYPO is specified, you can select any scalable font except members of the Courier family listed above in the FIXED description.

<Font Name>

Name of the scalable font you wish to use.  Below is a complete list of all font names supported by 4029UTIL (this includes the resident scalable fonts and those found on the Scalable-22 Font Card).  Please note that the font name parameter is CASE SENSITIVE.  In other words, specifying COURIER will NOT select Courier.

Courier

Courier-Oblique

Courier-Bold

Courier-BoldOblique

TimesNewRoman

TimesNewRoman-Italic

TimesNewRoman-Bold

TimesNewRoman-BoldItalic

Helvetica

Helvetica-Italic

Helvetica-Bold

Helvetica-BoldItalic

Helvetica-Narrow

Helvetica-Narrow-Italic

Helvetica-Narrow-Bold

Helvetica-Narrow-BoldItalic

AvantGarde-Book

AvantGarde-BookOblique

AvantGarde-Demi

AvantGarde-DemiOblique

Bookman-Light

Bookman-LightItalic

Bookman-Demi

Bookman-DemiItalic

Helvetica-Light

Helvetica-LightOblique

Helvetica-Black

Helvetica-BlackOblique

NewCenturySchlbk-Roman

NewCenturySchlbk-Italic

NewCenturySchlbk-Bold

NewCenturySchlbk-BoldItalic

Palatino-Roman

Palatino-Italic

Palatino-Bold

Palatino-BoldItalic

ZapfChancery-MediumItalic

ZapfDingbats

SymbolSet

<Code Page>

Number ID of the code page you wish to use with the scalable font your are selecting.  Below is a list of all valid scalable font code pages which are supported by 4029UTIL.

437

819

850

863

1004

899

1035

One important note regarding code page selection.  Code page 899 can only be used and must be used when selecting the SymbolSet font.  Code page 1035 can only be used and must be used when selecting the ZapfDingbats font.

POINT ###[.##]

This parameter will scale any selected scalable font to the specified point size (###[.##]).

PITCH ###[.##]

This parameter will scale any selected scalable font to the specified pitch (###[.##]) or characters per inch.

WIDTH ###[.##]

This parameter will scale any selected scalable font to a width of ###[.##] points.  This allows the user the scale fonts horizontally with better control.  Please see examples below.

Examples:

To selecting Helvetica-Bold using code page 850 with a point size of 14:

4029UTIL TYPO Helvetica-Bold 850 POINT 14

To change just the point size of the above selection to 12 point:

4029UTIL POINT 12

To select TimesNewRoman-Italic with a point size of 10 and a double-wide width (width of 20):

4029UTIL TYPO TimesNewRoman-Italic POINT 10 WIDTH 20

Notice above that once TYPO or FIXED is specified, any of the other scalable font parameters can be changed freely and any changes made will affect the currently selected font.  Feel free to experiment to see what you can do!!!

What 4029UTIL Does Not Do

To keep the program relatively small and to increase performance, there is only minimal checking of the validity of parameters.  For example, invalid font position numbers are accepted by this utility but in all likelihood, will be ignored by the 4029 LaserPrinter.  This utility assumes the user knows what he or she is doing.  Whenever an invalid parameter is encountered, 4029UTIL will terminate execution at that point.

Also, there are a number of escape sequences that this utility currently does not support.  I am more than happy to take your suggestions on additions. If you have any suggestions, send them to me via VNET (NO PHONE CALLS!!!).

Also, if you notice any problems, send complete descriptions of problems to me also via VNET.  My VNET address is at the start of this document.  Additional support will be provided as needed on the Lexmark Bulletin Board.

PLEASE NOTE:

Once the 4029 has been set into HP, Plotter or PostScript emulation mode, this utility can only be used to change the emulation mode.  Once the emulation mode is set back to the PPDS (primary) data stream, then the utility can be used to its fullest.

For a utility to use in HP mode similar to the function of 4029UTIL, please look for HPUTIL.

On the font option output page, code point '1A'x (26) which is the right arrow character is not printed.  Instead, a space is printed.  This is because this character is recognized by DOS as the end of file character and attempting to print it during the font option causes the output to get fouled up.