This is a brief description of the speech synthesizers supported by

Tinytalk along with manufacturer contact information.  This is not intended

to be an evaluation and no endorsements are implied by these descriptions.

Prices aren't listed because they can change rapidly; contact the

appropriate manufacturer for pricing.

Synthesizers can be internal or external and "smart" or "dumb."  Internal

synthesizers plug directly into the computer's slots; some are designed for

standard slots such as those on desktop or high-end laptop computers, while

others are designed for specific laptops.  External synthesizers use either

serial or parallel ports to connect to the computer.

All synthesizers speak using a two-step process: they translate text into

phonetics and then pronounce the phonetics.  "Smart" synthesizers contain

circuits that do both, whereas "dumb" synthesizers use the computer itself

to do the text-to-phonetics translation with a software driver.  External

synthesizers are always smart; internals can be either smart or dumb.

Smart synthesizers usually cost more because they need to contain a small

computer of their own; dumb synthesizers usually require more memory to

hold the software driver and slow the computer down a bit (this isn't a

problem except when the computer is running multiple programs or receiving

fast-paced data).

ACCENT

Aicom Corporation

1590 Oakland Rd., Suite B112

San Jose, CA  95131

(408)453-8251

This line of synthesizers includes the Accent SA (external), Accent PC

(internal, standard slot, smart), Accent Mini (internal, standard slot,

dumb) and several laptop-specific internal dumb boards.  All of them are

based on the SSI263 chip.

ASP

Automated Functions, Inc.

6424 N. 28th Street

Arlington, VA  22207

(703)536-7741

This is a dumb internal board for Toshiba laptops.  It uses the SSI263

chip.

AUDAPTER

Personal Data Systems, Inc.

100 West Rincon Ave., Suite 103

P.O. Box 1008

Campbell, CA 95009

(408)866-1126

This is an external unit based on a proprietary digital signal

processor.  A battery-operated version is available.

BG MICRO COMPUTALKER

BG Micro

P.O. Box 280298

Dallas, TX  75228

(214)271-5546

This unit is an internal smart card, but uses its slot for power only; it

needs to be connected to a serial port (an external power supply is

available from the manufacturer, which avoids the need for a slot; however,

you then have a bare board with no case).  It speaks at a rather slow fixed

rate and has no settable parameters.  It uses the General Instruments chip

set (CTS256 and SP0256).  It costs significantly less than the other

synthesizers described here.

BRAILLE 'N' SPEAK

Blazie Engineering

109 E. Jarrettsville Rd.

Forest Hill, MD  21050

(410)893-9333

This is a dedicated-function battery-operated portable computer which can

be used as an external synthesizer.  It can emulate the Echo PC, though the

speech sounds different since it uses the SSI263 chip.  Newer models have a

native emulation mode which provides more speed control and better

handshaking.

DECTALK

Digital Equipment Corporation

146 Main Street

Maynard, MA  01754

(800)832-6277

This is an external unit based on a proprietary digital signal processor.

The current model has been discontinued, and two new models (external and

smart standard-slot internal) offering faster speech among other things

will be available shortly.

DOUBLETALK PC

RC Systems, Inc.

121 W. Winesap Rd.

Bothell, WA  98012

(206)672-6909

This is an internal standard-slot smart board using a proprietary

architecture.

ECHO GP/PC

Street Electronics

6420 Via Real

Carpinteria, CA  93013

(805)684-4593

This is an external unit based on the TMS5220 chip.  It is no longer in

production.  It has only two speeds, and squeaks when a program flushes the

speech buffer.

ECHO PCII

Street Electronics (see above)

This is an internal dumb board based on a more recent TI LPC design.  It

provides more parameter control than the Echo GP, but the speech quality is

not as good.

LAPTALK

Computersmith

115 N. Keats

Louisville, KY  40206

(502)893-7310

This is an external battery-operated unit based on an OEM speech board from

RC Systems (speech is identical to the Doubletalk PC).  This should not be

confused with the Laptalker from Automated Functions, which is a laptop

with a pre-installed ASP board.

LITETALK

Microtalk

337 S. Peterson

Louisville, KY  40206

(502)897-2705

This is almost identical to the Laptalk.

PSS

Votrax Inc.

24027 Research Drive

Harmington Hills, MI  48335

(313)442-0900

This is an external unit based on the SC01 chip.  It comes in two models, A

and B (B is apparently the current version) which differ primarily in the

available speech rates.

SOUNDBLASTER

Creative Labs, Inc.

2050 Duane Ave.

Santa Clara, CA  95054

(408)986-1461

This is an internal multimedia sound card which uses a proprietary digital

signal processor to provide digitized speech playback, multi-voice music

and a MIDI interface for musical instruments.  It is supplied with a

version of SmoothTalker, a text-to-speech driver manufactured by First

Byte.  SoundBlaster clones exist, but not all of them include SmoothTalker,

which is necessary for speech synthesis.  Unless your computer has expanded

(EMS) memory, SmoothTalker will require about 180K of main memory.  It may

not run well on slow XT-class machines.  SmoothTalker is rather processor-

intensive; it may interfere with telecommunications if you're trying to

listen to the output at the same time.

SOUNDINGBOARD

GW Micro

310 Racquet Drive

Fort Wayne, IN  46825

(219)483-3625

This an internal board based on the SSI263 chip.  It is available in both

standard and laptop versions.  Although a dumb board, it includes onboard

memory to hold its software driver which has a number of unusual features

such as the ability to "fast-forward" or "rewind" through the spoken text.

SPEAKEASY

Chip Orange

3227 Rain Valley Court

Tallahassee, FL  32308

(904)487-2680

This is an external synthesizer based on an Amiga 500 computer.  It can

emulate either a Dectalk or a Votrax PSS.  Since the translation and

pronunciation software is supplied on disk, the manufacturer can update the

unit without requiring it to be sent in.

SPEECH THING

Covox, Inc.

675 Conger Street

Eugene, OR  97402

(503)342-1271

This is a digital-to-analog converter module that connects to a parallel

port.  It is supplied with SmoothTalker, which provides text-to-speech

capability.  See the entry for the SoundBlaster for limitations.

SYNPHONIX

Artic Technologies

55 Park Street, Suite 2

Troy, MI  48083

(313)588-7370

This is a line of internal dumb boards (both standard and laptop) based on

the SSI263 chip.  Earlier models didn't talk well on fast computers.  Newer

models don't have this problem, but the Sonix software driver (required for

Tinytalk) which was bundled with the earlier boards is now an extra-cost

item for some models.

TYPE & TALK

Votrax Inc. (see above)

This is an external unit based on the SC01 chip.  It was one of the first

synthesizers available for personal computers.  Its speech settings cannot

be changed under software control, pitch and rate are not independent and

it sometimes mispronounces words after the buffer is flushed.

VOTALKER

Votrax Inc. (see above)

This is an internal standard-slot dumb board based on the SSI263 chip.  It

is no longer in production.  It uses the Sonix software driver (also used

by the Synphonix boards).