MKLAPI

M�rklin-Interface driver and

High Level Language

Application Programming Interface

for C-language and BASIC

Version 1.0

Document Number RH-MKLAPI 1.0

May 4, 1993

Rob Hamerling

Vianen-ZH

The Netherlands

Phone: 31-3473-72136 (voice)

FIDO: 2:512/4.1098

MKLAPI

MKLAPI

���������������������������������������������������������������������������

CONTENTS

Introduction  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

What is MKLAPI?   . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

Purpose   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

Realisation   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2

Registration  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2

Functional description  . . . . . . . . . . . . . . . . . . . . . . . . 3

Overview  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 3

Terminology   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 3

Functional Requirements   . . . . . . . . . . . . . . . . . . . . . . . 3

Provided Facilities   . . . . . . . . . . . . . . . . . . . . . . . . . 4

Deliverables  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 4

Prerequisites   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 5

Restrictions and Limitations  . . . . . . . . . . . . . . . . . . . . . 5

Tests   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 6

C-language framework  . . . . . . . . . . . . . . . . . . . . . . . . . 7

QuickBASIC framework  . . . . . . . . . . . . . . . . . . . . . . . . . 8

Sample programs   . . . . . . . . . . . . . . . . . . . . . . . . . . . 8

OS/2 and DOS differences   . . . . . . . . . . . . . . . . . . . . . . 10

Specifications   . . . . . . . . . . . . . . . . . . . . . . . . . . . 11

Available Functions  . . . . . . . . . . . . . . . . . . . . . . . . . 11

mklclose   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 11

mklflush   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 12

mklgetsn   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 13

mklgo  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 13

mklmsecs   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 14

mklopen  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 15

mklpoll  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16

mklpurge   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 16

mklputc  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 17

mklsleep   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

mklsound   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

mklstats   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19

mklstop  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

mkltime  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 22

mkltrace   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 22

Macro Specifications   . . . . . . . . . . . . . . . . . . . . . . . . 23

mklput1c   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 23

mklput2c   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

Use of timer   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

QuickBASIC Function Specifications   . . . . . . . . . . . . . . . . . 25

Errors and Signals   . . . . . . . . . . . . . . . . . . . . . . . . . 25

Installation   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 27

Application development considerations   . . . . . . . . . . . . . . . 27

C-Compiler considerations  . . . . . . . . . . . . . . . . . . . . . . 27

MicroSoft QuickBASIC compiler 4.5 Considerations   . . . . . . . . . . 27

Building an EXE-file   . . . . . . . . . . . . . . . . . . . . . . . . 27

C-language and DOS   . . . . . . . . . . . . . . . . . . . . . . . . 28

C-language and BASIC   . . . . . . . . . . . . . . . . . . . . . . . 28

C-language and OS2 1.3   . . . . . . . . . . . . . . . . . . . . . . 29

���������������������������������������������������������������������������

Contents  ii

MKLAPI

���������������������������������������������������������������������������

C-language and OS2 2.0+  . . . . . . . . . . . . . . . . . . . . . . 30

Storage and performance  . . . . . . . . . . . . . . . . . . . . . . . 30

Memory Utilisation   . . . . . . . . . . . . . . . . . . . . . . . . 30

Processor utilisation  . . . . . . . . . . . . . . . . . . . . . . . 30

Comments, Remarks, Bug-reports   . . . . . . . . . . . . . . . . . . . 31

Appendix A.  MKLAPI Internals  . . . . . . . . . . . . . . . . . . . . 32

Multitasking   . . . . . . . . . . . . . . . . . . . . . . . . . . . . 32

Reading M�rklin S88 encoders   . . . . . . . . . . . . . . . . . . . . 33

Communications Interface Specification   . . . . . . . . . . . . . . . 35

Problems with OS/2 COM[xx].SYS   . . . . . . . . . . . . . . . . . . . 37

OS/2 and DOS differences   . . . . . . . . . . . . . . . . . . . . . . 38

Appendix B.  Trace facility  . . . . . . . . . . . . . . . . . . . . . 40

Trace records  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 40

Sample   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 40

| Appendix C.  M�rklin-Interface peculiarities   . . . . . . . . . . . . 41

Appendix D.  Summary of changes  . . . . . . . . . . . . . . . . . . . 42

��������������������

(1)     This document is processed: on May 4, 1993 at 9:52 p.m.

���������������������������������������������������������������������������

Contents  iii

MKLAPI

���������������������������������������������������������������������������

INTRODUCTION

Copyright (C) R. Hamerling, 1991, 1993

All Rights Reserved.

WHAT IS MKLAPI?

MKLAPI is a combination of:

�  a driver for buffered communications, automatic reading of M�rklin S88

encoder-boxes, timer- and some other services.

�  a number of user callable routines for common elementary functions, with

a comfortable high-level programming interface.

So MKLAPI reliefs a programmer from 'low-level' tasks such as controlling

the communication with the M�rklin-Interface, so that he can concentrate on

the railroad-control logic.  This simplifies programming for

model-railroads which are controlled via a M�rklin-Interface.  Internally

it performs a kind of multi-tasking, which is highly desirable for this

realtime environment ('proces control').

PURPOSE

The driver and API has been developed for the following purposes:

�  have an application programming interface (API) which is independent of

the operating system (i.c. OS/2 or DOS).

�  provide a simple interface for a number of frequently used basic control

functions such as sending commands to trains and switch points, and

reading sensors.

�  allow the mainline to be written completely in high-level programming

language (i.c. C-language or QuickBASIC).

�  obtain a buffered communications facility to the M�rklin-Interface,

including automatic flow control as required by the M�rklin-Interface.

�  have 'background' tasks for a number of functions that should or could

be performed independently and asynchronously from the mainline.

�  provide an accurate and efficient timer facility for time-dependent

operations, events and interval-calculations.

���������������������������������������������������������������������������

Introduction  1

MKLAPI

���������������������������������������������������������������������������

REALISATION

This package offers a set of drivers with all the functionality of listed

"Purpose" on page 1.  Although the OS/2-drivers and DOS-drivers are

completely different implementations, due to the rather extreme differences

in functionality provided by OS/2 and DOS for this type of environment, the

mainline of your railroad control program may be unaware of the underlying

operating system: the application programming interface is identical.

You may read a number of minor differences in "OS/2 and DOS differences" on

page 10.

REGISTRATION

This package is of the 'SHAREWARE' type: You may try it for a reasonably

short time (max 3 months).  If you decide to use it for your own railroad

control program, you may show your appreciation and become a 'registered

user'.  After registration, I'll send you by mail:

�  A diskette (3-1/2", 720KB), with the following contents:

�  The most recent version of all the drivers.

�  Source-files (C-language) of a couple of sample programs provided as

executables in the base package.

�  Complete and up-to-date printed documentation, as well as 2

documentation files on the diskette:

�  A print-file, formatted for IBM Pro-Printer III.

�  A file in OS/2 INF format (VIEW-able as online documentation like the

inline OS/2 Command Reference Manual).

After registration you are allowed to distribute the driver as integral

part of your own railroad control application programs.  The package (not

the material you get after registration) may be distributed freely with

under the following conditions:

�  it remains a separate and complete package

�  you do not apply changes of any kind

�  you do not make profit with it

The registration fee is 45 Dutch Guilders or equivalent of 25 US-Dollars.

Bare money is accepted as well, but foreign money only in the form of

US-Dollar banknotes.

Please send your comments and registration fee to:

R. Hamerling

Vianen-ZH,

The Netherlands

Postbank acct#: 2087285

PC-Square: FIDOnet 2:512/4.1098

Mention the purpose of your payment ("MKLAPI"), and the destination

address!  MKLAPI-source is not included.

���������������������������������������������������������������������������

Introduction  2

MKLAPI

���������������������������������������������������������������������������

FUNCTIONAL DESCRIPTION

This chapter describes a possible use of this package.

OVERVIEW

The M�rklin-Interface-driver package is a set of functions to enable direct

communications support in a program written in C-language or BASIC for the

MicroSoft QuickBASIC compiler 4.5((2)) for the family of IBM Personal

System/2 and compatibles.

Please report problems with the driver as soon as you can.  and accompany

your report with as much details as might be relevant.

TERMINOLOGY

In this document I use a number of terms which should probably be clarified

first.

Interface The M�rklin-Interface. (M�rklin partnumber 6050)

Sensor    The M�rklin S88 encoder. (M�rklin partnumber 6088).  Sometimes

also called sensor-box or encoder.

Switch    The railroad facility, M�rklin K83 decoder (M�rklin partnumber

6083) is used to control these switch points.

FUNCTIONAL REQUIREMENTS

The M�rklin-Interface-driver must provide the following functionality:

�  Possibility to send commands to the M�rklin-Interface in any sequence

and mix.

�  Builtin buffered communication with the M�rklin-Interface.

�  Automatic flow control between PC and M�rklin-Interface.

�  Time-base (clock) for for time-dependent control, independent of

hardware (processor) speed.

�  Automatic readout of sensor-boxes (M�rklin S88 encoder): the mainline

must be able to obtain sensor positions instantaneously (without

waittime).

��������������������

(2)     Please note that this driver is delivered in the form of

.OBJ-files, and therefore can be used only with compiled programs

(other .OBJ-files).  QBASIC that comes with DOS 5.0 is not capable

of producing an .OBJ file, you'll need the MicroSoft QuickBASIC

compiler 4.5 to use this driver with BASIC programs.

���������������������������������������������������������������������������

Functional description  3

MKLAPI

���������������������������������������������������������������������������

�  Priority mechanism: mainline activities must be handled with higher

priority than 'background' service processes.

�  Ability for emergency stops, regardless of the state of the program or

the model railroad.

PROVIDED FACILITIES

To fulfil the functional requirements, a 'driver' has been built.  It

contains a number of service-routines (and for DOS also hardware interrupt

routines), and provides the following facilities:

�  Functions to send train and switch commands to the interface.

Two macro's for output of resp. 1 or 2 consecutive characters (the so

called 1-byte or 2-byte commands).  With the 2-byte form the subroutines

ensure that the 2 characters are sent as 2 consecutive characters (not

interrupted by a 'background' task).

�  Functions to read sensors (S88-boxes).

This can either be done by immediate reading the status of the last

polling operation (if polling started).  "Reading M�rklin S88 encoders"

on page 33 contains details about this mechanism.

If the mainline is sending a command to the M�rklin-Interface, polling

is automatically suspended (the command is treated with higher

priority).

�  A 'shadow' of the system timer, but expressed in milliseconds since

program start.  This timer has with the OS/2-driver of MKLAPI a

precision of 31.25 msecs, using standard OS/2 facilities (the timer

tick).  With the DOS-driver of MKLAPI the posibilities of the hardware

PC timer are explored, giving a precision of 1 millisecond.

�  A trace facility is built in for easy program debugging.  All output to

the interface can [optionally] be traced (OS/2 only) and formatted

later.

DELIVERABLES

The shareware components of the M�rklin-Interface-driver package are:

| �  This documentation file: MKLAPI.DOC.

| �  Header file for C-language: MKLAPI.H with function prototypes, data- and

|    macro- definitions.

| �  Include file for QuickBASIC: MKLAPI.INC with function prototypes.

| �  2 OS/2-object decks, MKLAPI2.OBJ (16-bits OS/2 version) and MKLAPI3.OBJ

|    (32-bits OS/2 version), with a buffered multitasking communication

|    interface, polling, timer-support and trace facility.

| �  2 DOS-object decks for C-language programs, SMKLAPI.OBJ and LMKLAPI.OBJ,

���������������������������������������������������������������������������

Functional description  4

MKLAPI

���������������������������������������������������������������������������

|    with the same functionality as the OS/2 drivers, (but without a trace

|    facility).  SMKLAPI is the object deck to be linked with your SMALL

|    memory model program, LMKLAPI.OBJ is for LARGE memory model C-language

|    programs.  LMKLAPI.OBJ is also used for QuickBASIC programs.

�  TRAIN.EXE and TRAIN2.EXE, which are ready to run sample programs for a

DOS and OS/2 environment respectively.  See "Sample programs" on page 8

for some characteristics of these programs.

�  MKLTRFMT.EXE, utility to interpret (format) the recorded trace file.

The registration package consists of:

�  Sources (in C-language) of a sample train control program that makes use

of the M�rklin driver, usable for OS/2 and DOS ('fullscreen'

application)

�  All include and header files

�  All MAKE and DEF files, used for compilation with MicroSoft C-compiler

6.00a or equivalent, IBM C SET/2 compiler for 32-bits programs and

MicroSoft QuickBASIC compiler 4.5 for QuickBASIC programs.

PREREQUISITES

The package has been built with the following assumptions:

�  Your PC-hardware is 100% compatible with the productline of IBM Personal

Computers (PC, PC/XT, PC/AT, and all current models of the Personal

System/2 family), in particular the asynchronous communication adapters,

their port addresses, etc. and the hardware timer.

�  The operating system is either OS/2 1.2 or higher, or PC-DOS 2.1 or

MS-DOS 2.1 or higher.  The package has been tested only with IBM PC/DOS

version 4.0 and 5.0, and with OS/2 version 1.3 and 2.0 (in OS/2

session).

�  Your (16-bits) C-programs (OS/2 and DOS) will be compiled with MicroSoft

C-compiler 6.00a.  For 32-bits C-programs IBM C SET/2 compiler is

assumed.  There may be some specific requirements on system hardware and

software for these.  Maybe Borland Turbo C compiler will work as well,

but this has not been tested.

�  Your BASIC programs will be compiled with the MicroSoft QuickBASIC

compiler 4.5.

RESTRICTIONS AND LIMITATIONS

The following restrictions apply:

�  One single port can be used between open and close of the COM-port to

the M�rklin-Interface, all other functions apply to the port chosen at

open-time.

�  The transmit buffer of the DOS-driver is 2048 bytes long, for OS/2 it

���������������������������������������������������������������������������

Functional description  5

MKLAPI

���������������������������������������������������������������������������

depends on COM[xx].SYS, or substitute.

�  Received data is transferred immediately to the sensor-array (64 bytes).

�  When the mainline is sending data to the M�rklin-Interface and a

buffer-full situation arises, the data will be discarded.  However it is

very unlikely that this will happen in real life.

�  If you read a sensor-box outside the polling-range, the response is

undefined.

TESTS

The drivers have been tested by the author with combinations of the

following hardware and software:

�  model railroad equipment:

�  M�rklin Combined Control

�  3 M�rklin Digital locs

�  3 M�rklin K83 decoders, each connected to 4 switch points

�  1 M�rklin S88 encoder, with railroad-section sensoring

�  computer hardware:

�  IBM PC/AT-3 (80286/8) with IBM serial-parallel adapter

�  IBM PS/2 model 80 (80386/16) with standard serial port

�  IBM PS/2 notebook N51SLC (80386SLC/16) with standard serial port and

second serial port

�  IBM compatible 80486/33 with multi-I/O card

Note: The DOS-drivers are written in 8086/8 Assembler and should also

work on XT-type systems.

�  Operation Systems:

|    �  IBM OS/2 2.0: 32-bits mode with MKLAPI3.OBJ, 16-bits mode with

|       MKLAPI2.OBJ, DOS-session with LMKLAPI.OBJ

|    �  IBM OS/2 1.3: MKLAPI2.OBJ

|    �  IBM PC/DOS 5.0: LMKLAPI.OBJ with QuickBASIC and C-language

Note: Not all combinations of hardware and software have been tested, but

it is not likely that any viable combination will not work.

���������������������������������������������������������������������������

Functional description  6

MKLAPI

���������������������������������������������������������������������������

C-LANGUAGE FRAMEWORK

A typical model railroad control program with this driver may look like:

#include "mklapi.h"

mklopen(1,2400);                  /* open COM1: */

mklpoll(5);                       /* 5 S88 boxes in yard */

while (key != ESC) {              /* end program from keyboard */

if (kbhit()) {                  /* keyboard input */

if (key = getch()) {          /* normal key (incl alt-numpad) */

switch(key) {

case '?':   - - - - -     /* your key, your function */

break;

case ESC:   break;        /* nothing now, exit program */

default:    fprintf(stdout,"%c",BEL);

break;        /* unsupported normal key */

}

}

else {                        /* 'special' key */

switch(key = getch()) {     /* extended character */

case PgDn:   - - - - -    /* your 'extended' key function */

break;

default:    fprintf(stdout,"%c",BEL);

break;        /* unsupported 'extended' key */

}

}

}

for (i=0; i<MAXTRAIN; i++) {    /* scan train matrix */

- - - - -                     /* train control function */

}

for (i=1; i<5, ++i) {           /* scan S88 boxes */

mklgetsn(i);                  /* read sensor bits */

- - - - - -                   /* process changes */

}

if (!(mkltime()%5000)) {        /* every 5 seconds */

- - - - -                     /* anything else */

}

mklsleep(25);                   /* give up remainder of timeslice */

}

mklflush();                       /* wait until output sent */

mklclose();                       /* terminate */

Note: Above code is just a framework to show the general setup of a model

railroad control program using the M�rklin-Interface.

���������������������������������������������������������������������������

Functional description  7

MKLAPI

���������������������������������������������������������������������������

QUICKBASIC FRAMEWORK

An equivalent framework in QuickBASIC might look like:

' $INCLUDE: 'MRKAPI.INC'

rc = mklopen(1,2400);

mklpoll(5);

key$ = INKEY$

while key$ <> ESC$

select case key$

case '?'

- - - - -

case ESC

- - - - -

case else

print "error key"

end select

for i=0 to MAXTRAIN

- - - - -

next i

for i=1 to 5

mklgetsn(i)

- - - - - -

next i

if MOD(mkltime,5000) < 50 then

- - - - -

end if

mklsleep(25)

key$ = INKEY$

wend

mklflush;

mklclose;

Note: This code may not be true and workable QuickBASIC!

The file MKLAPI.INC contains the DECLARE lines for all functions contained

in LMKLAPI.OBJ.  It should be included in a QuickBASIC program to obtain

the correct calling conventions and activates syntax checking by the

MicroSoft QuickBASIC compiler 4.5.  Subsequently the file LMKLAPI.OBJ

should be linked with your QuickBASIC program to include the MKLAPI

functions and subroutines.

SAMPLE PROGRAMS

After registration you will receive the sources a sample (C-language)

program:

�  Control of 8 trains and several other possibilities, with the following

details:

�  Speed is controlled from the computer keyboard with arrow (cursor)

keys: right-key is faster, left-key is slower, up-key is maximum for

this train, down is maximum backward for this train.

���������������������������������������������������������������������������

Functional description  8

MKLAPI

���������������������������������������������������������������������������

�  The train for which the command is meant is selected with a

number-key (program provides control for trains 1 through 8).  The

corresponding loc-address and other train properties are in a matrix

(with individual entries for each train), such as minimum and maximum

speed in M�rklin Central Control values.  The train addresses that

are programmed are 10 for train 1, 20 for train 2, etc until 80 for

train 8.

�  The speed-control is indirect: you tell the system the program

loc-speed, the program does mass-simulation ('slow' increase and

decrease of speed).  The parameters for enertia simulation are part

of the train definitions.

�  For each train its address, desired and actual train speed are

displayed (in color).  Direction changes are buffered and automatic.

�  The display also indicates direction and whether the extra function

is 'on' or 'off'.

�  Control of 4 M�rklin K83 decoder for 16 railroad switch points.  For

each switch the position ('normal' or 'bound' in color) and its address

is displayed.  The keys are A-P for adresses 253-256, and 1-12

respectively.

�  Polling of M�rklin S88 encoder each with 16 sensors (or isolated

railroad sections).  The position is displayed real time, nothing but

displaying is done with the information.  2 M�rklin S88 encoder-boxes

are being polled.

�  Sensor 16 of M�rklin S88 encoder-box 1 is designated for speed

measurement.  The display shows the time (in milliseconds) between 2

successive 'OFF-ON' transitions.  If there is 1 train running and moving

along the same road, this facility can be used for calibrating speed

calculations for each speed-command-value.

�  Other keys:

Insert    Set Function 'on'

Delete    Set Function 'off'

End       Stop polling

Home      Restart polling

F3        Stop the program (has to be entered twice consecutively)

Esc       Emergency Stop (issues the M�rklin Central Control STOP

command)

Enter     Resume (issues the M�rklin Central Control GO command)

The program may be started with 2 numeric parameters:

1. the first parameter indicates the COM-port to be used (1 for COM1, 2 for

COM2, etc.), The default is COM1.

2. The second parameter indicates the number of M�rklin S88 encoder-boxes

to include in the polling mechanism.  The default is 2.

���������������������������������������������������������������������������

Functional description  9

MKLAPI

���������������������������������������������������������������������������

OS/2 AND DOS DIFFERENCES

The OS/2-drivers differ from the DOS drivers:

�  The OS/2-drivers are written in C-language, the DOS drivers in

Assembler.

�  The OS/2-driver makes use of the services provided by the standard OS/2

communications driver COM02.SYS (for PS/2) or COM01.SYS for others under

OS/2 version 1.3, and COM.SYS for OS/2 version 2.0.  The DOS driver

interfaces directly with the PC hardware.

�  The OS/2-driver makes use of the standard OS/2 facilities for

multithreading (task signalling is done with semaphores).  The DOS

driver performs a form of multi-tasking, driven by interrupts (the

COM-port hardware and the standard DOS timer tick).

�  The DOS-drivers explore the possibilities of the hardware timer of the

PC to obtain a accurate time values when needed.  With the DOS

timer-tick of 55 msec the average deviation would be about 28 msec.

OS/2 does not allow access to the hardware by 'normal' applications,

therefore the OS/2-drivers use standard OS/2 function calls.  Since the

OS/2 timer tick is 31.25 msec, the average deviation will be about 16

msecs.

The sample program under DOS chokes the processor (takes all the cycles

that are available).  This is OK in a typical single-task environment, but

not very suitable for a multitasking system.  Under OS/2 a very simple

solution for this problem is to insert  for example a DosSleep(30L) at the

end of the infinite (keyboard-) loop.  There may be better solutions.

���������������������������������������������������������������������������

Functional description  10

MKLAPI

���������������������������������������������������������������������������

SPECIFICATIONS

This chapter describes the use of the provided facilities.

AVAILABLE FUNCTIONS

The following user callable functions are provided:

mklopen         Start communication with the M�rklin-Interface and the

background tasks.

mklclose        Stop background tasks and shutdown the communication with

the M�rklin-Interface.

mklputc         Transmit 1 or 2 character-commands to the

M�rklin-Interface.

mklflush        Wait until all data transmitted.

mklpurge        Discard remaining data in output buffer.

mklpoll         Start or stop polling of M�rklin S88 encoder boxes.

mklgetsn        Obtain the status of 1 M�rklin S88 encoder (when polling

active).

mklstop         Set M�rklin-Interface in 'stop'-state and stop polling

mklgo           Set M�rklin-Interface in 'go'-state and (optionally) resume

polling.

mklmsecs        Give accurate time in milliseconds since program start

(only in DOS-drivers).

mkltime         Give time of last timer tick in milliseconds since program

start.

| mklstats        Provide statistics (activity counters).

| mklsleep        Pause for a certain time (and give up the remainder of a

|                 time slice when running in a multitasking environment).

| mklsound        Sound generator like BASIC sound or OS/2 DosBeep()

mkltrace        Start or stop recording data that is being sent to

M�rklin-Interface  (only in OS/2-drivers).

The next sections contain the specifications of available functions of the

M�rklin-driver, in alphabetical sequence.

Note: The descriptions for C-language and QuickBASIC are given both, but in

the text mainly the C-language notations are used.

MKLCLOSE

Name            mklclose - close communications with the M�rklin-Interface.

C-language

#include "mklapi.h"

void mklclose(void);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE SUB MKLCLOSE CDECL ()

���������������������������������������������������������������������������

Specifications  11

MKLAPI

���������������������������������������������������������������������������

Description     The close-routine disables the communications task of the

driver.  It should be called before termination your

program.  Transmits and receives that were in progress will

be terinated, data might be lost.

Return value    Nothing.

Related functions "mklopen" on page 15

Warning: mklclose() is not a trivial function!  It must be used when

mklopen() has been called succesfully (under DOS).  Otherwise your system

may enter a hang-up situation when for no matter what reason your

communications port gives an interrupt.  So terminate your program always

'neatly' (so intercept Ctrl-Break in your program to prevent uncontrolled

program termination)!

MKLFLUSH

Name            mklflush - flush all buffered output.

C-language

#include "mklapi.h"

int mklflush(void);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION FLUSH% CDECL ()

Description     Wait until all output is sent to the M�rklin-Interface.  In

other words: wait for empty transmit buffer before

|                 returning to caller.

|                 When buffered data is apparently not transmitted anymore,

|                 the remaining bytes in the transmit buffer will be purged

|                 and the purge statistics are updated accordingly.

| Return value    Depending on success:

|                 �  0   when no more bytes to transmit (buffer empty)

|                 �  >0  number of bytes not transmitted (purged) buffer.

�  -1 (DOS) last byte in hardware buffer was not

transmitted

Related functions "mklpurge" on page 16

���������������������������������������������������������������������������

Specifications  12

MKLAPI

���������������������������������������������������������������������������

MKLGETSN

Name            mklgetsn - get bit-pattern of a single M�rklin S88 encoder.

C-language

#include "mklapi.h"

unsigned short int mklgetsn(short int sensor);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLGETSN% CDECL (BYVAL SENSOR%)

Description     Get a single sensor-box word (16 bits) from the sensor

array.

This function does not generate any action to the

M�rklin-Interface and returns the bitsetting immediately.

Polling will have to be started first, with mklpoll(),

before the reading of sensors will give meaningful results.

Reading outside the range 1..31, or higher than the value

as specified with the latest mklpoll(maxsensor) or

|                 mklgo(maxsensor), will give unpredictable results.

|                 The M�rklin-Interface is always read with 'reset'.  The

|                 resulting 1-bits ('on'-switches) are cumulated until a

|                 mklgetsn() is called for a specific S88.  The caller

|                 obtains the cumulative bit-setting, but the sensor word is

|                 refreshed immediately with the results of the last read

|                 command.  Depending on the type of switche used on the

|                 model railroad, and the frequency in which mklgetsn() is

|                 issued by the mainline, it may be necessary to issue

|                 mklgetsn() twice.  See for a further explanation: "Reading

|                 M�rklin S88 encoders" on page 33.

Return value    A 16-bit word containing the current switch positions.

MKLGO

Name            mklgo - send the 'go' command.

C-language

#include "mklapi.h"

int mklgo(short int maxsensor);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLGO% CDECL (BYVAL MAXSENSOR%)

Description     Send the 'go'-command (ASCII 96) to the M�rklin-Interface

���������������������������������������������������������������������������

Specifications  13

MKLAPI

���������������������������������������������������������������������������

immediately.

The value of MAXSENSOR has the same meaning and effect as

in mklpoll (see "mklpoll" on page 16).  Polling will be

resumed if a non-zero parameter is specified.

OS/2: The go-command is sent via a DevIOCtl operation (send

immediate), and will be followed by a simulated XON.  If

there was buffered output (before the stop-state or after

entering stop-state under OS/2 2.0), it will be transmitted

|                 now.

|                 OS/2: If the 'go'-command could not be sent, for example if

|                 the Interface is not ready to accept data, a permanent low

|                 beep will be heard until the 'go'-button on M�rklin Central

|                 Control is pressed.  If an other error occured, mklgo will

|                 return an error code.

DOS: The go-command is send just as regular data.  The

returncode is always zero.

Return value    Nothing.

Related functions "mklstop" on page 21, "mklpoll" on page 16

MKLMSECS

Name            mklmsecs - obtain time since program start in milliseconds.

C-language

#include "mklapi.h"

unsigned long int void mklmsecs(void);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLMSECS& CDECL ()

Description     (DOS only!) Returns the exact time in milliseconds after

program start without waiting for the next timer tick.

This might be useful for more accurate timing for special

purposes.  Of course it takes some time to execute the

necessary instructions to read the timer and format the

information.  On faster systems mklmsecs() gives the time a

little more precise than on slower systems, but even on

relatively slow systems it is much more precise than

probably needed for the purpose of mode railroad control!

If this very high precision is not needed, then use the

function mkltime().

(OS/2) This function is 100% equivalent to mkltime().

���������������������������������������������������������������������������

Specifications  14

MKLAPI

���������������������������������������������������������������������������

Return value    Time in milliseconds after program start.

Related functions "mkltime" on page 22

MKLOPEN

Name            mklopen - open communications line

C-language

#include "mklapi.h"

|                   int  mklopen(short int port, short int speed);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

|                  DECLARE FUNCTION MKLOPEN CDECL (BYVAL PORT%, BYVAL SPEED%)

Description     Open and initialise communications port driver, high

resolution timer (DOS only), and set all statistics

counters to zero.  The port-parameter may have a value of 1

|                 to 4, indicating COM1: to COM4:.  With DOS the port

|                 addresses are obtained from BIOS, with OS/2 the COM[xx].SYS

|                 device driver has them.  The interrupt levels are IRQ4 for

|                 COM1 and COM3, IRQ4 for COM2 and COM4.  As you see COM3 may

|                 conflict with COM1 (and COM2 with COM4).  You cannot use

|                 conflicting interrupt levels at the same time in most

|                 computers.

Any port will always be initialised with 8-bit, no-parity,

2 stop-bits.  The speed parameter should specify 2400,

since it is the only speed supported by the

M�rklin-Interface.  Maybe later other values?

The counterpart of mklopen() is mklclose(), which should be

called before finishing your (DOS-) program.

| Return value    Depending on success:

|                 �  0     - port opened succesfully

|                 �  32    - port probably occupied by another device

|                 �  110   - port not available

|                 �  other - see OS/2 or DOS documentation

|                 With OS/2 the port is opened by a separate thread.  When

|                 the port could not be opened, the whole process will be

|                 terminated.

Related functions "mklclose" on page 11, "mklstats" on page 19

���������������������������������������������������������������������������

Specifications  15

MKLAPI

���������������������������������������������������������������������������

MKLPOLL

Name            mklpoll - start periodic reading of M�rklin S88 encoder.

C-language

#include "mklapi.h"

void mklpoll(unsigned int maxsensor);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE SUB MKLPOLL CDECL (BYVAL MAXSENSOR%)

Description     M�rklin S88 encoderboxes 1..MAXSENSOR will be read by a

'background' process.

When starting the polling process a 'read with reset' is

issued.  The responses of the read-operation are stored in

sensor-arrays.  Each word of the array can be read

individually at any time with mklgetsn().  See for details

"Reading M�rklin S88 encoders" on page 33.

Return value    Nothing

Related functions "mklgo" on page 13 and "mklgetsn" on page 13

MKLPURGE

Name            mklpurge - purge output buffer

C-language

#include "mklapi.h"

void mklpurge(void);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE SUB MKLPURGE CDECL ()

Description     Discard current contents of output buffer.  If the output

buffer does contain any characters, these are not sent tot

the Interface.  After purging the current contents, control

is immediately returned.  See also "mklflush" on page 12.

Note: This function is currently available in the driver

for DOS.

Return value    Nothing.

Related functions "mklflush" on page 12

���������������������������������������������������������������������������

Specifications  16

MKLAPI

���������������������������������������������������������������������������

MKLPUTC

Name            mklputc - send 1 or 2 characters to the interface

C-language

#include "mklapi.h"

int mklputc(unsigned int word);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLPUTC% CDECL (BYVAL WORD%)

Description     Send 1 or 2 characters to the M�rklin-Interface.  The

'word' is interpreted as two contiguous characters.  The

low order byte of the word is sent first, followed by the

high-order byte.  If the high is not sent when zero.  The

exception to this rule is: if the first byte is a

switch-command (ASCII 33 or 34), then the second byte will

be sent, even when zero.

This function returns immediately, and does not wait for

the bytes to be actually transmitted to the interface.  The

communications task will take care to send the buffered

character(s) to the M�rklin-Interface.

Coding example to signal speed 8 to loc 27:

rc = mklputc( 27*256 + 8)

For C-language programs it is highly recommended to use the

provided macro's for sending 1 or 2 character commands (see

"Macro Specifications" on page 23).

Warning: When there is no room in the output buffer, then

both  characters will be discarded.

Return value     0   character(s) are accepted by the driver.

-1   characters NOT accepted: output discarded.

Note: If you use mklputc() while the driver is in 'stop-state' (see

"mklstop" on page 21):

�  the OS/2-drivers issue a short high-pitch beep.  With OS/2 2.0 the data

might be buffered and transmitted after mklgo().  This depends on the

used device driver for the COM-port.  COM[xx].SYS does not buffer, but

SIO.SYS does (until its 4KB buffer is full).

�  the DOS-drivers do not generate a warning and will discard the data

until the stop-state is left via a call to mklgo().

���������������������������������������������������������������������������

Specifications  17

MKLAPI

���������������������������������������������������������������������������

MKLSLEEP

| Name            mklsleep - pause some time and yield processor

| C-language

|                   #include "mklapi.h"

|                   void mklsleep(short int pause);

| QuickBASIC

|                   ' $INCLUDE: 'MKLAPI.INC'

|                   DECLARE SUB MKLSLEEP CDECL (BYVAL PAUSE%)

| Description     This function provides two facilities:

|                 �  It does not return to the caller before the indicated

|                    pause-interval (im milliseconds) has expired.

|                 �  In a multitasking environment (OS/2, DesqView, Windows)

|                    it releases the processor for this task.

|                 In the OS/2-driver this function is equivalent to

|                 DosSleep().  In the DOS-driver it is implemented as a loop

|                 with an imbedded test for multitasking, in the OS/2-driver

|                 as DosSleep().

|                 The pause should be specified in milliseconds, and thus

|                 cannot exceed about 32 seconds!  The specified

|                 pause-interval is rounded to the next timer tick, but may

|                 be even larger when the multitasker decides not to give

|                 control yet to this task after expiration of the interval!

When running a DOS-program in a multitasking environment

(OS/2, Desqview, Windows, etc.), it is desirable to give

control back to the multitasker, when the program does not

need the processor for some time.  This part of mklsleep()

is experimental, it may not work with all multitaskers.

The use of mklsleep is not absolutely needed with

pre-emptive multitaskers (like OS/2), but is strongly

recommended.  In all cases it reduces CPU utilisation and

leaves processor time for other tasks.

Return value    Nothing.

MKLSOUND

Name            mklsound - give a 'beep'

C-language

#include "mklapi.h"

void mklsound(short int freq, short int time)

QuickBASIC

���������������������������������������������������������������������������

Specifications  18

MKLAPI

���������������������������������������������������������������������������

' $INCLUDE: 'MKLAPI.INC'

DECLARE SUB MKLSOUND CDECL (BYVAL FREQ%, BYVAL TIME%)

Description     Produces a beep-tone with frequency 'freq' expressed in

Hertz, during a period 'time' expressed in milliseconds.

The tone is played in background, all processing continues

without delays or slowdown of the system.

DOS: A beep can be stopped prematurely by calling

mklsound() with a zero frequency, or by mklclose().  A beep

can be changed by mklsound() with a different frequency.

|                 This will stop any previous beep.

|                 OS/2: mklsound is implemented as DosBeep and thus behaves

|                 exactly like it.  It is included in the OS/2 driver only

|                 for source compatibility between OS/2 and DOS programs

|                 using MKLAPI.

Return value    Nothing.

MKLSTATS

Name            mklstats - obtain content of statistics counter.

C-language

#include "mklapi.h"

long int mklstats(int counter);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLSTATS& CDECL (BYVAL COUNTER%)

Description     Obtain value of an internal counter in MKLAPI for

statistical purposes.  All counters will be reset to zero

with mklopen(), and can also be reset by a user program

through specification of 0 (zero) as parameter.

To read a counter specify a number in the range: 1 to 6.

The counters (by number) have the following contents:

1   Number of accepted commands, including polling-commands

(commands to read S88's).  Since the we use a buffering

mechanism, not all of these commands may have been

transmitted actually!

2   Number of internally generated commands to read S88's

('polls').

3   Number of commands that have been discarded.  Reasons

may be:

�  system is in 'stop-state'

�  transmit buffer is full

4   Number of bytes that were transmitted.  With the

DOS-driver these bytes are actually transmitted to the

���������������������������������������������������������������������������

Specifications  19

MKLAPI

���������������������������������������������������������������������������

COM-port, with OS/2 they were accepted by the device

driver of the COM-port.

5   Number of received bytes (most likely as result

commands to read S88's).

6   Number of bytes that were purged (discarded after first

being accepted, but before actually being transmitted

to the M�rklin-Interface).  Reasons may be:

�  there were commands buffered while mklstop() was

called

�  mklflush() could not complete (timeout)

�  commands were in the transmit buffer during a call

to mklpurge()

Discarded commands are not counted as 'purged', since

they were not accepted and never put into the transmit

system!

Note: Purging is not supported in the OS/2-drivers, so

this counter should remain zero with OS/2 programs.

Counters in the range 1 to 6 can be considered as permanent

feature of MKLAPI.  Counters in the range 7 to 15 are

mainly for debugging purposes.  The description below is

for information purposes only, do not use them in your

program, as they may be incorrect, their meaning be changed

in a next version of MKLAPI, or may not be active at all in

the distributed drivers (for performance reasons).

7   DOS: Number of calls to mklputc while the driver's

transmit buffer was empty.

8   DOS: Number of calls to mklputc while the CTS signal is

'high'.

9   DOS: Number of calls to mklputc while the

hardware communications buffer was empty.

10  DOS: Number of calls to mklputc whereby the first or

only byte had to be put in the driver's communications

buffer.

11  Number of calls to mklputc with a request for 2 bytes

(second character always buffered).

12  DOS: Total number of interrupts processed by the

interrupthandler for the asynchronous communications

port.

13  DOS: Number of THRE interrupts.

14  DOS: Number of MSR interrupts (number of CTS signal

changes).

15  DOS: Situations with a pending interrupt after a

previous interrupt was processed.

Specification of a counter number outside the indicated

range will give unpredictable results.

Return value    Contents of the specified counter (zero after reset).

Related functions "mklopen" on page 15

���������������������������������������������������������������������������

Specifications  20

MKLAPI

���������������������������������������������������������������������������

MKLSTOP

Name            mklstop - send the 'stop' command.

C-language

#include "mklapi.h"

int  mklstop(void);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLSTOP% CDECL ()

Description     Send the 'stop'-command (ASCII 97) to the M�rklin-Interface

immediately.  If polling was active at the time, it will be

|                 stopped.

|                 �  OS/2:  A simulated XOFF command is used to indicate the

|                    COM-driver to hold yet buffered data.  The stop-command

|                    is then sent via a 'send-immediate' operation, ahead of

|                    data that might be in the transmit buffer.  The transmit

|                    buffer is NOT purged, held data is preserved and sent

|                    after a 'go'-command.

|                    You may hear a low beep while the mklstop operation is

|                    waiting to get access to the M�rklin-Interface.  If an

|                    other error occured, mklstop will return an error code.

|                 �  DOS: The transmit buffer is purged and the stop-command

|                    is then sent as regular data.  New output is not

|                    accepted anymore (it will be discarded).

Warning: The 'stop' command could be sent by the mainline

directly,  but then would bypass the administrative

functions of the driver.  Any train or switchpoint control

command received by the M�rklin-Interface will cause a

permanent drop of the CTS signal, making the control over

the railroad from the computer permanently

unavailable.  The only possibility to resume control then

is by manually hitting the 'go' key on the M�rklin Central

Control.

Note: The timer does not stop with a mklstop().  This may

cause strange effects from time-dependent calculations in

your mainline!

The 'stop' command might interfere with a 2-byte command of

which the first byte was already transmitted, but the

second byte not!  In that case the 'stop' would not be

recognised by the M�rklin-Interface.  Two consecutive

'stop'-commands will prevent the acceptation by the

M�rklin-Interface of any further commands from the

computer.  To make sure that the 'stop' command will be

processed by the M�rklin-Interface, with with the least

���������������������������������������������������������������������������

Specifications  21

MKLAPI

���������������������������������������������������������������������������

inconvenience, MKLAPI precedes the 'stop'-command with a

'go'-command:

�  If there was no command in transmission, then it won't

harm.

�  If a 'broken' 2-byte command was in progress, then the

'go'-byte will be interpreted by the M�rklin-Interface

as the second byte.  This will probably have a different

effect than was foreseen, but since a 'stop' will

follow, this will be of minor importance.

A secundary effect of this setup is that it allows the

mainline to call mklstop() more than once.  The

intermediate 'go' ensures that the M�rklin-Interface

doesn't block permanently.

Return value    Error code

Related functions "mklgo" on page 13

MKLTIME

Name            mkltime - obtain time since program start in milliseconds.

C-language

#include "mklapi.h"

unsigned long int mkltime(void);

QuickBASIC

' $INCLUDE: 'MKLAPI.INC'

DECLARE FUNCTION MKLTIME& CDECL ()

Description     Returns the time in milliseconds after program start since

it was updated with the last timer 'tick' (OS/2: 31.25

msecs, DOS: 55 msecs).

This is a little less accurate than mklmsecs, but consumes

also less processor cycles.

Return value    Time in milliseconds after program start.

Related functions "mklmsecs" on page 14

MKLTRACE

Name            mkltrace - start or stop trace

C-language

#include "mklapi.h"

���������������������������������������������������������������������������

Specifications  22

MKLAPI

���������������������������������������������������������������������������

void mkltrace(short int signal);

QuickBASIC      trace facility not provided in DOS drivers

Description     Start or stop tracing of all data that is being sent to the

M�rklin-Interface and record the information, including a

timestamp into the file MKLAPI.TRC.

When signal is NOT zero, then tracing will be started, or

restarted (old trace data will be discarded in that case).

When signal is zero the trace is stopped and the trace file

closed.

It is my intention to replace this 'on/off' switch with a

trace level facility (selection of events or data to be

traced).

Note: Since a buffered communications interface is used,

the recording of trace data takes place at the moment the

data is sent to the buffer.  So the timestamp does not

reflect the exact transmission time.  Especially at moments

when M�rklin-Interface does not accept data (for example

when it is transmitting M�rklin S88 encoder-data), there

may be a significant time difference!

The utility MKLTRFMT.EXE can be used to produce a formatted

trace report.  All commands are printed hexadecimal and

interpreted.

See for details about the trace facility and a sample

output: "Appendix B.  Trace facility" on page 40.

Return value    Nothing.

Note: Tracing is not provided by the DOS-drivers, but a dummy function is

provided for combined OS/2 and OS/2 programs.

MACRO SPECIFICATIONS

A number of C-language only macro's is provided to make specification of

some function-calls a little more simple, and to take care of possibly

needed data conversion.

MKLPUT1C

Name            mklput1c - send a single character to the interface.

Usage           #include "mklapi.h"

void mklput1c(char);

Prototype in    mklapi.h

���������������������������������������������������������������������������

Specifications  23

MKLAPI

���������������������������������������������������������������������������

Description     Macro to send a single character in the output buffer.  The

macro will expand into a mklputc().

Return value    See "mklputc" on page 17.

MKLPUT2C

Name            mklput2c - send two characters to the interface.

Usage           #include "mklapi.h"

void mklput2c(char1,char2);

Prototype in    mklapi.h

Description     Macro to send two consecutive character in the output

buffer.  The characters will guaranteed be sent in the

given order, without the risk of intervening characters by

the polling mechanism.  The macro will expand into a

mklputc().

Return value    See "mklputc" on page 17.

USE OF TIMER

At program start with mklopen(), the system time is taken as base for the

functions mkltime and mklmsecs.  These functions return a value in

milliseconds after mklopen().

The OS/2-driver uses standard OS/2 facilities, which will allow a precision

of about 16 msecs (the time is updated in OS/2 control blocks every 31.25

msecs).

The DOS-driver uses the system hardware to obtain the system time.  This

allows a precision of 1 msec (with the function mklmsecs()).  The function

mkltime() gives a precision of about 28 msecs, and takes less machine

instructions, which might be sufficient for most purposes.

The use of this timer facility provided by the MKLAPI-drivers has the

following advantages:

�  It is independent of system (processor) speed and other tasks running in

your system.  Many sample programs use some kind of processing loop to

obtain a 'wait'-period, which might be dependent on processor speed and

'choke' the processor, during which time no other useful work is

possible, or at least it is pretty complicated.

�  It is available with very little processor cycles, so it is not needed

to call a 'universal' compiler-provided timer routine, which are

generally much more cycle consuming.

���������������������������������������������������������������������������

Specifications  24

MKLAPI

���������������������������������������������������������������������������

The timer can be used for example for:

�  The application of an event-queue (for timed operations).

�  'Wait'-intervals during without programming a loop.

�  Simulation of train mass (inertia) through gradually increasing or

decreasing loc-speed.

If you register this package, you'll receive a sample program which makes

use of this feature.

It is recommended to use mklsleep() in cyclic operations when running in a

multitasking environment sucha as Windows, DesqView or in a DOS-session

under OS/2.

QUICKBASIC FUNCTION SPECIFICATIONS

The functionality of the M�rklin-driver for QuickBASIC is the same as for

the drivers for C-language.  However the following should be noted:

�  BASIC does not allow the underscore-sign ("_") in names, as is very

common in C-language.  Therefore the FUNCTIONS and DATA fields in the

M�rklin-driver for QuickBASIC have been adapted a little to BASIC

conventions since in 0.9 of these drivers.  All other differences are

'under the cover' (but see next item).

�  C-language uses by default a different way of parameter passing than

BASIC, PASCAL and some other languages.  But MicroSoft QuickBASIC

compiler 4.5 allows the C-language convention to be used, and the

M�rklin-driver for QuickBASIC expects this convention to be used.  This

is reflected in the function specifications in MKLAPI.INC.  If you use

the MKLAPI.INC file (strongly recommended!), then you won't have to

worry about these conventions.

ERRORS AND SIGNALS

The driver is designed not to disturb your screen with error-messages.

| Therefore audiable signals are used to indicate unexpected problems.

| The OS/2 and DOS-drivers produce the same warning-signals:

| very high pitch, short 3000 Hz, 20 msec: M�rklin-Interface  The driver is

|             in stop-state, but some routine tries to send commands, which

|             are discarded.

| medium high pitch, long 2000 Hz, 300 msec: M�rklin-Interface seems to be

|             long-term blocked.  The transmit task could not deliver its

|             data.  Most likely the transmit buffer is full.  Check the

|             'stop'-light on the M�rklin Central Control if a stop-condition

|             is the cause (manually set, or electrical problem).

| high pitch, short 1000 Hz, 20 msec: A command-transmission did not finish

|             within the normal time.  Too many commands may be queued, maybe

���������������������������������������������������������������������������

Specifications  25

MKLAPI

���������������������������������������������������������������������������

|             because of a program loop.

| medium pitch, short 440 Hz, 100 msec: Flushing of the buffer could not take

|             place in the expected time.  Either transmission of buffered

|             data doesn't take place, or the buffer is being filled during

|             flushing faster than can be transmitted.

| low pitch, short 200 Hz, 100 msec: a communications error occurred (OS/2).

|             Debugging information might be sent to STDERR (which you might

|             have redirected to a log-file to prevent disturbance of your

|             screen).

���������������������������������������������������������������������������

Specifications  26

MKLAPI

���������������������������������������������������������������������������

INSTALLATION

APPLICATION DEVELOPMENT CONSIDERATIONS

This MKLAPI is a program development tool with a C-language or QuickBASIC

compiler.  You need MKLAPI.H (MKLAPI.INC for BASIC) at compile-time and one

of the .OBJ files at LINK-time.  It is an addition to the compiler and

possibly other toolkits you may have for other purposes.

C-COMPILER CONSIDERATIONS

The following compiler properties apply.

�  The file MKLAPI.H must be #included in the program that references the

driver routines (you can use the same header-file for OS/2 and DOS).

�  Memory model: OS/2: LARGE; DOS: SMALL or LARGE (your choice)

�  Calling conventions: C

�  OS/2:  The '/MT' parameter should be specified with MicroSoft C-compiler

6.00a  With IBM C SET/2 compiler '/Gm' should be specified among other

parameters.

�  Generate underbars: On (Turbo-C option).

Other compiler options are (probably) not of influence on the use of the

M�rklin-driver.

MICROSOFT QUICKBASIC COMPILER 4.5 CONSIDERATIONS

The QuickBASIC compiler is needed to create an object deck of your BASIC

program.  This OBJect file is combined with LMKLAPI.OBJ file as provided by

this MKLAPI package to build an EXE-file with the LINK utility.

�  Calling conventions: C (DECLARE SUB|FUNCTION xxx CDECL ... etc)

BUILDING AN EXE-FILE

One of the OBJect files should be included in the link-step with your own

OBJect file(s).  I prefer to Compile and LINK with the help of the (N)MAKE

utility.

���������������������������������������������������������������������������

Installation  27

MKLAPI

���������������������������������������������������������������������������

C-LANGUAGE AND DOS

For the DOS sample application ("TRAIN") I use the following MAKE-file:

�������������������������������������������������������������������������Ŀ

�                                                                         �

� #                                                                       �

� # MAKE TRAIN: Marklin Digital Train Control program (DOS version)       �

� #             - both SMARKLIN.OBJ and LMARKLIN.OBJ generated            �

� #             - LARGE memory model version used for TRAIN.EXE           �

� #             - With the compiler variable __DOS__ specified, certain   �

� #               #include-file selections are made in train2.c. When     �

� #               __DOS__ not specified the OS/2-inlcudes are selected.   �

� #                                                                       �

�                                                                         �

� MDL=L                                                                   �

� CL=-c -Zl -Zp -G0 -W3 -A$(MDL) -D__DOS__ -Fotrain.obj -Ot               �

� LK=/NOD /PACKC:65500 /ST:4000                                           �

�                                                                         �

� train.exe: train.obj smklapi.obj lmklapi.obj                            �

�   LINK  train+$(MDL)mklapi, train $(LK), nul, $(MDL)LIBCER;             �

�                                                                         �

� train.obj: train2.c train.h train.mak attrib.h attrib2.h common.h\      �

�            matrix.h mklapi.h                                            �

�   CL $(CL) train2.c                                                     �

�                                                                         �

� smklapi.obj: mklapi.asm mklapi.h train.mak                              �

�   MASM -Dsmall mklapi.asm,smklapi;                                      �

�                                                                         �

� lmklapi.obj: mklapi.asm mklapi.h train.mak                              �

�   MASM -Dlarge mklapi.asm,lmklapi;                                      �

�                                                                         �

�                                                                         �

���������������������������������������������������������������������������

Note: It contains switches to control the use of large or small memory

models.

C-LANGUAGE AND BASIC

For a simple test-program in QuickBASIC I use the following batch-file:

�������������������������������������������������������������������������Ŀ

�                                                                         �

� @echo off                                                               �

� rem                                                                     �

� rem Build Marklin Digital Test program for DOS QuickBASIC               �

� rem Note: LMKLAPI.OBJ fetched from \C2\MARKLIN subdirectory             �

� rem                                                                     �

� BC    test /S;                                                          �

� if errorlevel 0  LINK  test+\c2\marklin\lmklapi;                        �

�                                                                         �

���������������������������������������������������������������������������

���������������������������������������������������������������������������

Installation  28

MKLAPI

���������������������������������������������������������������������������

Note: With MicroSoft QuickBASIC compiler 4.5 the LMKLAPI.OBJ need to be

included in the link.  models.

C-LANGUAGE AND OS2 1.3

For the 16-bits OS/2-sample application ("TRAIN2") I use the following

MAKE-file for MicroSoft C-compiler 6.00a and IBM OS/2 Program Development

Toolkit 1.2/1.3:

�������������������������������������������������������������������������Ŀ

�                                                                         �

� #                                                                       �

� # MAKE TREIN: Marklin Digital Train Control                             �

� #                                                                       �

� # To be used with MicroSoft C-compiler 6.00a                            �

� #                                                                       �

� # With IBM C/2 1.1 compiler specify '__IBMC2__' compiler option         �

� #                                                                       �

� # 'OS2' compiler variable needed for OS/2 object of TRAIN2!             �

� #                                                                       �

�                                                                         �

� CL=-c -Zp -Zl -W3 -MT                                                   �

� LK=/A:16 /PACKC:65000 /NOI /NOD                                         �

� L1=LLIBCMT OS2                                                          �

�                                                                         �

� train2.exe: train2.obj mklapi2.obj train2.def                           �

�    LINK train2+mklapi2, $@ $(LK), NUL, $(L1), train2.def                �

�                                                                         �

� train2.obj: train2.c train2.h attrib2.h mklapi.h common.h matrix.h      �

�    CL $(CL) train2.c                                                    �

�                                                                         �

� mklapi2.obj: mklapi2.c                                                  �

�    CL $(CL) mklapi2.c                                                   �

�                                                                         �

�                                                                         �

���������������������������������������������������������������������������

The contents of the corresponding LINK .DEF-file are:

�������������������������������������������������������������������������Ŀ

�                                                                         �

� ;                                                                       �

� ;******* Marklin Train Control Program  Definition File (.DEF) ******** �

� ;        For: 1)  16-bit version for OS/2 1.3 and MS C-compiler         �

� ;             2)  32-bit version for OS/2 2.0 and IBM C Set/2 compiler  �

�                                                                         �

� NAME    TRAIN2  WINDOWCOMPAT                                            �

�                                                                         �

� DESCRIPTION 'Marklin Train Control. Copyright (1991,93) Rob Hamerling'  �

� PROTMODE                                                                �

� HEAPSIZE   4096                                                         �

� STACKSIZE  8192                                                         �

�                                                                         �

���������������������������������������������������������������������������

���������������������������������������������������������������������������

Installation  29

MKLAPI

���������������������������������������������������������������������������

C-LANGUAGE AND OS2 2.0+

For the 32-bits OS/2-sample application ("TRAIN3"), the following MAKE-file

is appropriate for IBM C SET/2 compiler and IBM OS/2 Program Development

Toolkit 2.0:

�������������������������������������������������������������������������Ŀ

�                                                                         �

� #                                                                       �

� # MAKE TREIN: Marklin Digital Train Control program                     �

� #                                                                       �

�                                                                         �

� CL=/c /G3 /Gd+ /Gm+ /Re /Se /Sm /Ss                                     �

� LK=/NOI /PACKC:65500 /ALIGN:4 /BASE:0x10000 /EXEPACK                    �

�                                                                         �

� train3.exe: train3.obj mklapi3.obj                                      �

�    link386 $**, $@ $(LK), NUL, , train2.def                             �

�                                                                         �

� train3.obj: train2.c                                                    �

�    icc $(CL) /Fotrain3.obj train2.c                                     �

�                                                                         �

� mklapi3.obj: mklapi2.c mklapi.h                                         �

�    icc $(CL) /Fomklapi3.obj mklapi2.c                                   �

�                                                                         �

�                                                                         �

���������������������������������������������������������������������������

Note: The same .DEF-file is used as for the 16-bit version.

STORAGE AND PERFORMANCE

There is yet not much to say about storage and performance of the driver.

It is highly optimised code, and probably only a minor part of the 'real'

railroad control program.

MEMORY UTILISATION

It is difficult to say how many storage is occupied by the OS/2-version of

the M�rklin-driver.

The DOS-version requires about 4K bytes of storage.

PROCESSOR UTILISATION

You might be curious about the cycles that will be consumed by the polling

task.  I've done some measurements under OS/2 version 1.3 with the sample

program TRAIN2.  On an 8 Mhz AT (80286) the polling takes between 10% and

12% of the processor time (difference in processor utilisation between

polling ON and OFF).  This number is practically independent of the number

of M�rklin S88 encoder's that are specified for polling, but the polling

frequency is also lower!

���������������������������������������������������������������������������

Installation  30

MKLAPI

���������������������������������������������������������������������������

COMMENTS, REMARKS, BUG-REPORTS

If there are bugs, requirements or questions about this M�rklin-driver, the

documentation or the sample program (that you get when you register),

please send me a message through the FIDO network.  You can find my name

and node-(point)-number in the introduction of this document.

Enjoy and good luck with your program development!  May this driver lead to

a very attractive and succesful model railroad control program!

Do not 'forget' to register!

���������������������������������������������������������������������������

Installation  31

MKLAPI

���������������������������������������������������������������������������

APPENDIX A.  MKLAPI INTERNALS

This appendix describes a number of design arguments for this driver and

explains decisions where a choice had to be made.

MULTITASKING

There are many situations in real-time programming, where you want the

system to do something for you, while in the meantime you would like to do

something else in the mainline (before the first piece of program

completes).

An example of this is reading sensor boxes.  While you have to wait for the

response of the M�rklin S88 encoder, you might want to update your screen,

read data from disk, or process keyboard input.  In many sample programs

that I have seen, the wait is frequently done in the form of a short loop

(a for- or while-statement).  This means that the processor is working, in

fact consuming cycles, while you cannot do anything (unless you make a more

'complicated' loop).  And in many cases you have to experiment with this

loop to get the needed wait-interval (not too long and not too short).

Running the same program on a machine with a different processor-speed,

requires a change in the loop.

A solution for this is to have multiple programs to run simultaneously.

Executing multiple programs (tasks) in parallel is called multi-tasking.

Since we are generally dealing with a single micro processor, the tasks are

simultaneaously present in memory, but only 1 of them is actually

processing.  There must be some mechanism to switch the processor between

the tasks.

Some operating environments have standard facilities for this (OS/2,

Windows), but DOS doesn't.  Under DOS you have to do it yourself.  By

intercepting hardware interupts, or the timer-tick interrupt it is possible

to implement a kind of task-switching.

The polling mechanism in the M�rklin-driver is a form of multi-tasking.

For OS/2 it uses the standard OS/2 facilities (called multi-threading), for

DOS it uses the system timer to execute periodical actions independently

from the main-program, and also the hardware interrupts of the asynchronous

communications port to perform some communications work.  So in fact there

are two 'extra' tasks in the polling mechanism:

�  Periodically send read-commands to the M�rklin-Interface.

�  Receive the responses and put them in the appropriate place in the

sensor-array.

Although you don't have to bother about the polling mechanism, it does have

some influence on the design of your railroad control program.  These

practical aspects are explained below.

���������������������������������������������������������������������������

Appendix A.  MKLAPI Internals  32

MKLAPI

���������������������������������������������������������������������������

READING M�RKLIN S88 ENCODERS

The driver generates read-sensor commands in the following way:

1. After polling is activated periodically a sensor-read command is

transmitted to read multiple M�rklin S88 encoders.

2. The receive-part of the communications task is signalled that a response

from the M�rklin-Interface should be expected.  The receive continues as

long as not all expected input is received, or an unexpected long time

had to be waited.

3. After processing the response, the system waits some time before issuing

a new read command (typically 100 msec).

4. Even if for some reason the previous poll did not finish within

reasonable time (typically 500 msec), a new read-command is issued.

5. If the transmit routine is in use by the mainline program (such as

sending train-commands), polling is suspended for a short while (shorter

than the normal polling interval).

The M�rklin-Interface documentation indicates that the average time to read

the last poll-response is between 45 and 300 milliseconds, depending on how

many M�rklin S88 encoder's are being read.  From 'life'-tests I have

concluded:

�  The total polling cycle (the time between two consecutive read commands)

is so short that the delay between the change of a switch and making the

change represented in the sensor-array is short enough for most

practical situation.

�  The poll wait interval is long enough to allow sending of many commands

from the mainline between read commands.  To make this sure there is a

built-in priority mechanism in the polling routine: it waits with

generating a poll when another program uses mklputc() routine!

�  The frequency of the polls is low enough to ensure that not all CPU

cycles are spent on polling, in stead of 'real' work.

Polling is programmed as follows:

1. After calling mklpoll() of mklgo() with a non-zero parametervalue a

'192'-byte is transmitted, meaning that the Interface will RESET the

bits with next read-sensor commands.

2. Periodically a '128 + MAXSENSOR' byte, meaning that the Interface will

report a group of sensors, up till the limit that was specified with the

latest mklpoll() or mklgo().

| The received data is collected in 2 sensor-arrays:

| 1. Collection of 'raw' data from the M�rklin-Interface.  Array-1 always

|    represents the current switch positions of all polled S88's, apart from

|    a slight delay between consecutive reads.

���������������������������������������������������������������������������

Appendix A.  MKLAPI Internals  33

MKLAPI

���������������������������������������������������������������������������

|                    �������������������ͻ

|                    �                   �

|                    � Marklin Interface �

|                    �                   �

|                    ��������#��#�������ͼ

|                            �  �

|                            �  �

|     ����������������������ͺ�ͺ������������������������������������ͻ

|     �    READ S88 1...6 ��ͼ  �                         Computer    �

|     �  (by polling task)      �                                     �

|     �                         �                                     �

|     �                         �                                     �

|     �          �����������������������������ͻ                      �

|     �  'move'  �     �     �     �     �     �                      �

|     �          �     �     �     �     �     �                      �

|     �       ���#�����#�����#�����#�����#�����#����� �               �

|     �       �  1  �  2  �  3  �  4  �  5  �  6  �        array-1    �

|     �       ��������������������������������������� �               �

|     �                       �                                       �

|     �   'or'   �     �     ��(2) �     �     �                      �

|     �                       �                                       �

|     �       ���#�����#�����##����#�����#�����#����� �               �

|     �       �  1  �  2  �  3  �  4  �  5  �  6  �        array-2    �

|     �       ��������������������������������������� �               �

|     �                      �                                        �

|     �                      �(1)   actions by: mklgetsn(3)           �

|     �                      �     1. return word 3 of array-2        �

|     �                      #     2. copy word 3 array-1 to array-2  �

|     �                                                               �

|     ���������������������������������������������������������������ͼ

| Figure 1. Organisation of reading S88's (polling): In this sample a

|           group-read of 6 S88-boxes is used.

| 2. Cumulative 'on'-bits since most recent mklgetsn() for each specific S88

|    in Array-2.

| The incoming data of the 'read group' is stored in array-1, and all 1-bits

| are copied immediately to array 2 with an 'or'-operation.  So array-2 is a

| cumulation of 'on' switch positions during a certain period.

| When mklgetsn(x) is executed, the contents of word-x in array-2 are

| returned to the caller.  At the same time word-x in array-2 is refreshed

| with the then current contents of array-1.  So a second mklgetsn()

| immediately after the first gives the actual status ((3)).

| A second call is required if you want to know for sure if the switch is

| currently in the 'off'-position after it has been 'on' since the most

| recent mklgetsn().

��������������������

(3)     Function mklgetsn might be changed later to be able to specify that

you need data from the first or from the second array.

���������������������������������������������������������������������������

Appendix A.  MKLAPI Internals  34

MKLAPI

���������������������������������������������������������������������������

| This mechanism is suitable for all types of switches that are applied in

| model-railroads, and ensures that you won't miss any 'switch-on' condition,

| even if it is 'on' during a very short period.  This organisation is

| illustrated in Reading M�rklin S88 encoders  on page 33.

You have to be careful with specifying more M�rklin S88 encoder boxes in

mklpoll() or mklgo() than you really have in your model railroad yard.

�  It is not useful, since the M�rklin-Interface will return all zeroes for

not-connected M�rklin S88 encoder's.

�  During the time the M�rklin-Interface sends the responses to the

computer, no commands are accepted by the Interface.

So specifying more M�rklin S88 encoders than really active will unnecessary

downgrade the responsiveness of your program.  You might decide to stop

| polling during processor-intensive work, or not to use polling at all.

| Note: The following consequences of this mechanism:

| �  If you use this driver, you should not issue read-commands from the

|    mainline.  The driver does not intercept read-commands from the

|    mainline, but it will intercept all data from the Interface and not

|    return it to the mainline!  This will disturb the administration of the

|    internal sensor numbering, which is depending entirely on the number of

|    incoming bytes.

| �  This driver does not yet support physically reading of individual

|    M�rklin S88 encoders, and it is not possible to use polling with

|    no-reset (command 128 in stead of 192).  This may look like a

|    limitation, especially with the use of reed-switches or contact-rails.

|    But once you are familiar with the convenience of the polling mechanism

|    and the functionality of mklgetsn(), this should not be a practical

|    limitation.

COMMUNICATIONS INTERFACE SPECIFICATION

The M�rklin-Interface contains a device driver level communication facility

with between computer and M�rklin-Interface, but with a rather high level

interface for the application programmer.  It contains many basic

functions, that otherwise have to be programmed yourself.  However, it

enforces a specific organisation of your program and has some limitations.

Some of the internals of the communications task are briefly explained to

show why it works as it works now, and may prevent questions.  On the other

hand, it may trigger suggestions for changes!  Feel free to go in

discussion with me!

Wiring          Only the Read (RD), write (TD) and Clear-to-Send (CTS) lead

are connected.  Thus using DTR and RTS from the computer

will have no effect of any kind and obviously DSR and DCD

(sometimes also called CD or RLSD) will not be used by the

M�rklin-Interface  Nevertheless DTR and RTS are set by the

driver with mklopen().  This allows for the use of a

break-out box and/or another computer in stead of the

���������������������������������������������������������������������������

Appendix A.  MKLAPI Internals  35

MKLAPI

���������������������������������������������������������������������������

M�rklin-Interface for debugging of your program (since a PC

commmunications program usually needs these leads to be

active).  The driver itself works properly without the DSR

and DCD leads.

Speed           The M�rklin-Interface allows only 2400 bps, and 8-bits

data, NO parity, and 2 stop bits.  These are the built-in

defaults of the driver, however a different speed can be

specified at open-time (might be useful for test and

debugging).

FIFO            So called FIFO hardware buffering, provided by the newer

UART-chips (such as 16550 AFN) on asynchronous

communication ports is handled as follows by the drivers:

�  The DOS-driver does not actively support FIFO-mode, but

FIFO-support is in plan.

�  The OS/2-MKLAPI uses the facilities of the device driver

COM[xx].SYS for operation of the COM-port.  When FIFO is

is active for the COM-port before opening, it is left

active and uses this feature.  Use the MODE-command to

control this feature.

FIFO-mode may not provide a considerable improvement:

�  The M�rklin-Interface operates strictly half-duplex, it

is either in receive mode, or in transmit mode.  During

data transfer (either way) CTS is dropped while

transmitting each command-byte and may remain low for a

'considerable' time while command interpretation is in

progress and S88-positions are being sent back to the

computer.  This does not suit very well with the FIFO

for transmitbuffering.  It might be useful for receive,

since in many cases at least 2 bytes are received per

S88. being read.  The support for this is in

consideration.

�  Since the speed is 2400 bps, most systems can easily

keep up with the non-buffered mode of operation.  The

highly effective interrupt handler of the DOS-driver may

make the netto effect of reduced CPU-cycles for

interrupt handling only minimal.

But FIFO-mode may be helpful in a multitasking environment.

Use the MODE command with BUFFER=ON, to activate it, when

desired.

Output flow control The CTS signal is dropped by M�rklin-Interface during

data transfer (during receive and transmit: the

M�rklin-Interface is strictly half-duplex).  The CTS signal

is used for 'hardware output flow control' from the

computer.  So the transmit routines will not send data when

CTS is not high, but will automatically resume transmission

when CTS rises and the buffer contains any data to

transmit.

DOS Although not strictly needed, the DOS-version of the

M�rklin-driver has the transmit interrupt (THRE) active to

���������������������������������������������������������������������������

Appendix A.  MKLAPI Internals  36

MKLAPI

���������������������������������������������������������������������������

be able to run the driver against a full-duplex interface

(with continuous CTS), such as another PC with a diagnostic

communications program.  The transmit interrupt uses the

same code as the MSR interrupt.

For initial testing of programs without actually running a

model railroad, you could use a 'wrap-plug' to partially

simulate the M�rklin-Interface.  The RTS signal (raised by

the driver) is feeded back to CTS, so the driver is always

able transmit.

If also send and receive (generally pins 2 and 3) leads are

wrapped back, then the transmitted data is treated as S88

response.  This may cause

Input flow control There is no form of input flow control allowed by the

M�rklin-Interface: there is no way to signal the

M�rklin-Interface to stop sending.  This will not be a

problem for the driver, since there is enough buffering and

CPU-power to process the low volume input in time.

XON/XOFF flow control Not used.  This type of flow-control is not supported

by the M�rklin-Interface, since the commands for the

M�rklin-Interface may contain these characters!

OS/2: The standard COM[xx].SYS of OS/2 allows simulated

XON/XOFF.  A DosDevIOCtl-'XOFF' is issued to stop

transmitting buffered data when a 'stop'-command has to be

transmitted.  A DosDevIOCtl-'XON' is issued when a

subsequent 'go'-command is issued.

Null stripping  The OS/2 COM[xx].SYS allows null stripping.  There are at

least 2 reasons for this feature not to be activated:

�  M�rklin S88 encoder's may respond with all bits zero

�  Switch 256 is addressed as ASCII 0.

Command parsing The driver does not interpret the data it sends to the

M�rklin-Interface.  It means that it is completely the

responsibility of the mainline to ensure proper command

sequences.

Note: The driver does not send automatically a

switch-'relax' command (ASCII 32) after a switch command

(ASCII 33 or 34)!  You may write a shell for M�rklin

command syntax checking.

PROBLEMS WITH OS/2 COM[XX].SYS

The Com-port driver of some versions of OS/2 for non-PS/2 machines

(COM01.SYS) does have some known problems, such as:

�  READ and WRITE timout specifications cannot be changed from the built-in

default of 1 minute.

���������������������������������������������������������������������������

Appendix A.  MKLAPI Internals  37

MKLAPI

���������������������������������������������������������������������������

When encountering this kind of problems, I could recommend Gerry Rozema's

replacement driver for AT-type machines (COM16540.SYS, or COM16550.SYS, as

appropriate for your system's COM-port).  For OS/2 2.0 there is also a

driver SIO.SYS by Ray Gwinn, that might be useful when having problems with

the standard OS/2 2.0 drivers.

These alternative drivers are available on Bulletin Boards such as

PC-Square in The Netherlands (FIDO node 2:512/4, telephone 31.79.424107).

OS/2 AND DOS DIFFERENCES

Although all versions of this M�rklin-driver offer a consistent application

programming interface for communication with the M�rklin-Interface, it is

most likely that there will be differences between the programs using

either of them.  Some of these differences might be:

�  When running in a multitasking environment, the program should give

other tasks the chance to run (yield the processor).

�  With OS/2 other tasks will get some control automatically

(pre-emptive multi-tasking), but nevertheless the program should at

least use DosSleep(0) when it does not need the processor for some

time.

�  DOS: apart from the built-in multitasking of the M�rklin-driver which

controls itself, you probably won't run other programs.  In that case

you will not have to release the processor.

�  When running a DOS-program under another multitasker (Windows,

Desqview), periodic calls of mklsleep() are recommended.

�  The OS/2 driver makes use of the standard multitasking (multithreading)

facilities.  Therefore any program using this driver should specify /MT

for MicroSoft C-compiler 6.00a.  The different include-search might be

indicated with a compiler option (-I for most C-compilers).

The DOS driver should use the normal include directory.

�  The OS/2 driver requires the LARGE memory model compiler options and use

the LLIBCMT library for LINK (automatic with /MT).  The DOS-version is

for either the SMALL (/AS) or LARGE memory model (/AL), use the

respectively the SLIBCE or LLIBCE library with LINK.

�  The standard OS/2-driver (COM[xx].SYS) will not accept new output data

in stop-state, but will transmit possibly retained data (from before

mklstop) after a call to 'mklgo'.  Some alternative drivers (such as

SIO.COM) do accept new data from mklputc, but keep it internally

buffered.  No more data is accepted when the buffer is full.  After

mklgo() all buffered data, old and new, will be transmitted.

The DOS-drivers refuse all new output data in stop-state.

�  It is an absolute must, that the M�rklin-driver under DOS is terminated

with a call to mklclose() to de-activate the interrupts for com-port and

timer.  Therefore it is recommended to use a routine to intercept Ctrl-C

or Ctrl-Break actions from the keyboard.

���������������������������������������������������������������������������

Appendix A.  MKLAPI Internals  38

MKLAPI

���������������������������������������������������������������������������

Under OS/2 this attitude is recommended, but not an absolute must.

In both cases it is recommended to direct the Ctrl-C signal to a

controlled finish of your railroad control program: you probably do not

want to terminate your program with uncontrolled running trains!

�  The display of information about the state of your railroad on the

computer screen will probably use different techniques, such as the way

cursor and color will be used:

1. OS/2: Presentation Manager (graphics) or VIO routines.

2. DOS: Windows (graphics)

3. ANSI escape-sequences (compatible between OS/2 and DOS)

The sample program you'll receive after registration is a single source

for environments 1 and 3.  My own programs are primarily for OS/2 and I

prefer to use the OS/2 facilities and those of the Program Development

Toolkit as good as possible.  For instance I use VIO-routines for color

and cursor manipulation.

To be able to run my sample program under DOS, I included some extra

routines in the header-file.  These routines translate color and cursor

manipulations with OS/2 VIO-calls into equivalent ANSI escape sequences.

When ANSI is active under DOS (with DEVICE=[path]\ANSI.SYS in

CONFIG.SYS), the visual effects are the same as with the OS/2 version.

When there is no possibility to use the same code for OS/2 and DOS, you

might consider using a compiler variable (with the -D compiler option) to

distinguish which pieces of codes are OS/2 or DOS specific.  See for an

example the MAKE-files in the MKLAPI package.

���������������������������������������������������������������������������

Appendix A.  MKLAPI Internals  39

MKLAPI

���������������������������������������������������������������������������

APPENDIX B.  TRACE FACILITY

TRACE RECORDS

The trace facility produces the following records (8 bytes each):

long int timestamp;  /* value of mkltime */

int  fl:1,           /* flag: 0=output, 1=input */

nn:7,           /* (in start-of-trace record) trace source */

/* MKLAPI: DOS = 1; OS/2 1.x = 2; OS/2 2.x = 3 */

sw:8;           /* S88 box number on input */

char c1,             /* first character */

c2;             /* second character */

The first record after starting a trace contains 0xFF as first and second

character.

SAMPLE

The output of the trace formatting program MKLTRFMT will look like:

time    I/O  cmd  data  description

------------ ---  ---  ----  --------------------------------------

45.050                 Start of trace (MKLAPI OS/2 1.x)

+0.045  O    1A    13  Loc 19  speed 10  Funktion=ON

+0.350  O    22    3F  switchpoint 63: branch

+0.760  O    20        switchpoint release

+1.850  O    C0        read S88's with reset

+4.850  O    82        read S88 1 t/m 2

+4.881  I        1801  S88  1 ON=4,5,16

+4.881  I        090E  S88  2 ON=,8,13,14,15

+5.000  O    61        STOP

This is not a complete trace, just some parts to show the essentials.

MKLTRFMT.EXE is a 'family mode' application: it will run under OS/2 and

DOS.  No parameters are required, but you may specify the input filename

[including path] if your trace is in another directory or named other than

MKLAPI.TRC, which is the default (output filename of MKLAPI).

MKLTRFMT writes the output to 'stdout', which is normally the computer

display.  But the output may be redirected to your printer or to a file.

So you could use for example the command:

MKLTRFMT >TRACE.FMT

so that you could look afterwards with a text editor or file browse utility

| in TRACE.FMT for the formatted trace.

���������������������������������������������������������������������������

Appendix B.  Trace facility  40

MKLAPI

���������������������������������������������������������������������������

| APPENDIX C.  M�RKLIN-INTERFACE PECULIARITIES

| When writing a model railroad control program (with or without this

| driver), you should be aware of some peculiarities, which are not very well

| (or not at all) documented by M�rklin.  With experiments during development

| of MKLAPI I have found for example:

| �  Sending 2 consecutive change-direction commands to the same loc does not

|    put the loc back in the original direction.  A zero-speed command

|    between them is a good circumvention.

| �  When the M�rklin-Interface is in 'stop'-state:

|    �  Permanent blocking (having to push the 'go'-button on the M�rklin

|       Central Control)  does occur only with commands to train-decoders and

|       switch-point or light decoders (K83 or K84).

|    �  Reading M�rklin S88 encoders (polling) may continue.  But since the

|       changes are not sent from the control-box to the M�rklin-Interface,

|       it is not very useful.

���������������������������������������������������������������������������

Appendix C.  M�rklin-Interface peculiarities  41

MKLAPI

���������������������������������������������������������������������������

APPENDIX D.  SUMMARY OF CHANGES

0.7   First beta release: June 1992

0.8   released: October 1992

�  The time-base value in mkltime has been changed from 55 msec to 1

msec!(updating of mkltime still on timer-tick basis: once per 55

msecs in DOS, once per 31.25 msecs in OS/2).

�  DOS: High precision timer routine added: mklmsecs().  This gives

the period since line to opening of communication with

M�rklin-Interface  with 1 msec accuracy.

�  OS/2: Trace facility added (see also "Appendix B.  Trace facility"

on page 40), start and stop of trace via procedure call

mkltrace().  Output to and input from the M�rklin-Interface is

traced.  A trace-file formatting utility (MKLTRFMT.EXE) is added

to package.  I have tried to add this trace facility also in the

DOS driver, but I did not cucceed so far.  It is rather

complicated to use DOS facilities during interrupt handling

(tracing must be located in the com-port and timer interrupt

routines).

�  A procedure 'mklsleep' added to make your DOS-program 'friendly'

for some popular multitasking environments (such DOS-box of OS/2,

Desqview, Windows).

�  DOS: mklflush times out after 3 seconds when transmit buffer could

not be flushed.

�  Some bugs fixed:

�  mklflush() of OS/2 version watched the INput in stead of OUTput

queue.

�  DOS: DTR and RTS now turned off before finishing program.

0.9   released: March 1993

�  A 32-bits driver for OS/2 2.0+ (MKLAPI3.OBJ) has been added, with

same functionality as MKLAPI2, including the new features below!

�  'MKLAPI' now also useable by programs translated with the

MicroSoft QuickBASIC compiler 4.5.  Since QuickBASIC does not

allow underscores in names, all externally used names have been

changed, for example: mklopen() is now called mklopen().  The

C-language calling convention must be specified in the source (see

MKLAPI.INC), and the file LMKLAPI.OBJ should be used with LINK.

All function specifications have been changed, and additions made

for program development with MicroSoft QuickBASIC compiler 4.5.

�  mkltime() is not anymore an external variable, but a function

returning the value of the time, expressed in milliseconds, but

with the precision of the timer tick.

�  mklmsecs() now also a function returning the time, but in exact

milliseconds (DOS only).

�  With the OS/2 drivers, the transmissions in STOP-state (after

mklstop) may now be buffered in stead of discarded (until buffer

full, then discarded).  This depends on the COM.SYS device

drivers, or replacements thereof.

�  After the first mklstop subsequent calls of mklstop are ignored

and nothing sent to the M�rklin-Interface until mklgo has been

called.

�  mklstop() and mklgo() now give a returncode (0 = OK, otherwise a

transmission problem exists).

���������������������������������������������������������������������������

Appendix D.  Summary of changes  42

MKLAPI

���������������������������������������������������������������������������

�  Simple Switchpoint test program (incl c-source) added to MKLAPI

package.

�  Added counters for and a function mklstats() for statistical

purposes and debugging.

�  Polling optimised: a single 'reset after read S88' (192) is issued

after mklpoll() and mklgo(), subsequently only 'read group of n

S88s' (128+n) commands are sent periodically (previously both

commands were sent after each interval).

1.0   released: May 1993

�  Names and references to 'MARKLIN' renamed to MKLAPI, where the

driver is meant (and not M�rklin-components).  Include-files and

sources that include header files should be changed.

�  DOS-drivers now take RS232 port addresses from the BIOS data-area,

and performs a simple check if the port is not already in use.

�  Sound support added to DOS-drivers, and the function made callable

by mainline.  Warning signals of OS/2- and DOS-drivers

'harmonised'.

�  Polling and storage of S88 bit-settings changed to get better

operation for all types of switches.

�  mklstop() and mklgo() made more reliable.

�  mklsleep() accepts now specification of an interval (pause) time

�  mklmsecs(), mklsound() and mklsleep() functions now present in

both OS/2 and DOS versions for more compatibility between OS/2 and

DOS versions of programs that make use of the MKLAPI-driver.

���������������������������������������������������������������������������

Appendix D.  Summary of changes  43