HOW TO CONFIGURE PTB FOR VARIOUS COM PORT SETUPS

PTB supports DigiBoard and other intelligent serial

communications boards as well as fossil drivers.

DIGIBOARDS

----------

DigiBoards contain 4 or 8 serial ports along with their own on-board

processor to control the ports.  These boards are actually a single-board

computer dedicated to handling serial port I/O.

By moving the serial I/O functions from the Host Computer to the

DigiBoard, the IRQ and Interrupt Latency bottleneck that plagues

Multi-Tasking users is eliminated.

PTB supports the DigiBoard in two ways:

Direct Programming and INT14/EBIOS calls.

Direct Programming is the fastest because it eliminates the need

for costly software interrupt calls to a slower BIOS. In addition

it does not require the presence of a Device Driver which basically

performs the same functions as the Direct Programming method.

INT14/EBIOS requires that a Device Driver be loaded (this driver

is supplied by DigiBoard). Serial Port I/O is performed by issuing

software interrupts (Interrupt 14).

Whether or not to use the Direct Interface or the INT14/EBIOS

Interface depends on the BBS which will be running the PTB Door

program. If the BBS is utilizing the Device Driver, then that is

how the PTB program must be configured. If the BBS is a GAP system

and the Sysop is using the Direct Interface, then the PTB program

must be configred the same.

The two interfaces can not be intermixed since the Direct Interface will

essentially put the Device Driver to sleep.

When using the INT14/EBIOS interface and the DigiBoard supplied Device

Driver, it is not necessary to turn on the "Character Waiting" flags

for each of the ports. PTB does not use these flags as they basically

defeat the purpose of using a multi-port board in a multi-tasking

system. To use these flags an IRQ must be dedicated to the DigiBoard.

If the flags are not used, the IRQ need not be set.

Imagine running 8 ports under a Multi-Tasker where all 8 ports

are in use. As characters are received by each of the 8 ports, an

interrupt is generated. The Device Driver must service the

interrupt request and since the driver runs on the Host Computer,

it will be spending a lot of precious time figuring out which of

the ports generated the interrupt and updating its "Character

Waiting" flag for that port.

By disabling the "Character Waiting" flags and not utilizing the

IRQ, the Host Computer will not know it has 8 serial ports running

at high speed at the same time.

If, however, the Sysop has the IRQ and Character Waiting flags enabled,

no harm is done. PTB simply will not utilize the flags.

The Character Waiting flags are provided because making Interrupt

14 calls takes longer than simply checking a memory address (which,

by the way, is what the Direct Programming interface does).

Here are the various config file setups.

Line 5 - The COM Port Interface to use:

NONE      = Regular Communications Port Interface.  Use this

if you run COM 1 or COM 2 with standard IRQ.

Lines 6 thru 10 will be ignored and need not be

defined.

STANDARD    = Regular Communications Port Interface. This

is what you would use if you run a standard

serial board but with non-standard IRQ or BASE

ADDRESS.

DIGIBOARD   = Direct DigiBoard Programming Interface. Must

be used in conjunction with a program called

RESETDIG.EXE. If you don't have this program

but have a DigiBoard, then use the INT14

option below.

INT14     = Indirect DigiBoard Support via a Device Driver.

All Communications to the DigiBoard (or any

board that supports INT14/EBIOS) are performed

via a Software Interrupt that the Device Driver

handles.

FOSSIL  = Supports most fossil drivers. (X00 is recommended)

NOTE :  Lines 6 and above are not needed for standard and fossil setups.

Line 6 - This is the port IRQ if NOT using the DIGIBOARD or INT14

interface.  This line is for systems that run standard serial

boards but with ports other than 1 or 2 or non-standard IRQ.

If you use DIGIBOARD or INT14 then set this line to 0.

Line 7 - This is the port base address if NOT using the DIGIBOARD or

INT14 interface.  This line is for systems that run standard

serial boards but with ports other than 1 or 2.

If you use DIGIBOARD or INT14 then set this line to 0.

Line 8 - This is the DigiBoard Memory Window, or the address the

card uses as a communications area between the card and

the Host Computer. Only needed if using an Interface set

to DIGIBOARD. If using STANDARD or INT14/EBIOS, this can

be set to 0. This address must be entered in Hexadecimal.

Valid entries are: C000, C800, D000, and D800

Line 9 - This is the DigiBoard I/O Port address that the Board is

attached to. Only needed if using an Interface set to

DIGIBOARD. If using STANDARD or INT14/EBIOS, can be set

to 0. This address must be entered in Hexadecimal.

Valid entries are: 100, 110, 120, 200, 220, 300 and 320

Line 10 - This is the DigiBoard Channel Number for the port this

program will be utilizing. This is REQUIRED for DIGIBOARD

and INT14/EBIOS Interfaces. This is 1 - 8 for the

DIGIBOARD Interface and usually 4 - 11 for the INT14/EBIOS

Interface. This number tells the program which of the

Com Ports on the DigiBoard it should use.

The new configuration file for PTB that will be using a

DigiBoard Serial port would look like the following:

PTB.CFG w/Interrupt 14 interface

-----------------------------------

C:\GAP

The Beckett BBS

GAP

0000

INT14

4

03F8

D000

320

4

Lines 6 and 7 are ignored by the program and can be set to 0.

Because this is an INT14/EBIOS interface, lines 8 and 9 are also

ignored. Line 10 is the Channel Number and in this case, it is the

first serial port on the DigiBoard (Channel Numbers 1 - 4 are

generally reserved for COM 1 - COM 4 on the Host Computer, although

this is up to the Sysop and how he/she configured the Device Driver).

To use the Direct Programming Interface, the configuration file

would look something like:

PTB.CFG w/DigiBoard direct interface

---------------------------------------

C:\GAP

The Beckett BBS

GAP

0000

DIGIBOARD

4

03F8

D000

320

1

Lines 6 and 7 are ignored by the program and can be set to 0.

Line 8 is the Memory Window that the DigiBoard and the program

use to communicate with each other. Line 9 is the I/O port that

the board uses. Line 10 is the Channel Number which, in this case,

is the first serial port on the DigiBoard.

To use the Standard COM Port Interface with ports other than 1 or 2

or with non-standard IRQs, the configuration file would look something

like:

PTB.CFG w/standard interface

-------------------------------

C:\GAP

The Beckett BBS

GAP

0000

STANDARD

4

03F8

0

0

0

Line 6 is the port number and  Line 7 is the IRQ. Line 5 tells the program

to use its own internal interrupt handler. Lines 8-10 are not used and

can be set to 0.  Note that even if the DigiBoard specific options are

not used or needed they must still be present in the Configration File.

To use the Standard Com Port Interface with ports 1 or 2 and standard

IRQ, the configuration file would look something like:

PTB.CFG w/Normal setup

-------------------------

C:\GAP

The Beckett BBS

GAP

0000

NONE

To use a fossil driver with PTB the configuration file

would look like this:

PTB.CFG w/fossil support

---------------------------

C:\GAP

The Beckett BBS

GAP

0000

FOSSIL