G8BPQ IP Router.

The IP router runs with BPQCODE versions 4.06c and above. It is

normally run in a Desqview window (currently needs about 64k).

Once loaded it looks rather like PAC4, and can display traffic passing

through. The system is initially configured by commands in a file

IPGATE.CFG. See below for format. The system will learn the IP addresses

of stations which attempt to use the router, and will pass both

preconfigured and learned addresses to neighbouring BPQ routers at regular

intervals. The Protocol used is derived from RIP, but is not compatible

with other RIP implementations.

There is a very primitive SYSOP interface with the following commands:

A	Display ARP Table

D	Diagnositic dump

N	Display RIP Destinations (Neighbours)

R	Display Route Table

S	Display Stats

T	Force a RIP Transmission

ROUTE .....		; Equivalent to the commands in IPGATE.CFG to add

ARP .....		; Route ARP or RIP  table entries. See below for

RIP ....		; details.

CTRL/Y shuts down IPGATE

The Stats display is pretty obvious. The ROUTES display is as follows:

Target/Subnet	Router         Port Type Flags Metric Time  Count

44.131.4.30/32  44.131.4.18      1    V    2     02     00    0

The first field is the target address, followed by the number of bits

to be used in comparing addresses. In the current version the number is

either 32, indicating a full address, or 24, indicating a route to a

subnet zone. The next is the router to use, or 0.0.0.0 for a direct path.

Port is the Radio port to use, and Type the session type (V=Virtual Circuit,

N=NETROM, D=Datagram). Flags indicated whether the route is learned (1),

or entered by SYSOP (2). Metric is the number of hops to the station. Time

is the number of hours left before a learned route is deleted, and Count is

the number of frames sent over this route.

The ARP display is as follows:

Callsign    IP Address		Timeout  Port  Type

G8BPQ-5     44.131.4.18          65535    00    N

The timeout is in seconds, and 65535 indicates a locked entry.

Please note that although the router supports both Virtual Circuit and

Datagram mode, I strongly recommend that you only use Virtual Circuit

mode on radio links.

The following is a commented extract from my IPROUTE.CFG

#

#	Any lines starting with a # are ignored.

#

#	You must tell the system its IP Address. Note that if you are

#	running a NOS (or similar) system as well, it must have a

#	different address

#

IPADDRESS=44.131.4.20

#

#	You can put in ROUTE commands to tell the system where to send

#	frames for each station. You specify the IP address, the next

#	Router in the path (if you cant reach the station directly), the

#	Radio Port to use (as defined in BPQCFG), and the frame type.

#

#	ROUTE 44.131.4.18 * 1 V

#

#	This means station 44.131.4.18 can be reached on port 1, using

#	Virtual Circuit mode.

#

#	ROUTE 44.141.5.1 44.131.27.1 0 N

#

#	This means station 44.131.5.1 is reachable via router 44.131.27.1,

#	which is via a NETROM link. Port is 0, as the NETROM system will

#	choose the best port.

#

#	If all the traffic for a zone is sent the same way you can define

#	a 'subnet route' by setting the last byte of the address to zero.

#	The following will route all traffic for zone 18 via 44.131.18.24

#

#	ROUTE 44.131.18.0 44.131.18.24 0 N

#

# 	You can define a 'Default Route' - somewhere for the Router to

#	send frames if it doesnt know where the station is.

#

ROUTE DEFAULT * 1 V			# Try port 1 for all unknown dests

#

ROUTE 44.131.4.15 * 1 V			# Directly reachable stations. You

ROUTE 44.131.4.32 * 1 V			# may wish to define all your local

ROUTE 44.131.4.33 * 1 V			# users.

ROUTE 44.131.4.18 * 2 V			# My WNOS system on a 'LOOPBACK' port

ROUTE 44.131.4.23 * 0 N 		# G0INA ROUTER. Must be defined, as

# system cant learn the address of

# NETROM stations.

#

ROUTE 44.131.4.92 44.131.4.57 1 V	# g1xow - need this one, as he is not

# running a version which uses RIP

#

ROUTE 44.131.18.0 44.131.18.24 0 N    	# G4MCF and zone 18 - running NOS, so

# cant be learned

#

#

#	The system will discover the callsign to be used to talk to

#	directly reachable stations using the ARP protocol. However it

#	cant discover the callsign to be used for NETROM links, so you

#	must include ARP entries for all your NETROM neighbours. You can

#	put in ARP commands for other stations, but that shouldn't be

#	necessary.

#

ARP 44.131.4.23 G0INA-1 0 N

ARP 44.131.18.24 G4MCF-5 0 N

#

#	The Router will exchange routing information with other Routers,

#	if they are defined using a RIP command:

#

#

RIP 44.131.4.23				# G0INA

RIP 44.131.4.91				# G4IRX

#

#	Note that G4MCF is running NOS, not my Router, so I cant send

#	him RIP info.

John Wiseman G8BPQ

31/12/92

#################